IEEE P802.11
Wireless LANs

| LB123: Normative Text Changes for Event Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-MAY-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document contains normative text for the comment resolutions in 08/0610.

- Event reporting

Event requests and reports allow a STA to request a non-AP STA to send particular real-time event messages when they occur. The types of events include: Transition, RSNA, Syslog, and Peer to Peer link events. A transition event is transmitted after a non-AP STA successfully completes a BSS Transition. Transition events are useful to diagnose transition performance problems. An RSNA event report gives authentication information for a non-AP STA. RSNA events are useful to diagnose security and authentication performance problems. A syslog event allows a non-AP STA to transmit a set of Syslog events to the requesting STA. Syslog events are used to access the contents of a STA's syslog when a STA is unable to establish Layer 3 connection or before a Layer 3 connection is established. A peer-to-peer link event allows a non-AP STA to inform the requesting STA that a peer-to-peer link has been established. Peer-to-peer link events are useful to monitor the use of peer-to-peer links in the network.

- Information Elements

Insert Element IDs <ANA> into Table 7-26 and change the Reserved row accordingly in Table 7-26:

| - Element IDs |  |  |
| --- | --- | --- |
| Information Element | Element ID | Length (in octets) |
| Event Request (see Error: Reference source not found) | <ANA> | 5 to 256 |
| Event Report (see Error: Reference source not found) | <ANA> | 5 to 256 |
| Diagnostic Request (see Error: Reference source not found) | <ANA> | 4 to 256 |
| Diagnostic Report (see Error: Reference source not found) | <ANA> | 5 to 256 |
| Location Parameters (see Error: Reference source not found) | <ANA> | 2 to 256 |
| Multiple BSSID (see ) | <ANA> | 3 to 256 |

- 802.11-2007 ends with 7.3.2.35. 11k adds 36-44. 11r adds 45-48. 11y adds 49-51. 11n adds 49-52 (52-55). 11w adds 55 (56). 11s adds 64-68 (57-61).

- Event Request element

- Event Request definition

The Event Request element contains a request to the receiving STA to perform the specified event action. The format of the Event Request element is shown in Event Request element format.

|  | Element ID | Length | Event Token | Event Type | Event Response Limit | Event Request |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |
| - Event Request element format |  |  |  |  |  |  |

The Element ID field is equal to the Event Request value in Table 7-26.

The value of the Length field is variable and depends on the length of the Event Request field.

The Event Token field is set to a nonzero number that is unique among the Event Request elements sent to each destination MAC address for which a corresponding Event Report element has not been received.

The Event Type field is set to a number that identifies the type of event request. The Event Types are shown in Event Type definitions for event requests and reports.

| - Event Type definitions for event requests and reports |  |
| --- | --- |
| Name | Event Type |
| Transition | 0 |
| RSNA | 1 |
| Peer-to-Peer Link | 2 |
| System Log (Syslog) | 3 |
| Reserved | 4 – 220 |
| Vendor Specific | 221 |
| Reserved | 222 – 255 |

The Event Response Limit field contains the maximum number of requested Event Reports to be included in the Event Report Element.

The Event Request field contains the event request corresponding to the Event Type as described in Transi through Peer-t. The Event Request field is not present when requesting a Syslog report.

The Event Request element is included in an Event Request frame as described in Error: Reference source not found. The use of the Event Request element and Event Request frame is described in 11.20.3.

- Transition event request

The Event Request field corresponding to the Transition event request contains zero or more Transition Event Request sub-elements. A transition event is a STA movement or attempted movement from one BSS (the source BSS) in one ESS to another BSS (the target BSS) within the same ESS.

The Transition Event Request sub-elements are defined to have a common format consisting of a 1 octet Sub-element ID field, a 1 octet length field, and a variable length sub-element specific information field. The set of valid Transition Event Request sub-elements is defined in Transition Event Request Sub-element.

| - Transition Event Request Sub-element |  |
| --- | --- |
| Transition Event Request Sub-element | Sub-element ID |
| Target BSSID Transition | 0 |
| Source BSSID Transition | 1 |
| Transition Time | 2 |
| Transition Result | 3 |
| Frequent Transition | 4 |
| Reserved | 5-255 |

The Transition Event sub-elements specify the conditions in which a Transition Event Report is sent by a STA.

The Target BSSID Transition sub-element is used to request that a Transition Event Report includes the transition event entry when the target BSSID is equal to the specific BSSID in the Target BSSID field. Excluding this sub-element from the event request element indicates a request for transition events for all target BSSIDs.. The format of the Target BSSID Transition sub-element is shown in Target BSSID Transition sub-element format.

|  | Sub-element ID | Length | Target BSSID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |
| - Target BSSID Transition sub-element format |  |  |  |

The Sub-element ID field is equal to the Target BSSID Transition value in Transition Event Request Sub-element.

The value of the Length field is set to 6.

The Target BSSID field contains a 6-octet BSSID.

The Source BSSID Transition sub-element is used to request that a Transition Event Report includes the transition event entry when the source BSSID is equal to the specific BSSID in the Source BSSID field. Excluding this sub-element from the event request element indicates a request for transition events for all source BSSIDs.. The format of the Source BSSID Transition sub-element is shown in Source BSSID Transition sub-element format

|  | Sub-element ID | Length | Source BSSID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |
| - Source BSSID Transition sub-element format |  |  |  |

.

The Sub-element ID field is equal to the Source BSSID Transition value in Transition Event Request Sub-element.

The value of the Length field is set to 6.

The Source BSSID field contains a 6-octet BSSID.

The Transition Time sub-element is used to request that a Transition Event Report includes the transition event entry when the Transition Time is greater than or equal to the Transition Time Threshold. The format of the Transition Time sub-element is shown in Transition Time sub-element format.

|  | Sub-element ID | Length | Transition Time Threshold |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |
| - Transition Time sub-element format |  |  |  |

The Sub-element ID field is equal to the Transition Time value in Transition Event Request Sub-element.

The value of the Length field is set to 2.

The Transition Time Threshold field contains a value representing the transition time to be used as the threshold value for the Transition Time condition in TUs. The Transition Time is defined in 11.20.3.2.

The Transition Result sub-element is used to request that a Transition Event Report includes the transition event entry that matches the transition result defined by this sub-element. The format of Transition Result sub-element is shown in Transition Result sub-element format.

|  | Sub-element ID | Length | Match Value |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| - Transition Result sub-element format |  |  |  |

The Sub-element ID field is equal to the Transition Result value in Transition Event Request Sub-element.

The value of the Length field is set to 1.

The Match Value field is set with each bit as defined in Match Value field definitions to request that the specified transition results that match the bit descriptions are included in the Transition Event Report.

|  | B0 | B1 | B2-B7 |
| --- | --- | --- | --- |
|  | Include Successful Transitions | Include Failed Transitions | Reserved |
| Bits | 1 | 1 | 6 |
| - Match Value field definitions |  |  |  |

The Frequent Transition sub-element is used to request that an alerting Transition Event report be generated when the total transition count during the specified time period is equal to the value given in Frequent Transition Count Threshold field. The format of the Frequent Transition sub-element is shown in Frequent Transition sub-element format

|  | Sub-element ID | Length | Frequent Transition Count Threshold | Time Interval |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 |
| - Frequent Transition sub-element format |  |  |  |  |

.

The Sub-element ID field is equal to the Frequent Transition value in Transition Event Request Sub-element.

The value of the Length field is set to 3.

The Frequent Transition Count Threshold field is a one octet field containing the number of transitions in the measurement duration after which a Transition Event Report is generated.

The Time Interval field is set to the time interval in TUs during which the STA determines if the Frequent Transition Count Threshold is exceeded.

- RSNA event request

The Event Request field corresponding to an RSNA event request contains zero or more RSNA Event Request sub-elements.

The RSNA Event Request sub-elements are defined to have a common format consisting of a 1 octet Sub-element ID field, a 1 octet length field, and a variable length sub-element specific information field. The set of valid RSNA Event Request sub-elements is defined in RSNA Event Request Sub-element.

| - RSNA Event Request Sub-element |  |
| --- | --- |
| RSNA Event Request Sub-element | Sub-element ID |
| Target BSSID RSNA | 0 |
| Authentication Type | 1 |
| EAP Method | 2 |
| RSNA Result | 3 |
| Reserved | 4 – 255 |

The RSNA sub-elements specify reporting conditions for RSNA Event Reports.

The Target BSSID RSNA sub-element is used to request that an RSNA Event Report includes the RSNA event entry when the target BSSID is equal to the BSSID specified in the Target BSSID field. The format of the Target BSSID RSNA sub-element is shown in Target BSSID RSNA sub-element format.

|  | Sub-element ID | Length | Target BSSID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |
| - Target BSSID RSNA sub-element format |  |  |  |

The Sub-element ID field is equal to the Target BSSID RSNA value in RSNA Event Request Sub-element.

The value of the Length field is set to 6.

The Target BSSID field contains a 6-octet BSSID.

The Authentication Type sub-element is used to request that an RSNA Event Report includes the RSNA event entry when the Authentication Type is equal to the authentication type specified in the Authentication Type field. The format of the Authentication Type sub-element is shown in Aut.

|  | Sub-element ID | Length | Authentication Type |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 4 |
| - Authentication Type sub-element format |  |  |  |

The Sub-element ID field is equal to the Authentication Type value in RSNA Event Request Sub-element.

The value of the Length field is set to 4.

The Authentication Type field contains one of the AKM suite selectors defined in Table 34 in 7.3.2.25.2.

The EAP Method sub-element is used to request that an RSNA Event Report includes the RSNA event entry when the EAP Method is equal to the EAP method specified in the EAP Method field. The format of the EAP Method sub-element is shown in EAP Method sub-element format.

|  | Sub-element ID | Length | EAP Type | EAP Vendor ID (optional) | EAP Vendor Type (optional) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 3 | 4 |  |  |  |  |  |
| - EAP Method sub-element format |  |  |  |  |  |  |  |  |  |  |

The Sub-element ID field is equal to the EAP Method value in RSNA Event Request Sub-element.

The value of the Length field is set to 1 or 8.

The EAP Type field contains a value that identifies a single EAP method and is set to any valid IANA assigned EAP type [B34].

The EAP Vendor ID field contains a value that identifies the EAP Vendor. The EAP Vendor ID field is included when EAP Type field is set to 254, and is excluded otherwise.

The EAP Vendor Type field contains a value that identifies the EAP Type as defined by the vendor. The EAP Vendor Type field is included when EAP Type field is set to 254, and is excluded otherwise.

The RSNA Result sub-element is used to request that an RSNA Event Report includes the RSNA event entry that matches the transition result defined by this sub-element. The format of RSNA Result sub-element is shown in RSNA Result sub-element format.

|  | Sub-element ID | Length | Match Value |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| - RSNA Result sub-element format |  |  |  |

The Sub-element ID field is equal to the RSNA Result value in RSNA Event Request Sub-element.

The value of the Length field is set to 1.

The Match Value field bits are set as defined in Match Value Field Definitions to request that the specified RSNA results that match that bit descriptions are included in the RSNA Event Report.

|  | B0 | B1 | B2-B7 |
| --- | --- | --- | --- |
|  | Include Successful RSNA | Include Failed RSNA | Reserved |
| Bits | 1 | 1 | 6 |
| - Match Value Field Definitions |  |  |  |

- Peer-to-Peer Link event request

The Event Request field corresponding to Peer-to-Peer Link event request contains zero or more Peer-to-Peer Link Event Request sub-elements.

The Peer-to-Peer Link Event Request sub-elements are defined to have a common format consisting of a 1 octet Sub-element ID field, a 1 octet length field, and a variable length sub-element specific information field. The set of valid Peer-to-Peer Link Event Request sub-elements is defined in Peer-to-Peer Link Event Request Sub-element.

| - Peer-to-Peer Link Event Request Sub-element |  |
| --- | --- |
| Peer-to-Peer Link Event Request Sub-element | Sub-element ID |
| Peer STA Address | 0 |
| Channel Number | 1 |
| Reserved | 2 – 255 |

The Peer-to-Peer Link sub-elements specify reporting conditions for Peer-to-Peer Link Event Reporting.

The Peer STA Address sub-element is used to request that a Peer-to-Peer Link Event Report includes the Peer-to-Peer Link event entry whose peer STA address is equal to the specified MAC address. The format of the Peer STA Address sub-element is shown in Peer STA Address sub-element format.

|  | Sub-element ID | Length | Peer STA Address |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |
| - Peer STA Address sub-element format |  |  |  |

The Sub-element ID field is equal to the Peer STA Address value in Peer-to-Peer Link Event Request Sub-element.

The value of the Length field is set to 6.

The Peer STA Address field contains a 6-octet MAC address.

The Channel Number sub-element is present to request that a Peer-to-Peer Link Event Report includes the Peer-to-Peer Link event entry when the operating channel number is equal to the specified Channel Number. The format of the Channel Number sub-element is shown in Cha.

|  | Sub-element ID | Length | Regulatory Class | Channel Number |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |
| - Channel Number sub-element format |  |  |  |  |

The Sub-element ID field is equal to the Channel Number value in Peer-to-Peer Link Event Request Sub-element.

The value of the Length field is set to 2.

The Regulatory Class field indicates the channel set of the Peer-to-Peer link to be used for the Peer-to-Peer Link Event Report. The Regulatory Class is defined in Annex J.

The Channel Number field indicates the channel number of the Peer-to-Peer link to be used as the value for the Peer-to-Peer Link Event Report. A Channel Number of 0 indicates a request to report a Peer-to-Peer link event for each supported channel in the specified filtering Regulatory Class.

- Vendor Specific event request

The Event Request field corresponding to Vendor Specific event request contains zero or more Vendor Specific sub-elements as defined in 7.3.2.26.

- Event Report element

- Event Report Definition

The Event Report element is used by a STA to report an event. The format of the Event Report element is shown in Event Report element format.

|  | Element ID | Length | Event Token | Event Type | Event Report Status | Event Timestamp (optional) | Event Report (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 6 | variable |
| - Event Report element format |  |  |  |  |  |  |  |

The Element ID field is equal to the Event Report value in Table 7-26.

The value of the Length field is variable and depends on the length of the Event Report field. The minimum value of the Length field is 3.

The Event Token field is set to the Event Token in the corresponding Event Request element. If the Event Report element is being sent autonomously then the Event Token is set to 0.

The Event Type field is set to a number that identifies the type of event report. The Event Types are shown in Event Type definitions for event requests and reports.

The Event Report Status field is set to a value in Event Status, indicating the STA’s response to the Event Request.

| - Event Report Status |  |
| --- | --- |
| Event Report Status | Description |
| 0 | Successful |
| 1 | Fail |
| 2 | Refused |
| 3 | Incapable |
| 4 | Cancelled |
| 5–255 | Reserved |

The Event Timestamp and Event Report fields are only present when the Status field is set to 0.

The Event Timestamp field is set to the time that the Event occurred, in UTC, including hours, minutes, seconds and milliseconds, as shown in Error: Reference source not found.

The Event Report field contains the specification of the event report, as described in Transi through Error: Reference source not found.

The Event Report element is included in an Event Report frame as described in Error: Reference source not found. The use of the Event Report element and frame is described in 11.20.3.

- Transition event report

The format of the Event Report field corresponding to a Transition event report is shown in Eve.

|  | Source BSSID | Target BSSID | Transition Time | Transition Reason | Transition Result |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 2 | 1 | 2 |
|  | Source RCPI | Source RSNI | Target RCPI | Target RSNI |  |
| Octets: | 1 | 1 | 1 | 1 |  |
| - Event Report format for Transition event |  |  |  |  |  |

The Source BSSID field contains the 6-octet BSSID address of the associated AP prior to the attempted transition.

The Target BSSID field contains the 6-octet BSSID address of the AP that is the target of the attempted Transition.

The Transition Time field contains the transition time in TUs. The transition time is defined in 11.20.3.2.

The Transition Reason field indicates the reason why a transition attempt occurred and contains one of the values in Transition Reasons .

| - Transition Reasons |  |
| --- | --- |
| Transition Reason Value | Description |
| 0 | Unspecified |
| 1 | Excessive frame loss rates and/or poor conditions |
| 2 | Excessive delay for current traffic streams |
| 3 | Insufficient QoS capacity for current traffic streams (TSPEC rejected) |
| 4 | First association to ESS (the association initiated by an Association Request message instead of a Reassociation Request message) |
| 5 | Load balancing |
| 6 | Better AP found |
| 7 | Deauthenticated or Disassociated from the previous AP |
| 8 | AP failed IEEE 802.1X EAP Authentication |
| 9 | AP failed 4-way handshake |
| 10 | Received too many replay counter failures |
| 11 | Received too many data MIC failures |
| 12 | Exceeded maximum number of retransmissions |
| 13 | Excessive interference or noise |
| 14 | Received too many broadcast disassociations |
| 15 | Received too many broadcast deauthentications |
| 16 | Previous transition failed |
| 17-255 | Reserved |

The Transition Result field contains the result of the attempted transition and is set to one of the Status Codes specified in Table7-23 in 7.3.1.9.

The Source RCPI field indicates the received channel power of the most recently measured frame from the Source BSSID before the STA reassociates to the Target BSSID. The Source RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

The Source RSNI field indicates the received signal to noise indication of the most recently measured frame from the Source BSSID before the STA reassociates to the Target BSSID. The Source RSNI is reported in dB , as defined in 7.3.2.41.

The Source BSSID, Source RCPI, and Source RSNI fields are set to 0 if the transition is initiated by an Association Request message.

The Target RCPI field indicates the received channel power of the first measured frame just after STA reassociates to the Target BSSID. If association with target BSSID failed, the Target RCPI field indicates the received channel power of the most recently measured frame from the Target BSSID. The Target RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

The Target RSNI field indicates the received signal to noise indication of the first measured frame just after STA reassociates to the Target BSSID. If association with target BSSID failed, the Target RSNI field indicates the received signal to noise indication of the most recently measured frame from the Target BSSID. The Target RSNI is reported in dB , as defined in 7.3.2.41.

- RSNA event report

The format of the Event Report field corresponding to an RSNA event report is shown in Eve.

|  | Target BSSID | Authentication Type | EAP Method | RSNA Result | RSN Element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 4 | 1 or 8 | 1 | variable |
| - Event Report format for RSNA event |  |  |  |  |  |

The Target BSSID field contains the 6-octet BSSID address of the AP accepting the authentication attempt.

The Authentication Type field contains the Authentication type in use at the time of the authentication attempt and is set to one of the AKM suite selectors defined in Table 34 in 7.3.2.25.2.

When the Authentication Type field is set to the value of either 00-0F-AC:1 (Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in 8.4.6.2) or 00-0F-AC:3 (AKM suite selector for Fast BSS Transition as defined in 8.4.3), the EAP Method field contains the IANA assigned EAP type defined at http://www.iana.org/assignments/eap-numbers. The EAP type contains either the legacy type (1 octet) or the expanded type (1 octet type = 254, 3-octet Vendor ID, 4-octet Vendor-Type). The EAP Method field is set to 0 otherwise.

The RSNA Result field contains the result of the RSNA establishment attempt and is set to one of the Status Codes specified in Table 7-23 in 7.3.1.9.

The RSN Element field contains the entire contents of the negotiated RSN information element at the time of the authentication attempt.

- Peer-to-Peer Link event report

The format of the Event Report field corresponding to a Peer-to-Peer Link Event Report is shown in Event Report format for Peer-to-Peer Link event .

|  | Peer STA Address | Regulatory Class | Channel Number | STA Tx Power | Connection Time | Peer Status |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 1 | 1 | 1 | 3 | 1 |
| - Event Report format for Peer-to-Peer Link event |  |  |  |  |  |  |

The Peer STA Address field contains the 6-octet MAC address of the peer STA of a Peer-to-Peer Link.

The Regulatory Class field indicates the channel set of the Peer-to-Peer link. Valid values of the Regulatory Class are shown in Annex J.

The Channel Number field indicates the operating channel number of the Peer-to-Peer link. The Channel Number is defined within a Regulatory Class as shown in Annex J.

The STA Tx Power field indicates the target transmit power at the antenna in dBm with a tolerance of +/-5dB of the lowest basic rate of the reporting STA.

The Connection Time field contains the connection time in seconds. If the Peer-to-Peer link is still active (the STA has established an IBSS with the peer STA, or a Direct link is active (see 11.7)), the connection time is defined as the difference from the first time a frame could have been transmitted between the reporting STA and Peer STA and the current time. Otherwise, the connection time is defined as the difference from the first time a frame could have been transmitted between the reporting STA and Peer STA to the time of the Peer-to-Peer Link teardown.

The Peer Status field indicates the Peer link connection status as indicated in Peer Status definitions.

| - Peer Status definitions |  |
| --- | --- |
| Peer Status | Description, |
| 0 | Inactive |
| 1 | Active |
| 2-255 | Reserved |

- Syslog event report

The format of the Event Report field corresponding to a Syslog event report is shown in Even.

|  | Syslog Msg |
| --- | --- |
| Octets: | variable |
| - Event Report format for Syslog event |  |

The Syslog Msg field contains the entire syslog message, consisting of the PRI, HEADER, and MSG portion of a Syslog message as described in IETF RFC 3164 [RFC3164]. The MSG portion of the message begins with the ASCII representation of the STA MAC address.

- Vendor Specific event report

The Event Report field corresponding to Vendor Specific event report contains zero or more Vendor Specific sub-elements as defined in 7.3.2.26.

- Event Request and Report Procedures

- Event Request and Event Report

The Event Request and Event Report frames enable network real-time diagnostics. A STA that has a value of true for the MIB attribute dot11MgmtOptionEventsEnabled is defined as a STA that supports Event Reporting and indicates this support by setting the dot11MgmtOptionEventsEnabled bit of the Extended Capabilities element to 1 in transmitted Beacon, Association Request, Association Response, Reassociation Request, Reassociation Response, and Probe Response frames. While dot11MgmtOptionEventsEnabled is true, a STA shall continuously detect and log all transition, RSNA, Peer-to-peer, and Syslog events.

A STA that supports Event Reporting shall only send an Event Request or Event Report frame to a STA within the same infrastructure BSS or the same IBSS whose last received Extended Capabilities element contained a value of 1 for the Event bit in the Capabilities field. If the STA receives an Event Request frame without error and it supports the Event service, it shall respond with an Event Report frame that includes the Dialog Token that matches the one in the Event Request frame.

Within each Event Request frame there may be zero or more Event Request elements. Each Event Request element contains a unique Event Token that identifies the element within the Event Request Frame. Each Event Report element shall contain the same Event Token that was included in the original request.

Only a single Event Request frame from a STA is outstanding at a given STA at any time. If a STA receives a subsequent Event Request frame with a different Dialog Token before the completing Event Report for the previous request from the requesting STA, the STA shall only respond to the most recent Request frame.

Upon a BSS transition , any Event Requests in the latest Event Request frame shall be cancelled at the STA. Note that the STA log of events shall not be cleared as a result of BSS transitions .

All Event Report elements shall include a Status field that indicates the overall result of the event transaction. If the STA is able to return zero or more Event Report elements, then a value of Successful shall be returned. If the STA is unable to process the request at that time, a value of Fail shall be returned. If the request is unacceptable to the STA, a value of Refused shall be returned. If a STA has no logged events of the requested type, it shall return an event report with no events included in the successful Event Report element. If the STA is incapable of generating an Event Report of the type specified in the Event Request frame, the STA shall return a value of Incapable indicating that the requester should not request again.

The Event Report frame shall only be sent with an individually addressed destination address. If a non-AP STA receives an Event Request frame with a multicast destination address, it shall discard the frame. An Event Request shall only be sent from an AP to a non-AP STA. Event Request frames sent to an AP shall be discarded by the AP.

When a STA sends an Event Request frame to another STA it shall indicate the types of events requested by setting the Event Type field and shall indicate the maximum number of logged events to report by using the Event Response Limit field in each included Event Request element. If the number of available logged events of the requested type exceeds the Event Response Limit, the STA shall only report an Event Response Limit number of the most recent events. If there are no available logged events of the type specified in the Event Request frame, the STA shall send Event Report frame without any Event Report Element. The reporting STA shall send all available Event Report elements for the requested Event Type when Event Request field is not present in the Event Request element.

If the Event Report elements do not fit into a single MMPDU, the reporting STA shall send the remaining elements in additional Event Report frames until all Event Report elements have been returned to the requesting STA. Each subsequent Event Report frame and Event Report elements shall include the same Dialog Token and Event Token, respectively, that was sent in the corresponding Event Request frame. When multiple MMPDUs are required, the non-AP STA shall include complete Event Report elements and not fragment an element across multiple MMPDUs.

The Event Request and Report elements may contain conditions that specify events to be reported and conditions that establish event reporting when a STA experiences problems or failures. A STA sends an Event Request frame containing zero or more Event Request elements including one or more sub-elements. Sub-elements are defined for each event type. The corresponding Event Report element shall include the events that meet the specified event conditions within the current ESS or IBSS.

A STA shall not address an Event Request frame to a STA that is not a member of the same BSS or IBSS. The permitted source and destination STAs for an Event Request frame are shown in Allowed and Disallowed Event Request.

| - Allowed and Disallowed Event Request |  |  |  |
| --- | --- | --- | --- |
| Service Set | Source of Request | Destination of Request | Allowed |
| Infrastructure BSS | AP | Non-AP STA | Yes |
| Infrastructure BSS | Non-AP STA | Non-AP STA | No |
| Infrastructure BSS | Non-AP STA | AP | No |
| IBSS | Non-AP STA | Non-AP STA | Yes |
| ESS | AP | AP | No |

- 802.11-2007 ends with Table 79. 11k adds a. 11r adds none. 11y adds none. 11n adds n53 (b). 11w adds none. 11p adds none. 11s adds none.

- Transition Event Request and Report

The Transition Event report provides information on the previous transition events for a given non-AP STA. The Transition Event request and report are only permitted in the Infrastructure BSS.

Upon receipt of an Event Request frame containing an Event Request element including a Transition Event request, the non-AP STA shall respond with an Event Report frame that includes available Event Report elements within the current ESS for the Transition event type. Each STA supporting the Transition Event shall record up to and including the last 5 events occurring since the STA associated to the ESS.

If a Transition event sub-element is present in the Event Request field, the reporting non-AP STA shall include available Event Report elements that meet the specified condition for the transition event type. If no transition event sub-elements are present in the Event Request field, the reporting STA shall include all available Transition Event Report elements. A STA that encounters an unknown Sub-element ID value in a Transition Event Request frame received without error ignores that sub-element and parses remaining Event Request fields for additional information sub-elements with recognizable sub-element ID values.

A Transition Event Report may also be sent as a report when frequent transition is detected. The Frequent transition occurs when the number of BSS transitions exceeds the indicated Frequent Transition Count Threshold within a time interval indicated by the Time Interval defined in 7.3.2.62.2.

The Transition time is defined as the time difference between the starting time and the ending time of a transition between APs, even if the transition results in remaining on the same AP.

The starting time is one of the following items:

- The start of a search for an AP, when the transition reason is 4 (first association to WLAN).

- The latest time that a frame could have been transmitted on the source BSS.

- The start of a search for an AP, after determination that a transition has failed.

The ending time is one of the following items:

- The earliest time that a data frame can be transmitted on the target BSS, after completion of RSN, 802.1X, or other authentication and key management transmissions, when such are required by the target BSS.

- The time that a determination is made that the transition has failed.

- RSNA Event Request and Report

The RSNA Event Report provides authentication events for a given non-AP STA. The RSNA Event Request and Report are only permitted in an Infrastructure BSS.

Upon receipt of an Event Request frame containing an Event Request element including an RSNA Event request, the non-AP STA shall respond with an Event Report frame that includes available Event Report elements within the current ESS for the RSNA event type. Each STA supporting the RSNA Event shall record up to and including the last 5 events occurring since the STA associated to the ESS.

If an RSNA event sub-element is present in the Event Request field, the reporting non-AP STA shall include available Event Report elements that meet the specified condition for the RSNA event type. If no RSNA event sub-element is present in the Event Request field, the reporting STA shall include all available RSNA Event Report elements. A STA that encounters an unknown Sub-element ID value in a RSNA Event Request frame received without error ignores that sub-element and parses remaining Event Request fields for additional information sub-elements with recognizable sub-element ID values.

- Peer-to-Peer Link Event Request and Report

The Peer-to-Peer Event Report provides peer to peer connectivity events for a given non-AP STA. Peer-to-Peer link may be the Direct Link within a QBSS or the STA to STA communication in an IBSS.

Upon receipt of an Event Request frame containing an Event Request element including a Peer-to-Peer Link Event request, the non-AP STA shall respond with an Event Report frame that includes available Event Report elements within the current ESS or IBSS for the Peer-to-Peer event type. Each STA supporting the Peer-to-Peer Event shall record up to and including the last 5 events occurring since the STA associated to the ESS or IBSS.

If a Peer-to-Peer link event sub-element is present in the Event Request field, the reporting non-AP STA shall include available Event Report elements that meet the specified condition for the Peer-to-Peer event type. If no Peer-to-Peer link event sub-elements are present in the Event Request field, the reporting STA shall include all available Peer-to-Peer Event Report elements. A STA that encounters an unknown Sub-element ID value in a Peer-to-Peer Event Request frame received without error ignores that sub-element and parses remaining Event Request fields for additional information sub-elements with recognizable sub-element ID values.

- Syslog Event Request and Report

The purpose of the Syslog Event report is to provide the AP with system event information from a STA described in vendor specific, human readable (ASCII text) form.

A non-AP STA may be queried at any time for its current set of syslog messages, if any are available. The syslog messages returned by the non-AP STA may provide insight into the trouble being experienced by non-AP STA.

Upon receipt of an Event Request frame containing an Event Request element including a Syslog Event request, the non-AP STA shall respond with an Event Report frame that includes Syslog Event Report elements.

11.20.3.6 Vendor Specific Request and Report

The procedures for use of the Vendor Specific Event Request and Report are vendor specific and are not part of this standard.

Insert this new clause after A.4.17:

- last clause in 802.11k-D.70 is A.4.17.

- Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM1 | Extended Capabilities element | 7.3.2.62 | CFv:M | Yes, No, N/A |
| WNM2 | Management Diagnostic Alerts | 11.20.2 | CFv:M | Yes, No, N/A |
| WNM2.1 | Measurement Request frame | 7.4.5.1 | CFv:M | Yes, No, N/A |
| WNM2.2 | Measurement Report frame | 7.4.5.2 | CFv:M | Yes, No, N/A |
| WNM2.3 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.4 | Triggered STA Statistics Reporting | 7.3.2.21.8, 7.3.2.22.8, 11.10.8.5 | CFv:M | Yes, No, N/A |
| WNM2.5 | Triggered QoS Metrics Reporting | 7.3.2.21.10, 7.3.2.22.10, 11.11.9.8 | (CFv AND CF12):O | Yes, No, N/A |
| WNM2.6 | Multicast Diagnostics Reporting | 7.3.2.21.10a, 7.3.2.22.10a, 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.20.3 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request frame | 7.3.2.62, 7.4.11.1 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report frame | 7.3.2.63, 7.4.11.2 | (CFv AND CF12):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.20.4 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request | 7.3.2.64, 7.4.11.3 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report | 7.3.2.65, 7.4.11.4 | (CFv AND CF12):M | Yes, No, N/A |
| WNM4.3 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.65.2, 11.20.4.3 | CFv:M | Yes, No, N/A |
| WNM4.4 | Configuration Profile Diagnostic Type | 7.3.2.65.3, 11.20.4.2 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.64.2, 7.3.2.65.4, 11.20.4.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.64.3, 7.3.2.65.4, 11.20.4.4 | (CFv AND PC34):O | Yes, No, N/A |
| WNM5 | Location | 11.20.5, 7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Request | 7.4.11.5 | CFv:M | Yes, No, N/A |
| WNM5.2 | Location Response | 7.4.11.6 | CFv:M | Yes, No, N/A |
| WNM5.3 | Location Configuration Request | 7.4.11.7 | CFv:M | Yes, No, N/A |
| WNM5.4 | Location Configuration Response | 7.4.11.8 | CFv:M | Yes, No, N/A |
| WNM6 | Multiple BSSID Support | 11.20.6, 11.20.7 | CFv:O | Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.20.8 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report Element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query | 7.4.11.9 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request | 7.4.11.10 | (CFv AND CF12):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response | 7.4.11.11 | (CFv AND CF12):M | Yes, No, N/A |
| *WNM8 | FBMS | 9.2.7;11.20.9 | CFv:O | Yes, No, N/A |
| WNM8.1 | FBMS Request | 7.4.11.12 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FBMS Response | 7.4.11.13 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.2.1.4b | CFv:O | Yes, No, N/A |
| WNM10 | Co-located Interference Reporting | 7.4.11.14, 7.4.11.15, 11.20.10 | CFv:O | Yes, No, N/A |
| WNM11 | BSS Max Idle Period | 7.3.2.73, 11.20.15 | CFv:O | Yes, No, N/A |
| *WNM12 | TFS | 11.20.13 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.74, 7.4.11.16 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.75, 7.4.11.17 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.11.18 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | Sleep Mode | 11.20.14 | CFv:O | Yes, No, N/A |
| WNM13.1 | Sleep Mode Request frame | 7.3.2.76, 7.4.11.19 | WNM13:M | Yes, No, N/A |
| WNM13.2 | Sleep Mode Response frame | 7.3.2.76, 7.4.11.20 | WNM13:M | Yes, No, N/A |
| WNM14 | TIM broadcast | 7.3.2.77, 7.3.2.78, 7.4.11.21, 7.4.11.22, 7.4.11.23, 11.2.1.11a | CFv:O | Yes, No. N/A |
| WNM15 | Traffic Generation | 7.4.11.24, 11.20.11 | (CFv AND CF12):O | Yes, No, N/A |
| WNM16 | AC Station Count | 11.20.12 | (CFv AND CF12):O | Yes, No, N/A |