IEEE P802.11
Wireless LANs

| LB123 STA Statistics Comment Resolutions Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-May 13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |

- Abstract

This document contains normative text changes to resolve comment IDs from   LB123.

The submission is referenced to draft D2.0 and augments to all editor instructions as well as the actual base draft modifications.

All new text relative to the draft D2.0 is identified in blue color and underlined and all deleted text is identified by strikethrough.

- STA Statistics Request

Change Figure 62e in 7.3.2.21.8 as follows:

|  | Randomization Interval | Measurement Duration | Group Identity | Triggered Reporting (optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |
| - Measurement Request field format for a STA Statistics Request |  |  |  |  |

Insert a new row into Table 29c in 7.3.2.21.8 as shown below:

| - Group Identity for a STA Statistics Request |  |
| --- | --- |
| Statistics Group Name | Group Identity |
| RSNA Counters | 16 |
| Reserved | 1617 – 255 |

- Table 7-29c is defined in the 802.11k draft.

Change the paragraph starting with “The Measurement Duration field...” as follows:

The Measurement Duration field is set to the duration of the requested measurement in TUs except when triggered reporting is used. When triggered reporting is used, the measurement duration is set to 0.

Insert the following at end of 7.3.2.21.8:

The Triggered Reporting field is used to specify trigger conditions and thresholds for triggered STA Statistics measurements. It is present only when setting up triggered reporting from STA Counters, QoS STA Counters, or RSNA Counters, see 11.10.8.5.

The format of the Triggered Reporting field for STA Counters is shown in Triggered Reporting field for STA Counter. The fields marked as optional are the fields that are only present if the appropriate bit in the STA Counter Trigger Condition is set to 1.

|  | Measurement Count | Trigger Timeout | STA Counter Trigger Condition | dot11FailedCount Threshold (optional) | dot11FCS ErrorCount Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 2 | 2 | 0 or 4 | 0 or 4 |
|  | dot11Multiple RetryCount Threshold (optional) | dot11Frame DuplicateCount Threshold (optional) | dot11RTS FailureCount Threshold (optional) | dot11ACK FailureCount Threshold (optional) | dot11RetryCount Threshold (optional) |
| Octets: | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 |
| - Triggered Reporting field for STA Counter |  |  |  |  |  |

- Figure 7-79e is defined in the 802.11k draft.

The value in the Measurement Count field specifies the number of consecutive MSDUs/MPDUs transmitted and/or received by the STA. that are to be used to determine if one or more of the trigger conditions have been met.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met.

The STA Counter Trigger Condition field specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the STA Counter Trigger Condition field is shown in Error: Reference source not found.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11FailedCount | dot11FCSErrorCount | dot11Multiple RetryCount | dot11Frame DuplicateCount |
| Bits | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7-B15 |
|  | dot11RTSFailureCount | dot11ACKFailureCount | dot11RetryCount | Reserved |
| Bits | 1 | 1 | 1 | 9 |
| - STA Counter Trigger Condition field |  |  |  |  |

Each bit in the STA Counter Trigger Condition field is set to 1 to request that a STA Statistics Report frame be generated when the corresponding STA counter defined in Figure 85h and Figure 85i (in Error: Reference source not found) exceeds the value of the corresponding STA counter threshold defined in Triggered Reporting field for STA Counter, within the total number of MSDUs/MPDUs indicated in the Measurement Count field. One or more trigger conditions are set with specified thresholds. In the triggered STA Counters STA Statistics request, the value of the Group Identity field is set to either 0 or 1. When the group identity field value of the triggered STA Statistics request is set to 0, B2 – B6 in the STA Counter Trigger Condition field are set to 0. When the group identity of the triggered STA Statistics request is set to 1, B0 and B1 in the STA Counter Trigger Condition field are set to 0.

The dot11FailedCount Threshold field contains a value representing the number of dot11FailedCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11FailedCount bit in the STA Counter Trigger Condition field is set to 1.

The dot11RetryCount Threshold field contains a value representing the number of dot11RetryCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11RetryCount bit in the STA Counter Trigger Condition field is set to 1.

The dot11MultipleRetryCount Threshold field contains a value representing the number of dot11MultipleRetryCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11MultipleRetryCount bit in the STA Counter Trigger Condition field is set to 1.

The dot11FrameDuplicateCount Threshold field contains a value representing the number of dot11FrameDuplicateCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11FrameDuplicateCount bit in the STA Counter Trigger Condition field is set to 1.

The dot11RTSFailureCount Threshold field contains a value representing the number of dot11RTSFailureCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11RTSFailureCount bit in the STA Counter Trigger Condition field is set to 1.

The dot11ACKFailureCount Threshold field contains a value representing the number of dot11ACKFailureCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA This field is present when the dot11ACKFailureCount bit in the STA Counter Trigger Condition field is set to 1.

The dot11FCSErrorCount Threshold field contains a value representing the number of dot11FCSErrorCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11FCSErrorCount bit in the STA Counter Trigger Condition field is set to 1.

The format of the Triggered Reporting field for QoS STA Counters is shown in Error: Reference source not found. The fields marked as optional are the fields that are only present if the appropriate bit in the QoS STA Counter Trigger Condition is set to 1.

|  | Measurement Count | Trigger Timeout | QoS STA Counter Trigger Condition | dot11QoSFailedCount Threshold (optional) | dot11QoSRetryCount Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 2 | 2 | 0 or 4 | 0 or 4 |
|  | dot11QoSMultiple RetryCount Threshold (optional) | dot11QoSFrame DuplicateCount Threshold (optional) | dot11QoSRTSFailureCount Threshold (optional) | dot11QoSACK FailureCount Threshold (optional) | dot11QoSDiscardedCount Threshold (optional) |
| Octets: | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 |
| - Triggered Reporting Field for QoS STA Counters |  |  |  |  |  |

The UP of the QoS STA Counters for triggered QoS Statistics measurement is determined by the group identity of the measurement request field for a STA Statistics Request frame as defined in Figure 79e.

The value in the Measurement Count field specifies the number of consecutive MSDUs/MPDUs transmitted and/or received by the STA. that are to be used to determine if one or more of the trigger conditions have been met.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met.

The QoS STA Counter Trigger Condition field specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the QoS STA Counter Trigger Condition field is shown in QoS STA Counter Trigger Condition field.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11QoS FailedCount | dot11QoS RetryCount | dot11QoSMultiple RetryCount | dot11QoSFrame DuplicateCount |
| Bits: | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7-B15 |
|  | dot11QoSRTS FailureCount | dot11QoSACK FailureCount | dot11QoS DiscardedCount | Reserved |
| Bits: | 1 | 1 | 1 | 9 |

- QoS STA Counter Trigger Condition field

Each bit in the QoS STA Counter Trigger Condition field is set to 1 to request that a STA Statistics Report frame be generated when the corresponding QoS STA counter defined in Figure 85j (in Error: Reference source not found) exceeds the value of the corresponding QoS STA counter threshold defined in Error: Reference source not found, within the total number of MSDUs/MPDUs given in Measurement Count. One or more trigger conditions are set with specified thresholds.

The dot11QoSFailedCount Threshold field contains a value representing the number of dot11QoSFailedCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSFailedCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSRetryCount Threshold field contains a value representing the number of dot11QoSRetryCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSRetryCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSMultipleRetryCount Threshold field contains a value representing the number of dot11QoSMultipleRetryCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSMultipleRetryCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSFrameDuplicateCount Threshold field contains a value representing the number of dot11QoSFrameDuplicateCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSFrameDuplicateCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSRTSFailureCount Threshold field contains a value representing the number of dot11QoSRTSFailureCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSRTSFailureCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSACKFailureCount Threshold field contains a value representing the number of dot11QoSACKFailureCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSACKFailureCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSDiscardedCount Threshold field contains a value representing the number of dot11QoSDiscardedCount occurrences within the number of MSDUs indicated in the Measurement Count field that results in a triggered report sent by the measuring STA. This field is present when the dot11QoSDiscardedCount bit in the QoS STA Counter Trigger Condition field is set to 1.

The format of the Triggered Reporting field for RSNA Counters is shown in Error: Reference source not found. The fields marked as optional are the fields that are only present if the appropriate bit in the RSNA Trigger Condition is set to 1.

|  | Measurement Count | Trigger Timeout | RSNA Counter Trigger Condition | dot11RSNAStats CMACICVErrors Threshold (optional) | dot11RSNA StatsCMACReplays Threshold (optional) | dot11RSNA StatsBIPReplays Threshold (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 2 | 2 | 0 or 4 | 0 or 4 | 0 or 4 |
|  | dot11RSNA StatsRobustMgmt CCMPReplays Threshold (optional) | dot11RSNAStats TKIPICVErrors Threshold (optional) | dot11RSNAStats TKIPReplays Threshold (optional) | dot11RSNAStats CCMPDecryptErrors Threshold (optional) | dot11RSNAStats CCMPReplays Threshold (optional) |  |
| Octets: | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 |  |
| - Triggered Reporting field for RSNA Counter |  |  |  |  |  |  |

The value in the Measurement Count field specifies the number of consecutive MSDUs/MPDUs transmitted and/or received by the STA. that are to be used to determine if one or more of the trigger conditions have been met.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met.

The RSNA Counter Trigger Condition field specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the RSNA Trigger Condition field is shown in RSNA Trigger Condition field.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNA StatsCMACReplays | dot11RSNAStats BIPReplays | Dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5 | B6 | B7 | B8-B15 |  |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays | Reserved |  |
| Bits: | 1 | 1 | 1 | 8 |  |
| - RSNA Trigger Condition field |  |  |  |  |  |

Each bit in the Trigger Condition field is set to 1 to request that a STA Statistics Report frame be generated when the corresponding RSNA Error or Failure counter defined in Error: Reference source not found (see Error: Reference source not found) exceeds the value of the corresponding RSNA Failure Threshold in Error: Reference source not found within the total number of MSDUs/MPDUs given in Measurement Count. One or more trigger conditions are set with specified thresholds.

The dot11RSNAStatsCMACICVErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCMACICVErrors condition. This field is present when the dot11RSNAStatsCMACICVErrors bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCMACReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCMACReplays condition. This field is present when the dot11RSNAStatsCMACReplays bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsBIPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsBIPReplays condition. This field is present when the dot11RSNAStatsBIPReplays bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsRobustMgmtCCMPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsRobustMgmtCCMPReplays condition. This field is present when the dot11RSNAStatsRobustMgmtCCMPReplays bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsTKIPICVErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsTKIPICVErrors condition. This field is present when the dot11RSNAStatsTKIPICVErrors bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsTKIPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsTKIPReplays condition. This field is present when the dot11RSNAStatsTKIPReplays bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCCMPDecryptErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCCMPDecryptErrors condition. This field is present when the dot11RSNAStatsCCMPDecryptErrors bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCCMPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCCMPReplays condition. This field is present when the dot11RSNAStatsCCMPReplays bit in the RSNA Trigger Condition field is set to 1.

- Specific measurement usage

- STA Statistics Report

Insert the following at the end of 11.10.8.5:

A STA may request that a STA Statistics report be sent when statistics of interest reach a threshold as defined in the Measurement Request element of the STA Statistics Request frame (see 7.3.21.8). This is termed a triggered STA Statistics measurement and shall be requested by setting the Enable and Report bits to 1 within a Measurement Request Element containing the STA Statistics Measurement Type. The Measurement Request field shall contain a STA Statistics Request with the trigger conditions specified in the Triggered Reporting field, as defined in 7.3.2.21.8. One or more trigger conditions may be set with specified thresholds. See Error: Reference source not found. To prevent generation of too many triggered reports, the minimum value of the Trigger Timeout fiels is set to dot11MinTriggerTimeout.

Annex-D:

TGV Ed: Add the following to the end of “dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

dot11WirelessManagementImplemented TruthValue, 
	dot11MaxIdlePeriod 			INTEGER,
	dot11TIMBroadcastInterval     INTEGER,
	dot11TIMBroadcastOffset       INTEGER,

dot11MinTriggerTimeout     INTEGER}

TGV Ed: Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11MinTriggeredTimeout OBJECT-TYPE

SYNTAX UNSIGNED32

UNITS "seconds"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the minimum allowable value for Triggered Timeout. A Triggered STA Statistics report is generated by the STA after the timeout if none of the trigger conditions are satisfied.

."

DEFVAL { 10 }

::= { dot11StationConfigEntry xx }