IEEE P802.11
Wireless LANs

| Peer Power Save Mode for TDLS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-5-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| # Srinivasa Duvvuri | Atheros | 5480 Great America Parkway Santa Clara, CA 95054 USA | +1-408-222-5465 | sduvvuri@atheros.com |
| Michelle Gong | Intel Corporation | 2200 Mission College Blvd, Santa Clara, CA 95052 USA | +1-408-765-7994 | michelle.x.gong@intel.com |
| Jakub Majkowski | Nokia | Itamerenkatu 11-13, 00810 Helsinki, Finland | +358504878889 | Jakub.majkowski@nokia.com |
| Marc Jalfon | Intel Corporation |  |  | marc.jalfon@intel.com |
| Alexander Safonov | IITP RAS | B.Karetny 19, Moscow, 127994, Russia | +7 495 650 3617 | safa@iitp.ru |
| Yongho Seok | LG Electronics | Seocho-Gu, Seoul 137-724, Korea | +82-2-526-4225 | yhseok@lge.com |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |
| Jesse Walker | Intel Corporation | JF3-206, 2111 NE 25th Ave, Hillsboro, OR 97124 USA | +1-503-712-1849 | jesse.walker@intel.com |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document contains a normative text proposal for peer power save mode for TDLS. This document is based on TGz Draft 1.0.

This document addresses comment CIDs 418-476.

# 7 Frame formats

# 7.1 MAC frame formats

# 7.2 Format of individual frame types

### 7.2.1 Control frames

### 7.2.2 Data frames

### 7.2.2.1 TDLS frame format

Editor Note: Add two new clauses 7.2.2.1.11 and 7.2.2.1.12 as follows:

7.2.2.1.11 TDLS Peer PSM Request frame format

The TDLS Peer PSM Request frame contains the information shown in Table z1.

Table z1--Information for TDLS Peer PSM Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The link identifier is specified in section 7.3.2.z1 |
| 2 | Dialog Token | The Dialog Token is set to a non-zero value that is unique among TDLS Peer PSM Request frames for which a corresponding TDLS Switch Response frame has not been received. |
| 3 | Wakeup Schedule IE | The Wakeup Schedule IE describes a sleep state and/or a schedule for the wakeup time and is defined in Figure z1. |
| 4 | Vendor Specific | One or more vendor-specific information elements may be in this frame. |

### 7.2.2.1.12 TDLS Peer PSM Response frame format

The TDLS Peer PSM Response frame contains the information shown in Table z2.

Table z2—Information for TDLS Peer PSM Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The link identifier is specified in section 7.3.2.z1 |
| 2 | Dialog Token | The Dialog Token is set to the non-zero value contained in the TDLS Peer PSM Request frames to identify the request/response transaction |
| 3 | Status Code | The Status code identifies the result of the response as defined in Table 7-23. |
| 4 | Wakeup Schedule IE | The Wakeup Schedule IE describes a sleep state and/or a schedule for the wakeup time and is defined in Figure z1. |
| 4 | Vendor Specific | One or more vendor-specific information elements may be in this frame. |

Editor Note: Insert the following in Table 7-23, Section 7.3.1.9.

| Status Code | Meaning |
| --- | --- |
| 2 | Accept the TDLS Peer PSM Request with my own schedule |

### 7.3 Management frame body components

### 7.3.1 Fields that are not information elements

### 7.3.2 Information elements

Editor Note: Remove the last sentence in Section 7.3.2.27 Extended Capabilities Information Element

Editor Note: Insert a Table in Section 7.3.2.27 that defines the Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Support Peer PSM | The Support Peer PSM capability bit setting to 1 indicates that the STA supports Peer PSM as described in 11.2.1.12. The Support Peer PSM capability bit setting to 0 indicates that the STA does not support Peer PSM |
| <ANA> | Support “More Data” bit in ACK | Setting the Support “More Data” bit in ACK capability bit to 1 indicates that the STA supports operation on the “More Data” bit in ACK frames. Setting the Support “More Data” bit in ACK capability bit to 0 indicates that the STA does not support operation on the “More Data” bit in ACK frames. |

Editor Note: Add a new clause 7.3.2.z4 as follows:

7.3.2.z4 PSM Wakeup Schedule information element

|  | Element ID | Length | Sleep Mode | TSF Start Time | Offset | Periodicity | Fraction | Minimum Duration | Maximum Duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 4 | 1 | 1 | 4 | 4 |

Figure z1 PSM Wakeup Schedule Element Format

The Length field is 1 octet long and is set to 1 or 19 depending on the Sleep Mode.

The Sleep Mode field indicates the sleep mode that a peer STA will be in.  Table z3 describes the information of the Sleep Mode Field.  When the Sleep Mode is set to 0 or 1, the rest of the fields in the Wakeup Schedule IE are not included.  When the Sleep Mode is set to 2, a wakeup Schedule is specified in this IE.

The TSF Start Time represents the lower 4 octets of the TSF timer when a beacon TBTT occurs before the first wakeup time.

The Offset field indicates the offset between the start of the first wakeup time and the TSF Start Time, expressed in microseconds.

The combination of the Periodicity and the Fraction determines the wakeup time.  The valid range for the Periodicity field and the Fraction field is from 1 to 255.

The Minimum Duration field specifies the minimum duration of the wakeup period, expressed in microseconds, when a STA may remain awake.

The Maximum Duration field specifies the maximum duration of the wakeup period, expressed in microseconds, after which, a STA may return to doze state.

Table z3-- Information of Sleep Mode Field

| Sleep Mode | Information |
| --- | --- |
| 0 | Always Awake |
| 1 | Unscheduled Wakeup |
| 2 | Scheduled Wakeup |
| 3-255 | Reserved |

11 MLME

Editor Note: Insert the following clause after 11.2.1.11.

11.2.1.12 Peer Power Management with TDLS

A STA supports Peer Power Save Mode (PPSM) if dot11TDLSPeerPSMImplemented is true. When two STAs support PPSM they are capable of exchanging the TDLS Peer PSM Request/Response frames, as described in this section.  If two peer STAs support PPSM, either one or both peer STAs may go into power save mode while still being able to communicate via a direct link. The peer STAs shall negotiate one of the following combinations of sleep modes that they would be in before going into power save mode:

- One STA is in “Always Awake” mode and the peer STA is in “Unscheduled Wakeup” mode;

- One STA is in “Scheduled Wakeup” mode and the peer STA is in “Unscheduled Wakeup” mode;

- One STA is in “Always Awake” mode and the peer STA is in “Scheduled Wakeup” mode;

- Both STAs are in “Scheduled Wakeup” mode.

Note – The combination when both STAs are in “Always Awake” mode is valid. However, it is not considered here because it doesn’t result in a power save mode change by either STA. The combination when both STAs are in “Unscheduled Wakeup” mode is not supported for Peer Power Management over the direct link.

TDLS Peer PSM Request/Response Frame Exchange

A STA that intends to go into PPSM shall transmit a TDLS Peer PSM Request frame to the peer STA, indicating its Sleep Mode as “Always Awake”, “Unscheduled Wakeup”, or “Scheduled Wakeup.” A STA supports “Always Awake” if dot11TDLSPeerPSMAlwaysAwakeModeImplemented is true. A STA supports “Unscheduled Wakeup” if dot11TDLSPeerPSMUnscheduledModeImplemented is true.  A STA supports “Scheduled Wakeup” if dot11TDLSPeerPSMScheduledModeImplemented is true.

If the direct link is active, the TDLS Peer PSM Request/Response frames shall be sent via the direct link. If the direct link is not active, the TDLS Peer PSM Request/Response frames shall be sent over the AP path.

Upon receiving the TDLS Peer PSM Request frame, the peer STA replies with a TDLS Peer PSM Response frame indicating its own Sleep Mode. If the peer STA accepts the Sleep mode of the requesting STA as proposed in the TDLS Peer PSM Request frame, it shall set the Status Code in the response frame to “Accept”. In this case, the Sleep Mode field in the TDLS Peer PSM Response frame shall be set according to one of the valid combinations indicated above.

A peer STA may reject the TDLS Peer PSM Request by setting the Status Code to “Reject”.

If the Status Code field is set to “Accept the TDLS Peer PSM Request with my own schedule”, a Wakeup Schedule included in the TDLS Peer PSM Response frame indicates a different schedule selected by the recipient of the TDLS Peer PSM Request frame. In this case, the TDLS Peer PSM Request/Response frame exchange results in two independent Wakeup Schedules that are maintained by two peer STAs. The original requester should resubmit a second request frame that contains the Wakeup Schedule defined in the first TDLS PSM Response frame. The second TDLS Peer PSM Request/Response frame exchange results in a common Wakeup Schedule.

After two peer STAs have successfully negotiated Sleep Modes and/or Wakeup Schedules, they may go into power save mode.  A STA changing Power Management mode shall inform the peer STA of this fact using the Power Management (PM) bit within the Frame Control field of transmitted frames. The Power Management bit in the Frame Control field of the frame sent by the STA in this exchange indicates the Power Management mode that the STA shall adopt upon successful completion of the entire frame exchange.  A STA shall remain in its current Power Management mode until it informs the peer STA of a Power Management mode change via a frame exchange that includes an acknowledgment from the peer STA.  The peer STA may stay active to support the peer STA in PS mode or may go into power save mode following the same procedure.

If two STAs have negotiated their Sleep Modes, either of them may transmit another TDLS Peer PSM Request frame to update its Sleep Mode and/or its Wakeup Schedule. If the update fails, the previously negotiated Sleep Modes and Wakeup Schedules shall still be valid.

Operation of STAs in Peer Power Save Mode

STAs that support peer STAs in power management mode shall buffer traffic destined to the peer STAs and deliver the buffered traffic over the direct link.

If a STA is in Unscheduled Wakeup mode, the behavior of the peer STA is the same as that of an AP using U-APSD as defined in 11.2.1.4, with the following exception. When traffic becomes available for an AC and no service period has occurred for that AC for a period of Peer PSM Indication Window prior to the arrival of the new traffic, the STA in Always Awake mode shall send a unicast Peer Traffic Indication frame to the peer STA in Unscheduled Wakeup mode through the access point, indicating the AC(s) for which traffic is available.

The behavior of a STA in Unscheduled Wakeup mode with respect to traffic buffered at the peer STA is the same as that of a STA using U-APSD, with the following exception. The indication that traffic is buffered at the peer STA in Always Awake mode or in Scheduled Wakeup mode is obtained from received Peer Traffic Indication frames from the peer STA in Always Awake mode. If the peer STA is in Scheduled Wakeup mode, then the STA in Unscheduled Wakeup mode can only initiate a service period when the peer STA is awake.

- NOTE – To reduce the number of Peer Traffic Indication frames transmitted for a continuous uni-directional traffic stream without return traffic, a STA in Unscheduled Wakeup mode may trigger another service period within the Peer PSM Indication Window after the occurrence of a service period during which at least one MPDU was received, i.e. before the peer STA in Always Awake mode has transmitted a Peer Traffic Indication frame.

- A STA in Unscheduled Wakeup mode may request a STA in Always Awake mode or in Scheduled Wakeup mode to use U-APSD by setting individual U-APSD Flag bits in the QoS Info subfield of the QoS Capability element carried in TDLS Setup Request and Response frames.

A STA in Scheduled Wakeup mode shall wake up at or before the beginning of the scheduled wakeup time, when TSF mod floor(BI * Periodicity/Fraction) = offset. The STA shall remain awake either for the Minimum Duration after the beginning of the scheduled wakeup time or until any initiated service periods are terminated successfully.

Peer Service Period

A STA sets the “Support More Data bit in ACK” bit in the Extended Capabilities IE to 1 if dot11MoreDataACKOptionImplemented is true.  If a STA has one or more pending frames for the owner of the service period, the STA may set the More Data bit in an ACK frame to 1, in response to a data frame to indicate that it has one or more pending frames buffered for the peer STA, if both peer STAs set the “Support More Data bit in ACK” bit to 1.  If at least one of two peer STAs set the “Support More Data bit in ACK” bit to 0, the More Data bit in ACK frames is undefined.

When a STA is awake, the peer STA may initiate a service period.  Only one service period in one direction shall be active at any time for any peer STAs.  If both peer STAs set the “Support More Data bit in ACK” bit to 1, an acknowledged data frame or a QoS-Null frame may be used as a trigger frame for the service period.  If at least one of two peer STAs set the “Support More Data bit in ACK” bit to 0, a QoS-Null frame shall be used as a trigger frame for the service period.  The recipient of the trigger frame owns the service period. The owner shall set the EOSP bit in all data frames to 0, except the last data frame, within the same service period.  The EOSP bit in the last data frame within a service period shall be set to 1.

If both peer STAs set the “Support More Data bit in ACK” bit to 1, a service period is terminated via a frame exchange with the EOSP bit in the data frame set to 1 and the More Data bit in the ACK frame set to 0. If at least one of two peer STAs set the “Support More Data bit in ACK” bit to 0, a service period is terminated after the owner informs the peer STA via an acknowledged frame exchange with the EOSP bit in the data frame set to 1.  During a service period, the owner of the service period and the peer STA shall operate in Awake state.

If a STA does not receive an acknowledgment to a directed MPDU sent with the EOSP subfield set to 1, it shall retransmit that frame at least once within the same service period or the Minimum Duration, subject to applicable retry or lifetime limit.

Annex A

A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

A.4.3 IUT configuration

A.4.z1 Wireless Network Management extensions

Editor Note: Add a new entry to the PICS as follows:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TDLS1 | Tunneled Direct Link Setup | 7.2.2.1, 11.z1 | CFz:M | Yes, No. N/A |
| TDLS1.1 | TDLS Setup | 7.2.2.1.1, 7.2.2.1.2 | CFz:M | Yes, No. N/A |
| TDLS1.2 | TDLS Teardown | 7.2.2.1.3, 7.2.2.1.4 | CFz:M | Yes, No. N/A |
| TDLS1.3 | TDLS Tx Path Switch | 7.2.2.1.5, 7.2.2.1.6 | CFz:M | Yes, No. N/A |
| TDLS1.4 | TDLS Rx Path Switch | 7.2.2.1.7, 7.2.2.1.8 | CFz:M | Yes, No. N/A |
| TDLS1.5 | TDLS Peer Key Handshake | 8.5.9 | CFz:M | Yes, No. N/A |
| TDLS1.6 | Link RCPI Request/Report | 7.3.2.21.11, 7.3.2.22.11 | CFz:M | Yes, No. N/A |
| TDLS1.7 | TDLS Peer PSM | 7.2.2.1.11, 7.2.2.1.12 | CFz:O | Yes, No, N/A |

Annex D

Insert the following entry at the end of Annex D

dot11TDLSPeerPSMImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer PSM.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11TDLSPeerPSMAlwaysAwakeModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer PSM Always Awake Mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11TDLSPeerPSMUnscheduledModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer PSM Unscheduled Mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11TDLSPeerPSMScheduledModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer PSM Scheduled mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}