IEEE P802.11
Wireless LANs

| Peer Power Save Mode for TDLS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-7-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Srinivasa Duvvuri | Atheros | 5480 Great America Parkway Santa Clara, CA 95054 USA | +1-408-222-5465 | sduvvuri@atheros.com |
| Michelle Gong | Intel Corporation | 2200 Mission College Blvd, Santa Clara, CA 95052 USA | +1-408-765-7994 | michelle.x.gong@intel.com |
| Jakub Majkowski | Nokia | Itamerenkatu 11-13, 00180 Helsinki, Finland | +358504878889 | jakub.majkowski@nokia.com |
| Marc Jalfon | Intel Corporation | Haifa 31015, Israel | +972-4-865-5006 | marc.jalfon@intel.com |
| Alexander Safonov | IITP RAS | B.Karetny 19, Moscow, 127994, Russia | +7 495 650 3617 | safa@iitp.ru |
| Yongho Seok | LG Electronics | Seocho-Gu, Seoul 137-724, Korea | +82-2-526-4225 | yhseok@lge.com |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |
| Jesse Walker | Intel Corporation | JF3-206, 2111 NE 25th Ave, Hillsboro, OR 97124 USA | +1-503-712-1849 | jesse.walker@intel.com |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document contains normative text for peer power save mode for TDLS. This document is based on TGz Draft 1.0.

This document addresses comment CIDs 418-476.

# 7 Frame formats

# 7.1 MAC frame formats

# 7.2 Format of individual frame types

### 7.2.1 Control frames

### 7.2.2 Data frames

### 7.2.2.1 TDLS frame format

Add two new clauses 7.2.2.1.11 and 7.2.2.1.12 as follows:

7.2.2.1.11 TDLS Peer PSM Request frame format

The TDLS Peer PSM Request frame contains the information shown in Table zA.

Table zA—Information for Peer PSM Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Dialog Token | The Dialog Token contains a value which is unique among TDLS Peer PSM Request frames for which a corresponding TDLS Peer PSM Response frame has not been received. |
| 2 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 3 | Wakeup Schedule | The Wakeup Schedule is specified in 7.3.2.z4. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

### 7.2.2.1.12 TDLS Peer PSM Response frame format

The TDLS Peer PSM Response frame contains the information shown in Table zB.

Table zB—Information for Peer PSM Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Dialog Token | The Dialog Token is set to the value contained in the corresponding TDLS Peer PSM Request frame. |
| 2 | Status Code | The Status Code is specified in 7.3.1.9. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | Wakeup Schedule | The wakeup schedule is only present when the status code is set to “Wakeup schedule rejected but alternative schedule provided“. The Wakeup Schedule is specified in 7.3.2.z4. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.3 Management frame body components

7.3.1 Fields that are not information elements

In subclause 7.3.1.9, add the following status codes to Table 7-23, and update the reserved values accordingly:

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| <ANA> | Wakeup schedule rejected but alternative schedule provided |
| <ANA> | Wakeup schedule rejected because not going into power save |
| <ANA> | Wakeup schedule rejected for unspecified reason |

7.3.2 Information elements

In table 26, add a new information element as follows, and renumber the reserved values accordingly:

Table 26—Element IDs

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Wakeup Schedule (see 7.3.2.z4) | <ANA> | 16 |

7.3.2.27 Extended Capabilities information element

Replace the indicated change to Table 7-35a with the following:

Table 7-35a—Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Peer U-APSD Support | The Peer U-APSD Support field indicates support for Peer U-ASPD, as defined in 11.2.1.12. The Peer U-ASPD Support field is set to 1 to indicate support for Peer U-APSD. The Peer U-APSD Support bit is set to 0 to indicate a lack of support for Peer U-APSD. |
| <ANA> | Peer PSM Support | The Peer PSM Support field indicates support for Peer PSM, as defined in 11.2.1.13. The Peer PSM Support field is set to 1 to indicate support for Peer PSM. The Peer PSM Support bit is set to 0 to indicate a lack of support for Peer PSM. |

Insert the following new subclause after 7.3.2.z3:

7.3.2.z4 Wakeup Schedule information element

The Wakeup Schedule information element contains information regarding the wakeup schedule for Peer Power Save Mode. The element information format is defined in Figure zC.

|  | Element ID | Length | Start Time | Interval | Minimum Duration | Idle Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 4 | 4 | 2 |

Figure zC—Wakeup Schedule element format

The Length field is set to 16.

The Start Time field is set to the lower order 4 octets of the TSF timer value at the start of the first wakeup period.

The Interval field is set to the time in microseconds between two successive wakeup periods.

The Minimum Duration field is set to minimum duration of the wakeup period, in microseconds.

The Idle Count field is set to the number of consecutively empty wakeup periods before a peer STA deactivates the wakeup schedule.

11 MLME

Replace 11.2.1.12 with the following:

11.2.1.12 Peer U-APSD

A STA supports Peer U-APSD if dot11TDLSPeerUAPSDImplemented is true. Support of this capability shall be signaled through the Peer U-APSD subfield in the Extended Capabilities element included in the Association Request frame body inside the TDLS Setup Request or TDLS Setup Response frame. This is similar to an AP indicating support for U-APSD.

When a STA supports Peer U-APSD, the peer STA may enter a power save mode while the STA remains awake. Traffic is exchanged during U-APSD Service Periods, which are initiated by the peer STA, possibly after receipt of a Peer Traffic Indication frame. The peer STA may enter the doze state after setting the Power Management bit on MPDUs to the STA supporting Peer U-APSD, similar to U-APSD.

The behavior of a STA supporting Peer U-APSD shall be the same as for a U-APSD AP, with the following exception. When an AC becomes backlogged and no Service Period has occurred for that AC for a period of Peer PSM Indication Window prior to the arrival of the new traffic, the STA in AP mode sends a unicast Peer Traffic Indication frame to the peer STA through the access point, indicating the backlogged AC(s). For Peer U-APSD, all ACs shall be trigger and delivery enabled, as indicated by setting each of the ACs U-APSD Flag subfields inside the QoS Info field to 1 inside transmitted TDLS Setup Request or TDLS Setup Response frames. This is similar to a non-AP STA indicating to an AP that it intends to use U-APSD.

The behavior of a STA in PS mode with respect to traffic buffered at the peer STA supporting Peer U-APSD shall be the same as a regular U-APSD STA, with the following exception. The indication that traffic is buffered at the peer STA supporting Peer U-APSD is obtained from received Peer Traffic Indication frames from the peer STA supporting Peer U-APSD. To reduce the number of Peer Traffic Indication frames transmitted for a continuous unidirectional traffic stream without return traffic, a new Service Period may be started within Peer PSM Indication Window after the occurrence of a Service Interval during which a Data MPDU was received. Starting a new Service Period within Peer PSM Indication window avoids that a PTI frame is generated.

When a STA has entered Power Save mode by setting the PM bit on MPDUs transmitted to the peer STA, the peer STA can not also enter Power Save mode.

Insert a new subclause 11.2.1.13 as follows:

11.2.1.13 Peer PSM

A STA supports Peer Power Save Mode (Peer PSM) if dot11TDLSPeerPSMImplemented is true. Support of this capability shall be signaled through the Peer PSM subfield in the Extended Capabilities element included in the Association Request frame body inside the TDLS Setup Request or TDLS Setup Response frame.

When both peer STAs on a direct link support Peer PSM and have agreed on a wakeup schedule, they may go into power save mode while still being able to communicate over the direct link during periodic wakeup windows. The wakeup periods are automatically deactivated in the absence of traffic. Deactivated means that the periodic wakeup no longer occurs and that a schedule no longer exists.

A STA which intends to enter peer power save mode shall send a Peer PSM Request frame to the peer STA, including a proposed wakeup schedule. When the peer STA accepts the proposed wakeup schedule, it shall respond with a Peer PSM Response frame indicating status code 0. Otherwise it shall respond with a Peer PSM Response frame indicating the appropriate status code for rejecting the schedule. An alternative schedule shall be included in the Peer PSM Response frame when the reason code is set to “Wakeup schedule rejected but alternative schedule provided”. The alternative schedule may be used by the STA to generate a new Peer PSM Request frame.

After transmitting or receiving a Peer PSM Response frame indicating status code 0, peer STAs shall activate the wakeup schedule. When the wakeup schedule is active, both peer STAs shall be awake at the scheduled wakeup times, and stay awake for the Minimum Duration or until the end of a peer service period.

Either station may update an existing schedule, by transmitting a Peer PSM Request frame over the currently active path.

A peer service period (PSP) begins after an acknowledged frame exchange between the peer STAs. PSPs are bi-directional, and at most one PSP occurs per Wakeup Window. The PSP is terminated when both peer STAs received from the respective peer STA a frame with the EOSP bit set to one.

A STA shall remain awake until it has successfully transmitted to and received from the peer STA an MPDU with the EOSP bit set to one, and it has finished any ongoing frame exchange.

A STA shall cease transmitting Data MPDUs with the A1 field set to the MAC address of the peer STA after it has successfully transmitted to and received from the peer STA an MPDU with the EOSP bit set to one, and it has finished any ongoing frame exchange.

When both peer STA set the More Data ACK subfield in their QoS Capability information element, then the More Data bit inside an ACK frame set to zero shall have the same function as the EOSP bit inside a QoS frame set to one (i.e. to terminate a PSP).

To keep track of the connectivity over the direct link and to maintain the wakeup schedule, peer STAs may start an acknowledged frame exchange at least once per Idle Count consecutive wakeup windows, as a keepalive. For instance a Null frame may be used as a keepalive frame. When no peer service period has occurred for Idle Count consecutive wakeup periods, the STAs shall deactivate the wakeup schedule. When traffic arrives for a peer STA in Peer PSM mode with a deactivated wakeup schedule, the STA shall send a Peer PSM Request frame through the AP path.

11.z1 Tunneled Direct Link Setup

Insert a new paragraph as follows:

A TDLS Teardown Request message should be sent through the AP when one or more MPDUs are discarded on the direct link. The connectivity on a direct link may be tested using a (QoS) Null frame.

Annex A

A.4 PICS proforma—IEEE Std 802.11-2007

A.4.3 IUT configuration

A.4.z1 Wireless Network Management extensions

Replace PICS entries TDLS 1.7 and TDLS1.8 as follows:

| TDLS1.7 | TDLS Peer U-APSD | 11.2.1.12 | CFz:O | Yes, No, N/A |
| --- | --- | --- | --- | --- |
| TDLS1.8 | TDLS Peer PSM | 11.2.1.13 | CFz:O | Yes, No, N/A |

Annex D

Replace dot11PeerPSMAPModeImplemented and dot11PeerPSMClientModeImplemented with dot11PeerUAPSDImplemented  and dot11PeerPSMImplemented as specified below:

dot11TDLSPeerUAPSDImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer PSM.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11TDLSPeerPSMImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer PSM.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}