IEEE P802.11
Wireless LANs

| Power save for 802.11s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jarkko Kneckt | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland |  | Jarkko.Kneckt@nokia.com |
| Janne Marin | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland |  | Janne.Marin@nokia.com |
| Mika Kasslin | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland |  | Mika.Kasslin@nokia.com |
| Dee Denteneer | Philips | Philips Research, HTC 27 (WL 1.132), 5656 AE Eindhoven, The Netherlands |  | Dee.Denteneer@Philips.com |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa Shinagawa-ku, Tokyo, 141-0001 Japan |  | sako@wcs.sony.co.jp |

Abstract

The document contains proposed text as a comment resolution for simple editorial and technical comments that were received in LB126.

The target of the document is to clarify the 802,11s D2.0 descriptions for the power save, but not to introduce any new technical capabilities.

3. Definitions

Add the fields as shown and number the elements accordingly:

3.s14 Peer Service Period (PSP):  A peer service period is a contiguous period of time during which one or more unicast frames are transmitted between two peer MPs. A peer service period is directional and may contain one or more TXOPs. One MP transmits frames and initiates the termination of the peer service period. An MP may have multiple peer service periods ongoing in parallel. At most one peer service period can be set up in each direction with each peer MP.

3.s20 Power Mode: A power mode defines the used power management and power save level of an MP seen by neighboring MPs. For peer MPs the power mode may be link specific.

3.s21 Power Save Level: A power save level defines the level of the power save. The level of the power save defines the minimum beaconing frequency and the rules when the MP operates in Awake state.

7.1.3.1.6 Power Management field

Change the text in the clause as shown:

The Power Management field is 1 bit in length and is used to indicate the power management mode of a STA including MP. In a BSS network the following applies. The value of this field remains constant in each frame from a particular STA within a frame exchange sequence defined in 9.12. In case of a STA in a BSS, tThe value indicates the mode in which the station will be after the successful completion of the frame exchange sequence. A value of 1 indicates that the STA will be in PS mode. A value of 0 indicates that the STA  will be in active mode. This field is always set to 0 in frames transmitted by an AP.

In a mesh network the following applies. The value of this field indicates the mode in which the MP will be after the completion of the frame exchange sequence. In the case of an MP, a value of 0 in multicast or broadcast frame indicates that the MP will be in active mode. A value of 1 indicates that the MP will be in power save mode for non-peer MPs. For the peer MPs the link specific power mode rules are determined by the Power Management field in the unicast frame exchanged between these peers, as described in 11B13.3.  The Power Management field in a unicast frame indicates the Power Management mode of the MP for the recipient of the frame after the successful completion of the frame exchange sequence. A value of 0 indicates that the MP will be in active mode and a value of 1 indicates that the MP will be in power save mode.

7.1.3.5a.2 Mesh Flags field

Modify the figure and add descriptions for thePower Management Level as shown below.

| B0 B1 | B2 | B3 B7 |
| --- | --- | --- |
| Address Extension (AE) Mode | Power Save Level | Reserved |
| Bits: 2 | 1 | 5 |
| Figure s5—Mesh Flags field |  |  |

…

If the Power Management mode field is set to 1, the Power Save Level field indicates the power save level of the MP. The value 0 in the Power Save Level field indicates that the MP is operating in light sleep mode, the value 1 indicates that MP is operating in deep sleep mode. If the Power Management field is set to 0, the Power Save Level field is reserved.

…

7.2.1.4 PS-Poll frame format

Change the second paragraph in 7.2.1.4 as shown:

When the frame is transmitted by a STA in a BSS, tThe BSSID is the address of the STA contained in the AP. When the frame is transmitted by an MP in a mesh, the BSSID field is simply interpreted as RA. The TA field is the address of the STA transmitting the frame. When the frame is transmitted by a STA in a BSS, tThe AID is the value assigned to the STA transmitting the frame by the AP in the association response frame that established that STA’s current association. When the PS-Poll frame is transmitted by an MP in a mesh, he AID field is set to the AID of the MP that transmits the frame as defined in the peer link confirm frame between the transmitter and receiver MPs.

7.2.3.1 Beacon frame format

Change the Mesh ATIM field to Awake Window field as shonw.

Insert the following additional rows (preserving their order) in Table-7-8-Beacon frame body just before the Vendor Specific information element and insert contiguous numbering in the “Order” column:

| Order | Information | Notes |
| --- | --- | --- |
|  | Mesh ID | The Mesh ID information element may be present within Beacon frames when dot11MeshEnabled is true. |
|  | Mesh Configuration | The Mesh Configuration information element may be present within Beacon frames when dot11MeshEnabled is true. |
|  | Mesh TIM | The Mesh TIM information element may be present within Beacon frames when dot11MeshEnabled is true. |
|  | Awake Window | The Mesh Awake Window information element may be present within Beacon frames when dot11MeshEnabled is true. |
|  | Beacon Timing | The Beacon Timing information element may be present within Beacon frames when dot11MeshEnabled is true. |
|  | MDAOP Advertisements | The MDAOP Advertisements information element may be present within Beacon frames when dot11MeshEnabled is true and the MP supports MDA. |
|  | MSCIE | The MSCIE element may be present within Beacon frames when dot11MeshEnabled is true. |

7.2.3.5 Probe Response frame format

Insert the following additional rows (preserving their order) in before the last row of 7-15─Probe Response frame body just before the Vendor Specific information element and insert contiguous numbering in the “Order” column:

| - Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Mesh ID | The Mesh ID information element may be present within Probe Response frames when dot11MeshEnabled is true. |
|  | Mesh Configuration | The Mesh Configuration information element may be present within Probe Response frames when dot11MeshEnabled is true. |
|  | Awake Window | The Mesh Awake window element may be present within Probe Response frames when dot11MeshEnabled is true. |
|  | Beacon Timing | The Beacon Timing information element may be present within Probe Response frames when dot11MeshEnabled is true. |
|  | MDAOP Advertisements | The MDAOP Advertisements information element may be present within Probe Response frames when dot11MeshEnabled is true and the MP supports MDA. |
|  | MSCIE | The MSCIE element may be present within Probe Response frames when dot11MeshEnabled is true. |

- Mesh Capability

The mesh capability field comprises a set of values indicating whether an MP is a possible candidate for peer link establishment. The details of the mesh capability field are shown in .

| B0 | B1 | B2 | B3 | B4 | B5 | B6 B15 |
| --- | --- | --- | --- | --- | --- | --- |
| Accepting Peer Links | MDA Enabled | Forwarding | Beacon timing report enabled | TBTT adjustment enabled | Power Save Level | Reserved |
| Bits: 1 | 1 | 1 | 1 | 1 | 1 | 10 |
| - Mesh Capability field |  |  |  |  |  |  |

The “Accepting Peer Links” field is set to 1 if the MP is able and willing to establish peer links with other MPs and set to 0 otherwise.

The “MDA Enabled” field is set to 1 if the MP supports MDA services and set to 0 otherwise.

The "Forwarding" field is set to dot11MeshForwarding.

The "Beacon timing report enabled" field is set to 1 if the MP supports MBCA beacon timing reporting function and is set to 0 otherwise.

The "TBTT adjustment enabled" field is set to 1 if the MP supports TBTT adjusting function upon either of the detection of potential beacon reception at the neighboring MP or the reception of TBTT Adjustment Request, and is set to 0 otherwise.

If the Power Management mode field is set to 1, the Power Save Level field indicates the power save level of the MP. The value 0 in the Power Save Level field indicates that the MP is operating in light sleep mode, the value 1 indicates that MP is operating in deep sleep mode. If the Power Management field is set to 0, the Power Save Level field is reserved.

7.3.2.87 Mesh TIM element

The Mesh TIM element contains four fields: Mesh DTIM Count, Mesh DTIM Period, Bitmap Control, and Partial Virtual Bitmap. The field structure of the Mesh TIM element is the same as the TIM element.

The format of the Mesh TIM element is shown in Figure s20.

Figure s20—Mesh TIM element

The Element ID is set to the value given in Table 7-26 for this information element. The Length field indicates the length of this information element, which is constrained as described below.

The Mesh DTIM Count field indicates the number of Beacon frames (including the current frame) that appear before the next Mesh DTIM. A Mesh DTIM Count of 0 indicates that the current Mesh TIM is a Mesh DTIM. The Mesh DTIM Count field is a single octet.

The Mesh DTIM Period field indicates the number of Beacon intervals between successive Mesh DTIMs. If all Mesh TIMs are Mesh DTIMs, the Mesh DTIM Period has the value 1. The Mesh DTIM Period value 0 is reserved. The Mesh DTIM Period field is a single octet.

|  | AID 0 | Bitmap Offset |
| --- | --- | --- |
| Bit: | 0 | 1 - 7 |

FigureSS1─Bitmap Control field

The Bitmap Control field is a single octet. The format of the Bitmap Control field is shown in Figure─SS1. Bit 0 of the field contains the Traffic Indicator bit associated with AID 0. This bit is set to 1 in Mesh TIM elements with a value of 0 in the Mesh DTIM Count field when one or more groupcast frames are buffered at the MP. The remaining 7 bits of the field form the Bitmap Offset.

The traffic-indication virtual bitmap, maintained by the MP that generates a Mesh TIM, consists of 2008 bits, and is organized into 251 octets such that bit number N ( ) in the bitmap corresponds to bit number (N mod 8) in octet number N/8 where the low-order bit of each octet is bit number 0, and the high order bit is bit number 7. Each bit in the traffic-indication virtual bitmap corresponds to traffic buffered for a specific neighboring MP within the Mesh that the MP is prepared to deliver at the time the beacon frame is transmitted.

Bit number N is 0 if there are no directed frames buffered for the neighboring MP whose AID is N. If any directed frames for that neighboring MP are buffered and the MP is prepared to deliver them, bit number N in the traffic-indication virtual bitmap is 1.

The Partial Virtual Bitmap field consists of octets numbered N1 through N2 of the traffic indication virtual bitmap, where N1 is the largest even number such that bits numbered 1 through (N1 × 8) - 1 in the bitmap are all 0 and N2 is the smallest number such that bits numbered (N2 + 1) × 8 through 2007 in the bitmap are all 0. In this case, the Bitmap Offset subfield value contains the number N1/2, and the Length field is be set to (N2 - N1) + 4.

In the event that all bits other than bit 0 in the virtual bitmap are 0, the Partial Virtual Bitmap field is encoded as a single octet equal to 0, the Bitmap Offset subfield is 0, and the Length field is 4.

7.3.2.88 Awake Window element

The Awake Window parameter element is present in DTIM Beacon and may be present in Beacon and Probe Response frames.

The contents of the element are shown in Figure s23.

| Octets: 1 | 1 | 2 |
| --- | --- | --- |
| ID | Length | Awake Window |
| Figures21─Awake Window parameter element |  |  |

The Element ID is set to the value given in Table 7─26 for this information element. The length field is set to 2.

The Awake Window field is 2 octets in length and contains the Awake Window length in TUs.

10.3.52 MLME-MeshPeerLinkPOWERMGT

The following primitives describe how a mesh entity changes its power mode for a peer link.

10.3.52.1 MLME-MeshPeerLinkPOWERMGT.request

10.3.52.1.1 Function

This primitive requests a change in the MPs power mode for the peer link.

10.3.52.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshPeerLinkPOWERMGT.request(

local Link ID,

Power Mode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| local Link ID | Integer | 1 - 216−1 | Specifies the integer generated by the local MP |
| Power Mode | Enumeration | ACTIVE, LIGHT_SLEEP, DEEP_SLEEP | Specifies the power mode that the local MP is using for the peer link |

10.3.52.1.3 When generated

This primitive is generated when the mesh entity wishes to change its power mode for a peer link.

10.3.52.1.4 Effect of receipt

This primitive initiates the local MP power mode change for the peer link. The MLME subsequently issues an MLME-MeshPeerLinkPOWERMGT.confirm that reflects the results.

10.3.52.2 MLME-MeshPeerLinkPOWERMGT.confirm

10.3.52.2.1 Function

This primitive reports the result of a power mode change attempt.

10.3.52.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshPeerLinkPOWERMGT.confirm(

local Link ID,

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| local Link ID | Integer | 1 - 216−1 | Specifies the integer generated by the local MP |

10.3.52.2.3 When generated

This primitive is generated as a result of an MLME-MeshPeerLinkPOWERMGT.request.

10.3.52.2.4 Effect of receipt

The SME is notified of the results of the power mode change for a peer link procedure.

11B.13 Power save in a mesh

An MP is capable to buffer frames, track power mode of each peer MP, and use peer service periods for data transmission. These capabilities are referred as support for power save.

An MP, which operates in power save mode or is transitioning to power save mode, is referred to as a power saving MP. The operation in power save mode is optional.

11B.13.1  Link specific power modes

An MP is in one of two different power states:

- Awake: the MP is able to transmit or receive frames and is fully powered.

- Doze: the MP is not able to transmit or receive and consumes very low power.

The manner in which an MP transitions between power states is determined by the combination of peer link specific power modes. An MP shall be in Awake state if any of its peer links requires operation in Awake state.

A peer link is always associated with two MPs and both MPs have own power mode for the peer link. An MP defines a power mode in which it operates for the peer link and maintains the power mode its peer MP operates for the peer link. The power modes of peer links are independent and MP may operate in different power modes for other peer links. An MP maintains a power mode for non-peer MPs which is described in 11B.13.2. An example illustration of power modes use is shown in Figure SS1.

There are three power modes defined: active mode, light sleep mode, and deep sleep mode. The used power mode shall be indicated by the Power Management field and Power Save Level field. The difference between modes is as follows

- Active mode: The MP shall be in Awake state all the time.

- Light sleep mode: The MP alternates between Awake and Doze states, as determined by the frame transmission and reception rules. The MP shall listen to all the Beacons from its peer MP

- Deep sleep mode: The MP alternates between Awake and Doze states, as determined by the frame transmission and reception rules. The MP may choose not to listen to the Beacons from its peer MP

A power mode is defined as the combination of the Power Management mode indicated in the Frame Control field and the Power Save Level as shown in the table s76.

Table s76, Power Mode definition

| Activity Level | Power Mode | Power Management field | Power Save Level field |
| --- | --- | --- | --- |
| Highest Lowest | Active mode | 0 | Reserved |
|  | Light sleep mode | 1 | 0 |
|  | Deep sleep mode | 1 | 1 |

MP A

Non-peer PM A, Active mode

MP B

Non-peer PM B, Power save mode

PM B→A, Light sleep mode

MP C

Non-peer PM C, Power save mode

PM C→B, Deep sleep mode

PM B→C, Deep sleep mode

PM C→A, Deep sleep mode

PM A→C, Active mode

PM A→B, Active mode

Figure ss1, An example of Power Mode use. All MPs have peer links between each other.

11B.13.2 Non-peer power modes

An MP shall indicate its non-peer power mode with the Power Management field in Beacon and Probe Response frames and in groupcast frames it transmits. The non-peer power mode determines when non-peer MPs may send Probe Request and Peer Link Open Request frames to the MP. The non-peer power mode shall be equal to or lower than the lowest activity level in which the MP is operating for peer MPs.

11B.13.3 Power mode indications and transitions

When an MP is operating in active mode for a link, the MP shall set Power Management bit to 0 in all transmitted unicast frames on this link.

When an MP is operating in light sleep mode for a link, the MP shall set the Power Management bit to 1 and the Power Save Level field to 0 in all transmitted unicast frames on this link.

When an MP is operating in deep sleep mode for a link, the MP shall set the Power Management bit to 1 and the Power Save Level field to 1 in all transmitted unicast frames on this link.

The algorithm to trigger the change of power mode is beyond the scope of the standard.

11B.13.3.1 Transition to a higher activity level

An MP may use groupcast or unicast frames to change its power mode to a higher activity level, i.e. from deep sleep to light sleep or to active mode; or from light sleep to active mode.

Unicast frames may be used to temporarily raise the activity level of the MP for a peer link. This may be useful in cases, when a link temporarily requires efficient data transmission with the peer MP and the MP desires to be able to transit back to lower activity level without performing the power management transition signaling with all peer MPs.

11B.13.3.2 Transition to a lower activity level

An MP shall use acknowledged unicast frames to change its power mode for a link to a lower activity level, i.e. from active mode to light or deep sleep mode; or from light sleep to deep sleep mode.

If an MP is operating in deep sleep mode for a link or the MP intends to change the power mode of at least one link to deep sleep mode, the MP shall set the Power Management bit to 1 and Power Save Level field to 1 to all transmitted groupcast frames.

If an MP is not operating in deep sleep mode for any link and the MP is operating in light sleep mode for a link or the MP intends to change its operation mode for at least one link to light sleep mode, the MP shall set the Power Management bit to 1 and Power Save Level field to 0 in all transmitted groupcast frames.

11B.13.3 Mesh TIM transmissions

The Mesh TIM element shall identify the peer MPs operating in power save mode for which traffic is pending and buffered in the reporting MP. This information is coded in a partial virtual bitmap using AIDs. In addition, the Mesh TIM contains an indication whether groupcast traffic is pending. An MP shall assign an AID to every peer MP during the peer link establishment procedure. MPs shall assign AID values uniquely to each of the peer MPs. AID 0 (zero) is reserved to indicate the presence of buffered groupcast MSDUs. The MP shall identify those peer MPs for which it is prepared to deliver buffered MSDUs by setting bits in the partial virtual bitmap of the Mesh TIM that correspond to the appropriate AIDs.

11B.13.4 Mesh TIM types

There are two different Mesh TIM types: Mesh TIM and Mesh DTIM. An MP shall transmit a Mesh TIM with every Beacon frame. Every DTIMPeriod, a Mesh TIM of type Mesh DTIM is transmitted with a Beacon frame. After a Mesh DTIM the MP shall send the buffered groupcast MSDUs, before transmitting any unicast frames. The More Data field of each groupcast frame shall be set to indicate the presence of further buffered groupcast MSDUs.  The MP sets the More Data field to 0 in the last transmitted groupcast frame following the transmission of the DTIM beacon.

11B.13.5 Awake Window

A power saving MP shall be in Awake state during its own Awake Window. An Awake Window is present after the Beacon and Probe Response frames containing an Awake Window element. An awake Window element shall be in the DTIM Beacon frame and may be in TIM Beacon and Probe Response frames. The start of the Awake Window is measured from the end of the Beacon or Probe Response transmission. The duration of the Awake Window period is defined by the value of the Awake Window field in the Awake Window element.

The peer MPs may send trigger frames to the power saving MP during the Awake Window of the power saving MP in order to initiate a peer service period. A successfully transmitted trigger frame initiates a peer service period as described in 11B.13.9.

Non-peer MPs may send Probe Request and Peer Link Open frames to the power saving MP during its Awake Window. The power saving MP shall operate in Awake state at least until the completion of the peer link setup procedure (i.e. success or failure) or the transmission of the Probe Response.

11B.13.6 Power save support

As described in 11B.13.1 an MP indicates its power mode for each peer link and obtains the power modes of its peer MPs.

As described in 11B.13.3 an MP indicates the presence of buffered traffic in TIM elements for all peer MPs in power save mode. As described in 11B.13.4, an MP transmits its groupcast frames after its DTIM Beacon.

As described in 11B.13.9 peer service periods are used for frame transmissions towards an MP which operates in power save mode. Peer Service Periods are not used in frame exchange towards active mode MPs.

An MP may initiate a peer service period with a peer MP which operates in deep or light sleep mode by transmitting a trigger frame during the Awake Window of the peer MP.

11B.13.7 Operation in link specific power mode

11B.13.7.1 Operation in the active mode

An MP operating in active mode shall be in Awake state. Peer service periods are not used in frame exchange toward active mode MPs.

11B.13.7.2 Operation in deep sleep mode

An MP operating in deep sleep mode shall enter the Awake state prior to every TBTT that matches its own DTIM Beacon timing and shall remain in Awake state after the DTIM Beacon transmission for the duration of the Awake Window and the duration of its groupcast frame transmissions, whichever occurs later.

An MP in power save mode may receive trigger frames during its Awake Window as described in 11B.13.5.

11B.13.7.3 Operation in light sleep mode

If an MP indicated buffered traffic in Beacon frame for a peer MP in light sleep for the link, the MP shall remain in Awake state until it has received a trigger frame from the peer MP in light sleep.

An MP operating in light sleep mode shall enter the Awake state prior to every TBTT that matches the Beacon timing of its own and peer MP. The MP shall remain in Awake state after its beacon transmission for the duration of the Awake Window and the duration of its groupcast frame transmissions, whichever occurs later. The MP may return to the Doze state after the Beacon reception from a peer MP, to which it indicated to operate in light sleep mode, if the peer MP did not indicate buffered unicast or groupcast frames. If an indication of buffered frames is received, the light sleep mode MP shall send a trigger frame to initiate a peer service period with the beaconing MP.

If a light sleep mode MP enters the Awake state in order to listen to DTIM Beacon and groupcast transmissions from its peer MP, the MP shall remain Awake until the More Data field of the groupcast MSDUs indicated the last groupcast MSDU.

11B.13.7.4 Conditions for Doze state

An MP may enter Doze state if and only if all following conditions are fulfilled:

- the MP operates in power save mode for all of the links

- the MP has no peer service period ongoing

- the Awake Window indicated by the MP has expired

- the MP has terminated its groupcast frames delivery sequence after its DTIM Beacon

- the MP has received a trigger frame from all peer light sleep mode MPs, to which the MP indicated buffered traffic in Beacon frame

Guidance for the power save mechanisms use and default parameter values are given in Annex V.4.

11B.13.8 Beaconing in power modes

An MP indicating deep sleep to all its peer MPs shall transmit DTIM Beacons. If an MP has indicated active or light sleep mode to any peer MP, the MP shall transmit TIM and DTIM beacons.

11B.13.9 Peer service period

Peer service periods are used for frame exchange in a link in which at least one of the MPs operates in power save mode. A peer service period is a contiguous period of time during which one or more unicast frames are transmitted between two peer MPs. A peer service period is directional and may contain one or more TXOPs. One MP transmits frames and initiates the termination of the peer service period. An MP may have multiple peer service periods ongoing in parallel. At most one peer service period can be set up in each direction with each peer MP. A peer service period is initiated by a peer trigger frame.

A data frame or a QoS-Null frame that requires acknowledgement may be used as a peer trigger frame. If the EOSP bit in the peer trigger frame is set to 0, the peer trigger frame initiates two independent peer service periods; one is owned by the transmitter of the peer trigger frame and the other is owned by the receiver of the peer trigger frame,. If the EOSP bit in the peer trigger frame is set to 1, one peer service period is initiated which is owned by the receiver of peer trigger frame. Peer service period shall not be used in frame exchange toward active mode MP.

During the peer service period, the transmitter of the peer service period and peer MP shall operate in Awake state. The peer service period may contain one or more TXOPs. During the peer service period, the transmitter is able to transmit frames from any AC, but a TXOP shall contain frames from a single AC only.

The peer service period is terminated after a successfully acknowledged QoS-Null or data frame with the EOSP bit set to 1 from the transmitter of the peer service period.

If the MP does not receive an acknowledgement to a frame sent with the EOSP subfield set to 1, the MP shall retransmit that frame at least once within the same peer service period - subject to applicable retry or lifetime limit. The maximum number of retransmissions within the same peer service period is the lesser of the Max Retry Limit and the MIB attribute dot11MPMissingAckRetryLimit. If an Ack to the retransmission of this last frame in the same peer service period is not received, the MP may use the next peer service period to further retransmit that frame subject to the applicable retry or lifetime limit.

A.4.4.16a.1 General Mesh Support

Modify the MP8 as shown in the section and number the following elements accordingly

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| MP8 | Mesh Power Management | 11B.13 | MP1:O | Yes No N/A |
| MP9 | Mesh Power save Support | 11B.13 | MP1:M | Yes No N/A |

Annex D (normative) ASN.1 encoding of the MAC and PHY MIB

Remove the dot11MeshATIMWindow from the annex

V.4 Power Save parameters selection

Modify the V.4 annex as shown

Power save mechanisms enable MP operation in Doze state. In Doze state an MP may set the transreceiver off. The power save mechanism may adjust the power mode of an MP for various reasons which are beyond the scope of the standard including but not limited to traffic load or scanning and network maintenance needs.

The Annex specifies recommendations how to select a power mode for the MP based on traffic load, recommendations how to do scanning in mesh networks which may contain MPs in power save mode and recommended default values for power save related parameters.

V.4.1 Selecting the power mode based on traffic load

An MP may adjust its power mode based on the traffic load or the QoS requirements of the forwarded traffic. If an MP has high traffic load or an MP is congested, the MP should operate in active mode to reduce delays or overhead that may be created by the power save mechanisms.

An MP may consider operating in active mode for all peer MPs, even if only a single link is congested. When MP operates in active mode, the MP may receive frames at any time and peer service periods may be triggered on demand, as in case of an AP that receives a trigger frame at times controlled by a  non-AP STA.

If traffic load is moderate or low and best effort data is transmitted, an MP may consider operating in light sleep mode for all or some peer links. The MP operating in light sleep mode for a peer link receives beacons from the peer MP with periodic indication of buffered traffic.

If traffic load is low or no traffic is transmitted, an MP may consider operating in deep sleep mode for a peer link. The MP does not need to wake-up to receive a beacon from the peer MP relative to which it operates in deep sleep mode.

The MP may use deep sleep mode to control the number of times it enters the Awake state to receive Beacon frame from a peer MP. If the MP operates in deep sleep mode for all of its peer link, the MP needs only to operate in Awake state during its own beacon transmission and Awake Window.

The power saving MP should avoid real time traffic (AC3 or AC2) transmissions with EDCA when both MPs in a peer link are in power save mode. In this case the delays in frames forwarding will increase and the forwarded data frames have inappropriate QoS.

V.4.2 Scanning of mesh networks

A mesh network may have MPs in power save and active mode. With active scanning only devices in Awake state at the transmission time of Probe Request frame may be found. However, with passive scanning also MPs in power save mode can be found.

Since MPs in power save mode may transmit Beacons at long intervals, an MP seeking for candidate MPs for a new peer links should do passive scanning relatively longer time compared to passive scanning in BSS infrastructure mode. MPs in power save mode with long Mesh DTIM interval may not be discovered with short scanning durations. MPs in power save mode may use short Mesh DTIM interval, if it intends to establish new peer links..

V.4.3 Default parameters

The following are the recommended default values for power save related parameters for MPs:

Beacon Period: 200TU

Mesh DTIM period: 5

Awake Window: 10TU

Although MPs may utilize individual parameters regardless of the parameters used by neighbor MPs or peer MPs, each implementer should recognize balance between the power save efficiency and delay in the service initiation.