IEEE P802.11
Wireless LANs

| TGz Draft P802.11z_D1.1 with editorial comments addressed |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: June 19, 2008 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document contains TGz Draft P802.11z_D1.1 with editorial comments addressed. The comment resolutions are contained in accompanying document 11-08-0771-00-000z.

Instructions to the editor:

- Apply the changes as indicated in revision marks to TGz Draft 1.1.

IEEE P802.11z™/D1.1

Draft STANDARD for Information Technology- Telecommunications and information exchange between systems- 
Local and metropolitan area networks-
Specific requirements-

Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

Amendment 7: Direct Link Setup

Prepared by the 802.11 Working Group of the IEEE 802 Committee

Copyright © 2008 by the IEEE.

3 Park Avenue

New York, NY 10016-5997, USA

All rights reserved.

This document is an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! Because this is an unapproved draft, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for IEEE Standards Committee participnts to reproduce this document for purposes of international standardization consideration. Prior to adoption of this document, in whole or in part, by another standards development organization, permission must first be obtained from the IEEE Standards Activities Department (stds.ipr@ieee.org). Other entities seeking permission to reproduce this document, in whole or in part, must also obtain permission from the IEEE Standards Activities Department.

IEEE Standards Activities Department

445 Hoes Lane

Piscataway, NJ 08854, USA

Abstract: This amendment provides Direct Link Setup enhancements to the IEEE 802.11 MAC, and PHY, extending direct link setup to be independent of the access point, and adding power save capabilities. The direct link setup is made independent of the AP by tunneling the protocol messages inside data frames.

Keywords: Direct Link, Power Saving

Introduction

(This introduction is not part of IEEE P802.11z/D1.0, Draft Standard for Information Technology – Telecommunications and information exchange between systems - Local and metropolitan area networks – Specific requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications. Amendment 7: Direct Link Setup)

This amendment specifies enhancements to the following draft standard and draft amendments, in order to support direct link setup:

— IEEE P802.11-2007

— IEEE P802.11k (RevCom Mar08)

— IEEE P802.11r (RevCom Jun08)

— IEEE P802.11y (RevCom Jun08),

— IEEE P802.11w (RevCom Mar09)

— IEEE P802.11n (RevCom Jun09),

— IEEE P802.11u (RevCom Sep09),

The purpose of this document is to provide amendments to the IEEE 802.11 PHY/MAC layers related to direct link setup.

Notice to users

Errata

Errata, if any, for this and all other standards can be accessed at the following URL: http:// standards.ieee.org/reading/ieee/updates/errata/index.html. Users are encouraged to check this URL for errata periodically.

Interpretations

Current interpretations can be accessed at the following URL: http://standards.ieee.org/reading/ieee/interp /index.html.

Patents

Attention is called to the possibility that implementation of this amendment may require use of subject matter covered by patent rights. By publication of this standard, no position is taken with respect to the existence or validity of any patent rights in connection therewith. A patent holder or patent applicant has filed a statement of assurance that it will grant licenses under these rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses. Other Essential Patent Claims may exist for which a statement of assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions are reasonable or non-discriminatory. Users of this standard are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this draft amendment to standard was completed, the IEEE 802.11 Working Group had the following membership:

Bruce Kraemer, Chair

Adrian Stephens and John Rosdahl, Vice-chairs

Stephen McCann, Secretary

EDITORIAL NOTE—a three column list of voting members of 802.11 on the day the draft was sent for sponsor ballot will be inserted

The following were officers of Task Group z:

Menzo Wentink, Chair

Daniel Borges, Secretary

Menzo Wentink, Technical Editor

Major contributions were received from the following individuals:

EDITORIAL NOTE—a three-column list of individuals who made a major contribution will be inserted.

The following members of the balloting committee voted on this Standard. Balloters may have voted for approval, disapproval, or abstention.

EDITORIAL NOTE—a three-column list of responding sponsor ballot members will be inserted by IEEE staff

Editorial Notes

EDITORIAL NOTE—Two forms of editorial markup are used: Notes and Comments. Editorial Notes and Editorial Comments are not part of the amendment and will be removed before it is published, together with any other contents in this subclause. This paragraph is an example of how an Editorial Note is marked. Editorial Comments are marked (Ed:), and contain references to submissions or comment resolutions to track the origin of changes.

EDITORIAL NOTE—Headings with empty content or Headings preceding editing instructions that modify the contents of the referenced subclause are there to provide context to the reader of this document, they have no other significance.

EDITORIAL NOTE—Except when referring to tables and figures that exist in the baseline, figure and table numbers are preceded by “z” and are assigned sequentially. This will be changed prior to sponsor ballot.

EDITORIAL NOTE—The default IEEE-SA style for tables is to “float”. This means that they be repositioned later, usually at the head of the next page, to avoid splitting the table and reduce the amount of blank space. The table can appear to move out of the subclause it is referenced first from, and can even split a paragraph. This is the intended IEEE-SA behavior, please do not report it as a defect in the draft.

EDITORIAL NOTE—Line numbering is only approximate. This is a limitation of the FrameMaker tool. Whitespace between paragraphs is part of the IEEE-SA style, as defined in their templates. The combination of these two facts leads to the appearance of blank lines in the draft between every paragraph. Please do not report this as an editorial defect as it is the unavoidable behavior.

EDITORIAL NOTE—The <ANA> flags in this document will be replaced with the appropriate values once assigned.

Contents

EDITORIAL NOTE—Insert table of contents.

List of figures

EDITORIAL NOTE—Insert list of figures.

List of tables

EDITORIAL NOTE—Insert list of tables.

IEEE P802.11z™/D1.0

Draft STANDARD for Information Technology- Telecommunications and information exchange between systems- 
Local and metropolitan area networks-
Specific requirements-

Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

Amendment 7: Direct Link Setup

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

3. Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form “3.z<x>” where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline document.

Insert the following new definitions into Clause 3:

3.z1 tunneled direct link setup: A protocol which uses a specific Ethertype encapsulation to tunnel setup frames through any AP.

3.z2 peer power pave mode: A power save mode which can be used by STAs which support Tunneled Direct Link Setup (TDLS). There are two modes for Peer PSM operation, AP mode and client mode.

4. Abbreviations and acronyms

Insert the following new abbreviations and acronyms into Clause 4 so as to maintain alphabetic ordering:

Peer PSM		Peer Power Save Mode

TDLS			Tunneled Direct Link Setup

7. Frame formats

7.1 MAC frame formats

7.1.3 Frame fields

7.1.3.1 Frame Control field

Change 7.1.3.1.6 as shown:

7.1.3.1.6 Power Management field

The Power Management field is 1 bit in length and is used to indicate the power management mode of a STA. The value of this field remains constant in each frame from a particular STA within a frame exchange sequence defined in 9.12. The value indicates the mode in which the station will be after the successful completion of the frame exchange sequence.

A value of 1 indicates that the STA will be in PS mode or Peer PS mode. A value of 0 indicates that the STA will be in active mode. This field is always set to 0 in frames transmitted by an AP.

Change 7.1.3.5 and Table 7-4 as shown:

- 7.1.3.5 QoS Control field

The QoS Control field is a 16-bit field that identifies the TC or TS to which the frame belongs and various other QoS-related information about the frame that varies by frame type and subtype. The QoS Control field is present in all data frames in which the QoS subfield of the Subtype field is set to 1 (see 7.1.3.1.2). Each QoS Control field comprises five subfields, as defined for the particular sender (HC or non-AP STA) and frame type and subtype. The usage of these subfields and the various possible layouts of the QoS Control field are described 7.1.3.5.1 through 7.1.3.5.7 and illustrated in Table 4.

Table 7-4—QoS Control field

| Applicable Frame (sub) Types | Bits 0-3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8-15 |
| --- | --- | --- | --- | --- | --- |
| QoS CF-Poll and QoS CF-Ack+ CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP limit |
| QoS Data+CF-Poll and QoS Data+CF-Ack+CF-Poll frames sent by HC | TID | EOSP | Ack Policy | A-MSDU Present | TXOP limit |
| QoS Data and QoS Data+CF-Ack frames sent by HC or by a non-AP STA acting in Peer PSM AP mode | TID | EOSP | Ack Policy | A-MSDU Present | AP PS Buffer State |
| QoS Null frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS Data and QoS Data+CF-Ack frames sent by non-AP STAs not acting in Peer PSM AP mode | TID | 0 | Ack Policy | A-MSDU Present | TXOP duration requested |
|  | TID | 1 | Ack Policy | A-MSDU Present | Queue size |
| QoS Null frames sent by non-AP STAs | TID | 0 | Ack Policy | Reserved | TXOP duration requested |
|  | TID | 1 | Ack Policy | Reserved | Queue size |

7.2 Format of individual frame types

7.2.1 Control frames

7.2.2 Data frames

Insert the following new clauses at the end of 7.2.2:

7.2.2.3	TDLS frame format

The frame body of a TDLS frame is defined in Figure z1.

|  | LLC/SNAP | Remote Frame Type | TDLS Frame Type | Information |
| --- | --- | --- | --- | --- |
| Octets: | 8 | 1 | 1 | variable |

Figure z1—TDLS frame body

The LLC/SNAP field contains the LLC/SNAP header, with Ethertype 0x89-0d.

The Remote Frame Type field is set to <ANA> (TDLS).

The TDLS Frame Type field specifies the type of the TDLS frame, as specified in Table z1.

The Information field contains information which is specified for each TDLS Type individually in subclause 7.2.2.3.1 through 7.2.2.3.10.

Table z1—TDLS Frame Type values

| TDLS Type Value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown Request |
| 4 | TDLS Teardown Response |
| 5 | TDLS DL Path Switch Request |
| 6 | TDLS DL Path Switch Response |
| 7 | TDLS AP Path Switch Request |
| 8 | TDLS AP Path Switch Response |
| 9 | Peer Traffic Indication |
| 10 | TDLS Channel Switch Request |
| 11 | TDLS Channel Switch Response |
| 12 – 255 | reserved |

7.2.2.3.1 TDLS Setup Request frame format

The Information field of a TDLS Setup Request frame contains the information shown in Table z2.

Table z2—Information for TDLS Setup Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Dialog Token | The Dialog Token contains a unique nonzero value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.2.12. |
| 2 | Association Request frame body without RSN element | The Association Request frame body is specified in 7.2.3.4. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 4 | RSNIE_I | The RSNIE for Initiator STA is included if security is required on the direct link. |
| 5 | SMK Message 1 FTIE | The SMK Message 1 FTIE is included if security is required on the direct link. |
| 6 | DH_I | The Public value for Initiator STA is included if security is required on the direct link. |
| 7 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in TGy 7.3.2.51 (optional) |
| 8 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.2 TDLS Setup Response frame format

The Information field of a TDLS Setup Response frame contains the information shown in Table z3.

Table z3—Information for TDLS Setup Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Request. The Dialog Token is specified in 7.3.2.12. |
| 3 | Association Request frame body without RSN element | The Association Request frame body is specified in 7.2.3.4. Only present for Status Code 0 (Successful). |
| 4 | Link Identifier | The Link Identifier is specified in 7.3.2.66. Only present for Status Code 0 (Successful). |
| 5 | RSNIE_P | The RSNIE of Peer STA is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 6 | SMK Message 2 FTIE | The SMK Message 2 FTIE is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 7 | DH_P | The Public value for Peer STA is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 8 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in TGy 7.3.2.51 (optional) |
| 9 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. Only present for Status Code 0 (Successful). |

7.2.2.3.3 TDLS Setup Confirm frame format

The Information field of a TDLS Setup Response frame contains the information shown in Table z4.

Table z4—Information for TDLS Setup Confirm frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Response. The Dialog Token is specified in 7.3.2.12. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 4 | SMK Message 3 FTIE | The SMK Message 3 FTIE is included if security is required on the direct link. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.4 TDLS Teardown Request frame format

The Information field of a TDLS Teardown Request frame contains the information shown in Table z5.

Table z5—Information for TDLS Teardown Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 2 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.2.12. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.5 TDLS Teardown Response frame format

The Information field of a TDLS Teardown Response frame contains the information shown in Table z6.

Table z6—Information for TDLS Teardown Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Teardown Request. The Dialog Token is specified in 7.3.2.12. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.6 TDLS DL Path Switch Request frame format

The Information field of a TDLS DL Path Switch Request frame contains the information shown in Table z7.

Table z7—Information for TDLS DL Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 2 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.2.12. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.7 TDLS DL Path Switch Response frame format

The Information field of a TDLS DL Path Switch Response frame contains the information shown in Table z9.

Table z9—Information for TDLS DL Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Suspend frame. The Dialog Token is specified in 7.3.2.12. |
| 3 | Result | The Result field (1-octet) indicates the result of the Path Switch request and is set to one of values in Table z10. |
| 4 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.8 TDLS AP Path Switch Request frame format

The Information field of a TDLS AP Path Switch Request frame contains the information shown in Table z11.

Table z11—Information for TDLS AP Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 2 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.2.12. |
| 2 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.9 TDLS AP Path Switch Response frame format

The Information field of a TDLS AP Path Switch Response frame contains the information shown in Table z12.

Table z12—Information for TDLS AP Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Path Switch Request frame. The Dialog Token is specified in 7.3.2.12. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.3.10 Peer Traffic Indication

The Information field of a Peer Traffic Indication frame contains the information shown in Table z13.

Table z13—Peer Traffic Indication

| Order | Information | Notes |
| --- | --- | --- |
| 1 | AC_BK traffic available | 1 octet field which is set to 0 if AC_BK is empty and set to 1 if traffic is available in AC_BK. Values 2-255 are reserved. |
| 2 | AC_BE traffic available | 1 octet field which is set to 0 if AC_BE is empty and set to 1 if traffic is available in AC_BE. Values 2-255 are reserved |
| 3 | AC_VI traffic available | 1 octet field which is set to 0 if AC_VI is empty and set to 1 if traffic is available in AC_VI. Values 2-255 are reserved |
| 4 | AC_VO traffic available | 1 octet field which is set to 0 if AC_VO is empty and set to 1 if traffic is available in AC_VO. Values 2-255 are reserved |
| 5 | Peer PSM Indication Window | 1 octet field which indicates the minimum interval between successive Peer Traffic Indication frames sent to the same peer, expressed in Beacon Intervals. |
| 6 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 7 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

The Peer Traffic Indication frame indicates the power save buffer state at the STA buffering data for a peer STA in Peer PSM client mode.

Each of the ACs traffic available field is 1 octet in length. It is set to zero value if the corresponding AC (AC0, AC1, AC2, AC3) is empty and set to 1 if traffic is available for the specific AC. Values 2-255 are reserved.

### 7.2.2.3.11 TDLS Channel Switch Request frame format

The format of the TDLS Channel Switch Request frame contains the information shown in Table z10.

Table z10—Information for TDLS Channel Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Target Channel | 1-octet field which specifies the channel number of the target channel. |
| 2 | Regulatory Class | 1-octet field which specifies the regulatory class for the target channel. |
| 3 | Secondary Channel Offset | The secondary channel offset is included when switching to a 40 MHz direct link (optional). |

The TDLS Channel Switch Request frame shall be sent over the direct path. Switching back to the base channel shall always be accepted. The regulatory class for the base channel shall correspond to the regulatory class of the BSS to which the stations are associated.

### 7.2.2.3.12 TDLS Channel Switch Response frame format

The TDLS Channel Switch Request frame contains the information shown in Table z11.

Table z11—Information for TDLS Channel Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Result Code | 1-octet field which contains an integer which is set to 0 to indicate that the switch request is accepted and set to 1 if the switch request is rejected. |

The TDLS Channel Switch Response frame shall be sent over the direct path.

7.3 Management frame body components

7.3.1 Fields that are not information elements

In subclause 7.3.1.7, insert the following reason codes into Table 7-22, and update the reserved values accordingly:

Table 7-22—Reason codes

| Reason code | Meaning |
| --- | --- |
| <ANA> | Path switch requested for unspecified reason |
| <ANA> | Path switch requested due to change in power save mode |
| <ANA> | Path switch requested due to change in link state |
| <ANA> | Path switch requested for RCPI Measurement |

In subclause 7.3.1.9, insert the following status codes into Table 7-23, and update the reserved values accordingly:

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| <ANA> | Path switch request denied for unspecified reason |
| <ANA> | Path switch request denied because of entering power save mode |
| <ANA> | Path switch request denied because of link state |

7.3.2 Information elements

In table 7-26, inser the following New Information elements, and renumber the reserved values accordingly:

Table 7-26—Element IDs

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Link Identifier (see 7.3.2.66) | z1 | 20 |
| DH_I (see 7.3.2.67) | z2 | 192 |
| DH_P (see 7.3.2.68) | z3 | 192 |

7.3.2.21 Measurement Request element

In P802.11k D7.0, clause 7.3.2.21, Table 29, insert a new measurement request named Link RCPI Request, with Measurement Type 9, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly.

In P802.11k D7.0, renumber clause 7.3.2.21.11 into 7.3.2.21.12, and insert a new clause 7.3.2.21.11 as follows:

7.3.2.21.11 Link RCPI Request

The Measurement Request field corresponding to a Link RCPI Request is shown Figure z2.

|  | BSSID | STA Address |  |
| --- | --- | --- | --- |
| Octets: | 6 | 6 |  |

Figure z2—Measurement Request field for a Link RCPI Request

BSSID indicates the BSSID.

STA Address indicates the MAC address of the STA requesting the Link RCPI measurement.

7.3.2.22 Measurement Report element

In P802.11k D7.0, clause 7.3.2.22, Table 30, insert a new measurement report named  Link RCPI Report, with Measurement Type 9, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly.

In P802.11k D7.0, insert a new clause 7.3.2.22.11 as follows:

7.3.2.22.11 Link RCPI Report

The format of the Measurement Report field corresponding to a Link RCPI Report is shown in Figure z3.

|  | BSSID | RCPI 1 | STA Address | RCPI 2 |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 1 | 6 | 1 |  |

Figure z3—Measurement Report field for a Link RCPI Report

BSSID indicates the BSSID.

RCPI 1 indicates the RCPI on frames received from the AP. The RCPI field is defined in 802.11k clause 17.3.10.6.

STA Address indicates the MAC address of the STA requesting the Link RCPI measurement.

RCPI 2 indicates the RCPI on frames received from the STA requesting the Link RCPI measurement. The RCPI field is defined in 802.11k clause 17.3.10.6.

7.3.2.25 RSN information element

7.3.2.25.2 AKM suites

Insert the following new entry in Table 34 and update the reserved values accordingly:

Table 34—AKM suite selectors

| OUI | Suite type | Authentication type | Key management type |
| --- | --- | --- | --- |
| 00-0F-AC | <ANA> | N/A | SMK Handshake |

- 7.3.2.27 Extended Capabilities information element

Insert rows to Table 35a as follows and renumber the reserved values accordingly:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Peer PSM AP Mode | The Peer PSM AP mode capability bit set to 1 indicates that the STA supports Peer PSM AP mode as described in 11.2.1.12. The Peer PSM AP mode capability bit set to 0 indicates that the STA does not support this capability. |
| <ANA> | TDLS channel switching | The TDLS Channel Switching capability bit set to 1 indicates the STA supports Tunneled Direct Link Setup (TDLS) with Channel Switching as described in 11.20. The TDLS Channel Switching bit set to 0 indicates that the STA does not support this capability. |

7.3.2.46 Fast BSS transition information element

Add the following new entries to table 7-43d of 802.11r-D7.0:

Table 7-43d—Sub-element IDs

| Value | Contents of data field | Length (in octets) |
| --- | --- | --- |
| 4 | MAC_I | 6 |
| 5 | MAC_P | 6 |
| 6 | BSSID | 6 |
| 8-255 | Reserved |  |

MAC_I contains the MAC address of the Initiator STA.

MAC_P contains the MAC address of the Peer STA.

BSSID contains the BSSID.

Lifetime contains the lifetime of the SMKSA in seconds.

Insert the following new clauses after the last subclause of 7.3.2:

7.3.2.66 Link Identifier element

The Link Identifier element contains information which identifies the direct link. The element information format is defined in Figure z4.

|  | Element ID | Length | BSSID | Source Address | Destination Address | Regulatory Class | Channel Number |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 6 | 1 | 1 |  |

Figure z4— Link Identifier element format

The Length field is set to 20.

The BSSID field is set to the BSSID of the STA’s current association.

The Source Address field is set to the transmitting STA’s MAC address.

The Destination Address field is set to the MAC address of the destination STA.

The Regulatory Class field is set to the Regulatory Class of the STA’s current association.

The Channel Number field is set to the channel number of the STA’s current association.

7.3.2.67 DH_I element

The DH_I element contains the public value of the Initiator STA of a TDLS direct link. The element information format is defined in Figure z6.

|  | Element ID | Length | DH_I |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 192 |  |

Figure z6—DH_I element format

The Length field shall be set to 192.

The DH_I field is set to the public value of the Initiator STA.

7.3.2.68 DH_P element

The DH_P element contains the public value of the Peer STA of a TDLS direct link. The element information format is defined in Figure z7.

|  | Element ID | Length | DH_P |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 192 |  |

Figure z7—DH_P element format

The Length field shall be set to 192.

The DH_P field is set to the public value of the Peer STA.

8. Security

Add a new clause 8.5.9 after clause 8.5.8 as follows:

8.5.9 TDLS Peer Key Handshake

The TDLS Peer Key Handshake occurs after any other direct link setup procedures and is used to create an STKSA providing data confidentiality between the two STAs. The AP must establish an RSNA with each STA prior to PeerKey setup. The initiator STA starts the PeerKey Handshake, providing the key to use for securing the connection.

The TDLS Peer Key EAPOL-Key exchange provides a mechanism for obtaining the keys to be used for direct STA-to-STA communication. The initiator STA shall start a timer when it sends the first EAPOL-Key message and the peer STA shall do the same on receipt of the first EAPOL-Key message. On expiration of this timer, the STA shall transition to the STKINIT state.

A STA should use the TDLS Peer Key Handshake prior to transferring any direct STA-to-STA data frames. The STKSA should be deleted when the STA to STA connection is terminated.

The following assumptions apply:

- FTIE denotes an Fast BSS Transition information element as specified in Clause 7.3.2.46 of 802.11r-D7.0.

- STA_I is the initiator STA

- STA_P is the peer STA

- AP is the access point with which both the STA_I and the STA_P are associated

- MAC_I is the MAC address of STA_I

- MAC_P is the MAC address of STA_I

- INonce is the nonce generated by STA_I

- PNonce is the nonce generated by STA_P

- DH_I is the public value of STA_I

- DH_P is the public value of STA_P

The TDLS Peer Key Handshake has two components:

- SMK Handshake: This handshake is initiated by the initiator STA and, as a result of this handshake, the SMKSA is installed in both the STAs. This message exchange goes through the AP and is protected using the PTK.

- 4-Way STK Handshake: Using the installed SMKSA, the initiator STA initiates the 4-Way Handshake (as per 8.5.3.4) and, as a result of this, the STKSA gets installed in both the STAs. The STKSA is used for securing data exchange between the initiator STA and the peer STA. This message exchange goes directly between the peer STAs.

8.5.9.1 SMK Handshake

The Initiator STA initiates the SMK Handshake by sending first message to the Peer STA through the AP path. This is done to establish a SMKSA between Imitator and Peer STA associated with the same AP. Unlike the 4-Way Handshake and Group Key Handshake, the SMK Handshake is initiated by the initiator STA.

For SMK Handshake, the modulus p shall be 1536 bits (as per RFC 3526) in length and the generating element g shall be 2.

The prime is: 2^1536 - 2^1472 - 1 + 2^64 * { [2^1406 pi] + 741804 }. Its hexadecimal value is:

FFFFFFFF FFFFFFFF C90FDAA2 2168C234 C4C6628B 80DC1CD1

29024E08 8A67CC74 020BBEA6 3B139B22 514A0879 8E3404DD

EF9519B3 CD3A431B 302B0A6D F25F1437 4FE1356D 6D51C245

E485B576 625E7EC6 F44C42E9 A637ED6B 0BFF5CB6 F406B7ED

EE386BFB 5A899FA5 AE9F2411 7C4B1FE6 49286651 ECE45B3D

C2007CB8 A163BF05 98DA4836 1C55D39A 69163FA8 FD24CF5F

83655D23 DCA3AD96 1C62F356 208552BB 9ED52907 7096966D

670C354E 4ABC9804 F1746C08 CA237327 FFFFFFFF FFFFFFFF

The information flow of the SMK handshake is as follows:

SMK Message 1:	Initiator STA → Peer STA

RSNIE_I, FTIE(0, 0, 0, INonce, MAC_I, BSSID, Lifetime, DH_I)

SMK Message 2:	Peer STA → Initiator STA

RSNIE_P, FTIE(7, MIC, PNonce, INonce, MAC_I, MAC_P, BSSID, Lifetime, DH_P)

SMK Message 3:	Initiator STA → Peer STA

RSNIE_I, FTIE(4, MIC, PNonce, INonce, MAC_I, MAC_P, BSSID, Lifetime, DH_I)

8.5.9.1.1 SMK Handshake Message 1

The Initiator STA performs following steps before generating Message 1:

- Create RSNIE by filling the element id (fixed hex 30), length, version (1), and pairwise cipher suite list field. Since group cipher suit field is before pairwise cipher suite list field (so STA needs to fill it), STA shall fill this field with any value and on the other side STA processing this field shall ignore it.

- Generate 256 bit random number which is sent in Key Nonce field.

- Decides on lifetime of SMKSA, which is sent as part of Lifetime.

- Initiator STA selects a random integer A < p and computes public DH_I = gA mod p.

The initiator STA sends Message 1 to the peer STA P through AP. After sending the message 1, the Initiator STA starts a timer (different from the Lifetime-Timer) and waits for response message from the Peer STA.

On receipt of Message 1, the peer STA performs following actions:

- Verify the initiator MAC address against existing direct link. If no direct link exists, it silently discards the message.

- If all checks succeed,

  - Generate 256 bit random number which is sent in Key Nonce field.

  - Check the suggested lifetime value proposed by initiator STA. If needed select a smaller lifetime value or use the same lifetime value to create the Lifetime to be used in message 2.

  - Peer STA selects a random integer B < p and computes public DH_P = gB mod p.

  - Peer STA computes the SMK as

SMK-Key-Data = KDF-384(SHA-256(DH_IB mod p), "SMK Key Derivation", BSSID || MAC_I || MAC_P || INonce || PNonce)

SMK-KCK = L(SMK-Key-Data, 0, 128)

SMK = L(SMK-Key-Data, 128, 256)

where

—   KDF-384 is the KDF function as defined in 8.5.1.5.2 used to generate a key of length 384 bits.

—   L(-) is defined in 8.5.1.

—   "SMK Key Derivation" is 0x534d4b204b65792044657269766174696f6e.

—   BSSID is the basic service set identification as defined in 7.1.3.3.3.

  - Peer STA selects one of the ciphersuites from the ciphersuite list in the initiator RSNIE and creates peer RSNIE.

- Using all the information, peer STA creates Message 2 and sends it to initiator STA through AP.

8.5.9.1.2 SMK Handshake Message 2

The peer STA sends Message 2 to the initiator STA through the AP path. After sending Message 2, the Peer STA starts a timer (different from the Lifetime-Timer) and waits for response message from the Initiating STA.

On receipt of Message 2, the Initiator STA performs the following actions:

- Verify the Peer MAC address against existing direct link. If no direct link exists, it silently discards the message.

- Verify the Initiator MAC address and INonce from the FTIE and if it does not match, the Initiator STA silently discards the message.

- If all checks succeed,

  - The Initiator STA computes the SMK as

SMK-Key-Data = KDF-384(SHA-256(DH_PA mod p), "SMK Key Derivation", BSSID || MAC_I || MAC_P || INonce || PNonce)

SMK-KCK = L(SMK-Key-Data, 0, 128)

SMK = L(SMK-Key-Data, 128, 256)

- Verify the MIC on Message 2.  If the MIC verification fails, the Initiator STA silently discards the message. The MIC shall be calculated using the SMK-KCK and the AES-128-CMAC algorithm. The output of the AES-128-CMAC shall be 128 bits. The MIC shall be calculated on the concatenation, in the following order, of:

- MAC_I

- MAC_P

- BSSID

- RSNIE_P

- PNonce

- INonce

- Lifetime

- DH_P

- If the MIC verification succeeds,  the Initiator STA creates Message 3 and sends it to the Peer STA through the AP path.

8.5.9.1.3 SMK Handshake Message 3

The Initiator STA sends Message 3 to the Peer STA through the AP path. After sending Message 3, the SMK handshake is complete from initiator STA side.

On reception of Message 3, the peer STA performs following actions:

- Verify the Initiator MAC address against existing direct link. If no direct link exists, silently discard the message.

- Verify the Peer MAC address and PNonce from the FTIE and if either does not match, silently discard the message.

- Verify the MIC on Message 3.  If the MIC verification fails, the Peer STA silently discards the message. . The MIC shall be calculated using the SMK-KCK and the AES-128-CMAC algorithm. The output of the AES-128-CMAC shall be 128 bits. The MIC shall be calculated on the concatenation, in the following order, of:

- MAC_I

- MAC_P

- BSSID

- RSNIE_I

- PNonce

- INonce

- Lifetime

- DH_I

- If all checks succeed, the SMK handshake is complete from the Peer STA side.

8.5.9.2 SMK handshake error conditions

If the STA_P does not receive a valid SMK Message 3 or a 4-Way STK Message 1 after sending the EAPOL request message to initiate the TDLS Peer Key rekey within a 2000 millisecond timeout, the STA_P shall invoke a direct link teardown procedure.

If the STA_I does not receive an SMK Message 2 from the STA_P, the STA_I shall attempt dot11RSNAConfigSMKUpdateCount transmits of the SMK Handshake Message 1 plus a final timeout. If the STA_I still has not received a response after these retries, it shall invoke a direct link teardown procedure. The retransmit timeout value shall be 2000 milliseconds for the first timeout, the listen interval for the second timeout, and twice the listen interval for subsequent timeouts. If there is no listen interval, then 200 milliseconds shall be used for all timeout values.

Upon receipt of the SMK Message 3, the STA_I will transmit Message 1 of the 4-Way STK Handshake to the STA_P. If the STA_I does not receive Message 2 of the 4-Way STK Handshake from the STA_P, it shall attempt dot11RSNAConfigSMKUpdateCount transmits of 4-Way STK Handshake Message 1, plus a final timeout. If STA_I still has not received a response after these retries, it shall invoke a direct link teardown procedure. The retransmit timeout value shall be 1000 milliseconds for the first timeout, half the listen interval for the second timeout, and the listen interval for subsequent timeouts. If there is no listen interval, then 1000 milliseconds shall be used for all timeout values.

8.5.9.3 STKSA rekeying

Rekeying is always initiated by the STA_I. When needed, the STA_P sends an EAPOL request message to the STA_I to request rekeying. The STA_P shall wait a minimum of one half the IEEE 802.1X timeout after the STSL setup before initiating a TDLS Peer Key rekey procedure. To perform rekeying, there are two cases:

1) If the SMK timer has not expired, the STAs will initiate a 4-Way Handshake to create a new STK. The 4-Way Handshake is always initiated by STA_I. In this case, STA_P should not delete any existing STKSA prior to verifying Message 3 of the 4-Way Handshake with STA_I for this session.

2) If the SMK has expired, STA_I shall not use an existing STKSA and shall start the SMK Handshake followed by a 4-Way Handshake to create new keys.

The format of the EAPOL-Key request message in case 1 from STA_P to STA_I is as follows:

Request Message:	STA_P → STA_I

EAPOL-Key(1,1,0,0,1,0,0,0, MIC, PMKID KDE)

The request message uses the following values for each of the EAPOL-Key frame fields:

Descriptor Type = N - see 8.5.2

Key Information:

Key Descriptor Version = 1 (RC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128)

Key Type = 1 (PTK)

SMK Message = 0

Install = 0

Key Ack = 0

Key MIC = 1

Secure = 1

Error = 0

Request = 1

Encrypted Key Data = 0

Reserved = 0

Key Length = 0

Key Replay Counter = request replay counter of peer STA

Key Nonce = 0

EAPOL-Key IV = 0

Key RSC = 0

Key MIC = MIC computed over the body of this EAPOL-Key frame

Key Data Length = Length of Key Data field in octets

Key Data = SMKID in SMKID KDE

8.5.9.4 Error Reporting

Error reporting messages are defined in this sub-clause and used to report errors whenever STAs detect an error during the SMK Handshake.

The STA, upon receipt of the error messages defined in this sub-clause, shall tear down the direct link with the other STA and clear all the TDLS Peer Key states.

The format of EAPOL-Key request message for reporting an error message is as follows:

Error Message: EAPOL-Key(1,1,0,0,0,1,0, 0, MIC, Error KDE, MAC Address KDE).

The request message uses the following values for each of the EAPOL-Key frame fields:

Descriptor Type = N - see 8.5.2

Key Information:

Key Descriptor Version = 1 (RC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128)

Key Type = 0 (Group/SMK)

SMK Message = 1 (SMK)

Install = 0

Key Ack = 0

Key MIC = 1

Secure = 1

Error = 1

Request = 1

Encrypted Key Data = 0

Reserved = 0

Key Length = 0

Key Replay Counter = request EAPOL replay counter

Key Nonce = 0

EAPOL-Key IV = 0

Key RSC = 0

Key MIC = MIC computed over the body of this EAPOL-Key frame

Key Data Length = Length of Key Data field in octets

Key Data = Error KDE (different types defined in Table 64), MAC Address KDE

8.5.9.4.1 Error ERR_CPHR_NS

This error message is sent whenever a STA finds that it does not support any of the ciphersuites proposed by the other STA. In response to this error, the STA creates an Error KDE with error type ERR_CPHR_NS and sends the message back to the other STA. The MAC address KDE contains the MAC address of the other

STA.

8.5.8.4.4 Error ERR_NO_STSL

This error message is sent whenever a STA finds that it does not have an existing direct link with the other STA. In response of this error, the STA creates an Error KDE with error type ERR_NO_STSL and sends the message back to the other STA. The MAC address KDE contains the MAC address of the other STA.

10. Layer management

10.3 MLME SAP interface

Insert a new subclause at the end of clause 10.3 as follows:

10.3.z1 Management of tunneled direct link setup

The following MLME primitives support the signaling of tunneled direct link setup. Figure z8 depicts the TDSL setup process. The figure illustrates the basic protocol, which is only an example and is not meant to be exhaustive of all possible protocol uses.

Figure z8—Tunneled Direct Link Setup Protocol Exchange

EDITORIAL NOTE—Add TDLS Setup Confirm.

10.3.z1.1 MLME-TDLSSETUPREQUEST.request

10.3.z1.1.1 Function

This primitive requests that a TDLS Setup Request frame be sent to the candidate peer STA.

10.3.z1.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUP.request(

PeerSTAAddress,

TDLSSetupRequest

TDLSRequestTimeout

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TDLS Setup process. |
| TDLSSetupRequest | As defined in TDLS Setup Request element | As defined in TDLS Setup Request element | Specifies the proposed service parameters for the TDLS Setup. |
| TDLSRequestTimeout | Integer | ≥ 0 | Specifies a time limit (in TU) after which the TDLS Setup procedure is terminated. |

10.3.z1.1.3 When Generated

This primitive is generated by the SME to request that a TDLS Setup Request frame be sent to the candidate peer STA.

10.3.z1.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Setup Request action management frame. The STA then attempts to transmit this to the candidate peer STA.

10.3.z1.2 MLME-TDLSSETUPREQUEST.confirm

10.3.z1.2.1 Function

This primitive reports the result of a TDLS Setup procedure.

10.3.z1.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUP.confirm(

TDLSSetupResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSSetup- Response | As defined in TDLS Setup Response element | As defined in TDLS Setup Response element | Specifies service parameters for the TDLS Setup. |

10.3.z1.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-TDLSSETUP.request and indicates the results of the request.

This primitive is generated when the MLME-TDLSSETUP.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a TDLS Setup Response frame from the AP.

10.3.z1.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

10.3.z1.3 MLME-TDLSSETUPREQUEST.indication

10.3.z1.3.1 Function

This primitive indicates that a TDLS Setup Request frame was received from a candidate peer STA.

10.3.z1.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUP.indication(

PeerSTAAddress,

TDLSSetupRequest

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TDLS Setup Request frame was received. |
| TDLSSetup- Request | As defined in TDLS Setup Request element | As defined in TDLS Setup Request element | Specifies the proposed service parameters for the TDLS Setup. |

10.3.z1.3.3 When Generated

This primitive is generated by the MLME when a valid TDLS Setup Request frame is received.

10.3.z1.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.

10.3.z1.4 MLME-TDLSSETUPRESPONSE.request

10.3.z1.4.1 Function

This primitive requests that a TDLS Setup Request frame be sent to the initiating STA.

10.3.z1.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUPRESPONSE.request(

PeerSTAAddress,

TDLSSetupRequest

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TDLS Setup process. |
| TDLSSetupRequest | As defined in TDLS Setup Request element | As defined in TDLS Setup Request element | Specifies the proposed service parameters for the TDLS Setup. |
| ResultCode | Enumeration | SUCCESS, REFUSED, INVALID_ PARAMETERS, TIMEOUT |  |

10.3.z1.4.3 When Generated

This primitive is generated by the SME to request that a TDLS Setup Response frame be sent to the initiating STA.

10.3.z1.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Setup Response action management frame. The STA then attempts to transmit this to the initiating STA.

10.3.z1.5 MLME-TDLSSETUPRESPONSE.confirm

10.3.z1.5.1 Function

This primitive reports the result of a TDLS Setup procedure.

10.3.z1.5.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUPRESPONSE.confirm(

TDLSSetupResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSSetup- Response | As defined in TDLS Setup Response element | As defined in TDLS Setup Response element | Specifies service parameters for the TDLS Setup. |

10.3.z1.5.3 When Generated

This primitive is generated by the MLME as a result of an MLME-TDLSSETUP.request and indicates the results of the request.

This primitive is generated when the MLME-TDLSSETUP.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a TDLS Setup Response frame from the AP.

10.3.z1.5.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

10.3.z1.6 MLME-TDLSSETUPRESPONSE.indication

10.3.z1.6.1 Function

This primitive indicates that a TDLS Setup Request frame was received from a candidate peer STA.

10.3.z1.6.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUPRESPONSE.indication(

PeerSTAAddress,

TDLSSetupResponse

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TDLS Setup Request frame was received. |
| TDLSSetup- Request | As defined in TDLS Setup Request element | As defined in TDLS Setup Response element | Specifies the proposed service parameters for the TDLS Setup. |
| ResultCode | Enumeration | SUCCESS, REFUSED, INVALID_ PARAMETERS, TIMEOUT |  |

10.3.z1.6.3 When Generated

This primitive is generated by the MLME when a valid TDLS Setup Response frame is received.

10.3.z1.6.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.

Add TDLS Teardown

11. MLME

Insert the following after 11.2.1.11:

11.2.1.12 Power Management with TDLS

Peer STAs may be capable to use the direct link for data transmission when one of them is in peer power save mode. Support for Peer PSM and operation in Peer PSM are two different optional capabilities, Peer PSM AP mode and Peer PSM client mode, correspondingly.

When a peer of a STA supports the Peer PSM AP mode, a STA supporting the Peer PSM client mode can enter the Peer PSM by sending a frame with the power management bit set. Such a STA is called a STA in Peer PSM client mode. The Peer PSM client mode may be supported when dot11PeerPSMClientModeImplemented is true.

When a peer of a STA is in the Peer PSM, the STA shall buffer MSDUs for the peer locally without sending them to the access point and deliver the buffered MSDUs over the direct link. Such a STA is referred to as a STA in Peer PSM AP mode. The Peer PSM AP mode may be supported when dot11PeerPSMAPModeImplemented is true. A STA which supports Peer PSM AP mode shall signal this capability through the use of the Peer PSM AP mode subfield in the Extended Capabilities Information field in TDLS Setup Request and Response frames. The STA in Peer PSM AP mode shall be in active mode.

The behavior of a STA in Peer PSM AP mode with respect to traffic buffered for a peer STA in client mode is the same as a regular U-APSD AP, with the following exception. When traffic becomes available for an AC and no Service Period has occurred for that AC for a period of Peer PSM Indication Window prior to the arrival of the new traffic, the STA in AP mode sends a unicast Peer Traffic Indication frame to the peer STA in client mode through the access point, indicating the AC(s) for which traffic is available.

The behavior of a STA in Peer PSM client mode with respect to traffic buffered at the peer STA in AP mode is the same as a regular U-APSD STA, with the following exception. The indication that traffic is buffered at the peer STA in AP mode is obtained from received Peer Traffic Indication frames from the peer STA in AP mode. To reduce the number of Peer Traffic Indication frames transmitted for a continuous uni-directional traffic stream without return traffic, a new Service Period may be started within Peer PSM Indication Window after the occurrence of a Service Interval during which a Data MPDU was received.

A STA in Peer PSM client mode may configure a STA in Peer PSM AP mode to use U-APSD using two methods. First, a STA in Peer PSM client mode may set individual U-APSD Flag bits in the QoS Info subfield of the QoS Capability element carried in TDLS Setup Request and Response frames. Alternatively, a STA in Peer PSM client mode may designate one or more AC as trigger-enabled and one or more AC as delivery-enabled by sending an ADDTS Request frame per AC to the STA in Peer PSM AP mode.

The Peer PSM Indication Window field is optionally present in the Peer Traffic Indication frame. The default Peer PSM Indication Window is equal to the DTIM interval.

Insert a new subclause at the end of clause 11 as follows:

11.20 Tunneled Direct Link Setup

Tunneled Direct Link Setup (TDLS) is characterized by encapsulating signaling frames in Data frames, which allows them to be transmitted through an access point transparently. Therefore, the access point does not need to be direct link aware, nor does it have to support any of the capabilities that will be used on the direct link. TDLS also includes an option either to enter Peer Power Save Mode (Peer PSM) on the direct link or to suspend receiving over the direct link, so that the station can enter a power save mode.

The following terminology is used in this section:

- Initiator STA: STA which transmits a TDLS Setup Request

- Peer STA: STA which is involved in a direct link or a direct link establishment

A direct link transmission is a transmission in which the A1 field of the MAC header is set to the MAC address of the Peer STA.

A non-AP STA may act as Initiator STA or Peer STA when dot11TunneledDirectLinkSetupImplemented is true.

TDLS frames (as defined in 7.2.2.3) shall be transmitted via the AP (i.e. the A1 field of the MAC header contains the BSSID). TDLS frames should be transmitted at UP 7 (AC_VO).

A STA may transmit a Link RCPI Measurement Request to an (intended) peer STA to obtain an indication of the RCPI values at the peer STA. The Link RCPI measurement request and report are sent to the peer STA directly. The RCPI information may be used to decide whether to switch over to a direct link for communication with the peer STA.

To establish a direct link, the initiator STA shall send a TDLS Setup Request to the intended peer STA. If the peer STA accepts the direct link, it shall respond with a TDLS Setup Response frame with status code 0 (Successful). If the peer STA does not accept the direct link, it shall respond with a TDLS Setup Response with a status code other than 0. If there is no response within dot11DLSResponseTimeout, the initiator STA shall terminate the setup procedure. Otherwise, the initiator STA shall send a TDLS Setup Confirm to the peer STA to confirm the receipt of the TDLS Setup Response.

RSNIE, SMK Message FTIE and DH IE shall not be included in a TDLS setup message when the STA has no RSNA with the AP. If security is required, the TDLS setup messages shall include the SMK handshake. After a successful SMK handshake, the initiator STA further prepares the direct link for Data transmissions by starting the 4-way Handshake.

When the TDLS Setup Handshake has been completed, both STAs shall accept Data frames received over the direct link.

After transmitting a last Data frame through the AP path and before transmitting the first Data frame over the direct link, a STA may send a TDLS DL Path Switch Request. The first Data frame transmitted over the direct link should be transmitted after receipt of the TDLS DL Path Switch Response. This avoids potential reordering of frames between the AP path and the direct link. The STA may also use a message exchange that is part of the direct link establishment or the peer key handshake for this purpose (i.e. any request/response type message exchange between the Peer STAs and through the AP can be used to ensure that no data is in flight between the Peer STAs before switching over the the direct link). The first Data frame may be transmitted after some delay after receiving the TDLS Path Switch Response, to account for potential EDCA related reordering inside the AP, when frames have been transmitted for multiple ACs. The magnitude of this delay (if deemed required) is implementation specific. This delay is absent by default.

When a STA intends to disable its direct Rx path, for instance to suspend receiving over the direct link, it sends a TDLS AP Path Switch Request to the peer STA. Upon receipt of a TDLS AP Path Switch Request, the receiving STA shall cease queueing new MPDUs for transmission over the direct link. When no further Data frames will be transmitted over the direct link, the responding STA shall send a TDLS AP Path Switch Response. The requesting STA shall not enter a power-saving mode until the TDLS AP Path Switch Response frame has been received..

A STA may request a peer STA to enable its direct Rx path by sending a TDLS DL Path Switch Request. Upon receipt of the TDLS DL Path Switch Request, the receiving STA may enable the direct Rx path and respond with a TDLS DL Path Switch Response. The requesting station shall not transmit frames on the direct link before receiving a TDLS DL Path Switch Response indicating that the Rx path was enabled (reason code 0).

When a STA intends to enter a power save mode, it may suspend the direct link by shutting down the direct Rx path, or enter the Peer PSM by sending a frame with the power management bit set. See 11.2.1.12.

To tear down a direct link, the STA sends a TDLS Teardown Request to the peer STA, after which the STA shall no longer transmit on the direct link. Upon receipt of the TDLS Teardown Request, the peer STA shall disable the direct Rx and Tx paths, destroy the related security parameters, and then respond with a TDLS Teardown Response. Upon receipt of the TDLS Teardown Response, the STA that initiated the teardown shall disable the direct Rx path and destroy the related security parameters.

To obtain adequate protection against hidden nodes, a Peer STA may precede a direct link transmission with an RTS/CTS exchange with the AP.

TDLS frames shall be sent through the AP, which means that the A1 field is set to the BSSID.

If channel switching is supported (dot11TDLSChannelSwitchingImplemented is set to True), the STA sets the TDLS Channel Switching capability bit to 1 and includes a Supported Regulatory Classes element in TDLS Setup Request and Response frames.

The target channel is determined by the STA which initiates a channel switch, from the set of regulatory classes supported by both peer STAs. The target channel and regulatory class are specified in the TDLS Channel Switch Request frame. The Country and Coverage Class settings on the target channel are the same as in the BSS to which both peer STAs are currently associated. Both STAs are entitled to request for a channel switch.

The events occurring for a channel switch are:

- backoff – TDLS Channel Switch Request – SIFS – ACK1 – […] – backoff – TDLS Channel Switch Response – SIFS – ACK2 – SwitchTime | ProbeTime – backoff – first transmission

Both STAs shall be listening on the new channel not later than SwitchTime after the end of ACK2 (this instance is marked as “|” in the above frame exchange sequence). After switching to the new channel, a STA shall perform a clear channel assessment (CCA) until a frame sequence is detected by which it can correctly set its NAV, or until a period of time equal to at least ProbeTime has transpired (this combined event is indicated as “ProbeTime” in the above frame exchange sequence). A first transmission on the new channel shall be preceded by a random backoff, which shall start at the end of the ProbeTime. If no successful frame exchange has occurred on the off-channel within SwitchTimeout after the end of ACK2, the stations shall go back to the base channel. The values for SwitchTime, ProbeTime and SwitchTimeout are specified in Table z12.

Table z12—Parameter Values

| Parameter | Value |
| --- | --- |
| SwitchTime | ≤ 100 us |
| ProbeTime | ≥ 100 us |
| SwitchTimeout | 250 us |

Before switching to an off-channel (i.e. a channel which is not the base channel), the stations shall be in PS mode with the AP. To this end, a STA which receives a TDLS Channel Switch Request may enter PS mode with the AP prior to sending the TDLS Channel Switch Response. This possible event is indicated with “[…]” in the above frame exchange sequence.

Because there is a backoff between the request and response, there is a (small) probability that two stations issue a TDLS Channel Switch Request at more or less the same time. To reduce the probability for this event to occur, a pending TDLS Channel Switch Request should be cancelled when a TDLS Channel Switch Request is received. When two TDLS Channel Switch Requests still cross, then both TDLS Channel Switch Responses will be executed sequentially if they contain result code 0 (accepted). If a TDLS Channel Switch Response does not imply a channel switch (because the stations already are on the requested channel), then there is no prior switch and ProbeTime and both stations simply continue what they were doing on the channel.

If the base channel is a DFS channel, then STAs should take care to reside on the base channel enough to receive channel switch announcements by the AP.

On the off-channel, the channel access rules which govern the off-channel shall be observed at any time, including possible DFS, TPC and radar scanning requirements (see below).

Switching back to the base channel shall always be accepted, because the base channel is the only channel at which communications with the AP can take place.

A TDLS Path Switch Request frame and a TDLS Teardown Request frame shall not be transmitted on the off-channel. This avoids that such frames are implicitly also a Channel Switch request, which would potentially complicate their behavior.

TDLS channel switching shall not be invoked for coverage classes other than 0. This effectively limits channel switching to indoor use only.

General Behavior

When active on an off-channel, peer stations shall respond to received Probe Requests with a Probe Response indicating “off-channel” as the SSID. The timestamp may contain a random value, while other contents of the Probe Response may be copied from the APs beacon or probe response.

If the off-channel direct link is a 40 MHz direct link, the Probe response shall contain an HT Capabilities element with the Supported channel width set field set to 1, the Channel width field set to 1 and the Forty MHz Intolerant field set to 0, and an HT Information element with the Secondary Channel Offset field set to SCA or SCA, depending on the location of the secondary channel.

The off-channel may be changed for at least 60 minutes when Data frames with a TA other than the peer STA have been received on the off-channel, either during a scan or when active on the off-channel.

If dot11SpectrumManagementRequired is true, a peer STA shall not transmit in an off-channel where radar detection is required, unless the channel has been tested for the presence of radars according to regulatory requirements. If a peer STA which is operating in such a channel detects radar, the peer STA shall discontinue transmissions according to regulatory requirements, and it should send a TDLS Channel Switch Request indicating a switch to the base channel. The channel switch is advised because it avoids an interruption on the direct link.

Setting up a 40 MHz direct link

A 40 MHz off-channel direct link may be started if both peer STAs indicated 40 MHz support in the Supported Channel Width Set field of the HT Capabilities element (which is included in the TDLS Setup Request frame and the TDLS Setup Response frame).

Switching to a 40 MHz off-channel direct link is achieved by including the following information in the TDLS Channel Switch Request:

- Regulatory Class element indicating 40 MHz Channel Spacing

- Secondary Channel Offset element indicating SCA or SCB

The regulatory class shall not have a value of 2.407 GHz for the Channel starting frequency.

Basic 40 MHz functionality

On a 40 MHz direct link, peer STAs may transmit Clause 20 PPDUs using CH_BANDWIDTH = HT_CBW40. A peer STA shall not transmit a 20 MHz PPDU in the secondary channel of its 40 MHz direct link.

Channel selection for a 40 MHz direct link

If a peer STA chooses to start a 40 MHz direct link that occupies the same two channels as an existing 40 MHz network (i.e. a 20/40 MHz BSSs or a 40 MHz direct link), then it shall ensure that the primary and secondary channels of the new direct link are identical to the primary and secondary channels of the existing 40 MHz network, unless the peer STA discovers that on these two channels there are existing 40 MHz networks with different primary and secondary channels.

If a peer STA chooses to start a 40 MHz direct link, the selected secondary channel should correspond to a channel on which no beacons are detected.

Switching from a 40 MHz to a 20 MHz direct link

Switching from a 40 MHz direct link to a 20 MHz direct link is established through a TDLS channel switch. When on a 40 MHz direct link, a requested switch to a 20 MHz direct link shall always be accepted.

CCA sensing and NAV assertion in a 40 MHz direct link

When active on a 40 MHz direct link, the peer STAs shall follow the CCA rules as defined in 11.14.9 and the NAV rules as defined in 11.14.10.

Annex A

A.4 PICS proforma–IEEE Std. 802.11, 2007 Edition

A.4.3 IUT configuration

Insert the following entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFz | Is Tunneled Direct Link Setup supported? |  | O | Yes ℃ No ℃ N/A ℃ |

Add a new clause at the end of A.4.3 as follows:

A.4.z1 Tunneled Direct Link Setup extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TDLS1 | Tunneled Direct Link Setup | 7.2.2.3, 11.20 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.1 | TDLS Setup | 7.2.2.3.1, 7.2.2.3.2 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.2 | TDLS Teardown | 7.2.2.3.3, 7.2.2.3.4 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.3 | TDLS DL Path Switch | 7.2.2.3.5, 7.2.2.3.6 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.4 | TDLS AP Path Switch | 7.2.2.3.7, 7.2.2.3.8 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.5 | TDLS Peer Key Handshake | 8.5.9 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.6 | Link RCPI Request/Report | 7.3.2.21.11, 7.3.2.22.11 | CFz:M | Yes ℃ No ℃ N/A ℃ |
| TDLS1.7 | Peer PSM (AP mode) | 7.2.2.3.10 | CFz:O | Yes ℃ No ℃ N/A ℃ |
| TDLS1.8 | Peer PSM (client mode) | 7.2.2.3.10 | CFz:O | Yes ℃ No ℃ N/A ℃ |
| TDLS1.9 | TDLS Channel Switching | 7.2.2.3.11, 7.2.2.3.12 | CFz:O | Yes ℃ No ℃ N/A ℃ |

Annex D (normative) ASN.1 encoding of the MAC and PHY MIB

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID	MacAddress,

dot11MediumOccupancyLimit	INTEGER,

dot11CFPollable	TruthValue,

dot11CFPeriod	INTEGER,

dot11CFPMaxDuration	INTEGER,

dot11AuthenticationResponseTimeOut	Unsigned32,

dot11PrivacyOptionImplemented	TruthValue,

dot11PowerManagementMode	INTEGER,

dot11DesiredSSID	OCTET STRING,

dot11DesiredBSSType	INTEGER,

dot11OperationalRateSet	OCTET STRING,

dot11BeaconPeriod	INTEGER,

dot11DTIMPeriod	INTEGER,

dot11AssociationResponseTimeOut	Unsigned32,

dot11DisassociateReason	INTEGER,

dot11DisassociateStation	MacAddress,

dot11DeauthenticateReason	INTEGER,

dot11DeauthenticateStation	MacAddress,

dot11AuthenticateFailStatus	INTEGER,

dot11AuthenticateFailStation	MacAddress,

dot11MultiDomainCapabilityImplemented	TruthValue,

dot11MultiDomainCapabilityEnabled	TruthValue,

dot11CountryString	OCTET STRING,

dot11SpectrumManagementImplemented	TruthValue,

dot11SpectrumManagementRequired	TruthValue,

dot11RSNAOptionImplemented	TruthValue,

dot11RSNAPreauthenticationImplemented	TruthValue,

dot11RegulatoryClassesImplemented	TruthValue,

dot11RegulatoryClassesRequired	TruthValue,

dot11QosOptionImplemented	TruthValue,

dot11ImmediateBlockAckOptionImplemented	TruthValue,

dot11DelayedBlockAckOptionImplemented	TruthValue,

dot11DirectOptionImplemented	TruthValue,

dot11APSDOptionImplemented	TruthValue,

dot11QAckOptionImplemented	TruthValue,

dot11QBSSLoadOptionImplemented	TruthValue,

dot11QueueRequestOptionImplemented	TruthValue,

dot11TXOPRequestOptionImplemented	TruthValue,

dot11MoreDataAckOptionImplemented	TruthValue,

dot11AssociatedinNQBSS	TruthValue,

dot11DLSAllowdInQBSS	TruthValue,

dot11DLSAllowed	TruthValue,

dot11AssociateStation	MacAddress,

dot11AssociateID	INTEGER,

dot11AssociateFailStation	MacAddress,

dot11AssociateFailStatus	INTEGER,

dot11ReassociateStation	MacAddress,

dot11ReassociateID	INTEGER,

dot11ReassociateFailStation	MacAddress,

dot11ReassociateFailStatus	INTEGER,

dot11RadioMeasurementCapable	TruthValue,

dot11RadioMeasurementEnabled	TruthValue,

dot11RRMMeasurementProbeDelay	INTEGER,

dot11RRMMeasurementPilotPeriod	INTEGER,

dot11RRMLinkMeasurementEnabled	TruthValue,

dot11RRMNeighborReportEnabled	TruthValue,

dot11RRMParallelMeasurementsEnabled	TruthValue,

dot11RRMRepeatedMeasurementsEnabled	TruthValue,

dot11RRMBeaconPassiveMeasurementEnabled	TruthValue,

dot11RRMBeaconActiveMeasurementEnabled	TruthValue,

dot11RRMBeaconTableMeasurementEnabled	TruthValue,

dot11RRMBeaconMeasurementReportingConditionsEnabled	TruthValue,

dot11RRMFrameMeasurementEnabled	TruthValue,

dot11RRMChannelLoadMeasurementEnabled	TruthValue,

dot11RRMNoiseHistogramMeasurementEnabled	TruthValue

dot11RRMStatisticsMeasaurementEnabled	TruthValue,

dot11RRMLCIMeasurementEnabled	TruthValue,

dot11RRMLCIAzimuthEnabled	TruthValue,

dot11RRMTransmitStreamCategoryMeasurementEnabled	TruthValue,

dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled

TruthValue,

dot11RRMAPChannelReportEnabled	TruthValue,

dot11RRMMIBEnabled	TruthValue,

dot11RRMMaxMeasurementDuration	Unsigned32,

dot11RRMNonOperatingChannelMaxMeasurementDuration	Unsigned32,

dot11RRMMeasurementPilotTransmissionInformationEnabled TruthValue,

dot11RRMMeasurementPilotCapability	Unsigned32,

dot11RRMNeighborReportTSFOffsetEnabled	TruthValue,

dot11RRMRCPIMeasurementEnabled	TruthValue,

dot11RRMRSNIMeasurementEnabled	TruthValue,

dot11RRMBSSAverageAccessDelayEnabled	TruthValue,

dot11RRMBSSAvailableAdmissionCapacityEnabled	TruthValue,

dot11RRMAntennaInformationEnabled	TruthValue,

dot11FastBSSTransitionImplemented	TruthValue

dot11LCIDSEImplemented	TruthValue,

dot11LCIDSERequired	TruthValue,

dot11DSERequired	TruthValue,

dot11ExtendedChannelSwitchEnabled	TruthValue,

dot11RSNAProtectedManagementFramesEnabled	TruthValue,

dot11RSNAUnprotectedManagementFramesAllowed	TruthValue,

dot11AssociationPingResponseTimeout	Unsigned32,

dot11AssociationMaximumPingAttempts	INTEGER,

dot11HighThroughputOptionImplemented	TruthValue,

dot11InterworkingServiceImplemented	TruthValue,

dot11InterworkingServiceEnabled	TruthValue,

dot11QosmapImplemented	TruthValue,

dot11QosmapEnabled	TruthValue,

dot11EbrImplemented	TruthValue,

dot11EbrEnabled	TruthValue,

dot11GasTimPeriod	INTEGER,

dot11GasTimTimeToSuspend	INTEGER,

dot11ESNetwork	TruthValue,

dot11InterworkingQosmapImplemented	TruthValue,

dot11InterworkingEbrImplemented	TruthValue,

dot11GasMulticastRepititions	INTEGER,

dot11GasResponseBufferingTime	INTEGER,

dot11TunneledDirectLinkSetupImplemented	TruthValue,

dot11PeerPSMAPModeImplemented	TruthValue,

dot11PeerPSMClientModeImplemented 	TruthValue,

dot11PeerPSMIndicationWindow	INTEGER,

dot11TDLSChannelSwitchingImplemented	TruthValue

}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11TunneledDirectLinkSetupImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Tunneled Direct

Link Setup.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry <ANA> }

dot11PeerPSMAPModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting Peer PSM AP mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11PeerPSMClientModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting Peer PSM client mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11PeerPSMIndicationWindow OBJECT-TYPE

SYNTAX INTEGER (1...256)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the minimum interval in Beacon Intervals

between successive Peer Traffic Indication frames.

This value is transmitted in the Peer Traffic Indication frame.

The default value of this attribute is equal to one DTIM Interval.”

::= { dot11StationConfigEntry <ANA>}

dot11TDLSChannelSwitchingImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Channel Switching.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}