IEEE P802.11
Wireless LANs

| Proposed Resolutions for Coex CIDs on OBSS Non-HT STAs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 W. Tasman Dr, San Jose, CA 95134 USA | +1-408-527-9344 | dougchan@cisco.com |

Abstract

This document contains proposed resolutions to LB129 comments: 7291, 7293, 7295, 7296, 8106, 8107, and 8087.

Editor instructions are based on D5.02.

Note that changes from r0 to r1 are marked by green highlights.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Proposed resolutions for CIDs

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 8107 | Ji, Lusheng | 114.42 | 9.13.3.4 | This note is well intentioned but its language is too weak to be effective. | Make this note part of the specification using normative language, such as "If a non-HT device is detected and the STA determines that its HT-greenfield format or RIFS sequence transmissions are affecting the operation of the non-HT device, then the STA shall enable protection of its HT-greenfield format and RIFS sequence transmissions. | Reject. TGn believes it is sufficient to let the invocation of this protection mechanism be optional because a mandatory behavior may impose unnecessary constraints for certain scenarios, which could lead to sub-optimal performance. Therefore, we should leave the decision on when to invoke this protection to the implementers. |
| 8106 | Ji, Lusheng | 114.42 | 9.13.3.4 | No method defined for how a STA "determines that its HT-greenfield format or RIFS sequence transmissions". | either define such a mechanism, or change the "may" on line 39 to "shall" | Reject. There are many ways to determine whether a STA’s HT-greenfield and RIFS packets are affecting a non-HT device. However, specificying such methods are beyond the scope of an informative note. (And see resolution to CID 8107 for reason of not changing to “shall”.) |
| 8087 | Stephens, Adrian | 144.42 | 9.13.3.4 | "NOTE--If a non-HT device is detected and the STA determines that its HT-greenfield format or RIFS sequence transmissions are affecting the operation of the non-HT device, then the STA is advised to enable protection of its HT-greenfield format and RIFS sequence transmissions." The "is advised" sounds like this should be normative text with a "should", but due to a last minute compromise was forced into a note. Otherwise, who provides the advice? when? Passive voice is considered harmful. I am advised that Notes should not advise - they can only provide additional explanatory material. | Replace "is advised to" with "can". | Accept. |
| 7291 | Marshall, Bill | 144.23 | 9.13.3.4 | the recommendation at line 43 "is advised to" is more consistent with a normative "should" than a simple "may". | upgrade the "may" to "should" | Counter. With the change accepted for CID 8087, the tone of the note at line 43 is no longer normative but more informative now. So it is consistent with keep this “may” as is. |
| 7293 | Marshall, Bill | 144.26 | 9.13.3.4 | the recommendation at line 43 "is advised to" is more consistent with a normative "should" than a simple "may". | upgrade the "may" to "should" | Counter. With the change accepted for CID 8087, the tone of the note at line 43 is no longer normative but more informative now. So it is consistent with keep this “may” as is. |
| 7295 | Marshall, Bill | 144.39 | 9.13.3.4 | the recommendation at line 43 "is advised to" is more consistent with a normative "should" than a simple "may". | upgrade the "may" to "should" | With the change accepted for CID 8087, the tone of the note at line 43 is no longer normative but more informative now. So it is consistent with keep this “may” as is. |
| 7296 | Marshall, Bill | 144.42 | 9.13.3.4 | the text of this NOTE is inconsistent with the normative behavior given in this subclause | make the normative behavior match the NOTE, rather than make the NOTE match the normative behavior | Counter. With the change accepted for CID 8087, the tone of the note at line 43 is no longer normative but more informative now. As a result, the note is now consistent with the behavior in this subclause. |

# CID 8087

TGn Editor: make the following changes to the paragraph at line 2 on page 153:

NOTE—If a non-HT device is detected and the STA determines that its HT-greenfield format or RIFS sequence transmissions are affecting the operation of the non-HT device, then the STA is advised tocan enable protection of its HT-greenfield format and RIFS sequence transmissions.