IEEE P802.11
Wireless LANs

| Proposed Resolutions for LB 129 PHY CIDs in the Intro, MCS and Spec groups |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 W. Tasman Dr, San Jose, CA 95134 USA | +1-408-527-9344 | dougchan@cisco.com |

Abstract

This document contains proposed resolutions to LB129 comments: 7047, 7058, 7064, 7065, 7468, and 7539.

Editor instructions are based on D5.02.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Proposed resolutions for CIDs

PHY Intro group:

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 7047 | Chu, Liwen | 241.17 | 20.1.1 | Shall a HT STA be capable of transmitting and receiving frames that are compliant with mandatory PHY specifications as defined in clause 17 in a band different from 5GHz (11y band)? If yes, modify the draft. If no, how does a 11y STA implement the 11n feature? | Clarify it. | Reject. 11n's intention is only to specify the PHY behavior in the 2.4 and 5 GHz bands, as reflected in the 11n PAR. |
| 7468 | Marshall, Bill | 242.48 | 20.1.4 | normative statement needed here | change "determines" to "shall determine" | Reject. The usage in Clause 20 is consistent with the usage in Clauses 17 and 19, where the normative effect of the TXVECTOR parameters is implicit. |

PHY Specificaions group:

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 7058 | Ecclesine, Peter | 317.64 | 20.3.21.7.4 | The term "I/Q arm" is used but not defined. | Explain what "I/Q arm" means somewhere or rewrite the sentence. | Reject. The term "I/Q arm" is a common technical term that implementers should be familiar with, like the terms "amplitude" or "dc offsets" that follow it in this sentence. Furthermore, "I/Q arm" has been mentioned in the 802.11-2007 text (top of p. 616) without its definition provided. |
| 7064 | Erceg, Vinko | 314.45 | 20.3.21.1 | In the sentence ".. at 20 MHz frequency offset and the maximum of -45 dBr or -53 dBm/MHz at 30 MHz frequency offset and above." it is more accurate to say "and" instead of "or". Same was done in TGy draft. | Please the change sentence to "..at 20 MHz frequency offset and the maximum of -45 dBr and -53 dBm/MHz at 30 MHz frequency offset and above." | Accept. |
| 7065 | Erceg, Vinko | 315.27 | 20.3.21.1 | In the sentence ".., -28 dBr at 40 MHz offset and the maximum of -45 dBr or -56 dBm/MHz at 60 MHz frequency offset and above." it is more accurate to say "and" instead of "or". Same was done in TGy draft. | Please change the sentence to ".., -28 dBr at 40 MHz offset and the maximum of -45 dBr and -56 dBm/MHz at 60 MHz frequency offset and above." | Accept. |

PHY MCS group:

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 7539 | Marshall, Bill | 345.30 | 20.6 | these tables need to be normative for the rate dependent parameters | change to "The rate dependent parameters for … shall be as shown in Table 20-29." Similar change for tables through 30-43. | Accept. |