IEEE P802.11
Wireless LANs

| 802.11 TGn LB129 Submission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

- This document contains proposed changes to the IEEE P802.11n Draft to address comments related to the operation of the signal extension.

It is necessary to change the PLCP state machine description to include the operation of the signal extension.  The edits proposed in this document achieve this.

- PLCP transmit procedure

Submission note – ignore the “Error! Reference source not found” – these are an artifact of the cut and paste from frame to word.

Change 20.3.24 as follows:

There are three options for transmit PLCP procedure. The first two options, for which typical transmit procedures are shown in PLCP transmit procedure (HT-mixed and PLCP transmit procedure (HT-greenfield, are selected if the FORMAT field of PHY-TXSTART.request(TXVECTOR) is set to HT_MF or HT_GF, respectively. These transmit procedures do not describe the operation of optional features, such as LDPC or STBC (#5263). The third option is to follow the transmit procedure as in Clause 17 or Clause 19 if the FORMAT field is set to NON_HT. Additionally, if the FORMAT field is set to NON_HT, CH_BANDWIDTH(# 2952) indicates NON_HT_CBW20, and NON_HT_MODULATION indicates OFDM, follow the transmit procedure as in Clause 17. If the FORMAT field is set to NON_HT, CH_BANDWIDTH indicates NON_HT_CBW20, and NON_HT_MODULATION indicates other than OFDM, follow the transmit procedure as in Clause 19. And furthermore, if the FORMAT field is set to NON_HT and CH_BANDWIDTH indicates NON_HT_CBW40, follow the transmit procedure as in Clause 17, except that the signal in Clause 17 is generated simultaneously on each of the upper and lower 20 MHz channels that comprise the 40 MHz channel as defined in Error: Reference source not found and Error: Reference source not found(#773). In all these options, in order to transmit data, PHY-TXSTART.request shall be enabled so that the PHY entity shall be in the transmit state. Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME, as specified in Error: Reference source not found(#2752). Other transmit parameters, such as MCS Coding types and transmit(# 1976) power, are set via the PHY-SAP with the PHY-TXSTART.request(TXVECTOR), as described in Error: Reference source not found.

A clear channel shall be indicated by PHY-CCA.indication(IDLE). Note - under some circumstances, the MAC uses the latest value of PHY-CCA.indication(#2753) before issuing the PHY-TXSTART.request. Transmission of the PPDU shall be initiated after receiving the PHYTXSTART.request(TXVECTOR) primitive. The TXVECTOR elements for the PHY-TXSTART.request are specified in Error: Reference source not found.

The PLCP shall issue the parameters in the following PMD primitives to configure the PHY:

- PMD_TXPWRLVL

- PMD_TX_PARAMETERS

- (#2754, 11-07/554r2)

The PLCP shall then issue a PMD_TXSTART.request, and transmission of the PLCP preamble may start, based on the parameters passed in the PHY-TXSTART.request primitive. The data shall then be exchanged between the MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the MAC, and PHY-DATA.confirm primitives issued by the PHY. Once PLCP preamble transmission is started, the PHY entity shall immediately initiate data scrambling and data encoding. The encoding method shall be based on the FEC_CODING(#1810), CH_BANDWIDTH, and MCS parameter of the TXVECTOR. A modulation rate change, if any, shall be initiated starting with the SERVICE field data, as described in Error: Reference source not found.

The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The SERVICE field and PSDU are encoded by the encoder selected by the FEC_CODING(#1810), CH_BANDWIDTH, and MCS parameters of the TXVECTOR as described in Error: Reference source not found. At the PMD layer, the data octets are sent in bit 0–7 order and presented to the PHY through PMD_DATA.request primitives. Transmission can be prematurely terminated by the MAC through the primitive PHY-TXEND.request. PHY-TXSTART shall be disabled by receiving a (#5784) PHY-TXEND.request. Normal termination occurs after the transmission of the final bit of the last PSDU octet, according to the number supplied in the LENGTH field.

The packet transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHYTXSTART shall be disabled). Each PHY-TXEND.request is acknowledged with a PHY-TXEND.confirm primitive from the PHY. If the length of the coded PSDU (C-PSDU) is not an integral(#8066) multiple of the OFDM symbol length, bits shall be stuffed to make the C-PSDU length an integral(#8067) multiple of the OFDM symbol length.(# 2757)

In the PMD, the GI or short GI shall be inserted in every OFDM symbol as a countermeasure against delay spread.

In some PPDU formats (as defined in 20.3.2 (PLCP frame format)), a signal extension is present. When no signal extension is present, the PHY-TXEND.confirm is generated at the end of last symbol of the PPDU.  When present, the PHY-TXEND.confirm is generated at the end of the signal extension.

A typical state machine implementation of the transmit PLCP is provided in PLCP transmit state machine. Requests (.req) and confirmations(.confirm) are issued once per state as shown. This state machine does not describe the operation of optional features, such as LDPC or STBC.

Replace figure 20-20 with the following.  The changes comprise the addition of the “signal extension” box,  the moving of the PHY-TXEND.confirm to align with the end of the signal extension and the addition of two dotted lines.

- PLCP transmit procedure (HT-mixed (#5400) format PPDU)(# 3200,#2754)

Replace figure 20-21 with the following.  The changes comprise the addition of the “signal extension” box,  the moving of the PHY-TXEND.confirm to align with the end of the signal extension and the addition of two dotted lines.

- PLCP transmit procedure (HT-greenfield (#5400) format PPDU)(#3201,#2754)

Replace figure 20-22 with the following.  The changes comprise the addition of the Add Signal Extension state,  two arrows and two labels connecting this state.

- PLCP transmit state machine(#507, 5073)

- PLCP receive procedure

Change 20.3.24 as follows:

Typical PLCP receive procedures are shown in PLCP receive procedure for HT-mixed and PLCP receive procedure for HT-greenfield. The receive procedures correspond to HT-mixed (#5400) format and HT-greenfield (#5400) format, respectively. A typical state machine implementation of the receive PLCP is given in PLCP receive state machine. These receive procedures and state machine do not describe the operation of optional features, such as LDPC or STBC. If the detected format indicates a non-HT PPDU (#6119) format, refer to the receive procedure and state machine in Clause 17 or Clause 19.(#2759--sentence deleted) Further, through station management (via the PLME) the PHY is set to the appropriate frequency, as specified in Error: Reference source not found(#2760). Other receive parameters, such as RSSI and indicated DATARATE, may be accessed via the PHY-SAP.

Upon receiving the transmitted PLCP preamble, PMD_RSSI.indication shall report a receive signal strength (#5354) to the PLCP. This indicates activity to the MAC via PHY-CCA.indication. PHY-CCA.indication(BUSY, channel-list (#2470)) shall also be issued as an initial indication of reception of a signal(#2762). The channel-list parameter of the PHY-CCA.indication is determined as follows: (#2470)

- It is absent when the operating channel width is 20 MHz (#6079)

- It is set to {primary} when the operating channel width is 40 MHz (#6079) and the signal is present only in the primary channel

- It is set to {secondary} when the operating channel width is 40 MHz (#6079) and the signal is present only in the secondary channel

- It is set to {primary, secondary} when the operating channel width is 40 MHz (#6079) and the signal is present in both the primary and the secondary channels.

The PMD primitive PMD_RSSI is issued to update the RSSI and parameter reported to the MAC.

After the(# 2764) PHY-CCA.indication(BUSY, channel-list (#2470)) is issued, the PHY entity shall begin receiving the training symbols and searching for SIGNAL and HT-SIG in order to set the length of the data stream, the demodulation type, code type, and the decoding rate. If signal loss occurs before validating L-SIG and/or HT-SIG, the HT PHY shall maintain PHY-CCA.indication(BUSY, channel-list (#2470)) until the received level drops below the CCA sensitivity level (for a missed preamble) specified in Error: Reference source not found(# 2918). If the check of the HT-SIG CRC is not valid, a PHY-RXSTART.indication is not issued. The PHY shall issue the error condition PHY-RXEND.indication(FormatViolation). The HT PHY shall maintain PHY-CCA.indication(BUSY, channel-list (#2470)) until the received level drops below the CCA sensitivity level (for a missed preamble) specified in Error: Reference source not found(# 2918).

If the PLCP preamble reception is successful and a valid HT-SIG CRC is indicated:

- Upon reception of an HT-mixed (#5400) format preamble, the HT PHY shall maintain PHY-CCA.indication(BUSY, channel-list (#2470)) for the predicted duration of the transmitted frame, as defined by(# 2765) TXTIME in Error: Reference source not found(# 3256), for all supported and unsupported modes except Reserved HT-SIG Indication. Reserved HT-SIG Indication is defined in the fourth bullet below.(# 2766)

- Upon reception of a GF preamble by an HT STA that (#2189) does not support GF, PHY-CCA.indication(BUSY, channel-list (#2470)) shall be maintained until either the predicted duration of the packet from the contents of the HT-SIG field, as defined by(# 2765) TXTIME in Error: Reference source not found(# 3256), except Reserved HT-SIG Indication, or until the received level drops below the receiver minimum sensitivity level of BPSK, R=1/2 in Error: Reference source not found + 10 dB (–72 dBm for 20 MHz, –69 dBm for 40 MHz). Reserved HT-SIG Indication is defined in the fourth bullet below.(# 2766)

- Upon reception of a GF preamble by an HT STA that (#2189) supports GF, the HT PHY shall maintain PHY-CCA.indication(BUSY, channel-list (#2470)) for the predicted duration of the transmitted frame, as defined by(# 2765) TXTIME in Error: Reference source not found(# 3256), for all supported and unsupported modes except Reserved HT-SIG Indication. Reserved HT-SIG Indication is defined in the fourth bullet below.(# 2766)

- If the HT-SIG indicates a Reserved HT-SIG Indication, the HT PHY shall maintain PHY-CCA.indication(BUSY, channel-list (#2470)) until the received level drops below the CCA sensitivity level (minimum modulation and coding rate sensitivity + 20 dB) specified in Error: Reference source not found(# 2918). Reserved HT-SIG Indication is defined as an HT-SIG with MCS field in the range 77–127 or Reserved field = 0 or STBC field = 3.

Subsequent to an indication of a valid HT-SIG CRC, a PHY-RXSTART.indication(RXVECTOR) shall be issued. The RXVECTOR associated with this primitive includes the parameters specified in Error: Reference source not found. Upon reception of a GF preamble by an HT STA that(# 2768) does not support GF, the FORMAT field of RXVECTOR is set to HT_GF and the remaining fields may be empty, and the PHY shall issue the error condition PHY-RXEND.indication(FormatViolation). If the HT-SIG indicates an unsupported mode or Reserved HT-SIG Indication(#2769) the PHY shall issue the error condition PHY-RXEND.indication(UnsupportedRate).

Following training and signal fields, the coded PSDU (C-PSDU) (which comprises the coded PLCP SERVICE field and scrambled and coded PSDU)(#2770) shall be received. If signal loss occurs during reception prior to completion of the PSDU reception, the error condition PHY-RXEND.indication(CarrierLost) shall be reported to the MAC. After waiting for the intended end of the PSDU, the PHY shall set PHY-CCA.indication(IDLE) and return to RX IDLE state.

The received PSDU bits are assembled into octets, decoded, and presented to the MAC using a series of PHY-DATA.indication(DATA) primitive exchanges. The number of PSDU octets is indicted in the HT Length(# 1588) field of the HT-SIG. The PHY shall proceed with PSDU reception.

After the reception of the final bit of the last PSDU octet, and possible tail and padding bits,(#2771) the receiver shall be returned to the RX IDLE state, if no signal extension is present (as defined in 9.3.2 (PLCP frame formats)), as shown in PLCP receive state machine. Otherwise, the receiver waits for the duration of the signal extension before returning to the RX IDLE state. A PHY-RXEND.indication(NoError) primitive shall be issued on entry to the RX IDLE state.

While in the Signal Extension state, if the receiver detects a CS/CCA event, it leaves the Signal Extension state and enters the Detect SIG state.  This occurs when signal-extended PPDUs are transmitted separated by a RIFS.

If the binary convolutional code is used, any data received after the indicated data length are considered pad bits (to fill out an OFDM symbol) and should be discarded.

Replace figure 20-23 with the following.  The changes comprise the addition of the Signal Present box,  two dotted lines, a callout “issued at the same time” and moving the PHY-RXEND.ind and PHY-CCA.ind primitives to the end of the signal extension.

- PLCP receive procedure for HT-mixed (#5400) format PLCP format (#5481)

Replace figure 20-24 with the following.  The changes comprise the addition of the Signal Present box,  two dotted lines, a callout “issued at the same time” and moving the PHY-RXEND.ind and PHY-CCA.ind primitives to the end of the signal extension.

- PLCP receive procedure for HT-greenfield (#5400) format PLCP

Replace figure 20-25 with the following.  The changes comprise the addition of the Signal Extension state, 6  connecting arrows,  two “A” connectors and labels for “no signal extension”, “signal extension” and “CS/CCA detected”.

- PLCP receive state machine(#2776, #5265, #5267,7835)