IEEE P802.11
Wireless LANs

| TGp LB 125 Comment Resolution: clause 7.1.3.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Susan R Dickey | California PATH | 1357 S 46th St, UC Berkeley, Richmond, California | 510-665-3664 | dickey@path.berkeley.edu |

Abstract

This submission proposes resolutions to comments on clause 7.1.3.1.3. To satisfy these and other comments the WAVE BSS terminology is being removed from D 4.0. Changes to clause 7.1.3.1.3 consistent with this are specified.

|  | LB125 Comment Resolution |
| --- | --- |

- COMMENT:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07 | Goodall, David | 7.1.3.1.3 | 5 | 12 | T | The sentence regarding To and From DS bits does not make sense if you still wish to reference Table 7-7 for address field usage. In addition, leaving the setting of these bits to chance, i.e. different implementors, is likely to lead to future incompatibilities. | Define what the To and From DS bits should be set to, e.g. zeroed on transmit and ignored on receive, and define the address field usage for WAVE mode. | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS. |
| 108 | Engwer, Darwin | 7.1.3.1.3 | 5 | 12 | TR | "When in WAVE mode, this standard does not define procedures for using any combination of To DS and From DS field values, with the exception of the address field designations in Table 7-7." So a WAVE mode STA can use any combination of the ToDS/FromDS bit values? I suspect this is not what was intended. First, many clauses in 802.11p cite the BSSID as an important field. Per Table 7, with ToDS=1 and FromDS=1 there is no assigned address field for the BSSID. So that would tend to rule out that combination. I suspect that what was intended was for WAVE to use ToDS=0/FromDS=0, which provides for three useful address fields: RA, TA and BSSID, acknowledging that in some cases the RA or TA will be equal to the Address 3 (BSSID) value. | change "When in WAVE mode, this standard does not define procedures for using any combination of To DS and From DS field values, with the exception of the address field designations in Table 7-7." to "When in WAVE mode, all frames are sent using the combination To DS = 0 and From DS = 0 field values." | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS. |
| 109 | Fischer, Matthew | 7.1.3.1.3 | 5 | 12 | TR | "The sentence: ""When in WAVE mode, this standard does not define procedures for using any combination of To DS and From DS field values, with the exception of the address field designations in Table 7-7."" is sort of confusing and incorrectly addresses the problem that you have. | Delete the cited sentence. Instead, ADD the following sentence to the text that is already present in the Meaning column of the first row of table 7-2 of subclause 7.1.3.1.3 of the baseline: "A Data frame transmitted between any two WAVE STA." | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS.. |
| 110 | Hart, Brian | 7.1.3.1.3 | 5 | 12 | T | "Does not define procedures" - procedures do not belong in clause 7; I suspect all u need to say is that To/from DS are reserved | Make reserved | ACCEPT |
| 112 | Ptasinski, Henry | 7.1.3.1.3 | 5 | 12 | TR | It's unclear from this sentence which values of ToDS and FromDS are allowed in WAVE mode. Table 7-7 merely says how to select the addresses in the frame once the ToDS and FromDS values are chosen. | Clarify which values of ToDS and FromDS are allowed in WAVE mode, and when. Include details into table 7-2. | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS.. |
| 113 | Roy, Richard | 7.1.3.1.3 | 5 | 12 | TR | WAVE is not a separate "mode" of operation of a STA. The WAVE amendment provides additional specifications that allow STAs to communicate (i.e., send data, management, and control frames) outside the context of any BSS. Furthermore, proposed additional sentence reads ... "in a certain condition, this standard does not define procedures for doing something except for the stuff that is already in the standard". This is a bit confusing and certainly not necessary. | Remove the sentence, or rewrite it in the context of WC STAs and how they use the DS. | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS.. |
| 137 | Kenney, John | 7.1.3.1.3 | 5 | 12-13 | TR | There is no reason to exempt control and management frames sent by a STA in WAVE mode from the requirement that To DS and From DS be set to 0. The inserted sentence should only apply to data frames. This sentence should also be reworded for clarity. | Replace this sentence with: "For a data frame sent by a STA in WAVE mode the meaning of the values in the To DS and From DS fields is not defined. The address field designations in Table 7-7 nevertheless apply to such a frame." | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS.. |
| 138 | Rai, Vinuth | 7.1.3.1.3 | 5 | 12-13 | TR | This sentence does not seem compatible with the statement in 5.2.2a, "STAs in WAVE mode do not use DS", we would need to reword it so that the correct intent is conveyed | Reword sentence to convey correct intent | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS.. |
| 139 | Venkatesan, Ganesh | 7.1.3.1.3 | 5 | 12-13 | T | "When in WAVE mode, this standard does not define procedures for using any combination of To DS and From DS field values, with the exception of the address field designations in Table 7-7". Are the ToDS and FromDS fields reserved while in WAVE mode? | Replace with "Since there is no notion of DS while in WAVE mode, ToDS and FromDS bits are ignored in WAVE mode." | COUNTER: ACCEPT IN PRINCIPLE Define ToDS and FromDS as reserved in data frames sent outside of a BSS.. |

- Background

In 802.11p we define procedures for sending data frames outside of a BSS, without authentication and association. In this context, the ToDS and FromDS bits need not be used. The description of this in D4.0 was worded in a confusing way, as the comments point out.

- Recommended Resolution of the Comments:

Resolve the comments as noted in the above table and change the text in D4.0 as described below.

Replace Draft P802.11p D4.0, page 5, lines 10-13 which currently read:

- Insert the following sentence after the first sentence in clause 7.1.3.1.3

When in WAVE mode, this standard does not define procedures for using any combination of To DS and From DS field values, with the exception of the address field designations in Table 7-7.

with

- Insert the following sentence after the first sentence in clause 7.1.3.1.3

For data frames sent outside of a BSS, theTo DS and From DS field values are reserved.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D4. noted above and instruct the editor to make these changes to P802.11p D4.0.

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: