IEEE P802.11
Wireless LANs

| TGp LB 125 Comment Resolution: clause 7, BSSID issues |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Susan R Dickey | California PATH | 1357 S 46th St, UC Berkeley, Richmond, California | 510-665-3664 | dickey@path.berkeley.edu |

Abstract

This submission proposes resolutions to comments on clause 7.1.3.3.3 and 7.2.2 that are concerned with the BSSID field. To satisfy these and other comments the WAVE BSS terminology is being removed from D 4.0. Changes to clauses 7.1.3.3.3 and 7.2.2 consistent with this are specified.

|  | LB125 Comment Resolution |
| --- | --- |

- COMMENT:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 115 | Adrian, Stephens | 7.1.3.3.3 | 5 | 19 | TR | "The value of the BSSID in a WAVE BSS shall be the MAC address of the STA initiating the WAVE BSS or a locally administered IEEE MAC address formed from a 46-bit random number generated according to the procedure defined in 11.1.3 for an IBSS.”But how does a STA know? It doesn't know which STA initialized a BSS. | Relate strictly to parameters of MLME primitives or on-the-air signalling. | COUNTER: ACCEPT IN PRINCIPLE. WAVE BSS terminology has been removed. |
| 116 | McCann, Stephen | 7.1.3.3.3 | 5 | 19 | T | If the WAVE STA is operating outside of the BSS, then how can it use a BSSID based on its MAC address. This paragraph seems to imply that even if a WAVE STA is not operating within a BSS, as mentioned in clause 5.2.2a, then it still requires a BSSID. I don't understand why? | Some text could be added, to explain the use of the BSSID when the WAVE STA is not operating in the context of a BSS. | COUNTER: ACCEPT IN PRINCIPLE. The BSSID field in a data frame outside of a BSS may be used to decide whether to receive a frame with a broadcast destination address. |
| 117 | Roy, Richard | 7.1.3.3.3 | 5 | 19 | TR | The concept of a WBSS is unnecessary. The additional functionality required to make STAs WAVE capable neither depends on nor does it require any concept of associating in any way with other STAs. As stated, this amendment specifies functionality that allows STAs to communicate outside the context of any BSS, and the introduction of the term/concept WBSS only confuses the matter, not to mention the implementer. Furthermore, without a WBSS, there is also no associated BSSID. There is, however, a field in transmitted frames often referred to in the standard as the BSSID field as it often contains the same value as that stored in the BSSID variable. This does not change the fact that it is used to filter the packets at the MAC layer and as such, is a frame or packet filter field in reality, which for WC STAs operating outside the context of a BSS is set by higher layers. | Rewrite this sentence to read: "The value of the BSSID field in frames transmitted by WC STAs outside the context of a BSS is not specified in this standard." | COUNTER: ACCEPT IN PRINCIPLE New wording reflects the intent of this suggestion. |
| 118 | NONAME, 16610 | 7.1.3.3.3 | 5 | 19 | TR | "The value of the BSSID in a WAVE BSS shall be the MAC address of the STA initiating the WAVE BSS.” STA initiating the WAVE BSS leaves a WAVE BSS. Then, the STA initiates a new WAVE BSS. How can be two WAVE BSS differentiated ? " | Remove the following sentence, "The value of the BSSID in a WAVE BSS shall be the MAC address of the STA initiating the WAVE BSS." | COUNTER: ACCEPT IN PRINCIPLE WAVE BSS terminology has been removed. |
| 119 | Hamilton, Mark | 7.1.3.3.3 | 5 | 20 | TR | There is no discussion of when or why a random number would be used for the BSSID | Either delete this option, or explain when it would be selected to be used. | COUNTER: the addresses used in a data frame sent outside of a BSS are set by the SME or higher layers. |
| 120 | Yang, Zhiyu | 7.1.3.3.3 | 5 | 25 | TR | What is the BSSID used in the Link RCPI Request? Is it the BSSID of the AP? | Use the AP s BSSID. | REJECT: we are describing a method of communication outside of a BSS in which there is no AP. |
| 121 | Yang, Zhiyu | 7.1.3.3.3 | 5 | 25 | TR | A non AP STA decides to establish a DLS with another non AP STA after sending it a Link RCPI and then determines if it should establish a peer link or not. However the non AP STA may be in Power Save mode and might not respond. | The non AP STA should send the Link RCPI after the DTIM beacon. | REJECT: In Draft 802.11p we are not talking about Direct Link communication in which the peer link is established after negotiation through an AP, but a new way of sending data frames without prior authentication or association. |
| 126 | Roy, Richard | 7.1.3.3.3 | 5 | 30 | TR | WAVE is not a separate "mode" of operation of a STA. The WAVE amendment provides additional specifications that allow STAs to communicate (i.e., send data, management, and control frames) outside the context of any BSS. | Replace "2. Data frames transmitted in WAVE mode." with "Any frames transmitted by WC STAs operating outside the context of a BSS." | COUNTER: ACCEPT IN PRINCIPLE New wording reflects the intent of this suggestion. |
| 140 | Kenney, John | 7.1.3.3.3 | 5 | 25-30 | TR | This paragraph does not make it clear whether the two exceptions are allowed to use the wildcard BSSID or are required to use the wildcard BSSID | Reword the sentence to start: "The wildcard BSSID may only be used in the BSSID field in the following two cases:" | ACCEPT |
| 141 | Stephenson, Dave | 7.1.3.3.3 | 5 | 25-30 | TR | A wildcard BSSID should not be used in WAVE mode; the BSSID should be the BSSID in the WAVE Beacon. | Delete the text. | REJECT: communication using the wildcard BSSID need not be preceded by a WAVE Beacon ; communication outside of a BSS may require the use of wildcard BSSID for acceptance of broadcast packets. |
| 143 | Fischer, Matthew | 7.2.2 | 6 | 6 | TR | You need to add instructions regarding how to fill in address3 when in WAVE mode. This is in direct reference to the combination of the fact that you indicated that ToDS and FromDS will determine the address fields as shown in Table 7-7 and the fact that you may use the wildcard BSS value in address3 in some cases. I note that you do have a general statement about how to determine the BSSID value for a WAVE STA, but that is not sufficient in this case, because the behavior description limits the STAs to either those in a BSS or those in an IBSS, and you are neither, so the definition of the BSSID that you have added here, while a good start, is insufficient. | Alter the baseline text from subclause 7.2.2 that includes the following phrasing: The BSSID of the Data frame is determined as follows: a) If the STA is an AP or is associated with an AP, the BSSID is the address currently in use by the STA contained in the AP.b) If the STA is a member of an IBSS, the BSSID is the BSSID of the IBSS. By adding another condition as follows: c) If the STA is operating in WAVE mode, the BSSID is either the address of the associated WAVE mode BSS or is the wildcard BSS value. | COUNTER: ACCEPT IN PRINCIPLE Wording has been added to describe the use of address 3 when sending data frames outside of a BSS. The exact wording suggested by the commenter does not apply since we have removed the WAVE BSS concept. |
| 144 | Stephenson, Dave | 7.2.2 | 6 | 15 | TR | The text states that a STA in WAVE mode use the wildcard BSSID. This means that all STAs must pass these frames up the stack since the lower MAC cannot filter based on BSSID. Upper MAC will have to process all of these frames and if STA is AP, then AP (non-WAVE mode) has to determine whether the STA is associated in order to make the decision on whether to forward or drop the frame. | Delete the use of wildcard BSSID for data frames outside the context of a WAVE BSS. | REJECT: data communication outside of a BSS may require the use of the wildcard BSSID to allow the acceptance of broadcast packets. Whether this creates a problem for a nearby AP is a system integration and performance issue. |

- Background

As these comments point out, defining a WAVE BSS that does not authenticate and associate using frame exchange but instead chooses to consider itself associated on receiving the WAVE on-demand beacon from another STA creates some confusion with the usual conventions for BSS operation in 802.11. In response to these comment, we are revising Draft P802.11p to define only communication outside of a BSS, in which data frames may be sent to unicast destination addresses or as a broadcast with a BSSID as the third address that can be used by the receiver to decide whether to accept it. It is expected that these MAC addresses will be known to the STA from a previous WAVE information element, although this will not be enforced by the MAC but will be handled by the SME and upper layers. STAs will accept such data frames and deliver them to the SME when the MIB attribute dot11WAVEEnabled is true. There are also two comments in reference to section 7.2.2 demonstrating lack of clarity about reception behaviour for a frame sent with a wildcard BSSID, and an addition to section 7.2.2 to clarify this is specified below as well as changes to 7.2.2 to eliminate WAVE BSS terminology.

- Recommended Resolution of the Comments:

Resolve the comments as noted in the above table and change the text in D4.0 as described below.

Replace Draft P802.11p D4.0, page 5, lines 15-30,  which currently reads

- Insert after the second paragraph of 7.1.3.3.3

The value of the BSSID in a WAVE BSS shall be the MAC address of the STA initiating the WAVE BSS or a locally administered IEEE MAC address formed from a 46-bit random number generated according to the procedure defined in 11.1.3 for an IBSS.

- Change the last sentence of the last paragraph of 7.1.3.3.3 to:

A wildcard BSSID shall not be used in the BSSID field except for management frames of subtype probe request in the following two cases:

1.	 Management frames of subtype Probe request

2.	 Data frames transmitted in WAVE mode

with

- Insert after the second paragraph of 7.1.3.3.3

For data frames outside of a BSS, the value of the BSSID field is a value in the format of an IEEE 802 MAC address provided by the the SME or a higher layer.

- Change the last sentence of the last paragraph of 7.1.3.3.3 to:

A wildcard BSSID shall not be used in the BSSID field except for management frames of subtype probe request in the following two cases:

1. Management frames of subtype Probe request

2. Data frames outside of a BSS

Replace Draft P802.11p D4.0, page 6, lines 8-17,  which currently reads

Insert the following after a) and b) of subclause with heading, “The BSSID of the Data frame is determined as follows:”

c) If the STA is transmitting a data frame in the context of a WAVE BSS, the BSSID is the BSSID of the WAVE BSS.

d) If the STA is in WAVE mode and transmitting a data frame outside the context of a WAVE BSS, the BSSID is the wildcard BSSID.

with

Add the following clause at the end of the following paragraph from section 7.2.2, as show in underline.

A STA uses the contents of the Address 1 field to perform address matching for receive decisions. In cases

where the Address 1 field contains a group address, the BSSID also is validated to ensure that the broadcast

or multicast originated from a STA in the BSS of which the receiving STA is a member, or, if dot11WAVEnabled

is true and the BSSID is not that of a BSS of which the receiving STA is a member, to ensure that the BSSID is the

wildcard BSSID or some other BSSID that the SME accepts as valid.

Insert the following after a) and b) of subclause with heading, “The BSSID of the Data frame is determined as follows:”

c If the STA is transmitting a data frame outside of a BSS when dot11WAVEEnabled is true, the BSSID may be a MAC address that the receiver can filter on the packet or the wildcard BSSID.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D4. noted above and instruct the editor to make these changes to P802.11p D4.0.

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: