IEEE P802.11
Wireless LANs

| QoSMap in (Re)-Association Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

Putting the QoSMap in (Re)-Association Response frames for interworking capable STAs is more efficient as STA’s don’t have to request the element post association.  When this is done, only the MLME-QoSMap.confirm and MLME-QoSMap.response primitives are needed in order to support autonomous update of the QoSMap by the AP.

This proposal addresses the following CIDs: 729, 790, 802, 821 and 880.

This document is based on 802.11u-d3.0.

    - Management frames

      - Association Response frame format

Add the following contents to Table7- 11 as shown below:

Table 7-11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | QoS Map | QoS Map is present if dot11InterworkingServiceEnabled is true and the QoS Map bit in the Extended Capabilities element of the corresponding Association Request frame is set to 1. |

      - Reassociation Response frame format

Add the following contents to Table7- 12 as shown below:

Table 7-13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | QoS Map | QoS Map is present if dot11InterworkingServiceEnabled is true and the QoS Map bit in the Extended Capabilities element of the corresponding Reassociation Request frame is set to 1. |

7.4	Action frame format details

7.4.2	QoS Action frame details

Change table 7-45 with table by inserting new rows as shown below.

Table 7-45—QoS action field values

| Action field value | Meaning |
| --- | --- |
| 0 | ADDTS request |
| 1 | ADDTS response |
| 2 | DELTS |
| 3 | Schedule |
| 4 | QoS Map Configure |
| 5-255 | Reserved |

Renumber the following clause as appropriate (given that 7.4.2.5 is deleted).

7.4.2.6	QoS Map Configure frame format

The QoS Map Configure is used by an AP to provide the QoS Map Set to a non-AP STA using the procedures defined in 11.19.

Table 7-57z—QoS Map configure frame body

| Order | Information |
| --- | --- |
| 0 | Category |
| 1 | Action |
| 2 | Dialog Token |
| 3 | QoS Map Set |

The Category field is set to the value in Table 7-24 (representing QoS).

The Action field is set to the value in Table 7-45 (representing QoS Map Configure).

The Dialog Token field is set to 0.

The QoS Map Set element is defined in 7.3.2.64.

10.	Layer management

10.3	MLME SAP interface

Amend the text as shown below, renumbering clauses as necessary:

10.3.40	QoS Map Set element management

The QoS Map Set element is provided to non-AP STAs in (Re)-association response frames. However, if the SME of an AP detects a change of the QoS Map information while one or more non-AP STAs are associated to the BSS, then the AP’s SME may transmit an un-solicited QoS Map Set element to associated STAs. The AP’s SME invokes the MLME-QoSMap.reponse primitive to transmit individually-addressed frames containing a QoS Map Set element to associated STAs.  When a non-AP STA receives such unsolicited QoS Map information, its MLME generates a MLME-QoSMap.confirm to the STA’s SME.  In turn, the SME should take appropriate action, e.g., initiate an ADDTS or DELTS if admission control changes are necessary.

10.3.40.2	MLME-QoSMap.confirm

10.3.40.2.1	Function

This primitive reports the QoS Mapping information sent from the AP to the non-AP STA. It is valid at non-AP STA.

10.3.40.2.2	Semantics of the service primitive

The primitive parameters are as follows:

MLME-QoSMap.confirm	(
ResultCode
DialogToken
QoS Map Set
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE | Indicates the results of the corresponding MLMEQoSMap.request primitive. |
| Dialog Token | Integer | 0 - 255 | The dialog token to identify the QoS Map Set message transaction. |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.40.2.3	When generated

This primitive is generated by the SME as a result of an MLME-QoSMap.request primitive indicating the result of that request.

The primitive is generated when the non-AP STA receives a response in the form of a QoS Map Configure frame in the corresponding Action frame from the AP. This requires no prior MLME-QoSMap.request primitive from the non-AP STA.

The SME of the non-AP STA should use the information to decide proper actions. For example, an ADDTS or DELTS procedure should be activated if the QoS Map information indicates a change in the admission control.

10.3.40.2.4	Effect of receipt

The non-AP STA operates according to the procedures defined in .

10.3.40.4	MLME-QoSMap.response

10.3.40.4.1	Function

This primitive is used to transmit a QoS Map Set to a specified non-AP STA MAC entity. The specified non-AP STA MAC address is an individual MAC address.

10.3.40.4.2	Semantics of the service primitive

The primitive parameters are as follows:

MLME-QoSMap.response 	(
ResultCode,
Non-APSTAAddress,
DialogToken,
QoS Map Set
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE | Indicates the results of the corresponding MLMEQoSMap.request primitive. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which query message is received. |
| Dialog Token | Integer | 0 | The dialog token to identify the QoS Map Set message transaction. |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.40.4.3	When generated

This primitive is generated by the MLME at the AP as a result  of any change in the AP QoS Map configurations.

10.3.40.4.4	Effect of receipt

The primitive causes the MAC entity at the AP to send a QoS Map Configure frame in the corresponding QoS Map action management frame to the non-AP STA.

- 11.	MLME

11.19	Support for QoS Mapping from External Networks

Maintaining proper end-to-end QoS is an important factor when providing interworking services. This is because the external networks may employ different network-layer (Layer 3) QoS practices. For example, the use of a particular differentiated services code point (DSCP) for a given service may be different between different networks. To ensure the proper QoS over-the-air in the IEEE 802.11 infrastructure, the mapping from DSCP to UP for the corresponding network needs to be identified and made known to the STAs. If an inconsistent mapping is used then:

- Admission control at the AP may incorrectly reject a service request, because the non-AP STA used the incorrect UP.

- Non-AP STAs may use the incorrect value for User Priority in TSPEC and TCLAS elements.

- The user may be given a different QoS over the IEEE 802.11 network than expected, e.g., a lower QoS may be provided than the STA expected.

Therefore, APs with dot11QosmapEnabled set to TRUE, shall set the QoSMap bit in the Extended capabilities element to 1 .  The AP’s SME shall cause the QoS Map Set to be available to higher layer protocols or applications so they will be able to set the correct priority in an MA-UNITDATA.request primitive.

Non-AP STAs with dot11QosmapEnabled set to TRUE, shall set the QoSMap bit in the Extended capabilities element to 1 .  An AP receiving an Association request frame or Re-association request frame having this bit set to 1 shall include the QoS Map Set element in the corresponding Association response frame or Re-association response frame per 7.3.2.5 or 7.3.2.7 respectively.  Upon receiving the QoS Map Set element, the non-AP STA’s SME shall cause the QoS Map Set to be available to higher layer protocols or applications so they will be able to set the correct priority in an MA-UNITDATA.request primitive.

Whenever the AP detects a change in the QoS mapping information, it shall update the non-AP STA with the new QoS Map Set element.  It accomplishes this update by having its SME send to the MLME an MLME-QoSMap.response primitive.

When the MLME of the AP receives a MLME-QoSMap.response, it shall send a QoS Map Configure Action frame to the corresponding non-AP STA MAC entity.  QoS Map Configurate Action frames shall be transmitted to non-AP STAs using individual MAC addresses.

When the MAC entity at the non-AP STA receives a QoS Map Configure action frame from the AP, the MLME shall generate a MLME-QoSMap.confirm primitive to its SME.

When the non-AP STA's SME receives the QoS Map response, it shall make the QoS Map available to higher layers so that in turn, they can invoke the MA-UNITDATA.request with the correct priority.