IEEE P802.11
Wireless LANs

| LB133 Diagnostics Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains the text changes to resolve LB133 diagnostics comments, relative to TGv D3.0.

Front Matter, page iii:

Diagnostic Reporting: Sets of information, termed Diagnostic Types, about a non-AP STA are defined, and

new management action frames are defined for the AP to request a set of information from the non-AP STA.

The Diagnostic types are:

— Manufacturer Information STA Report

— Configuration Profile

— Association Diagnostic, and

— IEEE 802.1X Authentication Diagnostic

The Diagnostic reporting mechanism enables an AP to gather configuration and operational data about the associated non-AP STAs. Diagnostic reporting also enables an AP to instruct a non-AP STA to conduct association, and 802.1X authentication tests, and gather the corresponding diagnostic results.

- Diagnostic reporting

Diagnostic requests allow a STA to request a non-AP STA to report on information that may be helpful in diagnosing and resolving problems with the WLAN network. Diagnostic reports include information on hardware, configuration, and STA capabilities.

7.3.2.64 Diagnostic Request element

7.3.2.64.1 Diagnostic Request definition

The Diagnostic Request element contains a request that the receiving STA undertake the specified diagnostic action. The format of the Diagnostic Request element is shown in Figure v20.

|  | Element ID | Length | Diagnostic Token | Diagnostic Request Type | Diagnostic Timeout | Diagnostic Information Sub-elements (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 2 | variable |
| Figure v20—Diagnostic Request element format |  |  |  |  |  |  |

The Element ID field is equal to the Diagnostic Request value in Table 7-26.

The value of the Length field is variable and depends on the length of the Diagnostic Information Sub-elements field. The minimum value of the Length field is 4 (based on a minimum length for the Diagnostic Information Sub-elements field of 0 octets).

The Diagnostic Token field is set to a number that is unique among the Diagnostic Request elements sent to each destination MAC address for which a corresponding Diagnostic Report element has not been received.

The Diagnostic Request Type field is set to a number that identifies a type of diagnostic request. The defined Diagnostic Request Types are shown in Table v9.

| Table v9—Diagnostic Type definitions |  |
| --- | --- |
| Name | Diagnostic Type Values |
| Cancel Diagnostic Request | 0 |
| Manufacturer Information STA Report | 1 |
| Configuration Profile | 2 |
| Association Diagnostic | 3 |
| IEEE 802.1X Authentication Diagnostic | 4 |
| Reserved | 5 – 220 |
| Vendor Specific | 221 |
| Reserved | 222 – 255 |

The Diagnostic Timeout field contains the time, in seconds, after which no response is returned.

The Diagnostic Information Sub-elements field contains zero or more diagnostic information sub-elements depending on the specific Diagnostic Request Type.

The Manufacturer Information STA Report and Configuration Profile Diagnostic Request elements carry no Diagnostic Information sub-elements.

The Diagnostic Request element is included in a Diagnostic Request frame as described in 7.4.11.3. The use of Diagnostic Request element and frames is described in 11.20.4.

7.3.2.64.2 Association Diagnostic request

The Diagnostic Information Sub-elements field corresponding to an Association Diagnostic Request Type is shown in Table v10. The corresponding Diagnostic Information Sub-elements are defined in 7.3.2.64.5.

| Table v10—Association Diagnostic request contents |  |
| --- | --- |
| Order | Information Sub-element |
| 1 | AP Descriptor |
| 2 | Profile ID |

7.3.2.64.3 IEEE 802.1X Authentication Diagnostic request

The Diagnostic Information Sub-elements field corresponding to an IEEE 802.1X Authentication Diagnostic Request Type is shown in Table v11. The corresponding Diagnostic Information Sub-elements are defined in 7.3.2.64.5.

| Table v11—IEEE 802.1X Authentication Diagnostic request contents |  |
| --- | --- |
| Order | Information Sub-element |
| 1 | AP Descriptor |
| 2 | EAP Method |
| 3 | IEEE 802.1X credentials |
| 4 | Profile ID |

7.3.2.64.4 Vendor Specific Diagnostic request

The Diagnostic Information Sub-elements field corresponding to a Vendor Specific Diagnostic Request Type contains zero or more Vendor Specific sub-elements. The Vendor Specific sub-elements have the same format as their corresponding elements (see 7.3.2.26)

7.3.2.64.5 Diagnostic Information Sub-element descriptions

The following sections describe the sub-elements that may be included in Diagnostic Information Sub-elements field of a Diagnostic Request element or a Diagnostic Report element. The format of a Diagnostic information sub-element is shown in Figure v21.

|  | Diagnostic Information Sub-element ID | Length | Diagnostic Information Sub-element Contents |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v21—Diagnostic Information Sub-element format |  |  |  |

The Diagnostic Information Sub-element ID field indicates the Diagnostic Information Sub-element type and is set to any allocated value in Table v12.

The Length field is set to length in octets of the Diagnostic Information Sub-element Contents field.

The Diagnostic Information Sub-element Contents values are described in Table v12.

| Table v12—Diagnostic Information Sub-element ID values |  |  |
| --- | --- | --- |
| Identifier | Sub-element Name | Length (in octets) |
| 0 | IEEE 802.1X Credential Type | 3 to 251 |
| 1 | AKM Suite | 6 |
| 2 | AP Descriptor | 10 |
| 3 | Antenna Type | 3 to 251 |
| 4 | Antenna Gain | 3 |
| 5 | Cipher Suite | 6 |
| 6 | EAP Method | 3 to 10 |
| 7 | Firmware Version | 3 to 251 |
| 8 | MAC Address | 8 |
| 9 | Manufacturer ID String | 3 to 251 |
| 10 | Manufacturer Model String | 3 to 251 |
| 11 | Manufacturer OUI | 5 |
| 12 | Manufacturer Serial Number String | 3 to 251 |
| 13 | Power Save Mode | 4 |
| 14 | Profile ID | 3 |
| 15 | Supported Regulatory Classes | 3 to 251 |
| 16 | Status Code | 4 |
| 17 | SSID | 4 to 36 |
| 18 | Tx Power Capability | 3 to 251 |
| 19-220 | Reserved |  |
| 221 | Vendor Specific | 3 to 251 |
| 221-255 | Reserved |  |

The format for the IEEE 802.1X Credential Type sub-element is shown in Figure v22.

|  | Sub-element ID | Length | Credential TYpe |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v22—IEEE 802.1X Credential Type Sub-element format |  |  |  |

The Credential Type field indicates one or more credential type(s). Each Credential Type value is encoded as a single octet, and each value is chosen from those shown in Table v13.

| Table v13—IEEE 802.1X Credentials Values |  |
| --- | --- |
| Value | Description |
| 0 | None |
| 1 | Pre-shared key |
| 2 | Username and password |
| 3 | X.509 certificate |
| 4 | Other certificate |
| 5 | One time password |
| 6 | Token |
| 7–255 | Reserved |

The format for the AKM Suite sub-element is shown in Figure v23.

|  | Sub-element ID | Length | OUI | AKM Suite |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 1 |
| Figure v23—AKM Suite Sub-element format |  |  |  |  |

The OUI and AKM Suite fields identify the authentication method and is set to one of the values in Table 34 in 7.3.2.25.2.

The format of the AP descriptor sub-element is described in Figure v24.

The BSSID field is a 6-octet field as described in 7.1.3.3.3 that identifies the BSS indicated in the AP Descriptor sub-element.

|  | Sub-element ID | Length | BSSID | Regulatory Class | Channel Number |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 | 1 |
| Figure v24—AP Descriptor Sub-element format |  |  |  |  |  |

The Regulatory Class field contains an enumerated value from Annex J specifying the frequency band in which the Channel Number is valid.

The Channel Number field indicates the current operating channel of the AP identified by the BSSID in the AP Descriptor.

The format for the Antenna Type sub-element is shown in Figure v25.

|  | Sub-element ID | Length | Antenna Type |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v25—Antenna Type Sub-element format |  |  |  |

The Antenna Type field contains an ASCII string (truncated to less than 251 octets as required) indicating the type of antenna connected to the wireless network adapter. This string is not null terminated.

The format for the Antenna Gain sub-element is shown in Figure v26.

|  | Sub-element ID | Length | Antenna Gain |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure v26—Antenna Gain Sub-element format |  |  |  |

The Antenna Gain field contains the peak gain in dBi of the antenna.

The format for the Cipher Suite sub-element is shown in Figure v27.

|  | Sub-element ID | Length | OUI | Suite Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 1 |
| Figure v27—Cipher Suite Sub-element format |  |  |  |  |

The OUI and Suite Type fields identify the cipher suite and is set to one of the values in Table 32.

The format for the EAP method sub-element is shown in Figure v28.

|  | Sub-element ID | Length | EAP method |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 or 8 |
| Figure v28—EAP Method Sub-element format |  |  |  |

The EAP Method field contains an IANA assigned EAP type [B34]. The EAP Method field contains either the legacy type (1 octet) or the expanded type (1 octet type = 254, 3-octet Vendor ID, 4-octet Vendor-Type).

The format for the Firmware Version sub-element is shown in Figure v29.

|  | Sub-element ID | Length | Firmware Version |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v29—Firmware Version Sub-element format |  |  |  |

This Firmware Version field contains an ASCII string (truncated to less than 251 octets as required) identifying the version of firmware currently installed on the wireless network adaptor. This string is not null terminated.

The format for the MAC Address sub-element is shown in Figure v30.

|  | Sub-element ID | Length | MAC Address |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |
| Figure v30—MAC Address Sub-element format |  |  |  |

This MAC Address field contains the 6-octet IEEE 802 MAC address of the STA.

The format for the Manufacturer ID String sub-element is shown in Figure v31.

|  | Sub-element ID | Length | ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v31—Manufacturer ID String Sub-element format |  |  |  |

The ID field contains an ASCII string (truncated to less than 251 octets as required) indicating the manufacturer identifier of the wireless network adaptor. This string is not null terminated.

The format for the Manufacturer Model String sub-element is shown in Figure v32.

|  | Sub-element ID | Length | Model |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v32—Manufacturer Model String Sub-element format |  |  |  |

The Model field contains an ASCII string (truncated to less than 251 octets as required) indicating the model of the wireless network adaptor. This string is not null terminated.

The format for the Manufacturer OUI sub-element is shown in Figure v33.

|  | Sub-element ID | Length | OUI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 |
| Figure v33—Manufacturer OUI Sub-element format |  |  |  |

The OUI field contains an Organizational Unique Identification, the first 24-bits of the network connected device, which indicate the specific vendor for that device.

The format for the Manufacturer Serial Number String sub-element is shown in Figure v34.

|  | Sub-element ID | Length | Serial Number |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v34—Manufacturer Serial Number String Sub-element format |  |  |  |

The Serial Number field contains an ASCII string (truncated to less than 251 octets as required) indicating the serial number of the wireless network adaptor. This string is not null terminated.

The format for the Power Save Mode sub-element is shown in Figure v35.

|  | Sub-element ID | Length | Power Save Mode |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |
| Figure v35—Power Save Mode Sub-element format |  |  |  |

The Power Save Mode field is a bitmap field that indicates the power save mode(s) in use by the STA, as defined in Table v14.

| Table v14—Power Save Mode definition |  |
| --- | --- |
| Power Save Mode | Bit |
| Unknown | 0 |
| None | 1 |
| PS mode (ReceiveDTIMs=1, see 11.2.1) | 2 |
| PS mode (ReceiveDTIMs=0, see 11.2.1) | 3 |
| U-APSD (see 11.2.1.4) | 4 |
| S-APSD (see 11.2.1.4) | 5 |
| U-PSMP (see 9.16) | 6 |
| S-PSMP (see 9.16) | 7 |
| WNM-Sleep Mode (see 11.20.15) | 8 |
| FBMS (9.2.7.2) | 9 |
| TIM Broadcast (11.2.1.11a) | 10 |
| TFS (11.20.14) | 11 |
| SM Power Save (11.2.3) | 12 |
| Reserved | 13-15 |

The format for the Profile ID sub-element is shown in Figure v36.

|  | Sub-element ID | Length | Profile ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure v36—Current Profile Indication Sub-element format |  |  |  |

The Profile ID field contains a unique identifier for referencing a configuration profile available on a device. The value of the identifier can be any arbitrary value, as long as it is uniquely associated to a single configuration profile on the device sending the identifier.

The format for the Supported Regulatory Classes sub-element is shown in Figure v37.

|  | Sub-element ID | Length | Supported Regulatory Classes Element |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure v37—Supported Regulatory Classes Sub-element format |  |  |  |

The Supported Regulatory Classes Element field contains the Supported Regulatory Classes element, as defined in 7.3.2.51, excluding the Element ID and Length in the Supported Regulatory Classes Element.

EDITORIAL NOTE—7.3.2.51 is defined in 802.11y D11.0.

The format for the Status Code sub-element is shown in Figure v38.

|  | Sub-element ID | Length | Status Code |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |
| Figure v38—Status Code Sub-element format |  |  |  |

The Status Code field contains the final IEEE 802.11 Status code, as defined in Table 7-23 in 7.3.1.9, received at the end of the applicable operation.

The format for the SSID sub-element is shown in Figure v39.

|  | Sub-element ID | Length | SSID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 0 to 32 |
| Figure v39—SSID Sub-element format |  |  |  |

The SSID field contains a Service Set Identifier with a maximum length of 32 octets, as defined in 7.3.2.1.

The format for the Tx Power Capability sub-element is shown in Figure v40.

|  | Sub-element ID | Length | Tx Power Mode | Tx Power |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure v40—Tx Power Capability Sub-element format |  |  |  |  |

The Tx Power Mode field identifies the transmit power mode of the non-AP STA and is set to one of the values in Table v15.

| Table v15—Tx Power Modes |  |
| --- | --- |
| Tx Power Mode | Value |
| Discrete | 0 |
| Range | 1 |
| Reserved | 2-255 |

"The Tx Power field indicates the target transmit power at the antenna in dBm with a tolerance of +/-5dB. If the Tx Power Mode field is set to 0 then the TxPower field contains one or more Tx power levels, in increasing numerical order. If the Tx Power Mode field is set to 1, the TxPower field is two octets in length and contains the STA’s minimum and maximum TxPower levels, in that order. The TX Power Levels are encoded as 2’s complement values,, rounded to the nearest integer.

The Vendor Specific sub-elements have the same format as their corresponding elements (see 7.3.2.26).

"The Tx Power field indicates the target transmit power at the antenna in dBm with a tolerance of +/-

5dB.  If the Tx Power Mode field is set to 0 then the TxPower field contains one or more Tx power levels, in increasing numerical order. If the Tx Power Mode field is set to 1, the TxPower field is two octets in length and contains the STA’s minimum and maximum TxPower levels, in that order. The TX Power Levels are encoded as 2’s complement values, rounded to the nearest integer."

7.3.2.65 Diagnostic Report element

7.3.2.65.1 Diagnostic Report element definition

The Diagnostic Report element contains a Diagnostic report. The format of the Diagnostic Report element is shown in Figure v41.

|  | Element ID | Length | Diagnostic Token | Diagnostic Report Type | Diagnostic Status | Diagnostic Information Sub-elements |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |
| Figure v41—Diagnostic Report element format |  |  |  |  |  |  |

The Element ID field is equal to the Diagnostic Report value in Table 7-26.

The value of the Length field is variable and depends on the length of the Diagnostic Information Sub-elements field. The minimum value of the Length field is 3.

The Diagnostic Token field is set to the Diagnostic Token in the corresponding Diagnostic Request element.

The Diagnostic Report Type field is set to a number that identifies the Diagnostic report. The Diagnostic Report Types are defined in Table v9.

The Diagnostic Status field is set to a value in Table v5 (see 7.3.2.63), indicating the STA’s response to the Diagnostic Request indicated by the Diagnostic Token.

The Diagnostic Information Sub-elements field contains the results of the diagnostic request. (see 7.3.2.64.5) corresponding to the Diagnostic Report Type field value.

The Diagnostic Report element is included in a Diagnostic Report frame as described in 7.4.11.4. The use of Diagnostic Report element and frames is described in 11.20.4.

7.3.2.65.2 Manufacturer Information STA Report

The Diagnostic Information Sub-elements field corresponding to a Diagnostic Report element of type Manufacturer Information STA Report is shown in Table v16. The corresponding Diagnostic Information Sub-elements are defined in 7.3.2.64.5.

| Table v16— Manufacturer Information STA Report contents |  |
| --- | --- |
| Order | Information Sub-element |
| 1 | Manufacturer OUI |
| 2 | Manufacturer ID string |
| 3 | Manufacturer model string |
| 4 | Manufacturer serial number string |
| 6 | Firmware Version |
| 7 | Antenna Type |
| 8 | Antenna gain |

7.3.2.65.3 Configuration Profile Report

The Diagnostic Information Sub-elements field corresponding to a Diagnostic Report element of type Configuration Profile Report is shown in Table v17. The corresponding Diagnostic Information Sub-elements are defined in 7.3.2.64.5.

| Table v17—Configuration Profile Report contents |  |
| --- | --- |
| Order | Information Sub-element |
| 1 | Profile ID |
| 2 | Supported Regulatory Classes |
| 3 | Tx Power Capability |
| 4 | Cipher Suite |
| 5 | AKM Suite |
| 6 | EAP Method |
| 7 | SSID |
| 8 | Power Save Mode |

7.3.2.65.4 Association Diagnostic Report

The Diagnostic Information Sub-elements field corresponding to a Diagnostic Report element of type Association Diagnostic Report is shown in Table v18. The corresponding Diagnostic Information Sub-elements are defined in 7.3.2.64.5.

| Table v18—Association Diagnostic Report contents |  |
| --- | --- |
| Order | Information Sub-element |
| 1 | AP Descriptor |
| 2 | Status Code |

7.3.2.65.5 IEEE 802.1X Authentication Diagnostic Report

The Diagnostic Information Sub-elements field corresponding to a Diagnostic Report element of type IEEE 802.1X Authentication Diagnostic Report is shown in Table v19. The corresponding Diagnostic Information Sub-elements are defined in 7.3.2.64.5.

| Table v19—IEEE 802.1X Authentication Diagnostic Report contents |  |
| --- | --- |
| Order | Information Sub-element |
| 1 | AP Descriptor |
| 2 | EAP Method |
| 3 | IEEE 802.1X Credentials |
| 4 | Status Code |

7.3.2.65.6 Vendor Specific Diagnostic report

The Diagnostic Information Sub-elements field corresponding to a Diagnostic Report element of type Vendor Specific Diagnostic Report contains zero or more Vendor Specific sub-elements as defined in 7.3.2.26.

7.4.11.4 Diagnostic Report frame format

The Diagnostic Report frame uses the Action frame body format transmitted by a STA in response to a Diagnostic Request frame. The format of the Diagnostic Report frame body is shown in Figure v77.

|  |  |  |  |  | Zero or more Vendor Specific Elements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Category | Action | Dialog Token | Diagnostic Report Elements | Vendor Specific Elements |  |  |  |  |  |
| Octets: | 1 | 1 | 1 | variable | variable |  |  |  |  |  |
| Figure v77—Diagnostic Report frame body format |  |  |  |  |  |  |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Diagnostic Report, as specified in Table v34 in 7.4.11.

The Dialog Token field is set to the nonzero value in the corresponding Diagnostic Request frame.

The Diagnostic Report Elements field contains one or more of the Diagnostic Report elements described in 7.3.2.65. The number and length of the Diagnostic Report elements in a Diagnostic Report frame is limited to 2304 octets.

The Vendor Specific Elements field contains zero or more Vendor Specific element, as defined in 7.3.2.26.

11.20.4 Diagnostic Request and Report Procedures

11.20.4.1 Diagnostic Request and Diagnostic Report

The Diagnostic Request and Diagnostic Report protocol provides a tool to diagnose and debug complex network issues. A STA that has a value of true for the MIB attribute dot11MgmtOptionDiagnosticsEnabled is defined as a STA that supports Diagnostics Reporting. A STA for which the MIB attribute dot11MgmtOptionDiagnosticEnabled is true shall set the Diagnostics field of the Extended Capabilities element to 1.

A STA that supports Diagnostics shall only send a Diagnostics Request or Diagnostics Report frame to a STA within the same infrastructure BSS or the same IBSS whose last received Extended Capabilities element contained a value of 1 for the Diagnostics bit in the Capabilities field. A STA gathers information from another STA by sending a Diagnostic Request frame containing a unique Dialog Token to another STA that supports the Diagnostics Capability. The source and destination of a Diagnostic Request frame shall both be members of the same BSS. The permitted source and destination STAs for a Diagnostic Request frame are shown in Table 79d. Within each Diagnostic Request frame there may be one or more Diagnostic Request elements. Each Diagnostic Request element contains a unique Diagnostic Token that identifies the element within the Diagnostic Request Frame.

If a STA receives a Diagnostic Request frame without error and it supports the Diagnostic service, the STA shall respond with a Diagnostic Report frame that includes the Dialog Token that matches the one in the Diagnostic Request frame. Each Diagnostic Report element that corresponds to the Diagnostic Request element shall contain the same Diagnostic Token that was included in the original request.

Only a single Diagnostic Request frame from a STA is outstanding at a given STA at any time. If a STA receives a subsequent Diagnostic Request frame with a different Dialog Token before completing the Diagnostic Report for the previous request from the requesting STA, the STA shall only respond to the most recent Request frame. A Dialog Token value of zero is a reserved value and shall not be used. The STA need not send a Diagnostic Report frame for the previous Diagnostic Request frame.

All outstanding Diagnostic Requests, as indicated by received MLME-DIAGREQUEST.ind primitives, are cancelled upon a BSS transition, except when the BSS transition occurs as a result of responding to or initiating a Diagnostic Request. All outstanding Diagnostic Requests, as indicated by received MLME-DIAGREQUEST.ind primitives, are cancelled after the time indicated in the Diagnostic Timeout field, in the Diagnostic Request frame. When a STA receives a Diagnostic Request Frame with a Diagnostic Request Type of Cancel Diagnostic Request, the STA cancels all outstanding Diagnostic Requests, and discards all pending Diagnostic Report Requests, as indicated by received MLME-DIAGREQUEST.ind primitives.

All Diagnostic Report elements shall include a Status field that indicates the overall result of the transaction. If the STA is able to complete the diagnostic request made in the Diagnostic Request element, then a value of Successful shall be returned. If the STA is unable to process the request at that time a value of Fail shall be returned. If the STA is incapable of generating a report of the type specified, it shall return a value of Incapable. If the STA cannot support the request for any other reason, the value of Refused shall be returned.

Both the Diagnostic Request frame and the Diagnostic Report frame shall only be sent with an individually addressed destination address. If a STA receives a Diagnostic Request or Report frame with a broadcast or multicast destination address, it shall discard the frame.

In the event that the Diagnostic Report elements do not fit into a single MMPDU, the reporting STA shall send the remaining elements in additional frames until all Diagnostic Report elements have been returned to the requesting STA. Each subsequent Diagnostic Report frame and Diagnostic Report elements shall include the same Dialog Token and Diagnostic Token, respectively, that was sent in the corresponding Diagnostic Request frame. When multiple MMPDUs are required, the STA shall include complete Diagnostic Report elements and shall not fragment an element across multiple MMPDUs.

A STA may cancel a previously sent Diagnostic Request frame for which it has not yet received a diagnostic report by sending a diagnostic request with the Reason Code value of “2”, indicating “Cancelled” in the Reason Code sub-element (see Table v15) to the STA to which it previously sent the diagnostic request. A STA shall inform a STA from which it has previously received a diagnostic request that the request has been locally cancelled by sending a diagnostic report with the Result Code value of "4", indicating "Cancelled" in the Result Code sub-element to that requesting STA. When either the Reason Code value of "2" or the Result Code value of "4" is used, only the Reason Code or Result Code Diagnostic Information sub-element is included in the Diagnostic Request or Response Element.

| Table 79d—Allowed and Disallowed Diagnostic Request |  |  |  |
| --- | --- | --- | --- |
| Service Set | Source of Request | Destination of Request | Allowed |
| Infrastructure BSS | AP | Non-AP STA | Yes |
| Infrastructure BSS | Non-AP STA | Non-AP STA | No |
| Infrastructure BSS | Non-AP STA | AP | No |
| IBSS | Non-AP STA | Non-AP STA | Yes |
| ESS | AP | AP | No |

11.20.4.2 Configuration Profile Report

The Configuration Profile report enables an AP to discover the current profile in use for an associated device, and additional profiles for the current ESS. Devices that support multiple configuration profiles for an ESS may include multiple Diagnostic Report elements in a single Diagnostic Report frame (or multiple frames if required).

A STA receiving a Diagnostic Request frame that indicates a Configuration Profile Request shall respond with a Diagnostic Response frame including a Configuration Profile Report for the current profile in use. The Diagnostic Response frame may also include Diagnostic Report elements containing additional Configuration Profile Report(s)  for the current ESS that are available on the device.

11.20.4.3 Manufacturer Information STA Report

The Manufacturer Information STA Report enables an AP to discover all the static manufacturer specific data about an associated device. A non-AP STA receiving a Manufacturer Information STA Report request type shall respond with a Diagnostic Report frame that includes all available Diagnostic Information elements allowed for the type.

11.20.4.4 Association diagnostic

The purpose of the Association diagnostic is to determine that the STA is able to associate with a designated BSS. This test directs an association to be completed with a specific AP.

To initiate the test, the AP shall send a Diagnostic Request frame containing a Diagnostic Request Type field set to 3 (Association Diagnostic) to the STA. An AP shall not send an Association diagnostic request with a designated BSS that is not part of the ESS and a STA receiving an Association diagnostic request shall reject requests to perform diagnostics on a BSS that is not part of the ESS. All parameters required to complete the association are included in the Diagnostic Request element.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an Association request, the STA determines whether or not to accept the request. If the STA declines the request, it shall send a Diagnostic Report frame with the Status field of a Diagnostic Report element set to Refused. If the STA accepts the request, it shall issue an MLME-DEAUTHENTICATE.request to deauthenticate from the current AP, and an MLME-AUTHENTICATE.request to authenticate to the AP indicated in the Diagnostic Request element. If successful, the STA shall issue an MLME-(RE)ASSOCIATE.request to the AP indicated in the Diagnostic Request element. The STA shall then issue an MLME-DEAUTHENTICATE.request to deauthenticate from the AP indicated in the Diagnostic Request element, reassociate to the AP from which it received the Diagnostic Request, and issue an MLME-DIAGREPORT.request primitive, indicating the results of the diagnostic

11.20.4.5 IEEE 802.1X Authentication diagnostic

The purpose of the IEEE 802.1X Authentication diagnostic is to determine that the STA is able to complete an IEEE 802.1X authentication with a designated BSS. This test directs an association and IEEE 802.1X authentication to be completed with a specific AP. If a STA that supports diagnostic reporting also supports RSN, the STA shall support the IEEE 802.1X Authentication diagnostic.

To initiate the test, the AP shall send a Diagnostic Request frame containing a Disgnostic Request Type field set to 4 (IEEE 802.1X Authentication Diagnostic) to the STA. A STA in an IBSS or an AP shall not send an IEEE 802.1X Authentication diagnostic request with a designated BSS that is not part of the ESS, or IBSS and a STA receiving a diagnostic request shall reject requests to perform diagnostics on other networks. All parameters required to complete the operation including the association and IEEE 802.1X authentication are included in the Diagnostic Request element.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an IEEE 802.1X Authentication Diagnostic Request, the STA determines whether or not to accept the request. If the STA declines the request, it shall send a Diagnostic Report frame with the Status field of a Diagnostic Report element set to Refused. If the STA accepts the request, it shall issue an MLME-DEAUTHENTICATE.request to deauthenticate from the current AP, and an MLME-AUTHENTICATE.request to authenticate to the AP indicated in the Diagnostic Request element. If successful, the STA shall issue an MLME-(RE)ASSOCIATE.request to the AP indicated in the Diagnostic Request element. The STA shall then issue an MLME-DEAUTHENTICATE.request to deauthenticate from the AP indicated in the Diagnostic Request element, reassociate to the AP from which it received the Diagnostic Request, and issue an MLME-DIAGREPORT.request primitive, indicating the results of the diagnostic.

References: