IEEE P802.11
Wireless LANs

| Text for RFI Comments Related to Sequence Numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Bahr | Siemens Enterprise Communications | Otto-Hahn-Ring 6 80200 München, Germany | +49-89-636-49926 | bahr at siemens dot com |
| Jan Kruys | Cisco Systems | Cisco Way Bld 14 San Jose, CA | + 31 348 453719 | jkruys at cisco dot com |

Abstract

This document contains changes to the text of the draft standard in order to resolve many comments related to sequence numbers and to the clarification of originator, destination, and target.

This document contains marked changes to the of the draft standard in order to resolve many comments related to (destination) sequence numbers and to the confusion of originator, destination and target.

Resolved Comments:

26 comments will be resolved with this document:

CIDs 98, 491, 510, 522, 524, 540, 548, 594, 596, 597, 598, 599, 600, 677, 680, 776, 1034, 1111, 1634, 1637, 1640, 1641, 1646, 1648, 1650, 1681

The spreadsheet 11-08/0950r0 contains proposed resolutions to 5 further comments related to the same topic: CIDs 487, 489, 490, 543, 549

Used Base Version:

The changes are based on draft standard version D2.0.

Notes to Editor:

- The changes are marked with Winword change tracking.

- Since the changes are based on D2.0., but the current version is already D2.02, some of the proposed changes might not be applicable directly. The editor shall

  - try to see if the changes can be adopted in a straightforward and unambiguous way to D2.02 as well. If yes, he should incorporate the changes in D2.02.

  - If this is not possible, he should discuss the implementation of the proposed changes with the authors.

  - If no acceptable solution can be found, a new document should be provided based on the then latest, available version of the draft standard.

Proposed Changes:

Instruction to Editor: Change the following acronym as shown

- Abbreviations and acronyms

SN	Sequence number

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.94 as shown:

- PANN information element

The Portal Announcement (PANN) element is used for announcing in the mesh the presence of an MP configured as a mesh portal (that has a live connection to an external network). MPs may use this information to increase the efficiency of communication with stations outside the mesh.

The format of the PANN element is shown in PANN  element.

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | Mesh Portal Address | PANN Sequence Number | Metric |
| - PANN element |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 17.

The Flags field is reserved for future use.

The Hop Count field is coded as an unsigned integer and indicates the number of hops from the originating mesh portal to the MP transmitting the request.

The Time to Live field is coded as an unsigned integer and indicates the maximum number of hops allowed for this element.

The Mesh Portal Address is represented as a 48-bit MAC address and is set to the MAC address of the MP that is collocated with the portal.

The PANN Sequence Number field is coded as an unsigned integer and is set to a sequence number specific for the mesh portal.

The Metric field is coded as an unsigned integer and indicates a cumulative metric from the originating mesh portal to the MP transmitting the announcement.

Detailed usage of the PANN element is described in 11B.7.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.95 as shown:

- RANN information element

The RANN information element is used for announcing the presence of an MP configured as Root MP. RANN elements are sent out periodically by the Root MP.

The format of the RANN element is shown in RANN  element.

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | Root MP Address | HWMP Sequence Number | Metric |
| - RANN element |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 21.

The Flags field is set as follows. Bit 0: Portal Role (0 = non-portal, 1 = portal). Bit 1 – 7: Reserved

The Hop Count field is coded as an unsigned integer and indicates the number of hops from the originating root MP to the MP transmitting the request.

The Time to Live field is coded as an unsigned integer and indicates the remaining number of times the RANN may be forwarded.

The Root MP Address field is represented as a 48-bit MAC address and is set to the Root MP MAC address.

The HWMP Sequence Number is coded as an unsigned integer and is set to the HWMP sequence number specific to the root MP.

The Metric field is coded as an unsigned integer and is set to the cumulative metric from the originating root MP to the MP transmitting the announcement.

Detailed usage of the RANN element is described in 11B.9.8.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.96 as shown:

- PREQ information element

The PREQ element is used for discovering a path to one or more target MPs, building a proactive (reverse) path selection tree to the root MP, and confirming a path to a target MP (optional).

The format of the PREQ element is shown in PREQ element.

| Octets:1 | 1 | 1 | 1 | 1 | 4 | 6 | 4 | 0 or 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | PREQ ID | Originator MP Address | Originator HWMP Sequence Number | Proxied Address | Lifetime |
| 4 | 1 | 1 | 6 | 4 | ... | 1 | 6 | 4 |  |
| Metric | Target Count | Per Target Flags #1 | Target Address #1 | Target HWMP Sequence Number #1 | ... | Per Target Flags #N | Target Address #N | Target HWMP Sequence Number #N |  |
| - PREQ element |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 37 to 255 octets.

The Flags field is set as follows. Bit 0: Portal Role (0 = non-portal, 1 = portal), Bit 1: (0 = group addressed, 1 = individually addressed) (see 11B.9.3), Bit 2: Proactive PREP (0 = off, 1 = on), Bit 3 – 5: Reserved, Bit 6: Address Extension (AE) (1= (target count ==1 && proxied device address present), 0 = otherwise), Bit 7: Reserved.

The Hop Count field is coded as an unsigned integer and is set to the number of hops from the originator to the MP transmitting the request.

The Time to Live field is coded as an unsigned integer and is set to the maximum number of hops allowed for this element.

The PREQ ID field is coded as an unsigned integer and is set to some unique ID for this PREQ.

The Originator MP Address field is represented as a 48-bit MAC address and is set to the originator MAC address.

The Originator HWMP Sequence Number is coded as an unsigned integer and is set to the HWMP sequence number specific to the originator.

The Proxied Address field is the MAC address of a proxied entity (e.g. STA) in case the PREQ is generated because of a frame received from outside the mesh (e.g. BSS) and the proxied entity is the source of the frame. This field is only present if the AE flag is set to 1 and is represented as a 48-bit MAC address.

The Lifetime field is coded as an unsigned integer and is set to the time for which MPs receiving the PREQ consider the forwarding information to be valid. The lifetime is measured in TUs.

The Metric field is coded as an unsigned integer and is set to the cumulative metric from the originator to the MP transmitting the PREQ.

The Target Count N field is coded as an unsigned integer and gives the number of targets (N) contained in this PREQ.

The format of the Per Target Flags field is shown in PREQ Per Target Flags field format.

| B0 | B1 | B2 | B3 B7 |
| --- | --- | --- | --- |
| TO | RF | USN | Reserved |
| Bits: 1 | 1 | 1 | 5 |
| - PREQ Per Target Flags field format |  |  |  |

Per Target Flags are set as follows.

Bit 0: TO (Target Only): If TO=0, an intermediate MP with active forwarding information to the corresponding target responds to the PREQ with a unicast PREP; if TO=1, only the target MP can respond with a unicast PREP. The default value is 1.

Bit 1: RF (Reply-and-Forward): The RF flag controls the forwarding of PREQ at intermediate MPs. When TO=0 and the intermediate MP has active forwarding information to the corresponding target, the PREQ is not forwarded if RF=0 and forwarded if RF=1. The default value is 1. When TO=1, the RF flag has no effect.

Bit 2: USN (Unknown Target HWMP Sequence Number): The USN flag indicates whether the Target HWMP Sequence Number field of the corresponding target can be interpreted as HWMP sequence number (USN=0) or not (USN=1), the latter meaning that a target HWMP sequence number is unknown at the originator MP.

Bit 2-7: Reserved

The Target Address is represented as a 48-bit MAC address.

The Target HWMP Sequence Number field is coded as an unsigned integer and is the latest known HWMP sequence number received in the past by the originator MP for any path towards the target MP. If such a target HWMP sequence number is not known, the USN bit is set to 1 and the contents of this field is ignored.

Detailed usage of the PREQ element is described in 11B.9.5.

The PREQ element may be transmitted to a neighbor peer MP via either unicast or broadcast. A “unicast PREQ” is a PREQ element contained in a management frame that is unicast to a neighbor peer MP. A “broadcast PREQ” is a PREQ element contained in a management frame that is broadcast to all neighbor peer MPs.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.97 as shown:

- PREP information element

The PREP element is used to establish a forward path to a target and to confirm that a target is reachable.

The format of the PREP element is shown in Path Reply element.

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 0 or 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Flags | Hopcount | Time to Live | Target MP Address | Target HWMP Sequence Number | Target Proxied Address | Lifetime |
| 4 | 6 | 4 |  |  |  |  |  |  |
| Metric | Originator MP Address | Originator HWMP Sequence Number |  |  |  |  |  |  |
| - Path Reply element |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 32 to 255 octets.

The Flags field is set as follows. Bit 0-5: Reserved. Bit 6: Address Extension (AE) (1=proxied address present, 0=otherwise). Bit 7: Reserved.

The Hop Count field is coded as an unsigned integer and is set to the number of hops from the path target to the local MP.

The Time to Live field is coded as an unsigned integer and is set to the maximum number of hops allowed for this element.

The Target MP Address is the MAC address of the target MP and is represented as a 48-bit MAC address.

The Target HWMP Sequence Number field is coded as an unsigned integer and is set to the HWMP SN of the target MP.

The Target Proxied Address field is set to the proxied address on behalf of which the PREP is sent. This field is present only if Bit 6 (AE) in Flags = 1.

The Lifetime field, if applicable, reflects the Lifetime of the PREQ this PREP responds to, and is coded as an unsigned integer. The lifetime is measured in TUs.

The Metric field is coded as an unsigned integer and indicates the cumulative metric from the path target to the local MP.

The Originator MP Address field is represented as a 48-bit MAC address and is set to the MAC address of the originator, which is contained in the PREQ.

The Originator HWMP Sequence Number field is coded as an unsigned integer and is set to the HWMP sequence number of the originator MP contained in the PREQ.

The detailed usage of the PREP element is described in 11B.9.6.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.98 as shown:

- PERR Information element

The Path Error (PERR) element is used for announcing a broken link to all traffic sources that have an active path over this broken link.

The format of the PERR element is shown in Path Error element.

| Octets: 1 | 1 | 1 | 1 | 6 | 4 |
| --- | --- | --- | --- | --- | --- |
| ID | Length | Mode Flags | Number of Destinations N | Destination Address #1 | HWMP Sequence Number #1 |
| - Path Error element |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 13 octets.

The Mode Flags field is reserved.

The Number of Destinations N field is coded as an unsigned integer and indicates the number of announced destinations in PERR (destination address and HWMP sequence number).

The Destination Address field is represented as a 48-bit MAC address indicates the detected unreachable destination MAC address.

The HWMP Sequence Number field is coded as an unsigned integer and indicates the HWMP sequence number of detected unreachable destination MP.

The detailed usage of the PERR element is described in 11B.9.7.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.99 as shown:

- Proxy Update (PU) information element

The Proxy Update information element is transmitted by a source MP to a destination MP to update its proxy information. This element is transmitted using individual addresses. The format of PU element is shown in Proxy Update (PU) Element.

| Octets: 1 | 1 | 1 | 1 | 6 | 2 | 6 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Flags | PU Sequence Number | Proxy Address | Number of proxied addresses (N) | Proxied MAC Address #1 | ... | Proxied MAC Address #N |
| - Proxy Update (PU) Element |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element.

The length is set to 12 + N*6.

The Flags field is set as follows. Bit 0: (0 = add proxy information, 1 = delete proxy information). Bit 1 – 7: Reserved

.

The PU Sequence Number field is coded as an unsigned integer and is set to the sequence number of the PU. The Source MP shall set the PU Sequence Number field in the PU Element to a value from a single modulo-256 counter that is incrementing by 1 for each new PU

The Proxy Address field is represented as a 48-bit MAC address and is set to the MAC address of proxy MP.

The Number of Proxied Address field is coded as an unsigned integer and is set to the number of proxied addresses reported to the destination MP.

The Proxied MAC Address is represented as a 48-bit MAC address and is the MAC address of the proxied entities.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.100 as shown:

- Proxy Update Confirmation (PUC) information element

The Proxy Update Confirmation element is transmitted by an MP in response to a PU. This element is transmitted using individual addresses. The format of PUC element is shown in Proxy Update Confirmation (PUC) element.

| Octets: 1 | 1 | 1 | 1 | 6 |
| --- | --- | --- | --- | --- |
| ID | Length | Flags | PU Sequence Number | Destination MP Address |
| - Proxy Update Confirmation (PUC) element |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element.

The length is set to 10.

The Flags field is set as follows. Bit 0 – 7: Reserved

The PU Sequence Number field is coded as an unsigned integer and is the PU sequence number of the received PU which is being confirmed.

The Destination MP Address is represented as a 48-bit MAC address and is set to the MAC address of the recipient of the PU.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.4.14.1 as shown:

- Path Request frame format

The Path Request frame is transmitted by an originator MP to discover the path to the target using the HWMP protocol defined in 11B.9. This frame may be transmitted using group addresses or individual addresses. The frame body of a Path Request frame contains the information shown in Path Request frame body.

| - Path Request frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | Path Request element |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Error: Reference source not found for this action frame type.

The Path Request element is set as described in Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.3.2.96 as shown:.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 7.4.14.1 as shown:

- Path Reply frame format

The Path Reply frame is transmitted by a target MP to a originator MP in the mesh to determine the path between the source and destination MP using the HWMP protocol defined in 11B.9. This frame may be transmitted using group addresses or individual addresses. The frame body of a Path Reply frame contains the information shown in Path Reply frame body.

| - Path Reply frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | Path Reply element |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Error: Reference source not found for this action frame type.

The Path Reply element is set as described in PREP information element.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.7.2.1 as shown:

- Function

The Portal Announcement (PANN) element, described in 7.3.2.94, is used to announce the presence of an MP configured as an MPP in the mesh. Portal Announcements allow MPs to select the appropriate MPP and build a path towards it. An MP which receives a Portal Announcement message shall propagate the Portal Announcement as described below but may ignore its content.

The PANN sequence number of the Portal Announcement shall be incremented by the MPP before each transmission.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.7.2.2 as shown:

- Conditions for generating and sending a PANN

An MP shall transmit a PANN in the following cases:

Case A: Original transmission

All of the following applies:

- The MP is configured as an MPP

- At every PORTAL_ANNOUNCEMENT_INTERVAL

The content of a PANN element in Case A shall be as shown in Content of a PANN element in Case A.

| - Content of a PANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PANN element |
| Length | As required |
| Flags | Not used |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for the Portal Announcement |
| Mesh Portal Address | MPP MAC address |
| PANN Sequence Number | A sequence number specific for the MPP |
| Metric | Initial value of active path selection metric |

Case B:	Forwarding

All of the following applies:

- The MP has received a Portal Announcement

- PORTAL_PROPAGATION_DELAY has expired

- The decremented TTL of the Portal Announcement is equal to or greater than 1

The content of a PANN element in Case B shall be as shown in Content of a PANN element in Case B.

| - Content of a PANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PANN element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Mesh Portal Address | As received |
| PANN Sequence Number | As received |
| Metric | As receivedown metric toward the transmitting MP |

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.7.2.3.1 as shown:

- Acceptance criteria

The Portal Announcement element shall not be accepted (and shall not be processed as described in Error: Reference source not found) if

- the PANN Sequence Number of the Portal Announcement is lower than the PANN Sequence Number of the previously received Portal Announcement from this MPP, or

- the PANN Sequence Number of the Portal Announcement is the same as the PANN Sequence Number of the previously received Portal Announcement from this MPP, and the metric is worse that the metric of the previously received Portal Announcement from this MPP

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.7.5.1.2 as shown:

- Conditions for generating and sending a PU

An MP shall send out a PU in the following cases:

- The MP needs to inform a destination MP of its proxy information

- .

- A change is made in the local proxy information of the MP due to addition or deletion of proxy entries.

- If a root receives a PU element with a add proxy information or if it detects a new proxied address in its DS (e.g. through bridge learning) and the root already has proxy information for one or more of these proxied address, it shall send a PU element with Bit 0 set to 1 (delete proxy information) to each of the old proxy. PU element in this case includes the list of corresponding proxied addresses which should be removed by the proxy.

- On a periodic basis to refresh the proxy information at the destination MP

The PU element is transmitted via unicast from the MP to a destination MP. This request may be repeated after every PU_ TIMEOUT for MAX_PU times until the MP receives a PUC element.

The content of a PU element shall be as shown in Content of a PU element.

| - Content of a PU element |  |
| --- | --- |
| Field | Value/description |
| ID | Value given in Table 7-26 for the PU element |
| Length | Length of the element |
| Flags | Bit 0: 0: add proxy information; 1: delete proxy information 1 – 7: Reserved |
| PU Sequence number | Sequence number of the PU |
| Proxy Address | MAC address of the proxy MP |
| Number of Proxied Device Address (N) | Number of proxied addresses reported to the destination MP |
| Proxied Device MAC Address | MAC address of proxied entities to which the MP is providing mesh services |

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.7.5.2.2 as shown:

- Conditions for generating and sending a PUC

A PUC is unicast from the destination MP to the MP that sent the PU.

The content of a PUC element shall be as shown in Content of a PUC element.

| - Content of a PUC element |  |
| --- | --- |
| Field | Value/description |
| ID | Value given in Table 7-26 for the PUC element |
| Length | Length of the element |
| Flags | Bit 0- 7: Reserved |
| PU Sequence number | PU Sequence number of the PU which is being confirmed |
| Destination MP Address | MAC address of the recipient of the PU |

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.7.5.2.3 as shown:

- PUC processing

On receiving a PUC element in a PUC frame, the MP shall no longer send any PU with the same PU sequence number.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.1.1 as shown:

- General

The Hybrid Wireless Mesh Protocol (HWMP) is a mesh path selection protocol that combines the flexibility of on-demand path selection with proactive topology tree extensions. The combination of reactive and proactive elements of HWMP enables optimal and efficient path selection in a wide variety of mesh networks (with or without infrastructure).

HWMP uses a common set of protocol primitives, generation and processing rules inspired by Ad Hoc On Demand Distance Vector (AODV) protocol [IETF RFC 3561] adapted for MAC address-based path selection and link metric awareness. HWMP is completely specified herein and does not require reference to AODV.

HWMP supports two modes of operation depending on the configuration.  These modes are:

- On demand mode: this mode allows MPs to communicate using peer-to-peer paths.  The mode is used in situations where there is no root MP configured.  It is also used in certain circumstances if there is a root MP configured and an on demand path can provide a better path to a given node in the mesh.

- Proactive tree building mode: this can be performed by using either the PREQ or RANN mechanism.

These modes are not exclusive: on demand and proactive modes may be used concurrently. One example of concurrent usage of on-demand and proactive mode is for two MPs that are part of the same mesh to begin communicating using the proactively built tree but subsequently to perform an on-demand discovery for a direct path between each other. This type of concurrent usage of the proactive and on-demand modes allows communication to begin immediately while an on-demand discovery finds a more optimal path between two MPs.

All HWMP modes of operation utilize common processing rules and primitives.  HWMP information elements are the Path Request (PREQ), Path Reply (PREP), Path Error (PERR) and Root Announcement (RANN).  The metric cost of the links determines which paths HWMP builds. In order to propagate the metric information between MPs, a metric field is used in the PREQ, PREP and RANN elements.

Path selection in HWMP uses a sequence number mechanism to assure that MPs can distinguish current path information from stale path information at all times in order to maintain loop-free connectivity.  Each MP maintains its own HWMP sequence number, which is propagated to other MPs in the HWMP information elements. Rules for maintaining HWMP sequence numbers are given in Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.2 as shown: HWMP Sequence Numberin).

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.1.2 as shown:

- On demand path selection mode

If a source MP needs to find a path to a destination MP using the on demand path selection mode, it broadcasts a PREQ with the target MP specified in the list of targets and the metric field initialized to the initial value of the active path selection metric.

When an MP receives a new PREQ it creates or updates its path information to the originator MP and propagates (re-broadcast) the PREQ to its neighbor peer MPs if the PREQ contains a greater HWMP sequence number, or the HWMP sequence number is the same as the current path and the PREQ offers a better metric than the current path. Each MP may receive multiple copies of the same PREQ that originated at the originator MP, each PREQ traversing a unique path.

Whenever an MP forwards a PREQ, the metric field in the PREQ is updated to reflect the cumulative metric of the path to the originator MP.  After creating or updating a path to the originator MP, the target MP sends a unicast PREP back to the originator MP.

The PREQ provides “Target Only” (TO) and “Reply and Forward” (RF) flags that allow path selection to take advantage of existing paths to the target MP by allowing an intermediate MP to return a PREP to the originator MP. If the TO flag is set to 1, only the target MP sends a PREP. The effect of setting the TO flag to 0 is the quick establishment of a path using the PREP generated by an intermediate MP, allowing the forwarding of data frames with a low path selection delay. The effect of setting the RF flag to 1 (in addition to setting the TO flag to 0) is the selection (or validation) of the best path after the path selection procedure has completed. If the RF flag is set to 1, the first intermediate MP that has a path to the target sends a PREP and propagates the PREQ with the TO flag set to 1 to avoid all intermediate MPs sending a PREP.

Intermediate MPs create a path to the target MP on receiving the PREP, and also forward the PREP toward the originator MP. When the originator receives the PREP, it creates a path to the target MP. If the target MP receives further PREQs with a better metric, then the target updates its path to the originator to the new path and also sends a new PREP to the originator along the updated path. A bidirectional, best metric end-to-end path is established between the originator and target MP.

In HWMP the PREQ processing at intermediate MPs is controlled per target.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.1.3.1 as shown:

- Proactive PREQ mechanism

The PREQ tree building process begins with a proactive Path Request element sent by the root MP, with the Target Address set to all ones, the TO flag set to 1 and the RF flag set to 1. The PREQ contains the path metric (set to the initial value of the active path selection metric by the root MP) and an HWMP sequence number.  The proactive PREQ is sent periodically by the root MP, with increasing HWMP sequence numbers.

An MP hearing a proactive PREQ creates or updates its forwarding information to the root MP, updates the metric and hop count of the PREQ, records the metric and hop count to the root MP, and then transmits the updated PREQ. Information about the presence of and distance to available root MP(s) is disseminated to all MPs in the network.

Each MP may receive multiple copies of a proactive PREQ, each traversing a unique path from the root MP to the MP. An MP updates its current path to the root MP if and only if the PREQ contains a greater HWMP sequence number, or the HWMP sequence number is the same as the current path and the PREQ offers a better metric than the current path to the root MP.  The processing of the proactive PREQ is the same as in the on-demand mode described in On demand path selection mode.

If the proactive PREQ is sent with the “Proactive PREP” bit set to 0, the recipient MP may send a proactive PREP if required (for example, if the MP has data to send to the root MP and requires establishing a bidirectional path with the root MP). If the PREQ is sent with a “Proactive PREP” bit set to 1, the recipient MP shall send a proactive PREP. The proactive PREP establishes the path from the root MP to the MP.  When the path from an MP to a root MP changes, and the root MP PREQ was sent with a “Proactive PREP” bit set to 1, the recipient MP shall send a proactive PREP to the root MP containing the addresses of the MPs that have established a path to the root MP through the current MP.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.1.3.2 as shown:

- Proactive RANN mechanism

The root MP periodically propagates a RANN element into the network.  The information contained in the RANN is used to disseminate path metrics to the root MP.

Upon reception of a RANN, each MP that has to create or refresh a path to the root MP sends a unicast PREQ to the root MP via the MP from which it received the RANN.

The unicast PREQ follows the same processing rules defined in the on demand mode.

The root MP sends a PREP in response to each PREQ.  The unicast PREQ creates the reverse path from the root MP to the originator MP, while the PREP creates the forward path from the MP to the root MP.

When the path from an MP to a root MP changes, it may send a PREP with the addresses of the MPs that have established a path to the root MP through the current MP.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.3 as shown:

- Terminology

This subclause describes terminology for HWMP, especially for the process of path discovery. Terms such as Path Originator or Path Target are assigned to very specific MPs within the path discovery process. They stay with the same assigned MP for the whole path discovery process.Illustration of definitions illustrates an example utilizing this terminology.

- Illustration of definitions

Note: Both the path target and the path originator are a path destination for the forward path and the reverse path respectively.

The following terms are used within the context of a single PREQ/PREP pair, a so-called HWMP path discovery.

- path originator:

- The path originator is the MP that triggers the path discovery.

- path originator address:

- The MAC address of the path originator.

- path target:

- The path target is the MP to which the path originator attempts to establish a path.

- path target address:

- The MAC address of the path target.

- intermediate MP:

- The intermediate MP is the MP which participates in path selection and is neither path originator nor path target.

- intermediate MP address:

- The MAC address of the intermediate MP.

- forward path:

- The forward path is the path to the path target, set up at the path originator and intermediate MPs.

- reverse path:

- The reverse path is the path to the path originator, set up at the path target and intermediate MPs.

- forwarding information: The forwarding information maintained by an originator MP, an intermediate MP, or a target MP that allows the MP to perform its path selection and forwarding functions.

The terminology used when discussing Forwarding Information is relative to the MP (reference MP, given MP, or local MP) and a particular target of the path. The following terms are specific to a given instance of the Forwarding Information.:

- destination MP:

- The end point of a (forward or reverse) path.

- destination MP address:

- The MAC address of the path destination.

- next hop MP:

- The next hop MP is a neighbor peer MP on the path to the destination MP.

- next hop MP address:

- The MAC address of the next hop MP.

- precursor MP:

- A precursor MP is an MP that identifies a given MP as the next hop MP to some destination MP.

- precursor MP address:

- The MAC address of the precursor MP.

Precursor and Next Hop Examples shows the roles of the various MPs in the Forward Path and Reverse Path generated as a result of the full PREQ and/PREP processing as shown in Illustration of definitions.  Each row in the table contains the roles of a forward/reverse path from the reference MP’s perspective.

| - Precursor and Next Hop Examples |  |  |  |
| --- | --- | --- | --- |
| Forward Path (to Path Target) |  |  |  |
| Reference MP | Precursor MP | Next Hop MP | Destination MP |
| Path Originator | N/A | Intermediate 1 | Path Target |
| Intermediate 2 | Intermediate 1 | Intermediate 3 | Path Target |
| Path Target | Intermediate 3 | N/A | Path Target |
| Reverse Path (to Path Originator) |  |  |  |
| Reference MP | Precursor MP | Next Hop MP | Destination MP |
| Path Originator | Intermediate 2 | N/A | Path Originator |
| Intermediate 2 | Intermediate 3 | Intermediate 1 | Path Originator |
| Path Target | N/A | Intermediate 3 | Path Originator |

- unknown destination: A destination MP is considered unknown if the MP does not have any forwarding information for that MP.

- unreachable destination: A destination MP is considered unreachable if the MP does not have valid forwarding information for that MP.

- HWMP Sequence Number (SN):

- Each mesh HWMP path selection element contains an HWMP sequence number that allows recipients to distinguish newer from stale information. An HWMP sequence number is specific to an MP. See also Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.2 as shown:.

- target sequence number:

- The HWMP sequence number of the target MP.

- time-to-live (TTL): An integer number that is used to limit the number of hops an HWMP Information Element may be processed and propagated.  Note that this TTL is not related to the TTL in the mesh header (see 7.1.3.5b).

- root MP:

- A root MP is the root of a path selection tree.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4 as shown:

- General rules for processing HWMP information elements

This subclause describes the rules for the processing of the following components of the HWMP information elements:

- HWMP Sequence Number

- TTL

- Metric

Note: It is assumed that the receiving MP only accepts HWMP elements from MPs with which it has established a peer link. Therefore, all HWMP elements accepted are presumed to have originated in the same mesh network that the receiving MP belongs to.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.2 as shown:

- HWMP Sequence Numbering

An HWMP sequence number is included in the PREQ, PREP, PERR, and RANN information elements. The HWMP sequence number in the forwarding information is updated whenever an MP receives new (i.e. not stale) information about the HWMP sequence number from a PREQ, PREP, or PERR that may be received relative to that target MP or destination MP. HWMP depends on each MP in the network to own and maintain its HWMP sequence number to guarantee the loop-freedom of all paths towards that MP. An MP increments its own HWMP sequence number in two circumstances:

—	Immediately before an originator MP starts a path discovery, it shall increment its own HWMP sequence number. This prevents conflicts with previously established reverse paths towards the originator MP. However, it might be advantageous not to increment the HWMP sequence number too frequently. An optional mechanism for achieving this is described in 11B.9.4.7.

—	Immediately before a target MP originates a PREP in response to a PREQ, it shall update its own HWMP sequence number to the maximum of its current HWMP sequence number and the target HWMP sequence number in the PREQ.

HWMP sequence numbers are processed as follows:

- HWMP sequence numbers are incremented monotonically as unsigned integers.

- Comparing HWMP sequence numbers is done using signed 32-bit arithmetic. This is necessary to assure correct results if the sequence number rolls over (changes to zero). If the result of subtracting the stored HWMP sequence number SN_1 from the incoming HWMP sequence number SN_2 is less than zero, then the information associated with SN_2 is stale and shall not be processed.

In general, when an MP receives an information element with an HWMP sequence number that is less than the last received HWMP sequence number for that MP, it discards the received information element. If they are the same, the outcome (information element processed or not) depends on the type of the information element and some additional conditions. These cases are noted in the applicable information element descriptions.

The only other circumstance in which an MP may change the HWMP sequence number in one of its forwarding information entries is in response to a lost or expired link to the next hop towards that destination MP. The MP determines which destinations use a particular next hop by consulting its forwarding information.  In this case, for each destination that uses the next hop, the MP increments the HWMP sequence number in the forwarding information and marks the path as invalid (see also Error: Reference source not found). Whenever any forwarding information containing an HWMP sequence number at least equal to the recorded HWMP sequence number for an affected destination is received by an MP that has marked that forwarding information as invalid, the MP shall update its forwarding information according to the information contained in the update.

An MP may change the HWMP sequence number in the forwarding information of a destination MP if the link to the Next Hop MP towards the destination MP breaks.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.4 as shown:

- Forwarding information

The forwarding information consists of at least a destination MP address, the HWMP sequence number (SN), the Next Hop address, the path metric, the precursor list, and the lifetime of this forwarding information. Stored forwarding information can be active or inactive. The latter means that the forwarding information is still known for future reference but not used for forwarding. The Forwarding Information becomes inactive when either the MP receives a RERR regarding the destination MP or the information is no longer active according to the lifetime.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.5 as shown:

- Creation and update of forwarding information

HWMP path selection information elements create or update the forwarding information in the MPs that process these information elements. The creation and update of forwarding information follows the same rules for PREQ, PREP, and RANN. These rules are given below. Unless otherwise noted, the term “HWMP_IE” stands for the information element under consideration (PREQ, PREP, or RANN). In the following text, the “” indicates an entry into the local forwarding data base, i.e., the path selection table.

- If the MP does not have any valid forwarding information to the originator MP (PREQ) or the target MP (PREP),  it creates this information from the field Originator MP Address (PREQ) or Target Address (PREP) (field: destination), the corresponding HWMP sequence number from the field Originator HWMP Sequence Number (PREQ) or Target HWMP Sequence Number (PREP) (field: HWMP sequence number), the transmitter address of the management frame containing the HWMP_IE (field: next hop), the accumulation of the value of field hwmp_ie.metric with the metric of the last link (field: path metric), and the value of field hwmp_ie.lifetime (field: lifetime).

- If the MP does not have any valid forwarding information to the transmitter of the HWMP_IE, it creates this information from the transmitter address of the management frame containing the HWMP_IE (field: destination and next hop), the HWMP sequence number is set to invalid (field: HWMP sequence number), the metric of the last link (field: path metric), and the value of field hwmp_ie.lifetime (field: lifetime).

- If the MP has valid forwarding information to the originator MP (PREQ) or target MP (PREP), and the acceptance criteria of the corresponding element as described in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.3.1 as shown:, Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.1 as shown:, Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.3.1 as shown:, Error: Reference source not found are satisfied, then the MP updates this forwarding information with the transmitter address of the management frame containing the HWMP_IE (field: next hop), the value of ion HWMP sequence number is setmberOriginator HWMP Sequence Number (PREQ) or Target HWMP Sequence Number (PREP) (fietionHMP _sequece _number), the accumulation of the value of field hwmp_ie.metric with the metric of the last link (field: path metric), and the larger one of the lifetime of the stored forwarding information and the value of field hwmp_ie.lifetime (field: lifetime).

- If the MP has valid forwarding information to the transmitter of the HWMP_IE and if the path metric of this information is worse than the metric of the last link, then the MP updates this forwarding information with the transmitter address of the management frame containing the HWMP_IE (field: next hop), the HWMP sequence number is set to invalid (field: HWMP sequence number), the metric of the last link (field: path metric), and the larger one of the lifetime of the stored forwarding information and the value of field hwmp_ie.lifetime (field: lifetime).

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.6 as shown:

- Repeated attempts at path discovery

Repeated attempts by an MP at path discovery towards a given (set of) target(s) shall be limited to dot11MeshHWMPmaxPREQretries and utilize a binary exponential backoff between transmissions. The minimum waiting time for the PREP corresponding to a PREQ is 2 * dot11MeshHWMPnetDiameterTraversalTime.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.4.7 as shown:

- Limiting the rate of HWMP sequence number increments (optional)

In order to improve path stability (and further reduce overhead), an MP may use the same originator HWMP sequence number for a certain time interval.  In this case, the originator HWMP SN shall be incremented only after at least dot11MeshHWMPnetDiameterTraversalTime has elapsed since the previous increment. This mechanism prevents MPs from changing the path frequently to the originator MP every time the originator MP sends a burst of PREQs within a very short time.  This element of the protocol allows an originator MP to immediately initiate on-demand path discovery to a new target without affecting recently refreshed paths to the originator in other MPs.

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.1 as shown:

- Function

The Path Request (PREQ) element, described in 7.3.2.96, is used for three purposes:

- Discovering a path to one or more targets

- Building a proactive (reverse) path selection tree to the root MP

- Confirming a path to a target (optional)

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.2 as shown:

- Conditions for generating and sending a PREQ

An MP shall send a PREQ element in a PREQ frame in the following cases:

Case A: Original Transmission (Path Discovery)

All of the following applies:

- The MP needs to establish an on-demand path to one or more target MPs or the proxy of a given target for which there is no ongoing path discovery initiated by this MP.

- The MP has not sent a PREQ element less than dot11MeshHWMPpreqMinInterval TUs ago. If this is the case, the transmission of the PREQ has to be postponed until this condition becomes true.

- The MP has not made more than (dot11MeshHWMPmaxPREQretries – 1) repeated attempts at path discovery towards the target of the PREQ.

The content of a PREQ element in Case A shall be as shown in Contents of a PREQ in Case E.

| - Content of a PREQ element in Case A |  |  |
| --- | --- | --- |
| Field |  | Value |
| ID |  | Value given in Table 7-26 for the PREQ element |
| Length |  | 27 + N*11 |
| Flags |  | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: Address Extension (AE) (1=if (target_count=1 and proxied address present), 0=otherwise) Bit 7: Reserved |
| Hop Count |  | 0 |
| Time to Live |  | Maximum number of hops allowed for this information element, e.g., HWMP_NET_DIAMETER. |
| PREQ ID |  | Previous PREQ ID + 1 |
| Originator MP Address |  | MAC address of the originator of the PREQ |
| Originator HWMP Sequence Number |  | Previous Originator MP HWMP SN + 1. See Note 2 |
| Proxied Address |  | Present only if Bit 6 in Flags = 1. This value is set to the proxied address which is the source of the frame. |
| Lifetime |  | The time for which MPs receiving the PREQ consider the forwarding information to be valid, e.g. dot11MeshHWMPactivePathTimeout. |
| Metric |  | Initial value of active path selection metric |
| Target Count |  | (N) |
| Per Target | Flags | The TO flag is set to dot11MeshHWMPTargetOnly and the RF flag is set to dot11MeshHWMPReplyAndForward Bit 0 (TO): dot11MeshHWMPTargetOnly Bit 1 (RF): dot11MeshHWMPReplyAndForward Bit 2 (USN): 0 if forwarding information for Target Address with valid HWMP sequence number exists, 1 otherwise |
|  | Target Address | MAC address of requested target MP |
|  | Target HWMP Sequence Number | The latest HWMP sequence number stored by the originator MP for the target MP from the forwarding information (see 11A.8.4.4 Forwarding information). |

Case B:	Original Transmission (Path Maintenance) (optional implementation enhancement)

All of the following applies:

- the MP has a path to a given target that is not a Root MP

- the last PREQ to this target was sent dot11MeshHWMPmaintenanceInterval TUs (or more) ago.

The content of a PREQ in Case B shall be as shown in Content of a PREQ element in Case B.

| - Content of a PREQ element in Case B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field |  | Value |  |  |
| ID |  | Value given in Table 7-26 for the PREQ element |  |  |
| Length |  | As required |  |  |
| Flags |  | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |  |  |
| Hop Count |  | 0 |  |  |
| Time to Live |  | Maximum number of hops allowed for this information element = HWMP_NET_DIAMETER. |  |  |
| PREQ ID |  | Previous PREQ ID +1 |  |  |
| Originator MP Address |  | MAC address of the originator of the PREQ |  |  |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Note 2 under Case A. |  |  |
| Lifetime |  | The time for which MPs receiving the PREQ consider the forwarding information to be valid, e.g. dot11MeshHWMPactiveRootTimeout. |  |  |
| Metric |  | Initial value of active path selection metric |  |  |
| Target Count |  | N |  |  |
| Per Target | Flags |  | TO flag = 1 RF flag = 0 Bit 0 (TO): 1 Bit 1 (RF): 0 Bit 2 (USN): 0 if forwarding information for Target Address with valid HWMP sequence number exists, 1 otherwise |  |
|  | Target Address |  | MAC Address of target MP |  |
|  | Target HWMP Sequence Number |  | The latest HWMP sequence number for this target known to the originator MP |  |

Case C: Root Path Confirmation.

All of the following applies:

- the MP has received a RANN from a root MP and does have a path to a root MP

- the MP has a path to a root MP and the last PREQ to the root MP was sent dot11MeshHWMPconfirmationInterval TUs (or more) ago.

The content of a PREQ element in Case C shall be as shown in Content of a PREQ element in Case C.

| - Content of a PREQ element in Case C |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Table 7-26 for the PREQ element |  |
| Length |  | As required |  |
| Flags |  | Bit 0: 0 (no portal role) Bit 1: 1 (individually addressed) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |  |
| Hop Count |  | 0 |  |
| Time to Live |  | 1 |  |
| PREQ ID |  | Not used |  |
| Originator MP Address |  | MAC address of the originator MP |  |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Note 2 under Case A. |  |
| Lifetime |  | The time for which MPs receiving the PREQ consider the forwarding information to be valid, e.g. HWMP_ACTIVE_ROOT_TIMEOUT. |  |
| Metric |  | Initial value of active path selection metric |  |
| Target Count |  | 1 |  |
| Per Target | Flags | TO flag = 1, RF flag = 0 Bit 0 (TO): 1 Bit 1 (RF): 0 Bit 2 (USN): 0 |  |
|  | Target Address | Root MP MAC Address |  |
|  | Target HWMP Sequence Number | The latest HWMP sequence number for this target known to the originator MP |  |

Case D:	PREQ Forwarding

Case D1 (target count = 1, no PREP generation):

All of the following applies:

- the MP has received and accepted a PREQ – See Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.1 as shown:

- Target_count = 1

- the MP is not the target of the PREQ OR the target of the PREQ is the MAC broadcast address (all 1’s)

- the MP is not the proxy of the target address

- the MP has no valid forwarding information for the requested target

- [Target Only flag of the target in the PREQ is ON (TO = 1)] 
OR
[{Target Only flag of the target in the PREQ is OFF (TO = 0)} AND {MP has no active forwarding information for the requested target }]

The content of a PREQ element in Case D1 shall be as shown in Content of a PREQ element in Case D1.

| - Content of a PREQ element in Case D1 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Table 7-26 for the PREQ element |  |
| Length |  | 37 |  |
| Flags |  | As received |  |
| Hop Count |  | As received + 1 |  |
| Time to Live |  | As received – 1 |  |
| PREQ ID |  | As received |  |
| Originator MP Address |  | As received |  |
| Originator HWMP Sequence Number |  | As received |  |
| Proxied Address |  | As received. This field is only present if Bit 6 of the Flags field (AE flag) is set to 1. |  |
| Lifetime |  | As received |  |
| Metric |  | As receivedown metric toward transmitter of received PREQ |  |
| Target Count |  | 1 |  |
| Per Target | Flags | As received |  |
|  | Target MAC Address | As received |  |
|  | Target HWMP Sequence Number | As received |  |

Case D2 (target count = 1, PREP generation as intermediate MP):

All of the following applies:

- the MP has received and accepted a PREQ – See Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.1 as shown:

- Target Count = 1

- the MP is not the target of the PREQ

- the MP has active forwarding information for the requested target

- Target Only flag of the target in the PREQ is OFF (TO = 0)

- Reply and Forward flag of the target in the PREQ is ON (RF = 1)

The contents of a PREQ element in Case D2 shall be as shown in Contents of a PREQ element in Case D2.

| - Contents of a PREQ element in Case D2 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Table 7-26 for the PREQ element |  |
| Length |  | 37 |  |
| Flags |  | As received |  |
| Hop Count |  | As received + 1 |  |
| Time to Live |  | As received – 1 |  |
| PREQ ID |  | As received |  |
| Originator MP Address |  | As received |  |
| Originator HWMP Sequence Number |  | As received |  |
| Proxied Address |  | As received. This field is only present if Bit 6 of the Flags field (AE flag) is set to 1. |  |
| Lifetime |  | As received |  |
| Metric |  | As receivedown metric toward transmitter of received PREQ |  |
| Target Count |  | 1 |  |
| Per Target | Flags | Bit 0 (TO): 1 (set to 1 before forwarding because MP sent a PREP) Bit 1 (RF): As received Bit 2 (USN): As received |  |
|  | Target MAC Address | As received |  |
|  | Target HWMP Sequence Number | As received |  |

Case D3 (target count > 1): All of the following applies

- the MP has received and accepted a PREQ – See Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.1 as shown:

- there is at least one requested target left after processing the PREQ according to Error: Reference source not found.

The contents of a PREQ element in Case D3 shall be as shown in Contents of a PREQ element in Case D3.

| - Contents of a PREQ element in Case D3 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Table 7-26 for the PREQ element |  |
| Length |  | 26 + N * 11 |  |
| Flags |  | As received |  |
| Hop Count |  | As received + 1 |  |
| Time to Live |  | As received – 1 |  |
| PREQ ID |  | As received |  |
| Originator MP Address |  | As received |  |
| Originator HWMP Sequence Number |  | As received |  |
| Lifetime |  | As received |  |
| Metric |  | As receivedown metric toward the transmitter of the received PREQ |  |
| Target Count |  | 1  target count  received target count received target count less the number of requested destinations, for which the processing MP - is the target or - has active forwarding information for the requested target and the corresponding Target Only flag is off (TO=0) and Reply and Forward flag is on (RF = 1) |  |
| Per Target #A | Flags #A | As received |  |
|  | Target MAC Address #A | As received |  |
|  | Target HWMP Sequence Number #A | As received |  |
| Per Target #B | Flags #B | Bit 0 (TO): 1 (set to 1 because MP sent PREP) Bit 1 (RF): As received Bit 2 (USN): As received |  |
|  | Target MAC Address #B | As received |  |
|  | Target HWMP Sequence Number #B | As received |  |

For the per target fields (per target flags, target MP MAC address, target HWMP sequence number) assume the following:

- target #A: If target A would have been the only requested target, it would generate a PREQ for propagation according to case D1.

- target #B: If target B would have been the only requested target, it would generate a PREQ for propagation according to case D2.

Case E:	Proactive PREQ (original transmission)

All of the following applies:

- The Root MP is configured to send proactive root PREQs

- The Root Announcement interval has expired

The contents of a PREQ in Case E shall be as shown in Contents of a PREQ in Case E.

| - Contents of a PREQ in Case E |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Table 7-26 for the PREQ element |  |
| Length |  | 37 |  |
| Flags |  | Bit 0: As needed (portal role) Bit 1: 0 (broadcast) Bit 2: As needed (proactive PREP) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |  |
| Hop Count |  | 0 |  |
| Time to Live |  | Maximum number of hops allowed for this information element, e.g. HWMP_NET_DIAMETER. |  |
| PREQ ID |  | Previous PREQ ID + 1 |  |
| Originator MP Address |  | root MP MAC address |  |
| Originator HWMP Sequence Number |  | Previous HWMP SN of root MP + 1 |  |
| Lifetime |  | dot11MeshHWMPpathToRootInterval |  |
| Metric |  | Initial value of active path selection metric |  |
| Target Count |  | 1 |  |
| Per Target | Flags | TO = 1, RF = 1 |  |
|  | Target Address | Broadcast address |  |
|  | Target HWMP Sequence Number | 0 |  |

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.1 as shown:

- Acceptance criteria

The PREQ element shall not be accepted (and shall not be processed as described in Effect of receipt) if any of the following is true:

- The Originator HWMP SN < HWMP SN in forwarding information for Originator MP

- The Originator HWMP SN = HWMP SN in forwarding information for Originator MP AND the updated path metric is worse than path metric in forwarding information for Originator MP

- (the target address of the PREQ is neither the recipient MAC address nor a MAC address proxied by the recipient) AND (dot11MeshForwarding is set to 0)

Otherwise, the PREQ information element is accepted.

See also General rules for processing HWMP information elements: General rules for message processing

Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.2 as shown:

- Effect of receipt

The following applies only to a PREQ element that was accepted according to the acceptance criteria in Instruction to Editor: Insert marked changes into text of paragraphs of 11B.9.5.3.1 as shown::

- The receiving MP shall record the PREQ ID and the Originator MP Address.

- The receiving MP shall update the active forwarding information it maintains for the originator MP and previous hop MP of the PREQ (see Creation and update of forwarding information)

- If the MP is addressed by the PREQ or is the proxy of the target MAC address it shall initiate the transmission of a PREP to the originator MP (Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.2 as shown: Case A). If the PREQ carries a proxied address (indicated by the AE flag), the MP shall update its proxy information with the proxied address and set the PREQ originator MP address as the corresponding proxy.

- If step 3 was not applicable for the MP and the AE flag in the PREQ is set to 1, the MP may update its proxy information with the proxied address and set the PREQ originator MP address as the corresponding proxy.

- If the MP has valid forwarding information to any of the requested targets and the TO flag for such a target is OFF (TO=0), it initiates the transmission of a PREP for each of these targets (see Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.2 as shown: Case A)

- If there are targets in the PREQ that have been not processed in steps 3 or 4 or that have been processed in step 4 but the corresponding Reply and Forward Flag is ON (RF = 1),  the receiving MP shall forward the PREQ as defined in Conditions for generating and sending a PREQ, Case D.

- If the MP is initiating a PREP transmission on behalf of another target (intermediate reply), it should update its forwarding information by placing the last hop MP (from which it received the PREQ) into the precursor list for the forward path entry for the target. In addition, this intermediate MP also updates its forwarding information for the originator MP by placing the next hop toward the target in the precursor list for the reverse path entry.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.1 as shown:

- Function

The Path Reply (PREP) element is transmitted in individually addressed frames and is described in 7.3.2.97. The purpose of the PREP is:

- to establish the forward path to a target and

- to confirm the reverse path to the originator.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.2 as shown:

- Conditions for generating and sending a PREP

An MP sends out a PREP element in a PREP frame in the following cases:

Case A: Original transmission

A PREP is transmitted if the MP has received a PREQ fulfilling all of the following conditions:

- One of the following applies:

- The Target Address of the PREQ is the same as MAC address of the receiving MP

- The Target  Address of the PREQ = all 1’s (broadcast) and the PREP flag is set to 1 (”Proactive PREP”)

- The Target Address of the PREQ is currently proxied by the MP

- One of the following applies:

- The Originator HWMP SN of the PREQ  is greater than the HWMP SN of the last PREQ received from the same originator MP address (that includes the case that there is no path to the originator MP)

- The Metric is better than the path selection metric currently associated with the Originator MP Address and the Originator HWMP SN of the PREQ  is equal to the HWMP SN of the last PREQ received from the same originator MP address

The content of the generated PREP in Case A shall be as shown in Contents of a PREP element in Case A.

| - Contents of a PREP element in Case A |  |  |
| --- | --- | --- |
| Field | Value |  |
| ID | Value given in Table 7-26 for the PREP element |  |
| Length | As required |  |
| Flags | Bit 0 – 5: Reserved Bit 6: Address Extension (AE) (1 = proxied device address present, 0 = otherwise) Bit 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this element |  |
| Target Address | MAC address of the target MP |  |
| Target HWMP Sequence Number | HWMP sequence number of the target MP after it has been incremented |  |
| Proxied Address | Proxied target address on behalf of which the PREP is sent. Present only if Bit 6 (AE) in Flags = 1. |  |
| Lifetime | As per the PREQ that triggered the transmission of this PREQ |  |
| Metric | Initial value of active path selection metric |  |
| Originator MP Address | MAC address of the originator MP |  |
| Originator HWMP Sequence Number | HWMP sequence number of the originator MP |  |

Note: the DA of the action frame carrying the PREP element is set to the address of the next hop to the Originator MP Address of the PREQ that triggered the PREP.

Case B:	PREP Propagation

A PREP is propagated if all of the following applies:

- the MP has received and accepted the PREP – See Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.3.1 as shown:

- the MP is not the target of the PREP

The contents of a PREP element in Case B shall be as shown in Contents of a PREP element in Case B.

| - Contents of a PREP element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PREP element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Target Address | As received |
| Target HWMP Sequence Number | As received |
| Proxied Address | As received |
| Lifetime | As received |
| Metric | As receivedown metric toward the transmitting MP |
| Originator MP Address | MAC address of the originator MP |
| Originator HWMP Sequence Number | HWMP sequence number of the originator MP |

Note: the DA of the action frame carrying the PREP element is set to the address of the next hop to the Originator MP Address of the PREQ that triggered the PREP.

Case C:	Intermediate reply

A PREP is transmitted if the MP has received a PREQ fulfilling all of the following conditions:

- The PREQ Target Only flag is set to 0

- The receiving MP has  active forwarding information with:

- A destination that is the same as the Target Address of the PREQ

- An HWMP SN that is greater than or equal to the Target HWMP SN of the PREQ

- A non-zero lifetime

The content of the generated PREP in Case C shall be as shown in Contents of a PREP element in Case C.

| - Contents of a PREP element in Case C |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PREP element |
| Length | As required |
| Flags | Bit 0: 0 Bits 1– 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this element |
| Target Address | Target MAC address from the PREQ |
| Target HWMP Sequence Number | HWMP SN of the stored forwarding information of the Target of the PREQ |
| Proxied Address | As received |
| Lifetime | As per the PREQ that triggered the transmission of this PREQ |
| Metric | Value of path metric taken from the active forwarding information for the target address of the PREQ |
| Originator MP Address | MAC address of the originator MP |
| Originator HWMP Sequence Number | HWMP Sequence number of the originator MP |

Case D:	PREP in Proactive PREP mode

All of the following applies:

- The MP has received a PREQ broadcast with the “Proactive PREP” flag set to 0.

- The MP needs the root MP to establish a path to itself

The content of the generated PREP in Case D shall be as shown in Contents of a PREP element in Case D.

| - Contents of a PREP element in Case D |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PREP element |
| Length | As required |
| Flags | Bit 0: 0 Bits 1– 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this element |
| Target Address | MAC address of the MP |
| Target HWMP Sequence Number | HWMP SN of the MP |
| Proxied Address | 0 |
| Lifetime | As per the PREQ that triggered the transmission of this PREQ |
| Metric | Initial value of active path selection metric |
| Originator MP Address | MAC address of root MP (originator MP of the PREQ) |
| Originator HWMP Sequence Number | HWMP sequence number of the root MP (originator HWMP SN of the PREQ) |

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.3.1 as shown:

- Acceptance criteria

The PREP element shall not be accepted (and shall not be processed as described in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.3.2 as shown:) if any of the following is true:

- The Target HWMP SN < HWMP SN in forwarding information for this target MP

- The Time to Live is 1 or less

- (the Originator MP address of the PREP is neither the recipient MAC address nor a MAC address proxied by the recipient) AND (dot11MeshForwarding is set to 0)

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.3.2 as shown:

- Effect of receipt

The following applies only to a PREP element that was accepted according to the acceptance criteria in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.6.3.1 as shown:.

- The receiving MP shall record the Target MP Address, together with the Target HWMP SN, hopcount and metric according to the rules defined in Creation and update of forwarding information.

- If the receiving MP is not the final destination of the PREP (Originator MP), the PREP is propagated as per Case B above.

- If the receiving MP is the final destination of the PREP (Originator MP) and the AE-flag is set, the MP shall store the proxy information with the target proxied address and as the proxy the corresponding target MP address.

- If the receiving MP is not the final destination of the PREP (Originator MP) and the AE-flag is set, the MP may store the proxy information with the target proxied address and as the proxy the corresponding target MP address.

- If the MP propagates the PREP, the precursor list for the Target MP Address is updated by adding the next hop MP to which the PREP is propagated. In addition, at the MP the precursor list for the originator MP address is updated by adding the next hop MP towards the Target Address.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.7.2 as shown:

- Conditions for generating and sending a PERR

An MP sends out a PERR in the following cases:

Case A: Original transmission

All of the following applies:

- The MP detects a link break to the next hop of an active path in its stored forwarding information while transmitting frames to it.  Note: the detection may be triggered by the fact that an MP is unable to forward a data frame to a next hop MP.

- The MP has not sent a PREQ element less than dot11MeshHWMPperrMinInterval TUs ago.

- dot11MeshForwarding is set to 1

Actions before sending the PERR:

- The MP first makes a list of unreachable destinations consisting of the unreachable MP and any additional destinations in the local forwarding table that use the unreachable MP as the next hop.

- The HWMP sequence numbers in all valid stored forwarding information of unreachable destinations announced in this PERR shall be incremented.

- The stored forwarding information for each unreachable destination announced in this PERR shall be invalidated.

The contents of a PERR element in Case A shall be as shown in Contents of a PERR element in Case A.

| - Contents of a PERR element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PERR element |
| Length | 2 + N * 10 |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced unreachable destinations in PERR. A destination is unreachable if its next hop in the stored forwarding information is an unreachable neighbor. |
| Destination Address | MAC address of detected unreachable destination #1 |
| HWMP Sequence Number #1 | Last used HWMP SN for Destination Address #1 + 1 |

Case B:	PERR propagation

All of the following applies:

- The MP creates a list of unreachable destinations consisting of those destinations in the PERR for which there exists valid forwarding information that has the transmitter of the PERR as the next hop.

- The MP received a PERR from a neighbor for one or more of its active paths in its stored forwarding information.

- The MP has not sent a PREQ element less than dot11MeshHWMPperrMinInterval TUs ago.

- dot11MeshForwarding is set to 1

The contents of a PERR element in Case B shall be as shown in Contents of a PERR element in Case B.

| - Contents of a PERR element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the PERR element |
| Length | 2 + N * 10 |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced unreachable destinations in PERR ( received value) A destination is unreachable if its next hop in the corresponding stored forwarding information is the transmitter of the received PERR. |
| Destination Address | MAC address of detected unreachable destination #1 (as received, but maybe at different position in destination list) |
| HWMP Sequence Number #1 | As received (but maybe at different position in destination list) |

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.7.3.2 as shown:

- Effect of receipt

The following applies only to a PERR element that was accepted according to the acceptance criteria in Error: Reference source not found.

- If the received HWMP sequence number for a referenced destination is higher than the current HWMP sequence number for that destination, the receiving MP shall consider that destination unreachable and update its stored information for that destination accordingly.

- The receiving MP shall transmit a PERR as defined in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.2 as shown:, Case B

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.2 as shown:

- Conditions for generating and sending a RANN

An MP sends out a RANN in the following cases:

Case A: Original transmission

All of the following applies:

- The MP is configured as a Root MP

- The MP has not sent a PREQ element less than dot11MeshHWMPperrMinInterval TUs ago.

- The root MP sent its previous RANN dot11MeshHWMPrannInterval TUs ago

The contents of a RANN element in Case A shall be as shown in Contents of a RANN element in Case A.

| - Contents of a RANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the RANN element |
| Length | 21 |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this element |
| Root MP Address | MAC address of the Root MP |
| HWMP Sequence Number | Last used HWMP SN + 1 |
| Metric | initial metric value (0 for airtime metric) |

Case B:	Forwarding

All of the following applies:

- the MP has valid forwarding information to a root MP

- The root MP sent its previous RANN dot11MeshHWMPrannInterval TUs ago

- dot11MeshForwarding is set to 1

The contents of a RANN element in Case B shall be as shown in Contents of a RANN element in Case B.

| - Contents of a RANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 for the RANN element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| TTL | As received – 1 |
| Root MP Address | As received |
| HWMP Sequence Number | As received |
| Metric | As receivedown link metric toward the transmitting MP |

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.3.1 as shown:

- Acceptance criteria

The RANN element shall not be accepted (and shall not be processed as described in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.3.2 as shown:) if any of the following is true:

- The HWMP SN < previous HWMP SN from this originating root MP

- (HWMP SN = previous HWMP SN) AND (updated path metric is worse than previous path metric)

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.3.2 as shown:

- Effect of receipt

The following applies only to a RANN element that was accepted according to the acceptance criteria in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.3.1 as shown:.

- The receiving MP may initiate a PREQ/PREP exchange with the root MP to set up or update a path to the root MP. See PREQ, when generated, case C.

- The receiving MP shall record the Root MP Address, together with the HWMP SN, hopcount, metric.

The receiving MP shall transmit a RANN as defined in Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.8.2 as shown:, Case B.

Instruction to Editor: Incorporate marked changes into text of paragraphs of 11B.9.9 as shown:

- Considerations for support of STAs without mesh functionality

The verification of disjunct MAC addresses between a non-AP STA without mesh functionality and MPs during authentication/association of the non-AP STA (11.3.3) may be done by issuing a PREQ for the MAC address of the non-AP STA by the AP with mesh functionality. The target only flag of the PREQ shall be set to 1.

The MAC address of the non-AP STA does already exist in the mesh network if the AP with mesh functionality receives a PREP for the MAC address of the non-AP STA and it can be derived from the PREP that the requested MAC address is originated from an MP. (The AE flag of the PREP is set to 0, see clause 7.3.2.97).

Instruction to Editor: Incorporate marked changes into text of rows of V.2 as shown:

- Recommended HWMP default values

| Parameter | Recommended value | Description |
| --- | --- | --- |
| dot11MeshHWMPmaxPREQretries | 3 | The number of action frames containing a PREQ that an MP can send to a particular path target |

References: