IEEE P802.11
Wireless LANs

| OBSS Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-526-3346 | brianh@cisco.com |
| Graham Smith | DSP Group | 2491 Sunrise Blvd, #100, Rancho Cordova CA 95742 | 916 851 9191 x209 | gsmith@dspg.com |

Abstract

During the July 802.11 meeting in Denver, document 08-0758 was presented, which contained some proposed technical requirements for a solution to the overlapping BSS problem. Following this as straw poll was taken with the question “Should the OBSS Requirements presentation (08/765r1) be used as the starting point for a formally written OBSS Requirements document?” To which the result was 19 yes, 0 no, 6 abstain.

Another document, 08-0864 was also presented that described a scenario where a chain or loop of overlapping BSS could occur. It was subsequently agreed that this should also be added to the OBSS requirements document.

This document contains the requirements from 08-0758 and 08-0864 plus the changes to 08-0758 discussed by the 11aa task group and recorded in document 08-0758.

# Introduction

The P802.11aa PAR has an item in its scope for “Increased robustness in overlapping BSS environments, without the requirement for a centralised management entity”.  An overlapping BSS is defined as two or more wireless networks operating on the centre same frequency in radio range of each other.

NOTE: Additional information is needed to provide a better definition of overlapping BSS, for example partial and complete overlapping.

This document contains a list of the technical requirements for improved overlapping BSS coexistence against which proposals can be evaluated.

Obviously it is preferable to avoid the overlapping BSS condition with appropriate use of channel selection and transmit power control. However, in 2.4 GHz band, overlapping BSS is routine. OBSS at 5 GHz is much less likely, but still occurs due to:

- 40 MHz operation

- DFS not implemented or certified (e.g. IBSS)

- Poor channel selection

- Moving APs

- Nearby radar

## Examples

  - Figure 1 - Example of a chain of overlapping BSS

# Criteria for evaluating Overlapping BSS proposals

The following items describe technical requirements that the 802.11aa task group intends to fulfil in its solution to improving robustness in overlapping BSS environments.

- Enables improved Quality of Service

  - When a stream has been accepted, traffic from other BSSs should not cause it to stop

- Number of Overlapping Networks

  - Shall improve network performance when there are two overlapping BSS as shown in condition 1 and 2.

  - Should improve network performance when there are three overlapping BSS as shown in conditions 3 to 7.

  - Should improve network performance when there are overlapping BSS where each BSS may overlap with further BSS, as shown in Figure 1.

- Contention based medium access protocols

  - Legacy EDCA-AC admissions shall be preserved

- Controlled medium access protocols

  - Shall support protection for controlled medium access reservations

  - Shall support a mechanism to avoid beacon collision and multicast collision

- Inter-AP communication for improved OBSS robustness

  - Shall be possible for APs to communicate via the WM when both BSS are in range of each other

  - Should be possible for APs to communicate via the wired DS

  - Shall be resistant to denial of service attacks and provide mitigation against denial of service attacks. (Note: Needs to be reviewed by people with security knowledge)

- Management

  - Shall not mandate that an AP to be configured as the supervisor controller

  - If the solution requires an AP to be automatically chosen as supervisor, it shall be robust to signal fade and AP removal

May support configuration of a supervisor controller

Shall provide a mechanism to control the dynamic aspects of the OBSS solution

- Dynamic

  - User experience shall be invarient to the order in which APs are switched on or off

  - Removal of an AP shall not cause video streams in other BSSs to terminate

  - Shall support dynamic stream creation and deletion

Shall not require detailed information about stream reservations to be exchanged between APs

  - Shall maintain current 802.11 levels of privacy (Note: may want to remove this, as it is covered by previous item)

- Fairness (cooperation) / Quality of Service / System Performance and Priority

  - For each 802.11 medium access mechanism, the right to access the WM shall be comparible with the current levels of fairness provided.

  - If the solution requires an AP to be chosen as supervisor, the act of providing the supervisor role does not gain increased priority in medium time allocation over other overlapping BSS.

  - The division of time may differ in each BSS, due to differing loads in each BSS.

  - The solution should include mechanisms to limit allocations based upon total capacity of the WM.

  - The solution must include mechanisms to enable APs to communicate BSS loads with each other with the intent to share access to the WM.

  - The solution should not significantly reduce the total system throughput nor significantly increase the delay or jitter compared to legacy systems.

- ESS / BSS

  - The solution shall support overlapping BSS that are in the same ESS or a different ESS.

- Security

  - The solution shall not require the reduction in the security policy of a BSS

  - The solution shall be robust to attacks from rogue AP / non-AP STA

  - The solution must not create a new denial of service technique

- Compatible with legacy non-11aa equipment

  - The solution should not require non-AP STA modification

  - The solution shall be tolerant of legacy non-AP STAs

  - The solution should be tolerant of legacy APs

- Frequency bands

  - The solution shall support 2.4GHz and 5GHz

  - The solution shall support 20 and 40MHz channel widths

  - The solution shall support 5 and 10 MHz channels when all BSSs are using the same channel width.

- Diagnostics

  - Shall provide a mechanism to provide metrics on the current performance of the OBSS solution

References: