IEEE P802.11
Wireless LANs

| OBSS Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-526-3346 | brianh@cisco.com |
| Graham Smith | DSP Group | 2491 Sunrise Blvd, #100, Rancho Cordova CA 95742 | 916 851 9191 x209 | gsmith@dspg.com |

Abstract

During the July 802.11 meeting in Denver, document 08-0758 was presented, which contained some proposed technical requirements for a solution to the overlapping BSS problem. Following this as straw poll was taken with the question “Should the OBSS Requirements presentation (08/765r1) be used as the starting point for a formally written OBSS Requirements document?” To which the result was 19 yes, 0 no, 6 abstain.

Another document, 08-0864 was also presented that described a scenario where a chain or loop of overlapping BSS could occur. It was subsequently agreed that this should also be added to the OBSS requirements document.

This document contains the requirements from 08-0758 and 08-0864 plus the changes to 08-0758 discussed by the 11aa task group and recorded in document 08-0758.

# Introduction

The P802.11aa PAR has an item in its scope for “Increased robustness in overlapping BSS environments, without the requirement for a centralized management entity”.  An overlapping BSS is defined as two or more BSSs that are operating on the same centre frequency in radio range of each other.

This document contains a list of the technical requirements for improved overlapping BSS coexistence against which proposals can be evaluated.

Obviously it is preferable to avoid the overlapping BSS condition with appropriate use of techniques such as channel selection and transmit power control. The scope of this requirements document for improving OBSS coexistence does not include techniques to avoid BSS overlap.

## Examples

The above diagrams represent idealized versions of overlapping BSS and in the real world OBSSs are more complex because the coverage of a BSS is not uniform. Also, an OBSS condition can change over time due to factors such as use of transmit power control, movement or changes in radio path propagation.

The above conditions can occur simultaneously in combination, such as in the example below:

  - Figure 1 - Example of a chain of overlapping BSS

# Criteria for evaluating Overlapping BSS proposals

The criteria below will be used to evaluate OBSS proposals..

- Enables improved Quality of Service

  - When a video transport stream has been accepted, additional traffic from other BSSs should not cause the video transport stream  to fail to meet its QoS requirements.

  - It is not required to provide OBSS protection for video transport streams that have not used an admission control protocol.

  - Should improve network performance when there are overlapping BSS.

- Contention based medium access protocols

  - Pre-11aa EDCA-AC admissions shall be preserved

  - Shall support a mechanism to avoid beacon collision and multicast collision

- Controlled medium access protocols

  - Shall support protection for controlled medium access reservations

  - Shall support a mechanism to avoid beacon collision and multicast collision

- Inter-AP communication for improved OBSS robustness

  - If inter-AP communication is required, it shall be possible to perform this communication using the WM.

  - If inter-AP communication is required, it should be possible to perform this communication using the DS.

  - Shall be resistant to denial of service and other attacks and provide mitigation against such attacks.

- Management

  - Shall not mandate that an AP to be configured as the supervisor controller, e.g. providing a centralized admission control function, or a timing reference

  - If the solution requires an AP to be automatically chosen as supervisor, it shall be robust to signal fade and AP removal May support configuration of a supervisor controller

  - Shall provide a mechanism to control the dynamic aspects (as described in section 7)  of overlapping BSSs,

- Dynamic

  - User experience shall be invariant to the order in which APs are switched on or off

  - Removal of an AP shall not cause video streams in other BSSs to terminate

  - Shall support dynamic stream creation and deletion

When it is necessary to exchange information to achieve performance requirements, this should maintain the current 802.11 levels of  privacy and security

- Fairness, Cooperation and Coexistence

  - For each 802.11 medium access mechanism, the right to access the WM shall be comparable with the currently provided levels of fairness. .

  - If the solution requires an AP to be chosen as supervisor, the act of providing the supervisor role does not gain increased priority in medium time allocation over other overlapping BSS.

  - The division of time may differ in each BSS, due to differing loads in each BSS.

  - The solution should include mechanisms to limit allocations based upon total capacity of the WM.

  - The solution should not significantly reduce the total system throughput nor significantly increase the delay or jitter compared to legacy non-11aa systems.

- ESS / BSS

  - The solution shall support overlapping BSS that are in the same ESS

  - The solution shall support overlapping BSS that are in different ESSs

- Security

  - The solution shall not require the reduction in the security policy of a BSS

  - The solution shall be as robust to attacks from rogue APs / non-AP STAs as legacy non-11aa systems

- Compatible with legacy non-11aa equipment

  - The solution shall be tolerant of legacy non-11aa non-AP STAs

  - The solution should be tolerant of legacy non-11aa APs

  - The solution should improve the performance of an 11aa BSS in the presence of overlapping non-11aa BSSs while still coexisting with them.

  - Provides a mode of operation that minimizes the potential for requiring hardware modifications

- Frequency bands

  - The solution shall support any frequency band in which 802.11 operates

  - The solution shall support any 802.11 physical layer with the possible exception of infra-red and frequency hopping

- Diagnostics

  - The solution shall provide a mechanism to enable metrics on the current performance of the OBSS solution

References: