IEEE P802.11
Wireless LANs

| Proposed Texts for Symmetrical Airtime Link Metric and Path loop Avoidance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics, Inc. | 1060 East Brokaw Road, MS 212, San Jose, CA 95131 | +1-408-467-8436 | liwen.chu@st.com |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408 451-8109 | george.vlantis@st.com |
| Jarkko Kneckt | Nokia | Itämerenkatu 11-13, 00180 Helsinki, Finland | + 358 50 4821550 | Jarkko.Kneckt@nokia.com |
| Janne Marin | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland |  | Janne.Marin@Nokia.com |
| Mika Kasslin | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland |  | Mika.Kasslin@Nokia.com |

Abstract

This submission provides updated texts to:

Provide symmetrical Airtime Link Metric,

Improve path stability,

Avoid path loop.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background

This contribution provides updated texts for providing symmetrical airtime link metric and avoiding path loop.

The following is normative text proposed as an amendment to P802.11s/D2.01.

Update Clause 11B.9.4.5 as follows:

- Creation and update of forwarding information

MPS path selection information elements create or update the forwarding information in the MPs that process these information elements. The creation and update of forwarding information follows the same rules for PREQ, PREP, and RANN. These rules are given below. Unless otherwise noted, the term “MPS_IE” stands for the information element under consideration (PREQ, PREP, or RANN). In the following text, the “” indicates an entry into the local forwarding data base, i.e., the path selection table.

- If the MP does not have any valid forwarding information to the originator of the MPS_IE (MPS_ie.originator_address), the value of the destination sequence number is set to MPS_ie.destination_sequence_number (field: destination_sequence_number), it creates this information from the field MPS_ie.originator_address (field: destination), the transmitter address of the management frame containing the MPS_IE (field: next hop), the accumulation of the value of field MPS_ie.metric with the metric of the last link (field: path metric), and the value of field MPS_ie.lifetime (field: lifetime).

- If the MP does not have any valid forwarding information to the transmitter of the MPS_IE, it creates this information from the transmitter address of the management frame containing the MPS_IE (field: destination and next hop), the destination sequence number is set to invalid, the metric of the last link (field: path metric), and the value of field MPS_ie.lifetime (field: lifetime).

- If the MP has valid forwarding information to the originator of the MPS_IE (MPS_ie.originator_address), and the acceptance criteria of the corresponding element as described in Error: Reference source not found, Error: Reference source not found, Acceptance criteria, Error: Reference source not found are satisfied, then the MP updates this forwarding information with the transmitter address of the management frame containing the MPS_IE (field: next hop), the value of the destination sequence number is set to MPS_ie.destination_sequence_number (field: destination_sequence_number), the accumulation of the value of field MPS_ie.metric with the metric of the last link (field: path metric), and the larger one of the lifetime of the stored forwarding information and the value of field MPS_ie.lifetime (field: lifetime).

- If the MP has valid forwarding information to the transmitter of the MPS_IE and if the path metric of this information is worse than the metric of the last link, then the MP updates this forwarding information with the transmitter address of the management frame containing the MPS_IE (field: next hop), the destination sequence number is set to invalid, the metric of the last link (field: path metric), and the larger one of the lifetime of the stored forwarding information and the value of field MPS_ie.lifetime (field: lifetime).

Each time a forwarding information is used to forward a frame, the lifetime of the forwarding information shall be updated by current time+dot11MeshHWMPactivePathTimeout. If the forwarding information becomes invalid, the destination sequence number shall be increased by 1. The invalid forwarding information should be kept in the forwarding table for at least dot11MeshHWMPinvalidPathTimeout.

Update Clause 11B.9.6.3.1 as follows:

- Acceptance criteria

The PREP element shall not be accepted (and shall not be processed as described in Error: Reference source not found) if any of the following is true:

- The DSN < previous DSN from this originator

- The DSN = previous DSN AND the updated path metric is worse than previous path metric

- The Time to Live is 1 or less

- (the destination address of the PREQ is neither the recipient MAC address nor a MAC address proxied by the recipient) AND (dot11MeshForwarding is set to 0)

V.6 Recommended HWMP default values

Insert the following row in the HWMP parameter  table:

| Parameter | Recommended value | Description |
| --- | --- | --- |
| dot11MeshHWMPinvalidPathTimeout | 15000 | The time (in TUs) for which MPs delete forwarding information after this forwarding information becomes invalid. |

References:

- L. Chu et al., “Symmetrical Airtime Link Metric Report and Path Loop Avoidance,” IEEE 802.11-08/636r4.