IEEE P802.11
Wireless LANs

| Proposed Texts for solving the problems in MDA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1-408-467-8436 | Liwen.chu@st.com |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408 451-8109 | George.vlantis@st.com |

Abstract

This submission provides updated texts to fix the problems in MDA created by variable length mesh beacon interval in mesh points.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background

This contribution provides updated texts to fix the problems in MDA created by variable length mesh beacon interval in different mesh points.

The following is normative text proposed as an amendment to P802.11s/D2.01.

Update Clause 3  as follows:

3.s25 Virtual mesh delivery traffic indication message (VDTIM) interval: The value in Time Unit that equals to the default Beacon Interval multiplied by default Mesh DTIM period.

3.s26 Virtual mesh target DTIM beacon transmission time: The start time of VDTIM interval. This time is not used for mesh beacon transmission.

Update Clause 7.3.2.90 as follows:

- MDAOP Setup Request element

The MDAOP Setup Request information element is used by an MP to request the setup of a set of MDAOPs, identified by a single MDAOP Set ID, between itself (transmitter) and a receiver. This information element is transmitted in individually addressed MDA action frames. The format of the information element is as shown in MDAOP Setup Request element.

| Octets: 1 | 1 | 1 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
| Element ID | Length | MDAOP Set ID | MDAOP Duration | MDAOP Periodicity | MDAOP Offset |
| - MDAOP Setup Request element |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 5 octets.

The MDAOP Set ID field is an eight bit unsigned number that represents the ID for the requested Set. It is determined by the MDAOP Set owner. When used in combination with the MAC address of the MDAOP set owner, the MDAOP Set ID uniquely identifies the MDAOP Set.

If this MDAOP Setup Request is for the unicast transmission, the MDAOP Set ID is ranged from 0 to 127. If this MDAOP Setup Request is for the multicast or broadcast transmission, the MDAOP Set ID is ranged from 128 to 254.

The MDAOP Duration field specifies the duration of the MDAOP in multiples of 32.

The MDAOP Periodicity field is a non-negative integer that specifies the number of subinterval periods into which the virtual Mesh DTIM interval is split. A value of zero indicates a non-repeated reservation for and MDAOP in the virtual mesh DTIM interval following the setup.

The MDAOP Offset field specifies the position of an MDAOP beginning from the beginning of the virtual Mesh DTIM interval and subsequent subinterval periods within the virtual Mesh DTIM interval. The value is specified in multiples of 32.

An example of periodicity, duration, and offset values for a periodic MDAOP Info field is shown in figure s24. In this particular example, the periodicity equals four, so that there are four subintervals within the virtual mesh DTIM interval. As further illustrated in the figure, the offset value indicates the start of the MDAOP relative to the start of these intervals.

Figure s24 ----Values for Periodic MDAOP Info field for an example MDAOP set

Update Clause 7.3.2.92  as follows:

- MDAOP Advertisements element

The MDAOP Advertisements Element is used by an MP to advertise its MDA state to its neighbors. This information element may be carried in selected Beacon frames with a chosen frequency. This information element may also be transmitted in an MDA action frame. The format of the information element is as shown in.Figure s27.

| B0 B7 | B8 B15 | B16 B47 | B48 B51 | B52 B55 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Virtual TDBTT offset | MDA Access fraction | MDA Access fraction limit | TX-RX times report | Interfering times report |  |
| Bits: 8 | 8 | 32 | 4 | 4 | variable | variable |  |
| - MDAOP Advertisements element |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 1 to 255 octets.

When MDAOP Advertisement element is carried in a beacon frame, Virtual TDBTT (Target Transmission Time of virtual Mesh DTIM Beacon) offset field shows the offset between this beacon’s transmission trime and the next virtual TDBTT. When MDAOP Advertisement element is carried in a MDAOP Advertisement action frame, Virtual TDBTT offset field is reserved.

MDA Access Fraction and MDA Access Fraction Limit fields are both 4 bit unsigned number fields. They denote a positive fraction of the virtual MESH DTIM interval length expressed in units of (1/16). The MDA Access Fraction field represents the current value of MDA Access Fraction at the MP rounded down (floor) to the nearest multiple of (1/16). The MDA Access Fraction Limit field represents the maximum MDA access fraction allowed at the MP. This number is always a multiple of (1/16).

The TX-RX Times Report field is a variable length field that advertises the times in the virtual Mesh DTIM interval that are busy for the MP as a transmitter or a receiver. These times may include known and otherwise un-advertised transmission and reception times besides MDAOPs. For example, an MAP may include its HCCA times in the advertisement.

Through the Interfering Times Report field, an MP reports the times when one of its neighbors is in TX or RX as reported by their (neighbors’) TX-RX times report fields. This field may not include times for which the MP is a transmitter or receiver (Such times are taken care of in the TX-RX times report).

The format of the TX-RX Times and the Interfering Times Report field is shown in The format of the TX-RX times report fields and Figure s29, correspondingly. The fields involved are similar to the fields involved in the MDAOP Setup Request element. While the fields are the same as in an MDAOP setup request element, the TX-RX times and Interfering times reports can more efficiently report information compared to MDAOP setup request elements. This is possible because different MDAOPs may be combined in an efficient way and reported. MDAOP Set IDs are not reported in the advertisements.

| Octets: 1 | 1 | 4 |  | 4 | 4 |  | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Unicast MDAOPs | Number of Broadcast MDAOPs | Unicast MDAOP 1 | … | Unicast MDAOP n | Broadcast MDAOP 1 | … | Broadcast MDAOP n |
| - The format of the TX-RX times report fields |  |  |  |  |  |  |  |

In the TX-RX Times Report field format, the Number of Unicast MDAOPs field indicates the number of MDAOPs which are used for unicast transmissions. The Number of Broadcast MDAOPs field indicates the number of MDAOPs which are used for broadcast transmissions.

| Octets: 1 | 4 |  | 4 |
| --- | --- | --- | --- |
| Number of MDAOPs | MDAOP 1 | … | MDAOP n |
| - The format of the Interfering times report fields |  |  |  |

In the Interfering Times Report field format, the Number of MDAOPs field indicates the number of MDAOPs which are used for both unicast and broadcast transmissions.

Update Clause 9.21 as follows:

- MDA

Mesh Deterministic Access (MDA) is an optional access method that allows supporting MPs that support MDA to access the channel at selected times with lower contention than would otherwise be possible. This standard does not require all MPs to use MDA. MDA can be used by a subset of MPs in a Mesh. However, MDA connections can only be setup among MDA-supporting MPs. The performance of MDA may be impacted by devices that do not respect MDA reservations. MDA sets up time periods in mesh neighborhoods when a number of MDA-supporting MPs that may potentially interfere with each others’ transmissions or receptions are set to not initiate transmission sequences. In order to use the MDA method for access, an MP shall maintain synchronization with its neighboring MP. The MDA method is described in detail below.

- MDA opportunity (MDAOP)

An MDAOP is a period of time within every virtual Mesh DTIM interval that is set up between the MDAOP owner and the addressed MP. Once an MDAOP is setup,

- Access to the channel by MDA-supporting MPs is governed by the procedures in Access by MDAOP Owners

- The MDAOP is advertised according to the procedures in MDAOP advertisements .

An MP shall randomly select its Virtual mesh target DTIM beacon transmission time and broadcast to its neighbor MPs before the first MDAOP is decided.

- MDAOP sets

A set of MDAOPs may be setup either for individually addressed transmissions from a transmitter to a receiver or for group addressed transmissions from a transmitter. Such a set is identified by a unique ID called the MDAOP Set ID. The MDAOP Set ID has to be unique for a transmitter, so that the MDAOP set ID and the transmitter (or set owner) MAC address uniquely identifies an MDAOP set in the mesh. The MDAOP set ID is a handle that allows operation such as setup and teardown to be conducted together for the entire set of MDAOPs in an MDAOP set.

MDAOP set ID is an 8-bit unsigned number. If this MDAOP Setup Request is for the unicast transmission, the MDAOP Set ID is ranged from 0 to 127. If this MDAOP Setup Request is for the multicast or broadcast transmission, the MDAOP Set ID is ranged from 128 to 254. The special value of MDAOP set ID, when all bits are set to 1, is reserved to mean all MDAOPs.

- Neighborhood MDAOP times at an MP

In an MP’s mesh neighborhood, all the TX-RX times reported by its neighbors (in their MDAOP advertisements) form a set of MDAOPs that are already being used in the neighborhood. No new MDAOPs may be set up by the MP during these times. These times are referred to as Neighborhood MDAOP times for the MP. In effect, Neighborhood MDAOP Times at an MP include all MDAOPs for which the MP and its neighbors are either the transmitters or receivers.

- Neighbor MDAOP interfering times for an MP

Through the Interfering Times in its MDAOP Advertisements, the MP reports the MDAOP times which its neighbors have advertised in their TX-RX reports and in which it is not involved itself. These times shall not be used for a new MDAOP with the reporting MP as they may experience interference.

- MDA access fraction (MAF)

The MDA access fraction at an MP is the ratio of the total duration of its ‘Neighborhood MDAOP Times’ (see definition above) in a virtual Mesh DTIM interval to the duration of the virtual Mesh DTIM interval. This parameter may be used to limit the use of MDA in an MP’s mesh neighborhood to a certain fraction of the total channel time. The maximum value for MAF that is allowed at an MP is specified by the dot11MAFlimit parameter.

The dot11MAFlimit is copied into the MDA Access Fraction Limit field of the MDAOP Advertisements information element. Before attempting to set up an MDAOP Set with a neighbor peer MP, an MP is required to ensure that the new MDAOP set does not cause the MAF of its neighbor peer MPs to exceed their MAF Limit. An MDAOP setup request shall be refused by the intended receiver if the MAF limit of its own neighbors is exceeded due to the new setup.

- MDAOP setup procedure

The setup of an MDAOP set is initiated by the intended transmitter, and is accepted/rejected by the intended receiver. Once accepted, the transmitter is referred to as the owner of the MDAOP. The setup procedure for an MDAOP set is as follows:

- The MP that intends to be the transmitter in a new MDAOP set builds a map of Neighborhood MDAOP times in the virtual Mesh DTIM interval after hearing Advertisements from all of its neighbor peer MPs that have MDA active. If no advertisement was heard from a neighbor peer MP in the last dot11MDAdvertPeriodMax, the MP may request the neighbor peer MP for MDAOP Advertisement.

- Based on traffic characteristics, it then chooses MDAOP starting times and durations in the virtual Mesh DTIM interval that do not overlap with either its Neighborhood MDAOP Times or the Neighbor MDAOP Interfering Times of the intended receiver, except when planning a Broadcast MDAOP which it already owns. If it owns a Broadcast MDAOP, it may choose to transmit an MDAOP Setup request for the same times setting the same MDAOP Set ID; such MDAOP Setup request may be sent in a unicast, or multicast, or broadcast manner.

- It then verifies that the new MDAOP Set will not cause the MAF limit to be crossed for its neighbor peer MPs. If MAF limit would be crossed for its neighbors peer MPs, due to the new MDAOP Set, it suspends the setup process.

- If the MAF limits at all neighbor peer MPs are respected despite the new MDAOP set, it transmits an MDAOP Setup request information element to the intended receiver with chosen MDAOP locations and durations.

- The receiver of the MDAOP Setup Request information element checks to see if the MDAOP times have overlap with its Neighborhood MDAOP Times. The receiver also checks if the new MDAOP Set will cause the MAF limit to be crossed for its neighbor peer MPs. The MDAOP Setup Reply information element is used to reply to a setup request.

- The receiver rejects the setup request for a Unicast MDAOP if there are overlaps of the requested MDAOP set with its Neighborhood MDAOP Times. It may suggest alternate times by including the optional field Alternate suggested MDAOP in the MDAOP Setup Reply element.

- The receiver rejects the setup request for a Broadcast MDAOP if there are overlaps of the requested MDAOP set with the Neighborhood MDAOP Times except when the intended transmitter itself is the owner of the overlapped times, i.e. it declares the times of the requested MDAOP set by transmitting the Broadcast MDAOP advertisement.

- The receiver also rejects the setup request if the MAF limit of itself or its neighbor peer MPs will be exceeded due to the new setup.

- If suitable, the receiver accepts the setup.

- If the receiver accepts a setup request for a Broadcast MDAOP, it includes the related times into its MDAOP advertisements only after the MDAOP advertisement from the owner of the Broadcast MDAOP is received.

- MDAOP advertisements

Every MP that has MDA active is required to advertise TX-RX and Interfering times using the MDAOP Advertisements information element, at least once in dot11MDAdvertPeriodMax. These advertisements are always transmitted in group addressed frames; either in Beacon frames or MDA action frames. The advertised times include:

- TX-RX times report:

- All MDAOP times for which the MP is the transmitter or the receiver.

- All other times that it knows are busy/reserved such that it is either the transmitter or the receiver. A non exhaustive list includes expected HCCA times for an MAP.

- Interfering times report:

- All TX-RX times reported by the MP’s neighbor peer MPs so that the MP is neither the transmitter nor the receiver during those times.

- MDAOP set teardown

Either the transmitter or the receiver may initiate a teardown by transmitting the MDAOP Set Teardown information element to the other communicating end. A teardown shall be initiated when either MDAOP owner or receiver learn of a conflicting MDAOP set from an advertisement by a neighbor with a lower MAC address. After a teardown, the transmitter and receiver stop advertising the MDAOP set in their TX-RX times.

The transmitter assumes a successful teardown and stops using or advertising (in TX-RX times report) an MDAOP set if any of the following happens:

- Its MDAOP Set Teardown information element is successfully Acked.

- The maximum retries for the teardown information element it is transmitting are exceeded.

- The receiver’s advertisement does not include the MDAOP set

- The receiver is unreachable for greater than dot11MDAOPtimeout time

The receiver assumes a successful teardown and stops advertising an MDAOP set if any of the following happens:

- Its MDAOP Set Teardown information element is successfully Acked.

- The maximum retries for the teardown information element it is transmitting are exceeded.

- The transmitter’s advertisement does not include the MDAOP set.

- The transmitter is inactive for greater than dot11MDAOPtimeout time

The interfering times are directly derived from neighbor peer MPs’ TX-RX times report. The interfering times report reflects the latest TX-RX times reports from the neighbor peer MPs.

- Access during MDAOP

MDA-supporting MPs shall track Neighborhood MDAOP times when either they or their neighbor peer MPs are transmitters or receivers. The access behavior for MPs during the Neighborhood MDAOP times is described below.

- Access by MDAOP Owners

If an MP is the owner of an MDAOP and has an MSDU associated with an MDA session to transmit, it shall attempt to access the channel during the time set up for the MDAOP and obtain a TXOP using EDCA contention and backoff parameters for the Access Category of the MSDU as defined in 9.9.1.3.

An MP successfully obtains a TXOP when the MP completes a frame exchange with the receiver of the MSDU. If the MP successfully obtains a TXOP, it may transmit until the EDCA TXOP limit for the Access Category of the MSDU is reached. If the MP reaches the TXOP limit before the end of the MDAOP, the MP should attempt to transmit additional MSDU(s) associated with the MDA session, if any are ready to be transmitted, by accessing the channel again during the MDAOP to obtain a subsequent TXOP.

If an MP accesses the channel during the MDAOP but fails to obtain either an initial or a subsequent TXOP, the MP shall perform the backoff procedure specified in 9.9.1.5.

After an MP successfully obtains the TXOP, if there are multiple MSDUs to be transmitted, the transmission and retransmission rules for multiple MSDU transmission should use the rules specified in 9.9.1.4 and 9.9.1.6.

When an MP prepares to access the channel for a retransmission outside an MDAOP, the MP shall avoid access the channel during its neighbor peer MP's MDAOP times by setting its NAV during these times.

When an MP’s MDAOP covers own TBTT, the MP shall stop the new frame transmission after its TBTT and transmit its beacon frame.

When an MP’s MDAOP covers its neighbor’s TBTT, the MP shall stop the new frame transmission after its neighbor’s TBTT.

When an MP’s MDAOP covers its neighbor’s neighbor’s TBTT, the MP should stop the new frame transmission after its neighbor’s TBTT.

- Access by non-owners of MDAOP

All MDA supporting MPs other than the MDAOP owner shall defer initiating tranmissions during the TXOP initiated in the MDAOP. MDA MPs that are not the owner of the current MDAOP can start contending for access to the channel after the conclusion of the TXOP initiated in the current MDAOP.

References:

- L. Chu et al., “MDA Issues,” IEEE 802.11-08/635r3.