IEEE P802.11
Wireless LANs

| 802.11 Update for ITU-R M.1801 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-8-7 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr. MS SJ-10-5 San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

This document contains the 802.11 Working Group Update for ITU-R M.1801

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: 15 August 2008 Question: Question ITU-R 238/8 | Document 5A/xxx |
|  | 15 JULY 2008 |
|  | English only |
| Institute of Electrical and Electronics Engineers (IEEE) |  |
| Revision of Recommendation ITU-R M.1801 |  |

# 1	Source information

This contribution was developed by IEEE Project 802®, the Local and Metropolitan Area Network Standards Committee (“IEEE 802”), an international standards development committee organized under the IEEE and the IEEE Standards Association (“IEEE-SA”).

The content herein was prepared by a group of technical experts in IEEE 802 and industry and was approved for submission by the IEEE 802.11™ Working Group on Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY), the IEEE 802.18 Radio Regulatory Technical Advisory Group, and the IEEE 802 Executive Committee, in accordance with the IEEE 802 policies and procedures, and represents the view of IEEE 802.

# 2	Background

IEEE is in receipt of the “LIAISON STATEMENT TO EXTERNAL ORGANIZATIONS –REQUEST FOR INPUT FOR A REVISION OF RECOMMENDATION ITU-R M.1801” and appreciates the opportunity to submit updated material for a revision of this Recommendation.

This contribution contains the proposed replacement of the material describing IEEE 802.11 RLANs, with additional modifications to Annex 6.

Revisions to:

https://mentor.ieee.org/802.18/file/08/18-08-0043-01-IMTA-802-20-m1801-revision.doc

IEEE 802.11 has developed a set of standards for RLANs, 802.11-1999 (R2003), which have been harmonized with IEC/ISO as specified in ISO/IEC 8802-11:2005, which is part of a series of standards for local and metropolitan area networks. IEEE 802.11 continues to develop the set of standards for RLANs, and expects continued harmonization with IEC/ISO. The medium access control (MAC) unit in IEEE 802.11 is designed to support physical layer units as they may be adopted dependent on the availability of spectrum. IEEE 802.11-2007 contains five physical layer units: four radio units, operating in the 2 400-2 500 MHz band and in the bands comprising 4.94-4.99 GHz, 5.03-5.091 GHz, 5.15-5.25 GHz, 5.25-5.35 GHz, 5.47-5.725 GHz and 5.725-5.850 GHz, and one baseband infrared (IR) unit. One radio unit employs the frequency hopping spread spectrum (FHSS) technique, two employ the direct sequence spread spectrum (DSSS) technique, and the others employ the orthogonal frequency division multiplexing (OFDM) technique. The 5 GHz OFDM radio unit operates with nominal RF channel bandwidths of 5-, 10- and 20-MHz.

Amendments to IEEE 802.11-2007 base standard include Radio Resource Measurement of Wireless LANs (11k), Fast Basic Service Set Transition (11r) and 3650-3700 MHz Operation in the USA (11y). The Enhancements for Higher Throughput (11n) radio unit operates with nominal RF channel bandwidths of 20- and 40-MHz in the 2 400-2 500 MHz band and the 5 GHz License-Exempt bands.

The URL for the IEEE 802.11 Working Group is http://www.ieee802.org/11 . The IEEE 802.11-2007 standard is available through the Get IEEE 802™ program at http://standards.ieee.org/getieee802/802.11.html .  Approved Amendments and some Draft Amendments are available for purchase at http://standards.ieee.org/getieee802/drafts.html .

TABLE 6 (continued)

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IEEE 802.11-2007 (Annex 1) | 22 MHz | Symmetric up and down: DQPSK CCK BPSK PBCC – 1/2 QPSK PBCC – 1/2 | Uncoded/ CC | 2.5 Mbit/s | No | No | TDD | CSMA/ CA, SSMA | Variable frame duration | Nomadic |

TABLE 6 (continued)

| Standard | Nominal RF channel bandwidth | Modulation/ coding rate(1) – upstream – downstream | Coding support | Peak channel transmission rate per 5 MHz channel (except as noted) | Beam-forming support (yes/no) | Support for MIMO (yes/no) | Duplex method | Multiple access method | Frame duration | Mobility capabilities (nomadic/ mobile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IEEE 802.11-2007 (Annex 1) | 5 MHz 10 MHz 20 MHz | Symmetric up and down: 64-QAM OFDM 2/3, 3/4 16-QAM OFDM –1/2, 3/4 QPSK OFDM – 1/2, 3/4 BPSK OFDM – 1/2, 3/4 | CC | 13.5 Mbit/s | No | No | TDD | CSMA/ CA | Variable frame duration | Nomadic |
| IEEE 802.11-2007 (Annex 1) | 20 MHz | Symmetric up and down: 64-QAM OFDM 2/3, 3/4 16-QAM OFDM – 1/2, 3/4 QPSK OFDM – 1/2, 3/4 BPSK OFDM – 1/2, 3/4 8-PSK PBCC – 2/3 64-QAM DSSS-OFDM – 2/3, 3/4 16-QAM DSSS-OFDM – 1/2, 3/4 QPSK DSSS-OFDM – 1/2, 3/4 BPSK DSSS-OFDM – 1/2, 3/4 | CC | 13.5 Mbit/s | No | No | TDD | CSMA/ CA | Variable frame duration | Nomadic |
| IEEE 802.11-2007 (802.11n) | 20 MHz 40 MHz | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM both upstream and downstream | CC and LDPC | 75 Mbit/s | Yes | Yes | TDD | CSMA/ CA | Variable frame duration | Nomadic |

ISO/IEC 8802-11:2005, Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.