IEEE P802.11
Wireless LANs

| LB134 MAC CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains proposed resolutions for a subset of the MAC comments from LB 134 of the TGn D6.0 document.

GREEN = Comment resolution reviewed and accepted in an adhoc

YELLOW = Comment resolution reviewed in adhoc, but needs changes before adhoc will accept it

RED = Comment resolution reviewed by adhoc with no consensus reached, and comment is unlikely to find consensus within adhoc, therefore best moved to full TGn

HOT PINK = Comment resolution not yet reviewed, but expected to be controversial

BLUE = Comment resolution not yet reviewed, outline of expected resolution provided, with additional changes needed

GRAY = Comment no longer part of this document (e.g. comment transferred to another person/doc/group)

NO COLOR = Comment resolution not yet reviewed, and not expected to be controversial

REVISION INFORMATION:

R4:

9067: resolution modified and accepted by adhoc

9068: resolution modified and accepted by adhoc

9105: resolution modified and accepted by adhoc

9060: deleted from document – previously transferred to PHY

9106: resolution modified and accepted by adhoc

9144: resolution modified and accepted by adhoc

9147: resolution modified and accepted by adhoc

9162: resolution modified and accepted by adhoc

9163: resolution modified and accepted by adhoc

9168: resolution modified and accepted by adhoc

9171: resolution modified and accepted by adhoc

9301: resolution modified and accepted by adhoc

R3:

Not recorded

R2:

9067: now has a proposed resolution

9075: complete rewrite of proposed resolution

9086: now has a proposed resolution

9087: now has a proposed resolution

9090: now has a proposed resolution

9094: now has a proposed resolution

9105: now has a proposed resolution

9106: now has a proposed resolution

R1:

Marked as green, comments that were discussed and agreed upon during conf call August 13, 2008.

9025, 9026: slight rewording of resolutions

9045: slight rewording of proposed text change to reference 11.14.4

9053: added a proposed resolution where there was none previously, including new proposed text changes

9054: slight modification to the wording of the proposed text change, no fundamental technical change in the originally proposed change

9055: removed words “of operation” at the end of the proposed sentence of insertion

9060: comment transferred to PHY

9066: proposed resolution rewritten to reflect the meaning of the complete description of the Order Field

| 9025 | Chaplin, Clint | 119.59 | 9.6.0g.5.3 | "An MCS may be selected according to 9.6.0g.5.4" | "An MCS may alternatively be selected according to 9.6.0g.5.4" | Counter – TGn editor to replace the cited sentence with the following: "An alternative MCS may be selected according to 9.6.0g.5.4" Commenter should review the resolution to CID 9026. |
| --- | --- | --- | --- | --- | --- | --- |
| 9026 | Chaplin, Clint | 118.48 | 9.6.0q.5.2 | "An alternative rate or MCS may be selected according to 9.6.0g.5.4" | "An MCS may alternatively be selected according to 9.6.0g.5.4" | Reject – the existing wording conveys the intended meaning, which is that the rate selected in 9.6.0g.5.4 is an altenative to the rate selected here. The rules of 9.6.0g.5.4 do not stand on their own. Note that the rules in the cited subclause require a specific rate to be selected for most instances, and that 9.6.0g.5.4 allows some deviation from that certainty. Also, this subclause is selecting either a rate or MCS, so the subject of the cited sentence needs to mention both rate and MCS. |
| 9045 | Lee, Jin | 218.00 | 11.7.2 | "STAs may use features, rates or MCSs that are supported by both of the STAs in the direct link, even when the AP does not support those features." When both STAs support 40MHz operation but AP (e.g., non-HT AP) does not support 40MHz operation, a direct transmission should use 40MHz operation independent of AP's capabilities. For using 40MHz operation even when AP does not support that fearture, DLS Setup Request and DLS Setup Response should include a Secondary Channel Offset IE. | DLS Setup Request and DLS Setup Response may include a Secondary Channel Offset IE. | Counter – TGn editor shall change the cited sentence to read: "STAs may use features, rates or MCSs that are supported by both of the STAs in the direct link, even when the AP does not support those features, except for transmission of a 40 MHz mask PPDU, which is governed by the rules found in 11.14.4." 40 MHz operation should be excluded from this allowance, because normally, the use or prohibition against using 40 MHz is centrally determined by the AP. If individual STAs are allowed to use 40 MHz without an associated AP monitoring trigger events, then the two STA could be allowed to operate 40 MHz when it would otherwise not be allowed due to trigger events. |

| 9053 | Lee, Jin | 85.03 | 7.4.1 | Extended Channel Switch Announcement frame also should include Secondary Channel Offset IE for switching between 20/40 MHz BSS operation and 20 MHz BSS operation. Especially, when a regulatory class of a 20/40 MHz BSS operation is changed, Extended Channel Switch Announcement frame shall include Secondary Channel Offset IE | Extended Channel Switch Announcement frame should include Secondary Channel Offset IE. | Counter – TGn editor to make the changes shown under any heading including CID 9053 found within document 11-08-0976r2. |
| --- | --- | --- | --- | --- | --- | --- |

CID 9053

TGn Editor: Instructions to TGn editor are in here eventually – keep reading – or just skip along until you find the editing instructions…

Some time ago, there was a question as to whether those regulatory classes that support 40 MHz operation also support 20 MHz operation. I.e. the behaviour limits for such regulatory classes specified the allowance of the use of a secondary channel, but other regulatory classes are specified that use exactly the same channels, but do not allow the use of a 40 mhz channel. The question was whether it was necessary to change to a new 20 MHz-only regulatory class when an AP switches a BSS from 40 mhz operation to 20 mhz operation, or does the AP stay on the same reg class that supported 40 mhz, but now only uses 20 mhz? That question was answered by making a change to the description of the 40 mhz reg classes by explicitly noting that a BSS operating on one of those classes could be operating as either a 20/40 Mhz BSS or as a 20 MHz BSS.

Because of this change, CID 9053 becomes relevant. Had the previous question been answered differently, that 40 mhz classes only allow 40 mhz operation, and a switch to 20 mhz BSS operation would have required a change to a reg class that only allowed 20 mhz operation, then CID 9053 could be rejected, because the regulatory class alone is enough to determine both the allowed operating width in the BSS and the secondary channel offset.

However, because of the change that explicitly allows 20 mhz operation within a 40-tolerant regulatory class, there is a new question (maybe it is not new, but simply clarified) that when using the extended channel switch announcement – i.e. if the switch is to a new regulatory class that supports 40 mhz operation, will the BSS be operating as 20 mhz only, or will it be a 20/40 MHz BSS? If the announcement uses the “switch at an upcoming TBTT” then the question can be answered by the presence of the BSS width and/or secondary channel IE in the Beacon at that TBTT.  BUT if the extended channel switch announcement uses the “switch immediately” option, then the question is unresolved.

After discussion the group feels that the existing text can be clarified to provide the necessary guidance – i.e. it is not necessary to make it possible to perform both a channel switch and a width switch in the same operation. But it is necessary to clarify what happens to the width in a few cases – i.e. those cases for which the new reg class/channel allows 20/40 MHz BSS operation. Basically, an explicit rule needs to state that the width does not change if the new reg class supports 20/40 MHz operation. The proposed text changes provide such a rule.

TGn editor: add two new paragraphs to subclause “11.14.3.3 Channel management at the AP and in an IBSS” of TGn D6.0 p 224 L 6, to appear immediately following the first paragraph of that subclause, as shown:

11.14.3.3 Channel management at the AP and in an IBSS

While operating a 20/40 MHz BSS, an IDO STA or an AP may decide to move its BSS, and an AP may decide to switch the BSS to 20 MHz operation either alone or in combination with a channel move. These channel move or BSS width switch operations can occur if, for example, another BSS starts to operate in either or both of the primary or secondary channels, or radar is detected in either or both of the primary or secondary channels or for other reasons that are beyond the scope of this standard. Specifically, the AP or IDO STA may move its BSS to a different pair of channels, and the AP may separately, or in combination with the channel switch, change from a 20/40 MHz BSS to a 20 MHz BSS using either the primary channel of the previous channel pair or any other available 20 MHz channel. While operating a 20 MHz BSS, an IDO STA or an AP may decide to move its BSS and an AP may decide to switch the BSS to a 20/40 MHz BSS, either alone or in combination with a channel move.

If an AP or IDO STA uses one or more Extended Channel Switch Announcement frames without also using Beacon and Probe Response frames to announce a change of regulatory class and/or a change in channel(s), and the new regulatory class supports either of the behavior limits 13 or 14 as identified in the appropriate table of annex J (i.e. table J.1, J.2 or J.3), then the BSS width (20 MHz BSS or 20/40 MHz BSS) immediately after the switch shall be the same as the BSS width immediately before the transmission of the first Extended Channel Switch Announcement frame that announced the change. The AP or IDO STA may subsequently perform a BSS width change.

NOTE - If an AP or IDO STA uses one or more Extended Channel Switch Announcement frames without also using Beacon and Probe Response frames to announce a change of regulatory class and/or a change in channel(s), then the AP or IDO STA cannot change from 20 MHz BSS operation to 20/40 MHz BSS operation as part of that change, even if the new regulatory class supports 20/40 MHz BSS operation, because Extended Channel Switch Announcement frames do not convey secondary channel information (i.e. information regarding whether a secondary channel, if permitted in the regulatory class, is to be used).

| 9054 | Lee, Jin | 219.03 | 11.9.2 | "The Quiet element defines an interval during which no transmission shall occur in the current channel." In 20/40 MHz BSS operation, which channel does Quiet element indicate? Primary channel or secondary channel or both? Quiet element should not allow a transmission on only primary channel. Then, there is no way to prohibit a transmission on a secondary channel. For quieting a secondary channel, add a channel number field in Quiet element. | Even though HT AP switchs a BSS to 20 MHz operation by a Secondary Channel Offset field set to SCN, a direct transmission can still use a secondary channel. For quieting a secondary channel, add a channel number field in Quiet element. Otherwise, make a new Extended Quiet IE having a quiet interval and a quiet channel number. | Counter – TGn editor to make the changes shown under any heading including CID 9054 found within document 11-08-0976r1. The existing element does not specifically call out any channel, and the language of 11.9.2 implies that all transmissions are to cease (without being specific with respect to channel), and it says that this is accomplished through the use of the NAV. For TGn 20/40 MHz BSS operation, each STA maintains only one NAV, which is used to prevent transmission in either of the two channels. So the need for another channel within the element does not seem necessary, but some clarification to the existing text might be helpful. |
| --- | --- | --- | --- | --- | --- | --- |

CID 9054

TGn editor: change subclause 11.9.2 of TGn D6.0 p 219 L 9 as shown:

11.9.2 Quieting channels for testing

Control of the channel is lost at the start of a quiet interval, and the NAV is set by all the STAs in the BSS for the length of the quiet interval. Transmission by any STA in the BSS of any MPDU and any associated acknowledgment within either the primary channel or the secondary channel (if present) of the BSS shall be complete before the start of the quiet interval. If, before starting transmission of an MPDU, there is not enough time remaining to allow the transmission to complete before the quiet interval starts, the STA shall defer the transmission by selecting a random backoff time, using the present CW (without advancing to the next value in the series). The short retry counter and long retry counter for the MSDU or A-MSDU are not affected.

| 9055 | Lee, Jin | 64.00 | 7.3.2.28 | "In BSS Load element, the channel utilization field is defined as the percentage of time, the AP sensed the medium was busy, as indicated by either the physical or virtual carrier sense mechanism." BSS Load element shall consider 40 MHz operation. For example, 40 MHz phase of PCO is considered as a busy time for 20 MHz STA. However, PCO active STAs shall not consider 40 MHz phase as a busy time. Is 40 MHz phase of PCO a busy time or not? If it is considered as a busy time for 20 MHz STA, it is necessary to make new 20/40 MHz BSS Load element that specify the channel utilization of 20/40 MHz BSS. | Redefine the BSS Load element for considering 40 MHz operation. | Counter – accept in principle – TGn editor to make the changes shown under any heading including CID 9055 found within document 11-08-0976r1. |
| --- | --- | --- | --- | --- | --- | --- |

CID 9055

TGn editor: insert the following new text and WG editor instruction to modify the third paragraph of subclause “7.3.2.28 BSS Load element” as shown, to appear just before subclause “7.3.2.29 EDCA Parameter Set element” of TGn D6.0 p 64 L 1 as shown:

7.3.2.28 BSS Load element

Change paragraph three as shown:

The Channel Utilization field is defined as the percentage of time, normalized to 255, the AP sensed the medium was busy, as indicated by either the physical or virtual carrier sense (CS) mechanism. When more than one channel is in use for the BSS, the Channel Utilization value is calculated only for the primary channel. This percentage is computed using the formula, ((channel busy time/(dot11ChannelUtilizationBeaconIntervals * dot11BeaconPeriod * 1024)) * 255), where channel busy time is defined to be the number of microseconds during which the CS mechanism, as defined in 9.2.1, has indicated a channel busy indication, and the MIB attribute dot11ChannelUtilization- BeaconIntervals represents the number of consecutive beacon intervals during which the channel busy time is measured. The default value of dot11ChannelUtilizationBeaconIntervals is defined in Annex D.

| 9066 | Marshall, Bill | 13.40 | 7.1.3.1.9 | making the meaning of the Order field in the Frame Control field of a received frame dependent on the settings of various internal parameters within the transmitting STA requires the receiver to know those settings before being able to interpret the meaning of the field in the received frame. Such advance knowledge is, in general, impossible. Further, the restriction is unnecessary, as the frame contains sufficient information to adequately decode the frame contents. (followup to CIDs 7073 and 7074) Leaving the definition of the Order field undefined for certain PHYs but defined for other PHYs is a likely source of confusion in the future. If there were some good technical reason to take this risk, then this text would be acceptable, but I see no such technical reason. The text here is a landmine waiting for the next unsuspecting amendment that defines higher throughput. If it defines backward compatibility modes for the HT PHY (as is likely), this text will need to be modified to include the new values of the TXVECTOR components that indicate such backward compatibility. At that point the receiver will not have a clue as to which TXVECTOR parameters were used to transmit the frame, and how to interpret the received frame. | delete "transmitted with a value of HT_GF or HT_MF for the FORMAT parameter of the TXVECTOR" | Reject –Transmissions of the sort that might cause receiver confusion are explicitly disallowed by the statement that says “otherwise, the order field is set to 0” – that last phrase means that if a receiver receives a QOS Data or Mgmt frame with order field set to 1, then it must have had TXVECTOR FORMAT parameter set to HT_GF or HT_MF, because those are the only two conditions for which this setting of the order field is allowed. The commenter’s assertion that the order field is undefined for other PHYs is incorrect, because the cited phrase of “otherwise the order field is set to 0” covers the case of those PHYs. |
| --- | --- | --- | --- | --- | --- | --- |
| 9067 | Marshall, Bill | 37.51 | 7.2.3.1 | The rejection of CID 7075 stated "Reject - The scenario of purchasing TGn equipment and then deploying it with the TGn functionality disabled is expected to be limited. The additional overhead of about 30 bytes to transmit these elements for this relatively infrequent scenario is minimal. The requested change disallows the option of a TGn AP from sending the beacon at a lower rate to allow for greater range of the BSS, and the existing amendment does not prevent an AP from taking the described action - that is - sending the beacon at a higher rate. This is an option left to the implementer and/or device manager." First, IEEE 802 is not a marketing organization, it is a technical organization. Technical comments need to be addressed on their technical merits, not on marketing projections (which are necessarily subject to different industry viewpoints). Second, there was nothing in the requested change that disallowed anything, and certainly did not prevent the AP from taking the action desired by the response. Third, the problem identified by CID 7075 is the excessive data that is being required to be included every low speed Beacon frame, merely because the AP also supports higher speed transmission as well. This is the technical problem that needs to be addressed. | Change the "Notes" for HT Capabilities and for HT Operation from "is present" to "may be present". Also acceptable is to change to "is present when dot11HighThroughputOptionImplemented is set to TRUE and dot11PhyType is set to 7 and the FORMAT parameter of the TXVECTOR is not NON-HT, and may be present otherwise". Similar change to Tables 7-10, 7-11, 7-12, 7-13, 7-14, and 7-15. | Reject – There is nothing in the standard or in the proposed amendment that requires a setting of TRUE for the MIB variable dot11HighThroughputOptionImplemented when hardware and/or software to implement the HT features are present – this allows a STA to have a value of FALSE for dot11HTOptionImplemented while still having the hardware and/or software present to perform these features. |
| 9068 | Marshall, Bill | 63.66 | 7.3.2.27 | CID 7080 observed: "The current text in TGn D5.0 adequately defines the requirements for interoperable behavior of STAs using PSMP, independent of whether the HT PHY is being used or not. However, there is no way for the non-AP STA know that the AP supports the feature prior to making the request; trial and error in settings in the ADDTS does not seem a good solution. The only missing piece for general use of PSMP is the advertisement of the capability in a way that is not tied to the advertisement of capability for the HT PHY. The 802.11 architecture defines a single MAC that interoperates with multiple different PHYs, and no pre-11n MAC feature is PHY-restricted as is this one. LB124/6182 rejection noted that it is possible to allow the feature for devices with non-HT PHY, but it was too much work to make the changes. The current text works; the only change needed is the addition of an entry in an existing table, less work than writing the text of that rejection." but was rejected stating: "Reject - The commenter is incorrect. A bit exists in the HT Capabilities element which indicates support or lack thereof, on the part of the AP for PSMP operation. PSMP operation is not completely separated from various other new features, including specifically, RIFS and MTBA. Without the inclusion of these mechanisms, the value of the PSMP mechanism is questionable, since its main objective is to reduce wake time for PS STAs and to reduce the latency of delivery of frames to PS STAs. The dependency of PSMP on these additional features is conveniently conveyed by the fact that the PSMP support bit exists in the HT Capabilities element which can only be transmitted by HT STA and when transmitted, indicates support for the other features. Additionally, when optional features are made available to broad classes of devices, the number of possible STA-STA combinations that might be encountered in the field increases to a point that exceeds the abililty to test the various possible combinations within a reasonable amount of development time.". CID 7080 was objecting to the lack of a way to advertise the capability of PSMP without also advertising HT Capabilities. However, the rejection stated "Reject - The commenter is incorrect. A bit exists in the HT Capabilities element which indicates support or lack thereof, on the part of the AP for PSMP operation." One of the major points of the comment is that the only advertisement is in the HT Capabilities element, which also indicates, at a minimum, support for 20MHz operation. The quoted rejection effectively endorsed this comment and refused to address it. The rejection of CID 7080 also incorrectly stated that RIFS is requied for PSMP, which is incorrect (see PICS, HTM12, where there is no dependency on RIFS); see 9.16.1.2 and 9.16.1.3 "PPDUs within a PSMP-DTT/PSMP-UTT may be separated using RIFS or SIFS."; also note that an HT AP can effectively say "no RIFS in my BSS" by setting the RIFS Mode bit to zero in the HT Capabilities; also note that the HT STA can say "no RIFS to me" by setting the RIFS Mode bit to zero in its HT Capabilities. While RIFS can be used by a STA during the PSMP exchanges, it is not required and the MAC procedures described handle PSMP both with and without RIFS. The rejection of CID 7080 also stated that MTBA is required for PSMP, which is correct. However, prior to D6.0, there was no prohibition against a non-HT STA using MTBA/MTBAR, as it was an additional mode defined within the existing BlockAck mechanism and was available to all STAs that implement the BlockAck mechanism. The prohibition in D6.0 was only added as a "Counter" to my CID 7081; there is no technical reason to disallow a non-HT STA from using PSMP due to its reliance on MTBA/MTBAR. No other technical reason was given for prohibiting the use of PSMP without the HT PHY. Finally, it is laughable that TGn, which is defining a multitude of optional features, should complain about the number of possible STA-STA combinations causing trouble with development and testing time. In fact the proposed change here would reduce the number of possible STA-STA combinations for development and testing, as it removes the distinction between a non-HT STA and an HT STA that does not support PSMP, leaving just STAs that support PSMP and STAs that don't. | Support for PSMP should be advertised separately with a bit in the Extended Capabilities element. Complete set of changes are given in CID 7080. | Reject – The proposed change to allow PSMP use by non-HT STA would require much more than adding a PSMP capability bit outside of the HT Capabilities element. Complexity of the rules of behavior specified in the amendment and the number of possible combinations of supported features of individual devices and the ensuing complexity of interactions among devices in the field is a technical consideration. |
| 9069 | Marshall, Bill | 124.49 | 9.7c | There is no technical reason to prohibit a STA without the HT PHY from sending and receiving A-MSDUs. I disagree with the resolution to CID 7079 that claimed such a technical reason exists, but didn't state the reason. | Either provide a technical justification for the prohibition, or delete the first sentence in this paragraph and change in the second sentence "a frame containing an HT Capabilities element" to "an indication of support of A-MSDUs". Other changes as in CID 7079. | Reject – The resolution to CID 7079 indicated a technical reason for rejecting that comment. CID 7079 requested bits to allow non HT STA to signal their support for the feature, and the response to the CID indicated that such STA were not allowed to support the feature. I.e. CID 7079 was different from CID 9069. For CID 9069, it is not clear that a technical reason is needed to either accept or reject the comment. A task group has many decisions to make when choosing various aspects of a protocol and all of those decisions are ultimately simply based on the collective opinion of the body. In this case, the collective opinion of the body is to not include this feature for the use case indicated by the commenter. |
| 9070 | Marshall, Bill | 135.41 | 9.10.6 | There is no technical reason to prohibit a STA without the HT PHY from sending and receiving MTBA/MTBAR frames. I disagree with the resolution to CID 7081 that claimed such a technical reason exists. | Either provide a technical justification for the prohibition, or delete ", and shall be set to 0 otherwise" from the text added by the Counter to CID 7081 (three places). Other changes as in the proposed resolution for CID 7081. | Reject –It is not clear that a technical reason is needed to either accept or reject the comment. A task group has many decisions to make when choosing various aspects of a protocol and all of those decisions are ultimately simply based on the collective opinion of the body as expressed through the voting process. In this case, the collective opinion of the body is to not include this feature for the use case indicated by the commenter. |
| 9075 | Marshall, Bill | 13.36 | 7.1.3.1.9 | Original text said "any non-QoS data frame", without restriction on the type of STA that is doing the transmitting. The change here, adding "transmitted by a non-QoS STA" requires the receiver to know the capabilities of the sending STA before being able to interpret the meaning of the Order field. Such advance knowledge is, in general, impossible. Further, the restriction is unnecessary, as the frame contains sufficient information to adequately decode the frame contents. (followup to CIDs 7073 and 7074) | delete "transmitted by a non-QoS STA" | Reject – The interpretation at a receiver is simple, since the distinction for determining the meaning of the bit is based on the subtype (QOS or non-QOS subtype). The portions of the text that the commenter refers to are simply restrictions on which STA may transmit frames with the value 1 in the field. I.e. the number of instances in which a receiver will encounter the value “1” in this field are reduced by these restrictions, but the meaning always depends on the subtype. |
| 9077 | Marshall, Bill | 31.22 | 7.2.1.8.1 | CID 7137 fixed one occurrence of this problem, but missed this one. | Apply the change agreed with CID 7137 to Table 7-6j | Counter – accept in principle – TGn editor to make the change requested- that is, change “shall not be” to “is not” as found in Table 7-6j of subclause 7.2.1.8.1 of TGn D6.0 |
| 9080 | Marshall, Bill | 70.59 | 7.3.2.57.2 | BSSs don't receive things. STAs receive. The Counter resolution to CID 7167 fixed one of the two problems identified here, but not the other | change "BSSs" to "STAs" | Counter – accept in principle - TGn editor to change the phrase “Indicates whether other BSSs receiving this information” to “Indicates whether APs receiving this information or reports of this information” as found within table 7-43k in subclause 7.3.2.57.2 (within the forty MHz intolerant bit description) in TGn draft D6.0. |
| 9084 | Marshall, Bill | 112.46 | 9.6.0a | In all of the rearranging and rewording done with CID 7215 (and others in this subclause) one very important statement was lost - that which appears as deleted text on page 113 lines 16-18 of D6.0: "No STA shall transmit a unicast frame at a rate that is not supported by the receiver STA, as reported in any Supported Rates and Extended Supported Rates element in the management frames transmitted by that STA." | Insert a new paragraph at page 112 line 47: "A STA shall not transmit a unicast frame at a rate that is not supported by the receiver STA, as reported in any Supported Rates and Extended Supported Rates element in the management frames transmitted by that STA." Alternatively, show that such a statement is extraneous, and that the subclauses of 9.6 cover all cases and include this general rule in their more specific statements. | Reject – the statement mentioned was not lost. It has been distributed among 9.6.0f.6., 9.6.0g.2, 9.6.0g.4 and 9.6.0g.5.3 |
| 9085 | Marshall, Bill | 114.03 | 9.6.0b | I disagree with the counter to CID 7204. The comment on D5.0 was "very badly worded normative statement. If the STA somehow goofs in the calculation of its 40MHz support, does that automaticaly give it the capability to transmit and receive at those extra rates? Suggest removing the normative language from the statement." and the suggested remedy was "change "shall support" to "supports"". Instead, it was resolved as "Counter - Replace cited sentence with the following: "An HT STA shall not indicate support for 40 MHz unless it supports reception and transmission of 40 MHz PPDUs using all MCSs within the BasicMCSSet and all MCSs that are mandatory for the attached PHY."" The indication of support for 40MHz is done by the SME, not by the MAC, through the procedures given in 11.14. The indication of 40MHz support is given to the MLME by the SME in the MLME-ASSOCIATE.request primitive. So the MAC (being defined here in clause 9) has no control over this indication of 40MHz support. A normative statement such as this is out of place. | change "shall not indicate" to "does not indicate", or delete the sentence completely. | Counter – tgn editor to move the cited sentence to become the new fourth paragraph of 11.14.2. |

| 9086 | Marshall, Bill | 114.09 | 9.6.0c | the implementation of CID 7205 made this statement extraneous, as every reference to selecting a rate from the BSSBasicRateSet now indlucdes a statement of what to do if that set is empty | delete this paragraph. Unless something new is added to 9.6.0c, delete the heading and renumber the subclauses. | Counter – accept in principle – TGn editor to make the changes shown under any heading including CID 9086 found within document 11-08-0976r3 |
| --- | --- | --- | --- | --- | --- | --- |
| 9087 | Marshall, Bill | 114.15 | 9.6.0d | the implementation of CID 7205/7206 made this statement extraneous, as every reference to selecting a rate from the BSSBasicMCSSet now indlucdes a statement of what to do if that set is empty | delete this paragraph | Counter – accept in principle – TGn editor to make the changes shown under any heading including CID 9087 found within document 11-08-0976r3 |

CID 9086, 9087

TGn editor: delete the heading and text for subclause “9.6.0c Basic Rate Set” as found in TGn D6.0 beginning on p 114 L 7, and renumber remaining subclauses as appropriate:

TGn editor: delete the first paragraph of subclause “9.6.0d Basic MCS Set” as found in TGn D6.0 beginning on p 114 L 15, which begins with the words: “In 9.6.0f and 9.6.0g when an MCS from the BSSBasicMCSSet parameter is required”

TGn editor: change subclause “9.6.0f.1 Rate selection for non-STBC Beacon and PSMP frames” as found in TGn D6.0 beginning on p 114 L 49, as shown:

9.6.0f.1 Rate selection for non-STBC Beacon and non-STBC PSMP frames with a group address in the Address1 field

This subclause describes the rate selection rules for non-STBC Beacon and non-STBC PSMP frames with a group address in the Address1 field.

If the BSSBasicRateSet parameter is not empty, a non-STBC Beacon or non-STBC PSMP frame with a group address in the Address1 field shall be transmitted in a non-HT PPDU using one of the rates included in the BSSBasicRateSet parameter.

If the BSSBasicRateSet parameter is empty, the frame shall be transmitted in a non-HT PPDU using one of the mandatory PHY rates.

TGn editor: change the phrase “non-STBC Beacon and PSMP frames” in  subclause “9.6.0f.3 Rate selection for other group addressed data and management frames” as found in TGn D6.0 beginning on p 115 L 11 as shown:

non-STBC Beacon and non-STBC PSMP frames with a group address in the Address1 field

TGn editor: change subclause “9.6.0f.4 Rate selection for polling frames” item b) as found in TGn D6.0 beginning on p 115 L 36 as shown:

b) Otherwise, the data frame shall be transmitted at a rate or MCS as defined in 9.6.0f.3, treating the frame as though it has a group address in the Address1 field, solely for the purpose of determining the appropriate rate or MCS.

TGn editor: change the second bullet item after the first paragraph of subclause “9.6.0g.5.4 Selection of an alternate rate or MCS” as found in TGn D6.0 beginning on p 120 L 4 as shown:

the alternative rate is in either the BSSBasicRateSet parameter or is a mandatory rate of the attached

PHY; and

| 9090 | Marshall, Bill | 135.24 | 9.10.4 | resolution of CID 7252 is wrong. Without the statement that comparisons are done circularly modulo 2**12, the word "lower" at line 12 really means "lower", and the existing block ack algorithms will no longer work. (Note: CID 7252 pointed out that an existing paragraph in the base standard was not correctly quoted in the amendment; the "Counter" resolution showed that missing paragraph as explicitely deleted with the amendment). | Undo the changes made in 0742r5 under CID 7252. Insert a paragraph at page 135 line 24 matching the base text: "All comparisons of sequence numbers are performed circularly module 2**12." (note the use of "**" should be replaced with superscript) | Reject – The resolution to CID 7252 was correct. The reason why the fifth paragraph of the baseline as cited by CID 7252 is shown as strikethrough in the current draft is because the same text has been inserted earlier in the document, so that it may have applicability from that point forward. The baseline location for the text is too far forward to correctly apply to all baseline and amendment references to sequence number comparisons – some references to comparisons occur before this location. See 9.10.1, which is the introduction to the entire set of block ack subclauses and is the current location of the modulo comparison statement. |
| --- | --- | --- | --- | --- | --- | --- |
| 9094 | Marshall, Bill | 201.58 | 11.1.3.4 | I disagree with the resolution to CID 7360. This MIB reference is wrong. See page 202 line 1 for a correct reference to the MIB variable. Claiming that errors already exist in the base document is not a technical resolution to this requested change. | If the claim is that the existing text is correct, explain why "dot11RegDomainsSupportEntry" does not appear in Annex A. Otherwise, fix it. CID #7360 provided the correction needed | Reject – The proposed resolution to this comment is outside of the scope of TGn since it is referring to text that is part of the baseline and to functionality that is not part of the TGn scope of work. |
| 9105 | Marshall, Bill | 113.65 | 9.6.0b | Text refers to "The BSSBasicRateSet parameter" without any explanation of what this is a parameter to. The text in D5.0 that was deleted from this subclause provided the context for this sentence | Either delete "parameter" from this sentence and the following one, or explain where this parameter appears. | Counter – TGn editor to make changes shown under any heading including CID 9105 in document 11-08-0976r4 – The text that was deleted was text that already exists in 10.3.3.1.4 – the amendment should not create redundant normative text, which is why the text from D5.0 mentioned by the commenter was deleted. Because of the normative description of this behavior in 10.3.3.1.4, any additional description of the meaning of the parameter would be unnecessary and potentially conflicting and/or redundant. Deleting the word “parameter” would be equivalent to ignoring all previous use of the parameter in the baseline, which described the origin of the parameter just once in 9.2, which TGn has deleted, as also redundant. However, such a reference shoud be present in 9.2, and another is needed for BSSBasicMCSSet, so, as resolution to this comment, that language is restored in a non-normative way, and BSSBasicMCSSet language is added in parallel. See also 10.3.10.1.4 |

CID 9105

TGn editor: Restore the deleted sentence in 9.2 DCF with some modifications and additions, of TGn D6.0 as shown:

All STAs that are members of a BSS are able to receive and transmit at all the data rates in the BSSBasicRateSet parameter of the MLME-START.request or BSSBasicRateSet parameter of the BSSDescription representing the SelectedBSS parameter of the MLME-JOIN.request, see 10.3.3.1.4 and 10.3.10.1.4. All HT STAs that are members of a BSS are able to receive and transmit using all the MCSs in the BSSBasicMCSSet parameter of the MLME-START.request or BSSBasicMCSSet parameter of the BSSDescription representing the SelectedBSS parameter of the MLME-JOIN.request, see 10.3.3.1.4 and 10.3.10.1.4.

TGn editor: Change the heading name and delete the first two sentences of 9.6.0b Support for Mandatory Rate / Mandatory MCS / Basic Rate / Basic MCS of TGn D6.0 as shown:

9.6.0b Support for 40 MHz MCSs

| 9106 | Marshall, Bill | 118.48 | 9.6.0g.5.2 | With the two "shall" paragraphs at lines 16 and 31, there is no room left for the alternative given in 9.6.0g.5.4. Similarly for the MCS computation in 9.6.0g.5.3, where there is no room left for the alternative given in 9.6.0g.5.4. | either delete 9.6.0g.5.4, or modify the normative statements in 9.6.0g.5.2 and 9.6.0g.5.4 to allow the alternative procedures to be used. | Counter - TGn editor to make the changes shown under any heading including CID 9106 found within document 11-08-0976r4 |
| --- | --- | --- | --- | --- | --- | --- |

CID 9106

TGn editor: change subclause “9.6.0g.5.2 Selection of a rate or MCS” as found in TGn D6.0 beginning on p 118 L 32, as shown:

9.6.0g.5.2 Selection of a rate or MCS

To allow the transmitting STA to calculate the contents of the Duration/ID field, a STA responding to a received

frame transmits its control response frame at a primary rate or MCS, or at an alternate rate or MCS, according to the following rules:

— If the control response frame (CTS, ACK or Immediate BlockAck including BlockAck sent as a

response to an implicit Block Ack request) is carried in a non-HT PPDU, the STA shall select the

highest rate in the BSSBasicRateSet parameter that is less than or equal to the rate (or non-HT reference

rate, see 9.6.2) of the previous frame to become the primary rate. If no rate in the BSSBasicRateSet parameter meets these

conditions, the STA shall select the highest mandatory rate of the

attached PHY that is less than or equal to the rate (or non-HT reference rate, see 9.6.2) of the previous

frame to be the primary rate. The STA may select an alternate rate according to the rules in 9.6.0g.5.4. The STA shall transmit the non-HT PPDU control response frame at either the primary rate or the alternate rate, if one exists.

— If the control response frame is carried in an HT PPDU, then it is transmitted at an MCS as

determined by the procedure defined in 9.6.0g.5.3.

The modulation class of the control response frame shall be selected according to the following rules:

— If the received frame is of a modulation class other than HT and the control response frame is carried

in a non-HT PPDU, the control response frame shall be transmitted using the same modulation class

as the received frame. In addition, the control response frame shall be sent using the same value for

the TXVECTOR parameter PREAMBLE_TYPE as the received frame.

— If the received frame is of the modulation class HT and the control response frame is carried in a

non-HT PPDU, the control response frame shall be transmitted using one of the ERP-OFDM or

OFDM modulation classes.

— If the control response frame is carried in an HT PPDU the modulation class shall be HT.

The selection of the value for the channel width (CH_BANDWIDTH parameter of the TXVECTOR) of the

response transmission is defined in 9.6.0g.6.

TGn editor: change subclause “9.6.0g.5.3 Control response frame MCS computation” as found in TGn D6.0 beginning on p 118 L 50, as shown:

9.6.0g.5.3 Control response frame MCS computation

If a control response frame is to be transmitted within an HT PPDU, the channel width (CH_BANDWIDTH

parameter of the TXVECTOR) shall be selected first according to 9.6.0g.6 and then the MCS shall be selected

from a set of MCSs called the CandidateMCSSet as described in this subclause.

The Rx Supported MCS Set of the STA that transmitted the frame eliciting the response is determined from

its Supported MCS Set field as follows:

— If a bit in the Rx MCS Bitmask subfield is set to 0, the corresponding MCS is not supported.

— If a bit in the Rx MCS Bitmask subfield is set to 1 and the integer part of the data rate (expressed in

units of Mb/s) of the corresponding MCS is less than or equal to the rate represented by the Rx Highest

Supported Data Rate field, then the MCS is supported by the STA on receive. If the Rx Highest

Supported Data Rate field is zero and a bit in the Rx MCS Bitmask is set to 1, then the corresponding

MCS is supported by the STA on receive.

The CandidateMCSSet is determined using the following rules:

— If the frame eliciting the response was an STBC frame and the Dual CTS Protection bit is set to 1, the

CandidateMCSSet shall contain only the Basic STBC MCS.

— If the frame eliciting the response had an L-SIG Duration value (see 9.13.5) and initiates a TXOP,

the CandidateMCSSet is the MCS Set consisting of the intersection of the Rx Supported MCS Set of

the STA that sent the frame that is eliciting the response and the set of MCSs that the responding

STA is capable of transmitting.

— If none of the above conditions is true, the CandidateMCSSet is the BSSBasicMCSSet parameter. If

the BSSBasicMCSSet parameter is empty, the CandidateMCSSet shall consist of the set of mandatory

HT PHY MCSs.

MCS values from the CandidateMCSSet that cannot be transmitted with the selected CH_BANDWIDTH parameter

value shall be eliminated from the CandidateMCSSet.

The choice of a response MCS is made as follows:

a) If the frame eliciting the response is within a non-HT PPDU,

1) Eliminate from the CandidateMCSSet all MCSs that have a Data Rate greater than or equal to

the Data Rate of the received PPDU (the mapping of MCS to Data Rate is defined in 20.6).

2) Find the highest indexed MCS from the CandidateMCSSet. The index of this MCS is the index

of the MCS that is the primary MCS for the response transmission.

3) If the CandidateMCSSet is empty, the primary MCS is the lowest indexed MCS of the mandatory MCSs.

b) If the frame eliciting the response is within an HT-PPDU,

1) Eliminate from the CandidateMCSSet all MCSs that have an index that is higher than the index

of the MCS of the received frame.

2) Determine the highest number of spatial streams (NSS) value of the MCSs in the CandidateMCSSet

that is less than or equal to the NSS value of the MCS of the received frame. Eliminate all

MCSs from the CandidateMCSSet that have an NSS value that is not equal to this NSS value.

The mapping from MCS to NSS is dependent on the attached PHY. For the HT PHY, see 20.6.

3) Find the highest-indexed MCS of the CandidateMCSSet for which the modulation value of

each stream is less than or equal to the modulation value of each stream of the MCS of the

received frame and for which the coding rate value is less than or equal to the coding rate value

of the MCS from the received frame. The index of this MCS is the index of the MCS that

is the primary MCS for the response transmission. The mapping from MCS to modulation and coding rate

is dependent on the attached PHY. For the HT PHY, see 20.6. For the purpose of comparing

modulation values, the following sequence shows increasing modulation values: BPSK, QPSK,

16-QAM, 64-QAM.

4) If there is no MCS that meets the condition in step 3), repeat step 3) beginning with the set of

MCS with the NSS value that is one less than the NSS value found in step 2) from the CandidateMCSSet

and continuing by successively decrementing NSS until step 3) produces a nonempty

result.

Once the primary MCS has been selected, the STA may select an alternate MCS according to 9.6.0g.5.4. The STA shall transmit the HT-PPDU control response frame using either the primary MCS or the alternate MCS, if one exists.

TGn editor: change subclause “9.6.0g.5.4 Selection of an alternate rate or MCS” as found in TGn D6.0 beginning on p 119 L 61, as shown:

9.6.0g.5.4 Selection of an alternate rate or MCS for a control response frame

An alternate rate

may be selected provided that the following conditions are met:

— the duration of frame at the alternate rate is the same as the duration of the

frame at the primary rate determined by 9.6.0g.5.2; and

— the alternate rate is in either the BSSBasicRateSet parameter or a mandatory rate of the attached

PHY; and

— the modulation class of the frame at the alternate rate is the same class as that of

the primary rate selected by 9.6.0g.5.2.

An alternate MCS

may be selected provided that the following conditions are met:

— the duration of the frame at the alternate MCS is the same as the duration of the

frame at the primary MCS; and

— the alternate MCS is in the CandidateMCSSet that was generated according to the procedure of

9.6.0g.5.3.

| 9144 | Nabar, Rohit | 140.00 | 9.10.7.6.3 | Actions to undertake when BAR is received is explained in this section but it is not clear if this is valid for all BAR variants like MTBAR | Add a statement here or in clause 9.10.6 that these rules/operations apply for all BAR variants like MTBAR too | Counter - TGn editor to make the changes shown under any heading including CID 9144 found within document 11-08-0976r4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

CID 9144

TGn editor: change subclause “9.10.6 Selection of BlockAck and BlockAckReq variants” as found in TGn D6.0 beginning on p 135 L 33, as shown:

9.10.6 Selection of BlockAck and BlockAckReq variants

The Compressed Bitmap subfield of the BA Control field or BAR Control field shall be set to 1 in all Block-

Ack and BlockAckReq frames sent from one HT STA to another HT STA, and shall be set to 0 otherwise.

The Multi-TID subfield of the BA Control field shall be set to 1 in all BlockAck frames related to an HTimmediate

agreement transmitted inside a PSMP sequence, and shall be set to 0 otherwise. The Multi-TID

subfield of the BAR Control field shall be set to 1 in all BlockAckReq frames related to an HT-immediate

agreement transmitted inside a PSMP sequence, and shall be set to 0 otherwise.

Where the terms BlockAck and BlockAckReq are used within 9.10.7 and 9.10.8, the appropriate variant according to this subclause (e.g. Basic, Compressed, Multi-TID) is referenced by the generic term.

| 9147 | Qian, Luke | 10.00 | 6.1.5 | There are potential DoS attacks identified on the receiving side of the data plane (Figure 6-1) where "Block ACK Reordering" occurs before "MPDU decryption and integrity". This comment is basically carried over from CID 8075 in LB129, CID 5899 in LB115 and CID 6232 in LB124. More types of potential DoS attacks have been identified since LB115, for example: -Forged packets with advanced Sequence Numbers (SN) -Captured and Replayed packets with modified SN. -False Block ACK Request (BAR) with advanced SN. Those DoS attacks possess a unique character of "hit-and-run" that a hacker only needs to transmit one frame to cause the receiver drops packets for a period of time. The proposed change is a part of a whole set of solutions to address the DoS issues. This comment was rejected in LB115 on the following basis: "It is accepted that a denial of service (DoS) attack exists in which a forger generates Data MPDUs with an arbitrary SN, forcing a STA to consider validly sent MPDUs to be outside its BA window. The proposed change correctly addresses this attack. However, the same DoS attack also exists as a replay attack. In this case the hacker captures a single encrypted Data MPDU addressed to the victim. It then replays this MPDU as much as it wants to, while changing its SN field. Because the SN field is not part of the AAD, this MPDU continues to pass through the integrity check logic, and will still cause the Block Ack receiver buffer to be flushed. Eventually the problematic MPDU reaches the replay logic, where it is discarded - but not before the damage to the BlockAck buffer has been done. Given that the proposed solution does not fully address the attack on the block ack reordering buffer." In responding to the resolution, multiple solutions have been proposed to address the whole set of the issues since LB115. Unfortunately, we didn't reach a converged solution good enough to be widely acceptable in the comment resolution process for LB 124. The issue remains unaddressed and the comment was rejected as: "GEN: 2008-05-15 17:35:58Z Reject - While the described DoS attack is a potential vulnerability, the additional complexity and cost of implementation of the jointly developed solutions in 08/0665r4 was considered to be unacceptable. ". With this guideline, we will work with TGn on a less complexity solution. Following up on this resolution, we submitted 802.11-08/0833r3 for a scaled down version of the joint proposal (802.11-08/0665r0) to simplify the solutions by focusing only on those DoS types witht he most significant damages. Unfortunately, this proposal was rejected with the following resolution during LB129: "MAC: 2008-07-17 15:43:49Z Reject - the group is unclear as to whether the issues cited by this CID are significant enough to warrant a change to the draft." We don't think this resolution is acceptable in that we have clearly described the issues in a number of submissions ( see 802.11-08/0703r0 for example) and a number of security experts from various companies have assessed their severities (see 802.11-07/2163r0 for example). While we have had a good start that multiple solutions from variuos companies have been proposed, as summarized in 802.11-08/0755r1, we should continue working on converging the solutions. | change the order of "Block ACK Reordering" and "MPDU Decryption and Integrity" Will work with TGn on a proposal for this. | Counter – Tgn editor to reverse the order of the ‘MPDU Decryption and Integrity (optional)’ and ‘Block Ack Reordering’ process blocks in figure 6-1, and add a callout to figure 6-1 that becomes a part of the diagram with the following wording: “The ‘MPDU Decryption and Integrity (optional)’ and ‘Block Ack Reordering’ processes may be performed in any order.” – the callout to appear next to the two blocks. |
| --- | --- | --- | --- | --- | --- | --- |
| 9148 | Qian, Luke | 10.00 | 6.1.5 | A hacker can transmit a captured and replayed packets with advanced SN without modification to cause a DoS. This is carried from CID 8076 in LB129 and CID 6233 in LB124. The issue remains unaddressed and the comment was rejected on the same basis for CID 8075 in LB129. We will work with TGn for a solution. | Specify a "Replay Detection" mechanism before "Block ACK Reordering" to prevent such a DoS. Will work with TGn on a proposal for this. | Luke to present. |
| 9162 | Stephens, Adrian | 117.21 | 9.6.0g.3 | The rules don't actually specify, in the case of dual CTS what rates or MCSs should be used - they only specify the vlaue of the STBC parameter. | Add rule that says something like: "If the STBC parameter is set 0, use a rate from the BSSBasicRateSet or an MCS from the BSSBasicMCSSet. Otherwise use the Basic STBC MCS." | Counter – TGn editor to make the changes shown under all headings that include CID 9162 within document 11-08-0976r4. |

CID 9162

TGn editor: change the third paragraph of subclause “9.6.0g.3 Rate selection for CF_End control frames” as found in TGn D6.0 beginning on p 117 L 21, as shown:

9.6.0g.3 Rate selection for CF_End control frames

A STA that transmits a CF-End control frame at the end of a TXOP that was obtained by a non-AP STA through the use of the dual CTS mechanism shall transmit the CF-End control frame with the same value for the TXVECTOR parameter STBC, TXVECTOR parameter MCS (if present), and TXVECTOR parameter RATE, as was used for the transmission of the matching control frame at the beginning of the TXOP. The matching control frame is defined as follows:

- For the first CF-End transmitted in the TXOP, the matching control frame is the first RTS transmitted in the TXOP.

- For the second CF-End transmitted in the TXOP, the matching control frame is the first CTS that follows the first RTS transmitted in the TXOP.

- For the third CF-End transmitted in the TXOP, the matching control frame is the second CTS that follows the first RTS transmitted in the TXOP.

A STA that transmits a CF-End control frame at the end of a TXOP that was obtained by an AP through the use of the dual CTS mechanism shall transmit the CF-End control frame with the same value for the TXVECTOR parameter STBC, TXVECTOR parameter MCS (if present), and TXVECTOR parameter RATE, as was used for the transmission of the RTS matching control frame at the beginning of the TXOP. The matching control frame is defined as follows:

- For the first CF-End transmitted in the TXOP, the matching control frame is the first CTS-to-self transmitted in the TXOP.

- For the second CF-End transmitted in the TXOP, the matching control frame is the first RTS transmitted in the TXOP.

| 9163 | Stephens, Adrian | 116.08 | 9.6.0g | In earlier drafts we moved the repeated statements of what to select if the BSSBasicRateSet or BSSBasicMCSSet is empty to 9.6.0c and 9.6.0d. New material has re-introduced this redunant specification. | remove "or a rate from the mandatory rate set of the attached PHY if the BSSBasicRateSet is empty" (116.60); "or using a rate from the mandatory rate set of the attached PHY if the BSSBasicRateSet is empty" (117.03) "or from the mandatory rate set of the attached PHY if the BSSBasicRateSet is empty." (117.29) "or from the mandatory rate set of the attached PHY if the BSSBasicRateSet is empty" (117.61) | Counter – see CID 9086. |
| --- | --- | --- | --- | --- | --- | --- |
| 9168 | Stephens, Adrian | 151.39 | 9.15.3 | "or as part of an aggregation" - this is informal and ambiguous given that there are two types of aggregation. | Replace "aggregation" with "A-MPDU". | Counter – change the phrase “either transmitted by itself, or as part of an aggregation” to “either transmitted as a non-A-MPDU frame or within an A-MPDU” |
| 9171 | Stephens, Adrian | 185.36 | 10.3.6.1.2 | "The parameter may be present only if the MIB attribute dot11HighThroughputOptionImplemented is TRUE." I don't believe this is strict enough. If an HT STA doesn't include this in the association request with an HT AP, then the HT AP will treat it like a non-HT STA. But the rules we have relate to an "HT STA", not a "STA that transmitted an HT Capabilities element". So this would put the STA in an ambiguous state. | Change so that the parameter shall be present if the STA is a HT STA and if the peer STA is an HT STA. Same comment for all the primitives carrying this parameter. | Counter - TGn editor to change the phrase “The parameter may be present only if the MIB attribute dot11HighThroughputOptionImplemented is TRUE” to “The parameter is present if the MIB attribute dot11HighThroughputOptionImplemented is TRUE” within all subclauses of clause 10 contained within TGn D6.0 that are either a SAP.request or SAP.response and that include such a statement. |
| 9189 | Vlantis, George | 91.38 | 7.4a.1 | Denial of Service (DoS) attacks caused by A-MPDU: DoS issues with A-MPDU are well-doumented within the IEEE (c.f. 11-08/0833; 11-08/0665; etc.). Fairness issues have been raised in the WFA testing. Even more troubling are the Internet websites that are popping up with how to implement DoS with A-MPDU (c.f. google "Denial of Service" and "A-MPDU" and refer to any of the first 8 hits.) | DoS attacks that rely on driver/application layer support can be hacked. Some duration limits (MAC/PHY), besides the 65 535 octet limit, should be put on A-MPDU. |  |
| 9301 | Ji, Lusheng | 69.06 | 7.3.2.57.2 | The resolution for my comment regarding the same matter from LB129 is not acceptable. The "reject" resolution cites that RIFS and MTBA are the reasons that PSMP is not separable from the rest of HT Capability. However 9.16 clearly suggests that PSMP does not depend on either RIFS or MTBA. PSMP will work with either RIFS or SIFS. MTBA is not required either. | repeating my proposed change from LB129: remove "PSMP support" bit from HT Capability IE, in stead in 7.3.2.27 make bit 1 of the extended capabilities IE "PSMP support" capability bit. | Reject – see CID 9068. |

References: