IEEE P802.11
Wireless LANs

| LB134 COEX 20 40 CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains proposed resolutions for a subset of the COEX comments from LB 134 of the TGn D6.0 document

COLORIZONIFICATIATION GUIDE:

GREEN = Comment resolution reviewed and accepted in an adhoc

YELLOW = Comment resolution reviewed in adhoc, but needs changes before adhoc will accept it, outline of those changes sometimes included

RED = Comment resolution reviewed by adhoc with no consensus reached, and comment is unlikely to find consensus within adhoc, therefore best moved to full TGn

HOT PINK = Comment resolution not yet reviewed, but expected to be controversial

BLUE = Comment resolution reviewed at least once, with suggested changes now implemented – ready for re-review

GRAY = Comment no longer part of this document (e.g. comment transferred to another person/doc/group)

NO COLOR = Comment resolution not yet reviewed, and not expected to be controversial

REVISION INFORMATION:

R0:

First revision

| 9046 | Lee, Jin | 220.33 | 11.14 | "While operating a 20/40 MHz BSS, an IDO STA or an AP may decide to move its BSS, and an AP may decide to switch the BSS to 20 MHz operation either alone or in combination with a channel move." "20 MHz BSS: a BSS in which the Secondary Channel Offset field is set to SCN." Even though HT AP switchs a BSS to 20 MHz operation by a Secondary Channel Offset field set to SCN, a direct transmission can still use 40 MHz operation. In 20 MHz BSS, the direct transmission shall not allow 40 MHz operation. | When AP moves its BSS to 20 MHz operation, the direct transmission also shall not allow 40 MHz operation. | Reject – No change is needed to the draft. 11.14.4.2 already clearly disallows the stated behavior. This subclause describes all of the conditions that must be TRUE before a STA may transmit a 40 MHz PPDU. Included among these conditions is the one that identifies the current setting of the Secondary Channel Offset field of the AP’s HT Operation element. |
| --- | --- | --- | --- | --- | --- | --- |
| 9047 | Lee, Jin | 224.31 | 11.14.3.3 | "through the New Regulatory Class field of transmitted Extended Channel Switch Announcement elements." It seems that New Regulatory Class field specifies Secondary Channel Offset. Do not change the semantics of a New Regulatory Class field of a Extended Channel Switch Announcement IE. | Do not change the semantics of a New Regulatory Class field of a Extended Channel Switch Announcement element. Extended Channel Switch Announcement frame should include Secondary Channel Offset IE. | Counter – see CID 9053 and document 11-08-0976r3 which discuss this issue and propose some clarifying text. The resolution of CID 9053 essentially reaffirms that some transitions of channel and width cannot be made with one change announcement, and other transitions can be made with a single announcement. The changes proposed for CID 9053 aim to clarify what happens in a few cases that were ambiguous. But clearly, for example, a change from a 40 MHz BSS to a regulatory class that only supports 20 MHz BSS operation forces a change of channel width for the BSS. |
| 9048 | Lee, Jin | 224.00 | 11.14.3.3 | "When changing to a new pair of channels, the New Regulatory Class field specifies the position of the secondary channel relative to the new primary channel, i.e., either above or below." It seems that New Regulatory Class field specifies Secondary Channel Offset. Do not change the semantics of a New Regulatory Class field of a Extended Channel Switch Announcement element. | Do not change the semantics of a New Regulatory Class field of a Extended Channel Switch Announcement element. Extended Channel Switch Announcement frame should include Secondary Channel Offset IE. | Counter – see CID 9047. |
| 9049 | Lee, Jin | 224.00 | 11.14.3.3 | "An IDO STA may inform HT STAs within its BSS that it is performing a channel pair relocation by including an Extended Channel Switch Announcement element in Beacon, Probe Response, and Extended Channel Switch Announcement frame transmissions until the intended channel switch time." Do not change the semantics of a New Regulatory Class field of a Extended Channel Switch Announcement IE. | Replace the sentence to the following. "An IDO STA may inform HT STAs within its BSS that it is performing a channel pair relocation by including an Secondary Channel Offset IE in Beacon, Probe Response, Channel Switch Announcement and Extended Channel Switch Announcement frame transmissions until the intended channel switch time." | Counter – see CID 9047 |
| 9050 | Lee, Jin | 226.00 | 11.14.4.2 | "The Extended Channel Switch Announcement action frame and the Extended Channel Switch Announcement element can each be used to indicate a transition from 20/40 MHz BSS operation to 20 MHz BSS operation and vice versa, and to indicate whether a secondary channel, when it exists, is above or below the primary channel in frequency." Do not change the semantics of a New Regulatory Class field of a Extended Channel Switch Announcement IE. | Extended Channel Switch Announcement frame should include Secondary Channel Offset IE. Replace the sentence to the following. "The Extended Channel Switch Announcement action frame can each be used to indicate a transition from 20/40 MHz BSS operation to 20 MHz BSS operation and vice versa, and to indicate whether a secondary channel, when it exists, is above or below the primary channel in frequency." | Counter – see CID 9047 |
| 9051 | Lee, Jin | 224.00 | 11.14.3.3 | "In order to maintain existing associations and/or minimize disruption to communications with other STAs while making a channel width change or while performing a channel pair relocation, an AP may inform HT STAs within its BSS that it is making the change by including an Extended Channel Switch Announcement element in Beacon, Probe Response, and Extended Channel Switch Announcement frame transmissions until the intended channel switch time." Extended Channel Switch Announcement element does not support a change pair relocation. | Replace the sentence to the following. "In order to maintain existing associations and/or minimize disruption to communications with other STAs while making a channel width change or while performing a channel pair relocation, an AP may inform HT STAs within its BSS that it is making the change by including an HT Operation element in Beacon, Probe Response, and Extended Channel witch Announcement frame transmissions until the intended channel switch time." | Counter – see CID 9047 |
| 9052 | Lee, Jin | 225.65 | 11.14.4.1 | Extended Channel Switch Announcement element does not specify the secondary channel offset. Remove "The Extended Channel Switch Announcement element". | Remove "The Extended Channel Switch Announcement element". | Counter – see CID 9047 |
| 9081 | Marshall, Bill | 84.32 | 7.3.2.61 | CID 7181 was accepted but not implemented; see CID 7181 for description of the problem here. | change "BSS" to "STA" | Counter – accept in principle. TGn editor to make the changes shown under any heading that includes CID 9081 within document 11-08-0994r0 |

CID 9081

TGn editor: change the fifth paragraph of subclause “7.3.2.61 20/40 BSS Coexistence element” of TGn D6.0 p 84 L 32 as shown:

The Forty MHz Intolerant field, when set to 1, prohibits APs receiving this information or reports of this information from operating as a 20/40 MHz BSS. When set to 0, it does not prohibit a receiving STA from operating a 20/40 MHz BSS. This field is used for inter-BSS communication. The definition of this field is the same as the definition of the Forty MHz Intolerant field in the HT Capabilities element (see 7.3.2.57) and its operation is described in 11.14.11.

| 9092 | Marshall, Bill | 146.31 | 9.13.3.4 | I disagree with the resolution to CID 8087 and the rejection of CID 7296 as a consequence. Coexistence should not be optional. Coexistence shall be mandatory. | change "may" to "shall" | Reject – The line drawn between mandatory and optional behaviors with respect to interactions of TGn draft devices with OBSS devices parallels the analogous line that was drawn between 802.11g devices and their OBSS neighbors. The decision to place the line where it is in the 802.11g instance has proved itself, through the currently installed base of 802.11 devices, to have been a correct choice, and the TGn group believes this to be ample support for the TGn draft language to which the commenter refers. |
| --- | --- | --- | --- | --- | --- | --- |
| 9096 | Marshall, Bill | 222.11 | 11.14.3.1 | Coexistence should not be optional. I disagree with the text proposed as a resolution to CID 7042. | change "switches" to "shall switch" | Reject – The behavior described in the cited subclause is written in declarative form to avoid being redundant with normative text that appears elsewhere in the document. The other normative text uses the verb “shall” to describe essentially the same behavior. See 11.14.12. |
| 9110 | Marshall, Bill | 220.07 | 11.9.7.3 | As worded, this says that a single clear scan is all that is needed forever | delete "if the HT STA starts a BSS on that channel", making the requirement apply either before starting a BSS or while a BSS is active. | Reject – Language already in place within 9.13.3.1 achieves the desired behavior. The differentiator is that 9.13.3.1 describes behavior required of a BSS that is already operating. |
| 9146 | Nabar, Rohit | 230.44 | 11.14.9 | If a STA was unable to transmit a 40 MHz mask PPDU because the secondary channel was occupied during the PIFS interval prior to backoff expiry on control channel, it has two options - transmit 20 MHz or restart backoff. When STA chooses the "Transmit 20 MHz" option it needs to make sure that the MAC duration value calculated is based on time duration it takes when using 20 MHz transmissions. There may be some ambiguity here so a note will make it clear. | Add a note below line 45 on the lines of "Note-When a STA chooses this option it needs to make sure that the MAC duration values are computed based on 20 MHz PPDU" | Group discussion? |
| 9149 | Raissinia, Ali | 230.00 | 11.14.9 | The draft states "the STA may transmit a pending 40 MHz mask PPDU only if the secondary channel has also been idle during the times the primary channel CCA is performed (defined in 9.2.10) during an interval of a PIFS immediately preceding the expiration of the backoff counter." The PIFS duration is chosen so that the time is longer than SIFS period (which is used for frame exchange sequence) plus the uncertainty between primary and secondary MAC clock as well as possible detection error on "absence of energy". If there's no mention of PIFS "using long timeslot" then the PIFS would be 19us (case where secondary channel consisted of 11.b only devices). The 19us is short to be used as ACK time-out value. | Add the phrase "(using long timeslot)" right after the word " PIFS". | Group discussion? |
| 9177 | Stephens, Adrian | 231.03 | 11.14.11 | The first para is redundant, as it is a subset of the first sentence of the second para. | Delete first para. | Accept. TGn editor to execute the requested change. |
| 9185 | van Zelst, Allert | 230.57 | 11.14.10 | By deleting "(using short timeslot for 5 GHz band and long timeslot for 2.4 GHz band)" after PIFS, the PIFS could now be 10 us SIFs plus a short slot time of 9 us is a total of 19 us. In my opinion this is too short to make sure to not stap on a data - ack sequence on the secondary channel. | Include "(using short timeslot for 5 GHz band and long timeslot for 2.4 GHz band)" after PIFS again, or change PIFS to DIFS, or introduce a fixed time of, say, 25 us | Group discussion. |
| 9188 | Vlantis, George | 532.47 | T.5.2 | Coexistence of non-802.11n networks: The intent to promote sharing of the 2.4GHz spectrum under the Bluetooth WPAN case is a laudable one. One can also foresee other Wireless standards (e.g 802.16) that will also want to share the 2.4 GHz spectrum. While a sensing mechanism is provided for sensing non-HT 802.11OBSS, the only mechanisms for reporting issues with WPAN, WMAN, WRAN, etc. is setting the "20/40 MHz BSS Intolerant Channel Report" element (Clause 7.3.2.59 on page 83) or the "Forty MHz Intolerant" bit in the HT capabilities info field (sub clause 7.3.2.57 on page 70). No guidance is given for under which circumstances this bit should be set, nor how to detect these networks. Surely some SINR criterion should be added. While the reason for rejection of a similar comment in LB124 and LB129 was that the designer has the freedom to do as they wish with the 40MHz Intolerant mechanisms, but coexistence with non-802.11 networks in the 2.4GHz band should be addressed in the standard. I've give this some futher thought, and I believe using a TDMA-approach, similar to 802.15.2's "Alternating wireless medium access" could be developed with the existing PCO mechanism. PCO uses the time-domain to manage 20MHz and 40MHz devices. An enhancement to the PCO mechanism, or a requirement for 802.15.2's AWMA should be provided in the draft. | The 802.15.2 mechanism(s) (e.g. "Alternating wireless medium access" should be required to allow Bluetooth (and other networks) to co-exist with 40MHz 802.11n devices. Consider extending PCO to include a quiet period for non-802.11n devices to operate. | Group discussion? |

References: