IEEE P802.11
Wireless LANs

| TGn LB134 Submission for CIDs 9148 in MAC ad-hoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Luke Qian | Cisco | 4125 Highlander Parkway, Richfield, OH 44286 | 1 330 523 2051 | lchia@cisco.com |
| Nancy Cam-Winget | Cisco | 225 East Tasman Drive, San Jose, CA95134 | 1 408 853 0532 | ncamwing@cisco.com |
| Doug Smith | Cisco | 500 Alden Rd, Ste 207 Markham, On L3R 5H5 Canada | 1 905 470 4803 | dsmit@cisco.com |
| Harish Ramamurthy | Marvell | 5488 Marvell Lane, Santa Clara, CA – 95054 | 408-222-9683 | harishr@marvell.com |
| Sandesh Goel | Marvell | Marvell India Pvt Ltd, Pune, India | +91-9960596612 | sgoel@marvell.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB134 comments:

- 9148

The changes marked in this document are based on TGn Draft version 6.00.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please delete all those indicating the related CID as "(# XXXX)" after appropriate time.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# 9148 (C)

# CID 9148

| CID | Page | Clause | Comment | Proposed change |
| --- | --- | --- | --- | --- |
| 9148 | 10.00 | 6.1.5 | A hacker can transmit a captured and replayed packets with advanced SN without modification to cause a DoS. This is carried from CID 8076 in LB129 and CID 6233 in LB124. The issue remains unaddressed and the comment was rejected on the same basis for CID 8075 in LB129. We will work with TGn for a solution. | Specify a "Replay Detection" mechanism before "Block ACK Reordering" to prevent such a DoS. Will work with TGn on a proposal for this. |

### Proposed Resolution: Counter

For reference, see 11-08/1021r1.

## Discussion for CID 9148:

For reference, see 11-08/1021r1.

## Edits for CID 9148:

### 7.3.2.25.3 RSN Capabilities

TGn Editor:

Replace Figure 7-74 with (the change being the addition of B12 as the PBAC and reassigning the Reserved bits as being B13-B15):

B8             B9                B10                        B11                B12         B13   B15

| Reserved | PeerKey Enabled | SPP A-MSDU Capable | SPP A-MSDU Required | PBAC | Reserved |
| --- | --- | --- | --- | --- | --- |

Insert the following list iterm after Iterm “Bit 11” on page 63:

- Bit 12: Protected Block Ack Agreement Capable (PBAC). A STA sets the PBAC subfield of RSN Capabilities field to 1 to indicate it supports PBAC. Otherwise this subfield is set to 0.

Change list item “ Bit 8 and 10-15” as follows:

- Bit 8 and 1213-15: Reserved. The remaining subfields of the RSN Capabilities field are reserved and shall be set to 0 on transmission and ignored on reception.

### 7.4.4 Block Ack Action frame details

Change the first paragraph of 7.4.4 as follows:

The ADDBA frames are used to set up or modify Block Ack for a specific TC or TS.  The Action field value associated with each frame format within the Block Ack category are defined in Table 7-54.

Add the following subclasue 9.10.9

9.10.9 Protected Block Ack Agreement

A STA defines support for a protected Block Ack agreement by setting the MFPC, MFPR and PBAC RSN Capabilities subfields to 1.  This section describes the behavior for when a STA has successfully negotiated protection of a Block Ack agreement.

A PBAC STA may choose to associate with a non-PBAC STA if dot11RSNAPBACRequired is set to 0; otherwise it shall only associate to PBAC enabled STAs.  If a PBAC STA is communicating with a non-PBAC STA, it shall follow the rules for an uprotected BA agreement.

In the following description, a hole in the receiver reordering buffer refers to one or more sequence numbers starting at WinStart that have not been received correctly by the receiver.

A STA that has successfully negotiated PBAC shall follow the following rules as a Block Ack originator:

- the STA shall not transmit a BAR which causes the WinStart at the receiver to jump over a hole

- the STA shall use a Robust Management ADDBA request action frame if it needs to move the WinStart at the receiver to jump over a hole

A STA that has successfully negotiated PBAC shall follow the following rules as a Block Ack recipient:

- the receiving STA shall respond to a BAR from a PBAC enabled originator with a BA. If the Block Ack Starting Sequence Control field value causes a request to change WinStart to jump over a hole, the value shall be ignored (e.g. WinStart is not updated) and dot11PBACErrors shall be incremented by 1.

- upon receipt of a valid Robust Management ADDBA request action frame for an established Block Ack agreement whose TID and Transmitter, the STA shall update its WinStart based on the Starting Sequence number in the Robust Management ADDBA request frame.

### Annex D

TGn Editor: add a comma (e.g. “,” to the dot11HighThroughputOptionImplemented “TruthValue”) and append as the last value to the Dot11StationConfigEntry table the following:

dot11RSNAPBACRequired		TruthValue

Insert the following element to the end of the dot11StationConfigTable element definitions as the last entry (e.g. after dot11HightThroughputOptionImplemented):

dot11RSNAProtectedManagementFramesEnabled			OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This variable indicates whether or not this STA Protects Block Ack agreements."

DEFAULT { FALSE }

::= { dot11StationConfigEntry <ANA>}

TGn Editor: add the following entry in Dot11CountersTable in the appropriate order:

dot11PBACErros		Counter32

TGn Editor: Insert the following definition of the new dot11CountersEntry in the appropriate order as follows:

dot11PBACErrors			OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This variable indicates the number or errors encountered in the PBAC procedures."

DEFAULT { 0 }

::= { dot11CountersEntry <ANA>}

## References:

[1] Draft P802.11n_D6.00.pdf

[2] Draft P802.11w_D6.0.pdf