IEEE P802.11
Wireless LANs

| TGn LB134 Submission for CIDs 9149 and 9185 in COEX ad-hoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tomoko Adachi | Toshiba Corporation | 1, Komukai Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582 Japan |  | tomo.adachi@toshiba.co.jp |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB134 comment:

- 9149 and 9185

The changes marked in this document are based on TGn Draft version 6.0.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please delete all those indicating the related CID as "(# XXXX)" after appropriate time.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# 9149 (R), 9185 (R)

# CID 9149

| CID | Page | Clause | Comment | Proposed change |
| --- | --- | --- | --- | --- |
| 9149 | 230.00 | 111.14.9 | The draft states "the STA may transmit a pending 40 MHz mask PPDU only if the secondary channel has also been idle during the times the primary channel CCA is performed (defined in 9.2.10) during an interval of a PIFS immediately preceding the expiration of the backoff counter." The PIFS duration is chosen so that the time is longer than SIFS period (which is used for frame exchange sequence) plus the uncertainty between primary and secondary MAC clock as well as possible detection error on "absence of energy". If there's no mention of PIFS "using long timeslot" then the PIFS would be 19us (case where secondary channel consisted of 11.b only devices). The 19us is short to be used as ACK time-out value. | Add the phrase "(using long timeslot)" right after the word " PIFS". |

# CID 9185

| CID | Page | Clause | Comment | Proposed change |
| --- | --- | --- | --- | --- |
| 9185 | 230.57 | 11.14.10 | By deleting "(using short timeslot for 5 GHz band and long timeslot for 2.4 GHz band)" after PIFS, the PIFS could now be 10 us SIFs plus a short slot time of 9 us is a total of 19 us. In my opinion this is too short to make sure to not stap on a data - ack sequence on the secondary channel. | Include "(using short timeslot for 5 GHz band and long timeslot for 2.4 GHz band)" after PIFS again, or change PIFS to DIFS, or introduce a fixed time of, say, 25 us |

Note: The place that the commenter is referring to is 11.14.9 not 11.14.10.

## Discussion:

From 11.14.3.2, if a 20 MHz OBSS is detected on the selected secondary channel, the value of 20/40 Operation Permitted will be false ("P == OTi for all values of i" will be false).

Therefore, the 20/40 MHz operation will be never used in 2.4 GHz whenever a 20 MHz OBSS is detected on the selected secondary channel. The only case for overlap is with 20/40 MHz BSSs using the same primary and secondary channels. (If there are 20/40 MHz BSSs that have different channel allocations, then you cannot operate in 40 MHz.)

So the data-ack sequence to occur in the secondary channel will be detected on the primary channel and there should be no issue.

One may be aware of the case when an OBSS just comes after the scanning, but it will be detected in the next scan and will be a corner case.

For the 19 us (10 us SIFS + 9 us Short Slot time) vs 16 us (11g frame exchange in 2.4 GHz), given that any legacy frames is at least 24 us in length, and taking into account backoff, the probability of 40 MHz transmission clobbering a legacy frame in the secondary channel comming 16 us just after you sensed idle in the primary will be quite small.

And multiplying the above two cases, the probability will be really small.

## Proposed resolutions to CIDs 9149 and 9185:

Reject.

The rules in 11.14 avoid having 40 MHz operation with a 20 MHz BSS overlap in the selected secondary channel and avoid having 40 MHz operation with other 20/40 MHz BSSs using different sets of primary and secondary channels. One may be aware of the case when an OBSS just comes after the scanning, but it will be detected in the next scan and will be a corner case. Given that any frames is at least 24 us in length, and taking into account backoff, the probability of 40 MHz CCA missing a legacy frame exchange separated in 16 us on the secondary channel after sensing the primary channel will be quite small, and under the above corner case, the probability will be really small.

## References:

[1] Draft P802.11n_D6.03.pdf

[2] IEEE Std 802.11TM-2007