IEEE P802.11
Wireless LANs

| Normative Text Proposal for LB#133 Comment Resolution on Co-Located Interference Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jing Zhu | Intel | 2111, NE 25th Ave., Hillsboro, OR 97124 | +1 (503) 265 7073 | jing.z.zhu@intel.com |
| Jon Rosdahl | CSR | Highland, UT |  | jrosdahl@ieee.org |

Abstract

This submission contains the normative text to resolve IEEE 802.11v LB#133 comments with CID 302~308.

The text is aligned with P802.11v-D3

7.3.2.80 Co-located Interference Report element

Change Figure v69 of 7.3.2.80 as follows:

|  | Element ID | Length | Report Period | Interference Level | Interference Level Accuracy/Interference Index / Receive Indicator |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |
|  | Interference Interval | Interference Burst Length | Interference Start Time | Interference Center Frequency | Interference Bandwidth |
| Octets | 4 | 4 | 4 | 2 | 2 |

Figure v69: Co-located Interference Report element format

Change Figure v70 of 7.3.2.80 as follows:

|  | B0-B3 | B4-B6 | B7 |
| --- | --- | --- | --- |
|  | Expected Accuracy | Interference Index | Receive Indicator |
| Bits | 4 | 4 3 | 1 |

Figure v70—Interference Level Accuracy/Interference Index/Receive Indicator field format

Add a new 9th paragraph as follows:

The Receive Indicator bit field set to 1 indicates that the reported co-located interference does not cause the performance degradation of its 802.11 PHY receive functionalities, for example, Receive PLCP . The value of 0 does not carry any specific meaning, and shall be ignored.

- Co-located Interference Reporting

Change the 5th paragraph as follows:

A STA accepting a request for automatic Co-located Interference reporting shall send a Co-located Interference Response frame to the requesting STA if it detects that a co-located radio or another co-located interferer is causing performance degradation to its 802.11 receiver the STA. The STA that has not received any Colocated Interference Request frame may send a Co-located Interference Response frame autonomously with the Dialog Token field set to zero. The Dialog Token field is set to the nonzero value received in the Colocated Interference Request frame which was used to enable automatic responses. If co-located interference is present, the reporting STA shall send Co-located Interference Response frames with interval indicated by the Report Period field or if the level of the interference caused by co-located interferer is significantly changing or if the time characteristics of the interference is significantly changing. The Report Period timer shall restart whenever the reporting STA sends out a Co-Located Interference Response frame. The criteria a reporting STA uses for determining significant changes are internal to the reporting STA and outside the scope of this standard.