IEEE P802.11
Wireless LANs

| LB134 PHY Comment Resolution General and Beamforming Set |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission suggests resolutions of LB129 PHY comments. The following CIDs are addressed: CID 9099, 9100, 9101, 9111, 9187, 9102, 9061, 9062, 9034, 9103, 9079, and 9170.

The changes marked in this document are based on TGn Draft 6.0

Revision History:

Rev 0: Initial version.

General Comments:

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9099 | 20.1.4 | 245 | 48 | CID 7468 was rejected with the statement "Reject - as shown in document 11-08/0785r1." Document 785r1 says merely "Reject. The usage in Clause 20 is consistent with the usage in Clauses 17 and 19, where the normative effect of the TXVECTOR parameters is implicit." There is no question that the text in clauses 17 and 19 define the normative behavior of certain TXVECTOR parameters. However, the procedures in those clauses are invoked when a different PHY is used in the STA. What is missing here is a normative chain of statements (i.e. sentences containing the word "shall") that connect a request from the MAC layer to transmit a PPDU with these various values of the FORMAT parameter to the normative procedures contained in those other clauses. Merely stating that the usage is consistent does not provide the normative chain needed to link the Clause 20 HT PHY with the previous PHY procedures. | change "determines" to "shall determine" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 245, line 48, modify text as follows:

“The FORMAT parameter determines shall determine the overall structure of the PPDU as follows: “

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9100 | 20.2.4 | 254 | 28 | CID 7471 was rejected with the statement "Reject - as shown in document 11-08/0743r2." Document 743r2 merely says "Reject. Reason for rejection: “When the FORMAT parameter is set to NON_HT, the behavior of the HT PHY is defined in other Clauses as shown in Table 20-3, ...” – This is a reference to the appropriate clauses." There is no question that the text in the other clauses define the normative behavior needed. What is missing here is a normative chain of statements (i.e. sentences containing the word "shall") that connect a request from the MAC layer to transmit a PPDU with these various values of the FORMAT parameter to the normative procedures contained in those other clauses. Merely stating that the reference is to the appropriate clauses does not provide the normative chain needed to link the Clause 20 HT PHY with the previous PHY procedures. | change "is defined" to "shall be as defined" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution:  Accept.

TGn Editor: on page 254, line 28, modify text as follows:

“When the FORMAT parameter is set to NON_HT, the behavior of the HT PHY is shall be as defined in other Clauses

as shown in Table 20-3, ..“

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9101 | 20.3.2 | 258 | 15 | CID 7474 was rejected with the statement "Reject - as shown in document 11-08/0743r2." Document 743r2 merely says "Reject. Reason for rejection: “..are followed by a period of no transmission with a length of 6 µs called the Signal Extension, which is defined in 19.3.3.4.5.” – This is a reference to the appropriate sub-clause" There is no question that the text in the other clauses define the normative behavior needed. What is missing here is a normative chain of statements (i.e. sentences containing the word "shall") that connect a request from the MAC layer to transmit a PPDU with these various values of the TXVECTOR to the normative procedures contained in those other clauses. Merely stating that the reference is to the appropriate clauses does not provide the normative chain needed to link the Clause 20 HT PHY with the previous PHY procedures. | change "are followed" to "shall be followed" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 258, line 15, modify text as follows:

“   .. with TXVECTOR parameter FORMAT values of HT_MF and HT_GF are shall be followed by a period of no transmission with a length of 6 μs called the Signal Extension,.. “

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9111 | 20.3.2 | 258 | 21 | normative statement needed here | change "are followed" to "shall be followed" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution:  Accept.

TGn Editor: on page 258, line 21, modify text as follows:

“Transmissions of frames with TX_VECTOR parameter NO_SIG_EXTN set to FALSE are shall be followed by a period of no transmission for a duration of Signal Extension, see to 9.2.10a.   “

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9187 | 20.3.9.5.3 | 287 | 13 | The line "In HT-greenfield format frames, the HT SIGNAL field is transmitted with the same number of subcarriers" is fussy, if the same number of occupied subcarriers is meant, then it is incorrect. | Please clarify. | Accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept in principle.

TGn Editor: on page 287, line 15, modify text as follows:

“In HT-greenfield format frames, the HT SIGNAL field is transmitted with the same number of subcarriers, the same cyclic shifts, and the same spatial mapping as the preceding portions of the preamble.    “

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9102 | 20.3.11.5 | 290 | 46 | CID 7519 was rejected with the statement "Reject - as shown in document 11-08/0734r1" Document 734r1 merely says "Reject – Regarding “… are each encoded by the rate-½ convolutional encoder defined in 17.3.5.5.”, it is a reference to the appropriate sub-clause." There is no question that the text in the other clauses define the normative behavior needed. What is missing here is a normative chain of statements (i.e. sentences containing the word "shall") that connect a request from the MAC layer to transmit a PPDU with BCC encoding to the normative procedures contained in those other clauses. Merely stating that the reference is to the appropriate clauses does not provide the normative chain needed to link the Clause 20 HT PHY with the previous PHY procedures. | change "are each encoded" to "shall each be encoded" | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 290, line 47, modify text as follows:

“..where applicable, are shall each be encoded by ..   “

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9061 | 20.3.11.7.3 | 297 | 14 | The N_{COL} and N_{ROW} specifications for MCS32 is not well defined in the Table 20-16. Reason: MCS32 be definition is a 40MHz HT transmission, however the interleaving needs to be done like a Clause17 device | Add note in Table 20-16 - 'For MCS32, the interleaving needs to be done as per Clause 17.3.5.6' | Accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution:  Accept in principle.

TGn Editor: on page 296, line 65, add the following sentence:

“If LDPC encoding was used, no frequency interleaving is performed,hence the parsed streams are immediately mapped to symbols as defined in 20.3.11.8. MCS32 interleaving shall be as defined  in Clause 17.3.5.6.“

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9062 | 20.3.22.7 | 326 | 61 | Request for clarification: Are there any rules for the power level of the packets in successive RIFS transmission? Am thinking that there are no rules and successive packets can have (a) different MCS (b) may/may not be beamformed (c ) May/may not use all the available antenna's etc (for eg 2 antenna for one packet and 3 for other etc) (d) different transmit power (e) etc etc I am thinking that having a similiar transmit power between successive RIFS transmission may facilitate simpler AGC/packet detection mechanism. May I request the opinion of the group on this aspect. |  | There are no rules that RIFS separated packets must use same MCS and beamforming weights as previous packet. Therefore, received power levels may vary. |
| --- | --- | --- | --- | --- | --- | --- |

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9034 |  | 331 | 62 | This paragraph and the following paragraph have a lot of repeated information between the two. I cannot tell if two different concepts are being elucidated here, or if they are supposed to be identical. | Either point out the differences between the two paragraphs better, or get rid of the repeated information. | Reject. Reason for rejection: it is clear that the first paragraph describes case when signal loss occurs (references to “CarrierLost” in the paragraph). Second paragraph describes proper data reception without errors (references to “NoError” in the paragraph). |
| --- | --- | --- | --- | --- | --- | --- |

Beamforming Comments:

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9079 | 7.3.1.29 | 54 | 35 | There are two alternative views of Table 7-25j, as seen from CID 7151 and the response. A STA that has calculated the angle values and desires to send them in a Compressed Beamforming Report field needs to know how to quantize the angles. A STA that has received a Compressed Beamforming Report field needs to understand how to convert the bits in that field into angles. The title of this table indicates the former. This table contents do the latter, and provide no guidance for the former. The rejection of CID 7151 stated "Reject - The standard specification needs to specify the operation of the transmission (at the beamformer); it should define how to calculate angles from fed back values of k." Before anyone can calculate the angles from fed back values of k, the standard needs to specify how the original measured angles are used to calculate the values of k. | as given in CID 7151 | Reject the comment. Reason for rejection: Commenter states: “Before anyone can calculate the angles from fed back values of k, the standard needs to specify how the original measured angles are used to calculate the values of k.” This is not necessary. Transmitter calculates angles directly from the formula given in Table 7-25j. There is no ambiguity and discrepancy in this portion of the spec. Sender of the k values knows that the recipient of the k values will interpret the angles according to the quantization formula. Therefore, it is up to sender of the k values to determine k values properly so that the beamforming gain is maximized. Furthermore, even if there was a connection between original measured angles and k values then this connection would be hidden as a part of the algorithm at the receiver, and therefore spec transparent (proprietary). |
| --- | --- | --- | --- | --- | --- | --- |

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 9170 | 9.19.3 | 173 | 61 | "— If the transmission of a control response frame is not required in response to the NDP request for feedback, the feedback response frame shall be sent a SIFS after the reception of the NDP." This provides no "get out" clause. A responder has to respond within a SIFS. Unlike the case of the previous dash list item at the same depth, which is a "may" and provides two "otherwise" alternatives. | Make it consistent with "there is a control response frame" list item. Recommend change the "shall" to "may" and copy the preceding two indented list items after this item. | Accept in principle. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept in principle.

TGn Editor: on page 173, line 61, modify text as follows:

“— If the transmission of a control response frame is required in response to the NDP request for feedback,

the control response frame is transmitted a SIFS after reception of the PPDU that elicited the

control response and the feedback response frame may be transmitted a SIFS after the reception of

the NDP.

— If the feedback response frame is not transmitted a SIFS after the reception of the NDP, and the

beamformee is subsequently required to transmit an ACK or BlockAck response in the same

TXOP, then the feedback response may be aggregated with the ACK or BlockAck.

— If the feedback response frame is not transmitted a SIFS after the reception of the NDP, and is

not transmitted as part of an aggregated ACK or BlockAck response in the same TXOP, then

the feedback response frame is delayed until the beamformee's next transmission within the

TXOP.

— If the transmission of a control response frame is not required in response to the NDP request for

feedback, the feedback response frame shall may be sent a SIFS after the reception of the NDP.”

— If the feedback response frame is not transmitted a SIFS after the reception of the NDP, and the

beamformee is subsequently required to transmit an ACK or BlockAck response in the same

TXOP, then the feedback response may be aggregated with the ACK or BlockAck.

— If the feedback response frame is not transmitted a SIFS after the reception of the NDP, and is

not transmitted as part of an aggregated ACK or BlockAck response in the same TXOP, then

the feedback response frame is delayed until the beamformee's next transmission within the

TXOP.