IEEE P802.11
Wireless LANs

| LB-132 GAS Protocol Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This proposal addresses a number of comments related to the GAS protocol, which resulted from LB132.  This proposal addresses the following CIDs: 444, 454, 467, 585, 609, 899 and 901.

Comment on CID 467: this comment is resolved by allocating to the AP’s MLME the function of transmitting frames multiple times when the group addressed delivery method is used—thus, the MAC sublayer will use a new sequence number for each frame; the non-AP STA’s MLME is responsible for query response re-assembly which includes duplicate frame detection and frame re-ordering if necessary.  This is conceptually analogous to reliability improvement via application-layer re-transmission (except here the “application layer” is the MLME).

This document is based on 802.11u-d3.01.

3	Definitions

Add thefollowing text, re-numbering as necessary:

3.u.7a GAS: Generic Advertisement Service is an 802.11 protocol which provides transportation for frames of higher-layer advertisement services.  GAS supports advertisement services employing a query/response protocol.  It provides transportation of these frames from a non-AP STA to and from an AP while the non-AP STA is in states 1, 2 or 3.

7	Frame formats

7.3	Management frame body components

7.3.1	Fields that are not information elements

7.3.1.33	GAS Query Response Fragment ID

Modify the text as shown below:

A GAS Query Fragment Response ID is used by the AP when a GAS Query Response spans multiple MMPDUs. APs use this field to inform the non-AP STA of the GAS fragment number (and thus the re-assembly order) of the transmitted frames as well as identifying the last GAS fragment of the query response. The maximum value permitted in the GAS Query Response Fragment ID is 127. The More GAS Fragments bit is set to 1 in GAS Comeback Response action frames that have another GAS fragment of the current query response to follow. Othewise, it is set to 0.

|  | B0 | B6 | B7 |
| --- | --- | --- | --- |
|  | GAS Query Response Fragment ID |  | More GAS Fragments |
| Bits: | 7 |  | 1 |

Figure 7-36s—GAS Query Response Fragment ID

7.3.3	Native query protocol query element

Modify the text, Figure 7-95ay and Table 7-43w as shown below:

Native Query Protocol query elements are defined to have a common format consisting of a two-octet Info ID field, a two-octet length field encoded following the conventions given in 7.1.1., and a variable-length element-specific information field. Each element is assigned a unique Info ID as defined in this standard. See Figure 7-95ay.

The Length field specifies the number of octets in the Information field and is encoded following the conventions given in 7.1.1.

|  | Info ID | Length | Information |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 |

Figure 7-95ay—Native Query Protocol element format

Table 7-43w—Native Query Protocol info ID definitions

| Info name | Info ID | Native info element (clause) |
| --- | --- | --- |
| Reserved | 0-255 | n/a |
| Capability List Information | 256 | 7.3.3.1 |
| Venue Name Information | 257 | 7.3.3.2 |
| Emergency Call Number Information | 258 | 7.3.3.3 |
| Emergency Public Network Access Information | 259 | 7.3.3.4 |
| Network Authentication Type Information | 260 | 7.3.3.5 |
| Roaming Consortium List | 261 | 7.3.3.6 |
| Reserved | 262-56796 | n/a |
| Vendor Specific Information | 56797 | 7.3.3.7 |
| Reserved | 56798-65535 | n/a |

Add the following sub-clause, renumbering as necessary:

7.3.3.7	Native Query Protocol vendor specific informatione

The Native Query Protocol vendor specific information is used to carry information not defined in this standard within a single defined format, so that reserved info IDs are not usurped for nonstandard purposes and so that interoperability is more easily achieved in the presence of nonstandard information. The element is in the format shown in Figure 7-95bk and requires that the first 3 octets of the information field contain the OUI of the entity that has defined the content of the particular Vendor Specific information. The length of the information field (n) is 3 ≤ n ≤ the maximum MMPDU size. The OUI field shall be a public OUI assigned by the IEEE. It is 3 octets in length. The length of the vendor-specific content is n–3 octets.

|  | Info ID | Length | OUI | Vendor-specific content |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 3 | n-3 |

Figure 7-95bk—Native Query Protocol vendor specific information

7.4	Action frame format details

7.4.7	Public action details

Modify the text as shown below:

7.4.7.10	GAS Initial Response frame format

The GAS Initial Response Action frame is transmitted by an AP to a non-AP STA. The format of the GAS Initial Response Action frame body is shown in Table  Error: Reference source not found

Table 7-57ac—GAS Initial Response frame body format

| Order | Information |
| --- | --- |
| 0 | Category |
| 1 | Action |
| 2 | Dialog Token |
| 3 | Status Code |
| 4 | GAS Query ID |
| 5 | GAS Group Address |
| 6 | GAS Comeback Delay |
| 7 | Advertisement protocol information element |
| 8 | Query Response Length |
| 9 | Query Response (optional) |
| 10 - 255 | Reserved |

The Category field is set to the value indicating a Public Action frame, as specified in Table 7-24.

The Action field is set to the value specified in Table 7-57aa for a GAS Initial Response Action frame.

The Dialog Token field is copied from the corresponding GAS Initial Request Action frame.

The Status Code values are defined in Table 7-23.

The GAS Query ID is defined in 7.3.1.32. It is set by the AP and provided to the non-AP STA in this frame; corresponding GAS Comeback Request and Response Action frames use the same value of GAS Query ID. If there is an advertisement query response provided in this GAS Initial Response Action frame, then the GAS Query ID is set to 0 by the AP and ignored upon reception by the non-AP STA.

The GAS group address is the group addressed MAC address (6 octets) used for the GAS group addressed delivery method (see 11.18.2.1.1). If the GAS delivery method of group address is used, then the 6-octet group address is provided in this field; if the unicast delivery method is used, the GAS group address field is set to all zeros.

The GAS Comeback Delay field specifies the delay time value in TUs. Upon expiry of this delay, the non-AP STA should attempt to retrieve the query response using a Comeback Request Action frame. The GAS Comeback Delay field format is provided in Figure 7-101r.

- A non-zero GAS Comeback Delay value indicates that the GAS query, which will be carried out by the AP on behalf of the non-AP STA, will be delivered using the unicast delivery method.

- A zero value will be returned by the AP when the group addressed delivery method is used or when a query response is provided in this frame. A value of zero means that no GAS Comeback Delay was provided and is ignored by the non-AP STA upon reception.

|  | B0 | B15 |
| --- | --- | --- |
|  | GAS Comeback Delay |  |
| Octets: | 2 |  |

Figure 7-101r—GAS Comeback Delay field

The Advertisement protocol information element is defined in 7.3.2.60.

The Query Response length field is defined in Figure 7-101s. The value of the Query Response length field is set to the total number of octets in the Query Response field.  If the Query Response length field is set to 0, then there is no Query Response included in this action frame.

|  | B0 | B15 |
| --- | --- | --- |
|  | Query Response length |  |
| Octets: | 2 |  |

Figure 7-101s—Query Response length field

The Query Response field is defined in Figure 7-101t. The Query Response field is a generic container whose value is the response to a GAS query and is formatted in accordance with the protocol specified in the Advertisement protocol information element.

|  | Query Response |
| --- | --- |
| Octets: | variable |

Figure 7-101t—Query Response field

7.4.7.12	GAS Comeback Response frame format

The GAS Comeback Response Action frame is transmitted by an AP to a non-AP STA. This Action frame is used for both individually addressed delivery and group addressed delivery methods. The format of the GAS Comeback Response Action frame body is shown in Table 7-57ae.

Table 7-57ae—GAS Comeback Response Action frame body format

| Order | Information |
| --- | --- |
| 0 | Category |
| 1 | Action |
| 2 | Status Code |
| 3 | GAS Query ID |
| 4 | GAS Query Response Fragment ID |
| 5 | GAS Comeback Delay |
| 6 | Advertisement protocol information element |
| 7 | Query Request Length (optional) |
| 8 | Query Request (optional) |
| 9 | Query Response Length |
| 10 | Query Response (optional) |
| 11 - 255 | Reserved |

The Category field is set to the value indicating a Public Action frame, as specified in Table 7-24.

The Action field is set to the value specified in Table 7-57aa for a GAS Comeback Response Action frame.

The Status Code values are defined in Table 7-23. The same status code value will be present in all fragments of a multi-fragment query response.

The GAS Query ID is defined in 7.3.1.32. This value is identical to that used in the corresponding GAS Initial Response Action frame. The same GAS Query ID will be present in all GAS fragments of a multi-fragment query response.

The GAS Query Response Fragment ID is defined in 7.3.1.33. If the AP has not received a response to the query that it posted on behalf of a non-AP STA, then the AP sets the GAS Query Response Fragment ID to 0. When there is more than one query response fragment, the AP sets the GAS Query Response Fragment ID to 1 for the initial fragment and increments it by 1 for each subsequent fragment in a multi-fragment Query Response. The More Fragments bit is set to 0 whenever the final fragment of a query response is being transmitted. A GAS Query Response Fragment ID field having a non-zero Fragment ID and the More Fragments bit set to 1 indicates to the non-AP STA that another GAS Comeback Action frame exchange should be performed to continue the retrieval of the query response.

The GAS Comeback Delay field format is provided in Figure 7-101r. A non-zero GAS Comeback Delay value is returned by the AP in this frame to indicate that the Non-Native GAS query being carried out on behalf of the non-AP STA is still in process and will be delivered using the individually addressed delivery method.

- A non-zero value indicates to the non-AP STA that another GAS Comeback Action frame exchange should be performed after expiry of the GAS Comeback Delay timer in order to retrieve the query response.

- This field is set to zero for all GAS Comeback Response Action frames containing a query response or a fragment of a multi-fragment query response.

- This field is set to zero for GAS Comeback Response Action frames transmitted by the AP using the group addressed delivery method.

The Advertisement Protocol information element is defined in Error: Reference source not found.

The Query Request length field is defined in Figure 7-101p. The value of the Query Request length field is set to the total number of octets in the Query Request field.  If the Query Request length field is set to 0, then there is no Query Request included in this action frame. If the AP has not received a response to the query that it posted on behalf of a non-AP STA, then the Query Request length field is set to 0. It is also set to 0 for the 2nd and subsequent fragments of a multi-fragment query response.  The Query Request Length field is not included in a GAS Comeback Request Action frame transmitted to an individual MAC address.

The Query Request field is defined in Figure 7-101q. The value of the Query Request field is dependent on the Advertisement Protocol specified in the Advertisement Protocol element.

The Query Response length field is defined in Figure 7-101s. The value of the Query Response length field is set to the total number of octets in the Query Response field.  If the Query Response length field is set to 0, then there is no Query Response included in this action frame.  The Query Request field is not included in a GAS Comeback Request Action frame transmitted to an individual MAC address.

The Query Response field is defined in Figure 7-101t. The value of the Query Response field is a generic container dependent on the Advertisement Protocol specified in the Advertisement protocol information element and the query itself. In a multi-fragment query response, the response to the query posted on behalf of a non-AP STA is fragmented such that each fragment to be transmitted fits within the MMPDU size limitation.

11	MLME

11.18	WLAN Interworking with External Networks Procedures

11.18.2	Interworking Procedures: Generic Advertisement Services

Modify the text as shown below:

11.18.2.2	Non-Native GAS Protocol

A non-AP STA obtains GAS advertisement capability information from Beacon or Probe Response frames. The Advertisement Protocol information element(s) present indicate the Advertisement Protocol IDs supported in the BSS. A non-AP STA transmits a Non-Native GAS query using GAS Initial Request Action frames and the AP provides information on how to receive the query response via a GAS Initial Response Action frame. Depending on the delivery method field in the Advertisement Protocol information element, a Non-Native GAS query response will be provided using either the group addressed delivery method procedures in 11.18.2.2.3 and 11.18.2.2.4 or the unicast delivery method procedures in 11.18.2.2.5 and 11.18.2.2.6.

11.18.2.2.1	Non-AP STA Procedures to Post a Non-Native GAS Query

Upon receipt of an MLME-GAS.request primitive with Advertisement Protocol ID not set to Native Query Protocol, the non-AP STA shall engage in the following procedure to transmit a query:

The non-AP STA sends a Non-Native GAS query by transmitting a GAS Initial Request Action frame containing a Dialog Token, an Advertisement Protocol information element containing an Advertisement Protocol ID not set to Native Query protocol and the Query Request field.

- Upon transmission of the GAS Initial Request Action frame, the STA shall set a timer, referred to as the dot11GasResponseTimer, equal to the dot11GasResponseTimeout MIB object or the QueryFailureTimeout parameter provided in the MLME-GAS.request primitive. If both values are present, the timer shall be set to the lesser of the two values.

- If the non-AP STA is not in State 3, it shall remain in active mode until the receipt of a GAS Initial Response Action frame with the same Dialog Token as in the GAS Initial Request Action frame or until the expiry of the timer, whichever occurs first. If the non-AP STA is in State 3, it may go into power save state while waiting the receipt of the GAS Initial Response Action frame or the timer expiry, whichever occurs first.

- If a GAS Initial Response Action frame is received with a status value corresponding to “successful”, the Non-Native query was successfully sent and the non-AP STA shall use the procedures outlined in 11.18.2.2.4 or 11.18.2.2.6 to retrieve the query response. Upon reception of the GAS Initial Response Action frame with a status value corresponding to “successful”, the non-AP STA shall reset the dot11GasResponseTimer to the value in the dot11GasResponseTimeout MIB object.

- If a GAS Initial Response Action frame is received with a status value corresponding to “Advertisement Protocol Not Supported”, the Non-Native GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive with status so indicating and shall set the Query Response Length field to 0.

- If a GAS Initial Response Action frame is received with a status value corresponding to “Server unreachable”, the Non-Native GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive with status so indicating and shall set the Query Response Length field to 0.

- If the dot11GasResponseTimer expires before a GAS Initial Response Action frame is received, the Non-Native GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive indicating “timeout” and shall set the Query Response Length field to 0.

11.18.2.2.2	AP Procedures to respond a Non-Native GAS Query

Upon receipt of a GAS Initial Request Action frame with Advertisement Protocol ID not set to Native Query Protocol, an MLME-GAS.indication primitive shall be issued to the AP’s SME. Upon receipt of an MLME-GAS.response primitive, the AP shall transmit a GAS Initial Response Action frame to the requesting non-AP STA according to the following procedures. If the requesting non-AP STA is in State 3 and in the power-save mode, the AP shall buffer the frame for transmission according to the procedures in 11.2.1; otherwise the AP shall queue the frame for transmission.

- If the Advertisement Protocol ID in the Advertisement Protocol information element does not correspond to the value contained in any dot11GasAdvertisementID MIB object, then the AP shall not post the query to an Advertisement server in the DS. The AP shall transmit a directed GAS Initial Response Action frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request Action frame, a Status Code corresponding to “Advertisement Protocol Not Supported” (see Table 11-17b), an Advertisement Protocol information element corresponding to the Advertisement Protocol ID contained in the GAS Initial Request Action frame and a Query Response Length set to 0. The following fields shall also be set to 0: GAS Query ID, group address and GAS Comeback delay.

- If the query request corresponds to an advertisement protocol whose server in the DS or external network is currently unreachable, the AP shall transmit a directed GAS Initial Response Action frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request Action frame, a Status Code corresponding to “Server Unreachable”, an Advertisement Protocol information element containing an Advertisement Protocol ID corresponding to the Advertisement Protocol ID contained in the GAS Initial Request Action frame and a Query Response Length set to 0. The following fields shall also be set to 0: GAS Query ID, GAS Group Address and GAS Comeback delay. The method used by the AP to determine the server is unreachable is out-of-scope of this specification

- If the Advertisement Protocol ID in the Advertisement Protocol information element corresponds to the value contained in any dot11GasAdvertisementID MIB object, then the AP shall post the query to the Advertisement server in the DS. The methods and protocols the AP uses to post the query are outside the scope of this specification.

- Upon posting the query to the server in the DS the AP initializes a timer, referred to as the PostReplyTimer, to the value in dot11GasResponseTimeout MIB object.

- The AP shall then transmit an individually addressed GAS Initial Response Action frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request Action frame, a Status Code set to “success”, a GAS Query ID, an Advertisement Protocol information element corresponding to the Advertisement Protocol ID contained in the GAS Initial Request Action frame, and a Query Response Length set to 0. If the group addressed delivery method is used, then the GAS Initial Response Action frame shall contain the group address to be used for group addressed delivery and the GAS Comeback Delay value shall be set to zero; if the individually addressed delivery method is used, then all octets of the group address shall be set to 0 and the GAS Comeback Delay shall be set to the value in dot11GasComebackDelay for this Advertisement Protocol.

11.18.2.2.3	AP Procedures for group addressed delivery method

In order to support the group addressed delivery method, a GASTIM information element is included in Beacon frames. The GASTIM count field is decremented at each TBTT from the value specified in dot11GasTimPeriod to zero and then re-initialized with that value at the next TBTT. The Non-Native GAS group-addressed delivery begins after the Beacon that includes the GASTIM count value set to zero.

After the GASTIM beacon, the AP transmits the group addressed GAS Comeback Response Action frames containing encapsulated query responses. For the sake of transmission reliability, each GAS Comeback Response Action frame is transmitted by the MLME dot11GasMulticastRepetitions times; as such, the AP’s MAC sublayer provides a new sequence number for each of the repeated frames. The AP shall not schedule GAS Comeback Response Action frames for transmission more than dot11GasTimTimeToSuspend TUs after the GASTIM Beacon frame. The AP shall transmit a GAS Comeback Response Action frame to the group addressed MAC destination address specified in each GAS Initial Response Action frame having the same GAS Query ID value.

When the AP schedules one or more GAS query response for transmission after the GASTIM beacon, it shall include the corresponding Query IDs in the GASTIM element. If no GAS query responses are scheduled for transmission, then no Query IDs shall be included in the GASTIM element. A non-AP STA receiving the GASTIM Beacon can thus determine if it should remain in active mode to receive the response to its transmitted query or transition to power-save mode.

The Non-Native GAS protocol supports query responses whose length is greater than the 802.11 maximum MMPDU size. In this case, the AP shall fragment the query response and transmit it using multiple GAS Comeback Response Action frames. This is accomplished by the AP’s use of the GAS Query Response Fragment ID field in the GAS Comeback Response Action frame. The GAS Query Response Fragment ID shall be set to 1 for the initial fragment and incremented by 1 for each subsequent fragment in a multi-fragment query response. A receiving STA can determine when a frame is re-transmitted (for improved reliability) since the GAS Query Response Fragment ID field is identical for these frames. If the query response is large enough to result in a multi-fragment response, an AP’s MLME shall first transmit all the frames belonging to a single query response until it exhausts all the fragments. The AP’s MLME shall then repeat transmitting the query response fragment(s) according to the number of times indicated by the dot11GasMulticastRepetitions MIB object.

An AP shall use the following procedures to deliver a Non-Native GAS Query Response using the group addressed delivery method.

- If the PostReplyTimer expires before the query response is received from the advertisement server in the DS, then the AP shall buffer for transmission a GAS Comeback Response Action frame with a status code set to “Timeout”. If the query response is subsequently received, it shall be dropped by the AP.

- If the query response received from the server is larger than dot11GasQueryResponseLengthLimit, the AP shall discard the query response and buffer for transmission a GAS Comeback Response Action frame with a status code set to “GAS Query Response larger than permitted per configured AP policy”.

- If the query response is received from the DS before the expiry of the PostReplyTimer and its length is less than dot11GasQueryResponseLengthLimit, then the query response shall be buffered for transmission in one or more GAS Comeback Action frames with status code set to “success”.

11.18.2.2.4	Non-AP STA Procedures for group addressed delivery method

A non-AP STA shall use the following procedures to receive a Non-Native GAS Query Response using the group addressed delivery method.

- If the non-AP STA receives a GAS Comeback Response Action frame with group addressed MAC destination address and GAS Query ID matching those in the corresponding GAS Initial Response Action frame and the status code corresponds to “Timeout”, “Server unreachable” or “Query Response too large” (see Table 11-17b), it shall issue an MLME-GAS.confirm primitive so indicating and shall set the Query Response Length to 0.

- If the non-AP STA’s dot11GasResponseTimer (set in 11.18.2.2.1 step d) expires prior to receiving a GAS Comeback Request Action frame whose group addressed MAC address and GAS Query ID match those in the corresponding GAS Initial Response Action frame, the STA shall issue an MLME-GAS.confirm primitive so indicating and shall set the Query Response Length to 0.

- A non-AP STA shall reset its timer to the dot11GasResponseTimeout MIB object value after each GAS Comeback Response Action frame it receives whose group addressed MAC destination address and GAS Query ID match those from the corresponding GAS Initial Response Action frame. If this timer expires before all the GAS query response fragments have been received, the partial GAS query response shall be dropped and the MLME shall issue an MLME-GAS.confirm primitive with result code set to “timeout” and shall set the Query Response Length to 0.

- Subsequent to a non-AP STA receiving all the fragments of a query response, its MLME shall issue an MLME-GAS.confirm primitive with status set to “success” along with the query response. A non-AP STA determines it has received all the fragments of a multi-fragment query response by confirming that all fragment IDs from 1 to the value in the GAS Query Response Fragment ID when the More GAS Fragments bit was set to 0 have been received, re-ordering GAS Fragments if necessary. The non-AP STA’s MLME shall drop any duplicate GAS Fragments it has received prior to re-assembly of the query response.

11.18.2.2.5	AP Procedures for individually addressed delivery method

After receiving a query response from a server in the DS, an AP shall buffer the query response for a minimum of dot11GasResponseBufferingTime after the expiry of the GAS Comeback Delay. If the AP does not receive a GAS Comeback Request frame whose source MAC address and GAS Query ID match the destination MAC address and GAS Query ID respectively of the corresponding GAS Initial Response Action frame within this time, it may drop the query response.

If an AP receives a Query Response which is larger than the configured Query Response Length Limit, it shall discard the response and instead return a status code of GAS Query Response larger than permitted per configured AP policy in the GAS Comeback Response Action frame. This behavior helps to prevent abuses of the medium which may be caused by overly general queries (which evoke a very large query response).

The GAS protocol supports Query Responses whose length is greater than the 802.11 maximum MMPDU size by the AP’s use of the GAS Query Response Fragment ID field in the GAS Comeback Response Action frame; the Query Response Fragment ID shall be set to 1 for the initial fragment and incremented by 1 for each subsequent fragment in a multi-fragment query response. If the Query Response is a multi-fragment response (i.e., contains more than 1 fragment), the AP shall transmit all fragments that belong to the same Query Response until all fragments are exhausted. The AP shall set the More GAS Fragments bit of the GAS Query Response Fragment ID to 0 when the transmitted fragment is the final fragment.

An AP shall use the following procedures to deliver a Non-Native GAS Query Response using the individually addressed delivery method.

- If the PostReplyTimer expires before the query response is received from the advertisement server in the DS, then the AP shall buffer  for transmission a GAS Comeback Response Action frame with a status code corresponding to “Timeout” (see Table 11-17b); this frame shall be transmitted to a non-AP STA in response to the AP receiving a GAS Comeback Request Action frame whose source MAC address and GAS Query ID match the destination MAC address and GAS Query ID respectively of the corresponding GAS Initial Response Action frame. If the query response is subsequently received from the server in the DS, it shall be dropped by the AP.

- If the query response received from the server is larger than dot11GasQueryResponseLengthLimit, it shall be dropped by the AP. Then the AP shall buffer for transmission a GAS Comeback Response Action frames with status code set to “Query Response too large”; this frame shall be transmitted to a non-AP STA in response to the AP receiving a GAS Comeback Request Action frame whose source MAC address and GAS Query ID match the destination MAC address and GAS Query ID respectively of the corresponding GAS Initial Response Action frame.

- If the query response is received from the DS before the expiry of the PostReplyTimer and its length is less than dot11GasQueryResponseLengthLimit, then the query response shall be buffered in one or more GAS Comeback Action frames with a status code set to “success”. These frames shall be transmitted to a non-AP STA in response to the AP receiving a GAS Comeback Request Action frame whose source MAC address and GAS Query ID matches the destination MAC address and GAS Query ID respectively of the corresponding GAS Initial Response Action frame.

- If a query response has not been received from the DS and PostReplyTimer has not expired, the AP shall transmit a GAS Comeback Response Action frame with status corresponding to “Query response outstanding” (see Table 11-17b) and GAS Comeback Delay set to indicate when the non-AP STA should comeback to obtain its query response. This frame shall be transmitted in response to the AP receiving a GAS Comeback Request Action frame whose source MAC address and GAS Query ID matches the destination MAC address and GAS Query ID respectively of the corresponding GAS Initial Response Action frame.

- If an AP receives a GAS Comeback Request Action frame whose source MAC address and GAS Query ID do not match the destination MAC address and GAS Query ID respectively of an outstanding GAS Initial Response Action frame, the AP shall transmit a GAS Comeback Action frames with a status code corresponding to “No request outstanding”.

11.18.2.2.6	Non-AP STA Procedures for individually addressed delivery method

A non-AP STA shall transmit a GAS Comeback Request Action frame including the GAS Query ID (provided in the earlier GAS Initial Response Action frame) immediately after the expiry of the GAS Comeback Delay. The AP shall provide the query response in one or more GAS Comeback Response Action frames with the corresponding GAS Query ID, and query request.

If a non-AP STA receives a GAS Comeback Response Action frame with status set to “Query response outstanding”, the non-AP STA shall wait for the GAS Comeback Delay from that frame and upon expiry of the GAS Comeback Delay, transmit another GAS Comeback Request Action frame. If the non-AP STA’s dot11GasResponseTimer (set in 11.18.2.2.1 step b) expires prior to receiving a GAS Comeback Response Action frame whose source MAC address and GAS Query ID match those in the corresponding GAS Initial Response Action frame not having a query response, the STA shall issue an MLME-GAS.confirm primitive with result code set to “timeout” and shall set the Query Response Length to 0.

The receiving non-AP STA’s MLME shall wait to receive all the fragments unicast from the AP to reassemble the entire query response. The non-AP STA’s MLME determines that all fragments have been received by confirming that all fragment IDs from 1 to the value in the GAS Query Response Fragment ID when the More GAS Fragments bit was set to 0 have been received. Upon receipt of the first GAS Comeback Response frame and every GAS Comeback Response Action frame thereafter, the dot11GasResponseTimer shall be reset. If all of the query response fragments were received before the expiry of the dot11GasResponseTimer, then the MLME shall issue an MLME-GAS.confirm with result code set to “success” along with the query response. If all of the query response fragments were not received before the expiry of the dot11GasResponseTimer, then the MLME shall issue an MLME-GAS.confirm with result code set to “transmission failure” and shall set the Query Response Length to 0.

If a non-AP STA receives a GAS Comeback Response with status set to “Timeout” or “Query Response too large”, then the MLME shall issue an MLME-GAS.confirm with result code so indicating and shall set  the Query Response Length to 0.

If a non-AP STA receives a GAS Comeback Response with status set to “No request outstanding”, then the MLME shall issue an MLME-GAS.confirm with result code set to “unspecified failure” and shall set the Query Response Length to 0.

Modify the text as shown below:

dot11GasMulticastRepetitions OBJECT-TYPE

SYNTAX INTEGER (0..31)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object is only used for GAS multicast delivery method and

identifies the number times each GAS Comeback Response

Advertisement MMPDU is to be transmitted in order to improve

communication’s reliability."

DEFVAL (1)

::= { dot11StationConfigEntry <ANA+3> }