IEEE P802.11
Wireless LANs

| TGz Draft 2.0 LB135 CID aa-bb (Clause 7) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: August 28, 2008 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document contains proposed comments resolutions for LB135 CID aa-bb.

Instructions to the editor:

- Modify TGz draft 2.0 according to the changes shown in this document in revision marks.

7. Frame formats

7.1 MAC frame formats

7.2 Format of individual frame types

Figure 7-z1—TDLS MSDU format

DLS Action field values

—Peer Traffic Indication

Table 7-z14—Information for TDLS Channel Switch Request frame

The TDLS Channel Switch Request frame is sent over the direct path.

Table 7-z15—Information for TDLS Channel Switch Response frame

7.3 Management frame body components

7.3.1 Fields that are not information elements

### 7.3.1.7 Reason Code field

In subclause 7.3.1.7, insert the following reason codes into Table 7-22, and update the reserved values accordingly:

Table 7-22—Reason codes

| Reason code | Meaning |
| --- | --- |
| <ANA> | Path switch requested for unspecified reason |
| <ANA> | Path switch requested due to change in power save mode |
| <ANA> | Path switch requested due to change in link condition |
| <ANA> | Path switch requested for RCPI Measurement |
| <ANA> | Teardown requested due to link condition |
| <ANA> | Teardown requested for roaming |

### 7.3.1.9 Status Code field

In subclause 7.3.1.9, inser the following status codes into Table 7-23, and update the reserved values accordingly:

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| <ANA> | Path switch request denied for unspecified reason |
| <ANA> | Path switch request denied because of entering power save mode |
| <ANA> | Path switch request denied because of link state |
| <ANA> | Direct link does not or no longer exist |
| <ANA> | Wakeup schedule rejected but alternative schedule provided |
| <ANA> | Wakeup schedule rejected for unspecified reason |

7.3.1.11 Action field

Insert the following row (ignoring the header row) in Table 7-24 in the correct position to preserve ordering by the “Code” column and update a “Reserved” range of codes appropriately.

Table 7-24—Category Values

| Code | Meaning | See subclause |
| --- | --- | --- |
| <ANA> | TDLS | 7.4.12 |

7.3.2 Information elements

In table 7-26, inser the following New Information elements, and renumber the reserved values accordingly:

Table 7-26—Element IDs

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Link Identifier (see 7.3.2.66) | < ANA > | 20 |
| Wakeup Schedule (see 7.3.2.67) | <ANA> | 16 |

7.3.2.21 Measurement Request element

In 7.3.2.21, Table 7-29, insert a new measurement request named Link RCPI Request, with Measurement Type 10, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly, as shown.

Table 7-29— Measurement Type definitions for measurement requests

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Link RCPI request | 10 | Radio Measurement |
| Reserved | 1011-254 | N/A |

Insert a new subclause 7.3.2.21.11a as follows:

7.3.2.21.11a Link RCPI Request

The Measurement Request field corresponding to a Link RCPI Request is shown Figure 7-z2.

|  | BSSID | STA Address |
| --- | --- | --- |
| Octets: | 6 | 6 |

Figure 7-z2—Measurement Request field for a Link RCPI Request

BSSID contains the BSSID of the BSS to which the STA is associated.

STA Address indicates the MAC address of the STA requesting the Link RCPI measurement.

7.3.2.22 Measurement Report element

In 7.3.2.22, Table 7-30, insert a new measurement report named  Link RCPI Report, with Measurement Type 10, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly, as shown.

Table 7-30— Measurement Type definitions for measurement reports

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Link RCPI report | 10 | Radio Measurement |
| Reserved | 1011-255 | N/A |

Insert a new subclause 7.3.2.22.11a as follows:

7.3.2.22.11a Link RCPI Report

The format of the Measurement Report field corresponding to a Link RCPI Report is shown in Figure 7-z3.

|  | BSSID | STA Address | RCPI from AP | RCPI from STA |
| --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 6 |

Figure 7-z3—Measurement Report field for a Link RCPI Report

Editorial note: The field “RCPI from AP” was swapped with “STA Address”, but this does not show as a change.

BSSID contains the BSSID of the BSS to which the STA is associated.

RCPI from AP indicates the RCPI on frames received from the AP. RCPI indicates the received channel power of the corresponding Link RCPI Request frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

STA Address indicates the MAC address of the STA requesting the Link RCPI measurement.

RCPI from STA indicates the RCPI on frames received directly from the STA requesting the Link RCPI measurement. RCPI indicates the received channel power of the corresponding Link RCPI Request frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type

7.3.2.25 RSN information element

7.3.2.25.2 AKM suites

Insert the following new entry in Table 34 and update the reserved values accordingly:

Table 34—AKM suite selectors

| OUI | Suite type | Authentication type | Key management type |
| --- | --- | --- | --- |
| 00-0F-AC | <ANA> | N/A | TPK Handshake |

- 7.3.2.27 Extended Capabilities information element

Insert rows to Table 35a as follows and renumber the reserved values accordingly:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Peer U-APSD Support | The Peer U-APSD Support field indicates support for Peer U-ASPD, as defined in 11.2.1.12. The Peer U-ASPD Support field is set to 1 to indicate support for Peer U-APSD. The Peer U-APSD Support bit is set to 0 to indicate a lack of support for Peer U-APSD. |
| <ANA> | Peer PSM Support | The Peer PSM Support field indicates support for Peer PSM, as defined in 11.2.1.13. The Peer PSM Support field is set to 1 to indicate support for Peer PSM. The Peer PSM Support bit is set to 0 to indicate a lack of support for Peer PSM. |
| <ANA> | TDLS channel switching | The TDLS Channel Switching capability bit set to 1 indicates the STA supports Tunneled Direct Link Setup (TDLS) with Channel Switching as described in 11.20. The TDLS Channel Switching bit set to 0 indicates that the STA does not support this capability. |

7.3.2.48 Fast BSS transition information element

Add the following new entries to table 7-43g:

Table 7-43g—Sub-element IDs

| Value | Contents of data field | Length (in octets) |
| --- | --- | --- |
| 4 | MAC_I | 6 |
| 5 | MAC_P | 6 |
| 6 | BSSID | 6 |
| 7-255 | Reserved |  |

MAC_I contains the MAC address of the TDLS initiator.

MAC_P contains the MAC address of the Peer STA.

BSSID contains the BSSID.

Lifetime contains the lifetime of the SMKSA in seconds.

Insert the following new clauses after the last subclause of 7.3.2:

7.3.2.66 Link Identifier element

The Link Identifier element contains information which identifies the direct link. The element information format is defined in Figure 7-z4.

|  | Element ID | Length | BSSID | TDLS initiator Address | Peer STA Address |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 6 |

Figure 7-z4— Link Identifier element format

Editorial note: The regulatory class and channel number fields have been removed, but this is not marked as a change.

The Length field is set to 18.

The BSSID field is set to the BSSID of the current association of the TDLS initiator.

The TDLS initiator Address field is set to the TDLS initiator’s MAC address.

The Peer STA Address field is set to the MAC address of the Peer STA.

7.3.2.67 Wakeup Schedule information element

The Wakeup Schedule information element contains information regarding the wakeup schedule for Peer Power Save Mode. The element information format is defined in Figure 7-z5.

|  | Element ID | Length | Start Time | Interval | Minimum Duration | Idle Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 4 | 4 | 2 |

Figure 7-z5—Wakeup Schedule element format

The Length field is set to 16.

The Start Time field is set to the lower order 4 octets of the TSF timer value at the start of the first wakeup period.

The Interval field is set to the time in microseconds between two successive wakeup periods.

The Minimum Duration field is set to minimum duration of the wakeup period, in microseconds.

The Idle Count field is set to the number of consecutively empty wakeup periods before a peer STA deactivates the wakeup schedule.

7.4 Action frame format details

Insert the following new clauses in 7.4:

7.4.12 TDLS Action frame details

Several Action frame formats are defined to support tunneled direct link setup (TDLS). The Action field values associated with each frame format within the TDLS category are defined in Table 7-z1. The frame formats are defined in 7.4.12.1 to 7.4.12.12.

Table 7-z1—TDLS Action field values

| Action field value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown Request |
| 4 | TDLS Teardown Response |
| 5 | TDLS DL Path Switch Request |
| 6 | TDLS DL Path Switch Response |
| 7 | TDLS AP Path Switch Request |
| 8 | TDLS AP Path Switch Response |
| 9 | Peer Traffic Indication |
| 10 | TDLS Channel Switch Request |
| 11 | TDLS Channel Switch Response |
| 12 | TDLS Peer PSM Request |
| 13 | TDLS Peer PSM Response |
| 14 – 255 | Reserved |

7.4.12.1 TDLS Setup Request frame format

The frame body of a TDLS Setup Request frame contains the information shown in Table 7-z2.

Table 7-z2—Information for TDLS Setup Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Dialog Token | The Dialog Token contains a unique nonzero value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.1.12. |
| 4 | Association Request frame body without RSN element | The Association Request frame body is specified in 7.2.3.4. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | RSNIE_I | The RSNIE for TDLS initiator is included if security is required on the direct link. |
| 7 | FTIE | The TPK handshake Message 1 (optional). Included if security is required on the direct link. |
| 8 | Timeout Interval IE | TPK Key Lifetime (optional). Included if security is required on the direct link. |
| 9 | Supported Channels | The Supported Channels element is defined in 7.3.2.19 (optional). Included if channel switching is supported. |
| 10 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in 7.3.2.51 (optional). Included if channel switching is supported. |
| 11 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.2 TDLS Setup Response frame format

The frame body of a TDLS Setup Response frame contains the information shown in Table 7-z3.

Table 7-z3—Information for TDLS Setup Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Status Code | The Status Code is defined in 7.3.1.9. |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Request. The Dialog Token is specified in 7.3.1.12. |
| 5 | Association Request frame body without RSN element | The Association Request frame body is specified in 7.2.3.4. Only present for Status Code 0 (Successful). |
| 6 | Link Identifier | The Link Identifier is specified in 7.3.2.66. Only present for Status Code 0 (Successful). |
| 7 | RSNIE_P | The RSNIE of Peer STA is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 8 | FTIE | The TPK handshake Message 2 (optional). Included if security is required on the direct link. |
| 9 | Timeout Interval IE | TPK Key Lifetime (optional) Included if security is required on the direct link. |
| 10 | Supported Channels | The Supported Channels element is defined in 7.3.2.19 (optional). Included if channel switching is supported. |
| 11 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in 7.3.2.54 (optional). Included is channel switching is supported. |
| 12 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. Only present for Status Code 0 (Successful). |

7.4.12.3 TDLS Setup Confirm frame format

The frame body of a TDLS Setup Confirm frame contains the information shown in Table 7-z4.

Table 7-z4—Information for TDLS Setup Confirm frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Status Code | The Status Code is defined in 7.3.1.9. |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Response. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | EDCA Parameter Set | The EDCA Parameter Set element is specified in 7.3.2.29. |
| 7 | FTIE | TPK Message 3 (optional). Included if security is required on the direct link. |
| 8 | RSN IE | RSN IE (optional). Included if security is required on the direct link. |
| 9 | Timeout Interval IE | TPK Key Lifetime (optional) Included if security is required on the direct link. |
| 10 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.4 TDLS Teardown Request frame format

The frame body of a TDLS Teardown Request frame contains the information shown in Table 7-z5.

Table 7-z5—Information for TDLS Teardown Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 4 | Dialog Token | The Dialog Token contains a unique non-zero value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | FTIE | If TPK handshake was successful for this session (optional). |
| 7 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.5 TDLS Teardown Response frame format

The frame body of a TDLS Teardown Response frame contains the information shown in Table 7-z6.

Table 7-z6—Information for TDLS Teardown Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Status Code | The Status Code is defined in 7.3.1.9. |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Teardown Request. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | FTIE | If TPK handshake was successful for this session (optional). |
| 7 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.6 TDLS DL Path Switch Request frame format

The frame body of a TDLS DL Path Switch Request frame contains the information shown in Table 7-z7.

Table 7-z7—Information for TDLS DL Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 4 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.7 TDLS DL Path Switch Response frame format

The frame body of a TDLS DL Path Switch Response frame contains the information shown in Table 7-z9.

Table 7-z9—Information for TDLS DL Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Status Code | The Status Code is defined in 7.3.1.9. |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Suspend frame. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.8 TDLS AP Path Switch Request frame format

The frame body of a TDLS AP Path Switch Request frame contains the information shown in Table 7-z11.

Table 7-z11—Information for TDLS AP Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 4 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.9 TDLS AP Path Switch Response frame format

The frame body of a TDLS AP Path Switch Response frame contains the information shown in Table 7-z12.

Table 7-z12—Information for TDLS AP Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Status Code | The Status Code is defined in 7.3.1.9. |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Path Switch Request frame. The Dialog Token is specified in 7.3.1.12. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.4.12.10 Peer Traffic Indication

The frame body of a Peer Traffic Indication frame contains the information shown in Table 7-z13.

Table 7-z13—Peer Traffic Indication

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | AC_BK traffic available | 1 octet field that is set to 0 if AC_BK is empty and set to 1 if traffic is available in AC_BK. Values 2-255 are reserved. |
| 4 | AC_BE traffic available | 1 octet field that is set to 0 if AC_BE is empty and set to 1 if traffic is available in AC_BE. Values 2-255 are reserved |
| 5 | AC_VI traffic available | 1 octet field that is set to 0 if AC_VI is empty and set to 1 if traffic is available in AC_VI. Values 2-255 are reserved |
| 6 | AC_VO traffic available | 1 octet field that is set to 0 if AC_VO is empty and set to 1 if traffic is available in AC_VO. Values 2-255 are reserved |
| 7 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 8 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

The Peer Traffic Indication frame indicates the state of the power save buffer at the STA supporting Peer U-APSD that is buffering data for a Peer STA in power save mode.

Each of the ACs traffic available fields is 1 octet in length. It is set to zero value if the corresponding AC (AC0, AC1, AC2, AC3) is empty and set to 1 if traffic is available for the specific AC. Values 2-255 are reserved.

7.4.12.11 TDLS Channel Switch Request frame format

The frame body of the TDLS Channel Switch Request frame contains the information shown in Table 7-z14.

Table 7-z14—Information for TDLS Channel Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Target Channel | 1-octet field that specifies the channel number of the target channel. |
| 4 | Regulatory Class | 1-octet field that specifies the regulatory class for the target channel. |
| 5 | Secondary Channel Offset | The secondary channel offset is included when switching to a 40 MHz direct link (optional). |

The TDLS Channel Switch Request frame is sent over the direct path.

7.4.12.12 TDLS Channel Switch Response frame format

The frame body of the TDLS Channel Switch Response frame contains the information shown in Table 7-z15.

Table 7-z15—Information for TDLS Channel Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Status Code | The Status Code is defined in 7.3.1.9. |

The TDLS Channel Switch Response frame is sent over the direct path.

7.4.12.13 TDLS Peer PSM Request frame format

The TDLS Peer PSM Request frame contains the information shown in Table 7-z16.

Table 7-z16—Information for Peer PSM Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Dialog Token | The Dialog Token contains a value that is unique among TDLS Peer PSM Request frames for which a corresponding TDLS Peer PSM Response frame has not been received. The Dialog Token is specified in 7.3.1.12. |
| 4 | Wakeup Schedule | The Wakeup Schedule is specified in 7.3.2.67. |
| 5 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 6 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

### 7.4.12.14 TDLS Peer PSM Response frame format

The TDLS Peer PSM Response frame contains the information shown in Table 7-z17.

Table 7-z17—Information for Peer PSM Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Dialog Token | The Dialog Token is set to the value contained in the corresponding TDLS Peer PSM Request frame. The Dialog Token is specified in 7.3.1.12. |
| 4 | Status Code | The Status Code is specified in 7.3.1.9. |
| 5 | Wakeup Schedule | The Wakeup Schedule is only present when the status code is set to “Wakeup schedule rejected but alternative schedule provided“. The Wakeup Schedule is specified in 7.3.2.67. |
| 6 | Link Identifier | The Link Identifier is specified in 7.3.2.66. |
| 7 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

11. MLME

11.20 Tunneled Direct Link Setup

Insert a new subclause 11.20.1 and renumber existing subclauses accordingly:

11.20.1	TDLS frame body

The TDLS frame body is specified in Figure 7-z1, omitting any possible security header and trailer.

|  | LLC | SNAP | Remote Frame Type | TDLS Information |
| --- | --- | --- | --- | --- |
| Octets: | 3 | 5 | 1 | variable |

Figure 7-z1—TDLS MSDU format

LLC is defined in ISO/IEC 8802-2:1998.

SNAP is defined in IEEE Std 802-2001. The formatting of the SNAP header is according to RFC 1042. The Ethertype is set to 89-0d.

The Remote Frame Type field is set to <ANA> (TDLS).

The TDLS Information field contains a TDLS action frame body, as specified in 7.4.12.1 through 7.4.12.14.

Modify 11.20 as follows:

11.20 Tunneled Direct Link Setup

Tunneled Direct Link Setup (TDLS) is characterized by encapsulating signaling frames in Data frames, which allows them to be transmitted through an access point transparently. Therefore, the access point does not need to be direct link aware, nor does it have to support any of the capabilities that will be used on the direct link. TDLS also includes an option either to enter Peer Power Save Mode (Peer PSM) on the direct link or to suspend receiving over the direct link, so that the station can enter a power save mode.

To set up and maintain a direct link, both STA need to be associated with the same AP.

A STA may be involved in multiple direct links at once.

Features may be used on a direct link which are not supported by the BSS. An example is the use of clause 20 MCS when associated with a Clause 19 BSS. When such features are used, the rules specified for these features in an IBSS shall be followed.

The following terminology is used in this section:

- TDLS initiator: STA which transmits a TDLS Setup Request

- Peer STA: STA which is involved in a direct link or a direct link establishment

A direct link transmission is a transmission in which the A1 field of the MAC header is set to the MAC address of the Peer STA.

A non-AP STA may act as TDLS initiator or Peer STA when dot11TunneledDirectLinkSetupImplemented is true.

TDLS frames may be transmitted at UP 7 (AC_VO). TDLS frames transmitted on a direct link shall use the formatting as specified in 11.20.1.

Modify 11.20.1 as follows:

11.20.1 TDLS Link Establishment

To establish a direct link, the TDLS initiator shall send a TDLS Setup Request to the intended peer STA. If the peer STA accepts the direct link, it shall respond with a TDLS Setup Response frame with status code 0 (Successful). If the peer STA does not accept the direct link, it shall respond with a TDLS Setup Response with a status code other than 0. If there is no response within dot11DLSResponseTimeout, the TDLS initiator shall terminate the setup procedure. Otherwise, the TDLS initiator shall send a TDLS Setup Confirm to the peer STA to confirm the receipt of the TDLS Setup Response.

The EDCA Parameter Set element inside the TDLS Setup Confirm shall contain the QoS parameters which are used on the base channel when the BSS is not QoS capable, and/or on the off-channel. The ACM bits inside the EDCA Parameter Set element shall be set to 0. When the BSS supports QoS, then the QoS parameters as indicated by the AP shall be used on the base channel, and the values indicated inside the the TDLS Setup Request apply only for the off-channel.

Modify 11.20.4 as follows:

11.20.4 TDLS Channel Switching

If channel switching is supported (dot11TDLSChannelSwitchingImplemented is set to True), the STA sets the TDLS Channel Switching capability bit to 1 and includes a Supported Regulatory Classes element in TDLS Setup Request and Response frames. The channel on which the AP operates is referred to as the base channel. If the AP operates in a 40 MHz band, then the base channel refers to the primary channel.