IEEE P802.11
Wireless LANs

| Synchronization related comment resolution MIB primitive |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |

Abstract

The current draft text does not contain some of the MIB and service primitives necessary for synchronization. This document provides the missing MIB and primitives relating to synchronization.

The suggested change potentially resolves CID 695, 699, 700, and 705.

## Summary of the intention of this document

- Provides missing MIB and service primitives rlating to synchronization.

- The suggested changes are based on draft D2.01.

## Suggested update to the draft spec

- Apply the following changes.
Corresponding changes to D2.01 is indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only.

- Layer management

- MLME SAP interface

Insert the following new clause after  10.3.38:

10.3.52 Mesh Neighbor Offset Synchronization

This mechanism supports the process of establishment and maintenance of the mesh neighbor offset synchronization status of an MP with targeted neighbor MP.

10.3.52.1 MLME-MeshNeighborOffsetSync.request

10.3.52.1.1 Function

This primitive requests an establishment or a change in the synchronization status with targeted neighbor MP.

10.3.52.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshNeighborOffsetSync.request(

SyncStatus,

PeerMPAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SyncStatus | Boolean | True,false | When true, the MP is ordered to be synchronized with the targeted MP, and maintains synchronizazation based on the neighbor offset protocol. When false, the MP is not synchronized with targeted MP. |
| PeerMPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which to perform the synchronization. |

10.3.52.1.3 When generated

This primitive is generated by the SME to implement the synchronization strategy with the neighbor MP.

10.3.52.1.4 Effect of receipt

This request sets the MP’s synchronization status. The MLME subsequently issues an MLME-MeshNeighborOffsetSync.confirm that reflects the results of this request.

10.3.52.2 MLME-MeshNeighborOffsetSync.confirm

10.3.52.2.1 Function

This primitive reports the results of a mesh neighbor offset synchronization request.

10.3.52.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshNeighborOffsetSync.confirm(

ResultCode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALD_PARAMETERS NOT_SUPPORTED | Indicates the result of the mesh neighbor offset synchronization request. |

10.3.52.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-MeshNeighborOfsetSync.request to establish a new synchronization status.

10.3.52.2.4 Effect of receipt

The SME is notified of the results of the mesh neighbor offset synchronization request.

10.3.52.3 MLME-MeshNeighborOffsetMeasure.request

10.3.52.3.1 Function

This primitive requests a measurement of TSF timer offset value for a specific neighbor MP.

10.3.52.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshNeighborOffsetMeasure.request(

PeerMPAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerMPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which to measure the TSF offset value. |

10.3.52.3.3 When generated

This primitive is generated by the SME to order the measurement of TSF timer offset value with the neighbor MP.

10.3.52.3.4 Effect of receipt

This request orders the measurement of the TSF timer offset value. The MLME subsequently issues an MLME-MeshNeighborOffsetMeasure.confirm that reflects the results of this request.

10.3.52.4 MLME-MeshNeighborOffsetMeasure.confirm

10.3.52.4.1 Function

This primitive reports the results of a mesh neighbor offset measurement request.

10.3.52.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshNeighborOffsetMeasure.confirm(

ResultCode

TSFOffsetValue

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALD_PARAMETERS NOT_SUPPORTED | Indicates the result of the mesh neighbor offset synchronization request. |
| TSFOffsetValue | Integer | 8 octets | Indicates the TSF offset value with the specified neighbor MP, expressed in 2’s complement in microsecond. |

10.3.52.4.3 When generated

This primitive is generated by the MLME as a result of an MLME- MeshNeighborOffsetMeasure.request to report a measurement result.

10.3.52.4.4 Effect of receipt

The SME is notified of the results of the mesh neighbor offset measurement request.

10.3.53 Mesh Beacon Timing Advertisement

This mechanism supports the process of establishment and maintenance of the beacon timing advertisement status of an MP.

10.3.53.1 MLME-MeshBeaconTimingAdvertisement.request

10.3.53.1.1 Function

This primitive requests an establishment or a change in the beacon timing advertisement status regarding the targeted neighbor MP.

10.3.53.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshBeaconTimingAdvertisement.request(

BeaconTimingAdvertisementStatus,

PeerMPAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BeaconTimingAdvertisementStatus | Boolean | True,false | When true, the MP is ordered to advertise the beacon reception timing of the targeted STA including MP, via beacon timing element. When false, the MP does not carry beacon reception timing information in the beacon timing element. |
| PeerMPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which to advertise the beacon reception time via beacon timing element. |

10.3.53.1.3 When generated

This primitive is generated by the SME to implement the beacon timing advertisement strategy of the neighbor MP.

10.3.53.1.4 Effect of receipt

This request sets the MP’s beacon timing advertisement status. The MLME subsequently issues an MLME-MeshBeaconTimingAdvertisement.confirm that reflects the results of this request.

10.3.53.2 MLME-MeshBeaconTimingAdvertisement.confirm

10.3.53.2.1 Function

This primitive reports the results of a mesh beacon timing advertisement request.

10.3.53.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshBeaconTimingAdvertisement.confirm(

ResultCode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALD_PARAMETERS NOT_SUPPORTED | Indicates the result of the mesh beacon timing advertisement request. |

10.3.53.2.3 When generated

This primitive is generated by the MLME as a result of an MLME- MeshBeaconTimingAdvertisement.request to establish a new beacon timing advertisement status.

10.3.53.2.4 Effect of receipt

The SME is notified of the results of the mesh beacon timing advertisement request.

10.3.54 Mesh TBTT Adjustment

This mechanism supports the process of establishment and maintenance of the mesh TBTT adjustment status of an MP.

10.3.54.1 MLME-MeshTBTTAdjustment.request

10.3.54.1.1 Function

This primitive requests an establishment or a change in the mesh TBTT adjustment status, or an issue of the TBTT adjustment request frame.

10.3.54.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.request(

TBTTAdjustmentStatus,

PeerMPAddress

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| TBTTAdjustmentStatus | Boolean | True,false | When true, the MP is ordered to initiate the TBTT adjustment request frame or repond with the TBTT adjustment request frame. When false, the MP is configured not to support any TBTT adjustment functions. |
| PeerMPAddress | MACAddress | Valid individual MAC address or 0xffffff | Specifies the address of the peer MAC entity with which to issue the TBTT adjustment frame. If set to 0xffffff, the MP is configured to support TBTT adjustment function and respond with the received TBTT adjustment request frame if any. |

10.3.54.1.3 When generated

This primitive is generated by the SME to implement the mesh TBTT adjustment strategy. This primitive is also generated by the SME when the SME order to issue a TBTT adjustment frame to a specific the neighbor MP.

10.3.54.1.4 Effect of receipt

This request sets the MP’s TBTT adjustment functionality status. The MLME subsequently issues an MLME- MeshTBTTAdjustment.confirm that reflects the results of this request. When this request is generated  in order to issue the TBTT adjustment frame, the MLME issue the TBTT adjustment frame to the targeted MP immediately.

10.3.54.2 MLME-MeshTBTTAdjustment.confirm

10.3.54.2.1 Function

This primitive reports the results of a mesh TBTT adjustment  request.

10.3.54.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.confirm(

ResultCode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALD_PARAMETERS NOT_SUPPORTED | Indicates the result of the mesh TBTT adjustment request. |

10.3.54.2.3 When generated

This primitive is generated by the MLME as a result of an MLME- MeshTBTTAdjustment.request to establish a new TBTT adjustment function status, or issue the TBTT adjustment request frame.

10.3.54.2.4 Effect of receipt

The SME is notified of the results of the mesh TBTT adjustment request.

10.3.54.3 MLME-MeshTBTTAdjustment.indication

10.3.54.3.1 Function

This primitive indicates that a specific peer MAC entity is requesting to adjust TBTT with the local Mac entity.

10.3.54.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.indication(

PeerMPAddress,

BeaconTiming

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerMPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the TBTT Adjustment request was received. |
| BeaconTiming | Beacon Timing information element | As defined in frame format | Specifies the beacon reception timing of the requesting MP. |

10.3.54.3.3 When generated

This primitive is generated by the MLME as a result of the receipt of a TBTT Adjustment request frame from a specific peer MAC entity.

10.3.54.3.4 Effect of receipt

The SME is notified of the results of the receipt of the TBTT adjustment request.

- (normative) ASN.1 encoding of the MAC and PHY MIB

Insert the following at the end of Annex D:

dot11MeshDelayedBeaconTxInerval OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DEFAULT {0}

DESCRIPTION

"This attribute shall specify the interval for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in unit of beacon interval, whereas the the value 0 indicates that the delayed beacon transmission is disabled exceptionally. "

::= { dot11MeshPointConfigEntry 22}

dot11MeshDelayedBeaconDelayAmount OBJECT-TYPE

SYNTAX INTEGER (0..4023)

MAX-ACCESS read-write

STATUS current

DEFAULT {0}

DESCRIPTION

"This attribute shall specify the delay amount for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in unit of micro second. "

::= { dot11MeshPointConfigEntry 23}

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D2.01, July 2008.

[2] “Letter Ballot 126 Comment Resolutions”,  11-08/0493r18, July 2008.

[3] “Proposed comment resolution relating to M-BS”,  11-08/1077r0, September 2008.