IEEE P802.11
Wireless LANs

| Proposal for Native-GAS Query for IP Address Type availability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

This proposal is for Native GAS query and response Info elements and provides the possibility for a non-AP STA to query the AP about the availability of IP version and the type, which can be requested from the network the AP belongs to, after successful authentication. This information will be useful for the IPv4 to IPv6 transition period, when the availability of IPv4 addresses will be scarce and ISPs are expected to either provide only an an IPv6 address, or allocate NATed private IPv4 addresses, or allocate port restricted public IPv4 addresses. The knowledge of such information before association may be useful as not all clients are expected to support all types of IP addresses.

This proposal is based on draft TGu version D3.01.

- 7. Frame Formats

  - Management frame body components

- 7.3.3	Native Query Protocol information elements

Insert the following to the contents of Table 7-43x as shown below:

Table 7-43x—Native query protocol info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 1 | 7.3.3.1 |
| Venue Name | 2 | 7.3.3.3 |
| Emergency Call Number Information | 3 | 7.3.3.4 |
| Emergency Public Network Access | 4 | 7.3.3.5 |
| Network Authentication Type | 5 | 7.3.3.6 |
| Roaming Consortium List | 6 | 7.3.3.7 |
| IPaddress type availability | 10 | 7.3.3.10 |
| Reserved | 12– 220 | n/a |
| Vendor Specific | 221 | n/a |
| Reserved | 222 – 255 | n/a |

Insert the following new subclauses, renumbering clauses as necessary:

- 7.3.3.10  IP Address type availability Information Element

The IP address type Availability Information element provides non-AP STA with the information about the availability of IP address version and type which could be allocated to the non-AP STA after successful association. This element may be returned in response to a Native GAS Query Request.  The format of the IP address type availablity Information element is provided in Figure 7-95bl.

|  |  |  |  | B1 B0 | B3 B2 | B5 B4 | B7 B6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Info ID | Length | Status Code | IPv6 address | Public IPv4 address | Port restricted public IPv4 address | NATed private IPv4 address |
| octets: | 1 | 2 | 2 |  |  |  |  |
| bits |  |  |  | 2 bit | 2 bit | 2 bit | 2 bit |

Figure 7-95bl—IP address type availability Information Element

The Length field is 2-octet. The value of the length field is 4 octets.

The Status Code is a 2-octet field whose value is drawn from Table 7-23.

The IPv6 address field is a 2 bit field, whose value is described in following table:

| IPv6 address field | Value |
| --- | --- |
| Address type not available | 0 |
| Address Type available | 1 |
| Availability of the address type not known | 2 |
| Reserved | 3 |

ThePublic IPv4 Address field is a 2 bit field, whose value is described in following table:

| Public IPv4 Address | Value |
| --- | --- |
| Address type not available | 0 |
| Address Type available | 1 |
| Availability of the address type not known | 2 |
| Reserved | 3 |

The Port Restricted Public IPv4 address field is a 2 bit field, whose value is described in following table:

| Port Restricted Public IPv4 Address | Value |
| --- | --- |
| Address type not available | 0 |
| Address Type available | 1 |
| Availability of the address type not known | 2 |
| Reserved | 3 |

The NATed private IPv4 address field is a 2 bit field, whose value is described in following table:

| NATed private IPv4 address | Value |
| --- | --- |
| Address type not available | 0 |
| Address Type available | 1 |
| Single NATed private IPv4 address available | 2 |
| Double NATed private IPv4 address available | 3 |