IEEE P802.11
Wireless LANs

| LB-132 MIB Comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Roke Manor Research Ltd | Old Salisbury Lane, Romsey, Hampshire, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |

Abstract

This proposal addresses a number of comments related to the MIB, which resulted from LB132.  This proposal addresses the following CIDs: 686, 687, 829, 833, 834,

This document is based on IEEE 802.11u-D3.01.

Update to the MIB table on page 105:

--**********************************************************************

-- * dot11Bssid TABLE

--**********************************************************************

dot11BssidTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BssidEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“This object is a table of BSSIDs contained within an Access Point (AP).”

::= { dot11imt 1 }

dot11BssidEntry OBJECT-TYPE

SYNTAX Dot11BssidEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“This object provides the attributes identifying a particular BSSID within an AP.”

INDEX { dot11BssidMacAddress }

::= { dot11NonApStaTable 1 }

Dot11BssidEntry ::=

SEQUENCE {

dot11BssidMacAddress

}

dot11BssidMacAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read only

STATUS current

DESCRIPTION

“This object specifies the MAC address of the BSSID represented on a particular radio interface and uniquely identifies this entry.”

::= { dot11BssidEntry 1 }

--**********************************************************************

-- * End of dot11Bssid TABLE

--**********************************************************************

Update to the MIB on page 135

-- **********************************************************************

-- * dot11GasAdvertisement TABLE

-- **********************************************************************

Insert the following entries as indicated:

dot11GasAdvertisementTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11GasAdvertisementEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“This object is a table of GAS counters implemented as a table to allow for multiple instantiations on an STA.”

::= { dot11gas 1 }

dot11GasAdvertisementEntry OBJECT-TYPE

SYNTAX Dot11GasAdvertisementEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“This object provides the attributes identifying a GAS counter within an STA.”

INDEX { dot11GasAdvertisementIndex }

::= { dot11GasAdvertisementTable 1 }

Dot11GasAdvertisementEntry ::=

SEQUENCE {

dot11GasAdvertisementIndex 		INTEGER,

dot11GasAdvertisementId			INTEGER,

dot11GasQueries				Counter32,

dot11GasQueryRate				INTEGER,

dot11GasResponses				Counter32,

dot11GasResponseRate			INTEGER,

dot11GasResponseTimeout			INTEGER,

dot11GasTransmittedFragmentCount	Counter32,

dot11GasFailedCount			Counter32,

dot11GasRetryCount				Counter32,

dot11GasMultipleRetryCount		Counter32,

dot11GasFrameDuplicateCount		Counter32,

dot11GasACKFailureCount			Counter32,

dot11GasReceivedFragmentCount		Counter32,

dot11GasTransmittedMsduCount		Counter32,

dot11GasDiscardedMsduCount		Counter32,

dot11GasRetriesReceivedCount		Counter32,

dot11GasDeliveryMethod			INTEGER,

dot11GasComebackDelay			INTEGER,

dot11GasQueryResponseLengthLimit	INTEGER

}

dot11GasAdvertisementIndex OBJECT-TYPE

SYNTAX INTEGER (1..20)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the

columnar objects in the GasAdvertisement Table.”

::= { dot11GasAdvertisementEntry 1 }

dot11GasAdvertisementId OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“The one-octet identification number for the GAS Advertisement protocol, as defined in Table 7-43t, for which statistics are stored the logical row of the GasAdvertisement table.”

::= { dot11GasAdvertisementEntry 2 }

dot11GasQueries OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of GAS queries sent or received for the protocol identified by dot11GasAdvertisementId.”

::= { dot11GasAdvertisementEntry 3 }

dot11GasQueryRate OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of GAS queries per minute received for the protocol identified by dot11GasAdvertisementId, averaged over the previous ten minutes.”

::= { dot11GasAdvertisementEntry 4 }

dot11GasResponses OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of GAS responses sent or received for the protocol identified by dot11GasAdvertisementId.”

::= { dot11GasAdvertisementEntry 5 }

dot11GasResponseRate OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of responses to GAS queries per minute received for the protocol identified by dot11GasAdvertisementId, averaged over the previous ten minutes.”

::= { dot11GasAdvertisementEntry 6 }

dot11GasTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented for an acknowledged MPDU, for a particular UP, with an individual address in the address 1 field or an MPDU with a group address in the address 1 field, either belonging to a particular TID. This counter has relevance only for TIDs between 0 and 7."

::= { dot11GasAdvertisementEntry 7 }

dot11GasFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment when an MSDU, for a particular UP, is not transmitted successfully due to the number of transmit attempts exceeding either the dot11ShortRetryLimit or dot11LongRetryLimit. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 8}

dot11GasRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment when an MSDU, of a particular UP, is successfully transmitted after more than one retransmissions. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 9}

dot11GasMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment when an MSDU, of a particular UP, is successfully transmitted after one or more retransmissions. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 10}

dot11GasFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment when a frame, of a particular UP, is received that the Sequence Control field indicates is a duplicate. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 11}

dot11GasACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment when an ACK is not received in response to an MPDU of a particular UP. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 12}

dot11GasReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented for each successfully Received MPDU of type Data of a particular UP. This counter has Relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 13 }

dot11GasTransmittedMsduCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment for each successfully transmitted MSDU of a particular UP. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 14 }

dot11GasDiscardedMsduCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment for each Discarded MSDU of a particular UP. This counter has relevance only for TIDs between 0 and 7.”

::= { dot11GasAdvertisementEntry 15 }

dot11GasRetriesReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment for each received MPDU of a particular TID with the retry bit set to 1.”

::= { dot11GasAdvertisementEntry 16 }

dot11GasResponseTimeout OBJECT-TYPE

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This parameter shall indicate the GAS response timeout value in ms.”

::= { dot11GasAdvertisementEntry 17 }

dot11GasDeliveryMethod OBJECT-TYPE

SYNTAX INTEGER {

Unicast(1),

Multicast(2)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object identifies the delivery method of the AP for each GAS

Advertisement Protocol in this row of the table.”

::= { dot11GasAdvertisementEntry 18 }

dot11GasComebackDelay OBJECT-TYPE

SYNTAX INTEGER {0..65535}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object identifies the GAS Comeback Delay (in TUs) to be used for this Advertisement Protocol”

DEFVAL (1000)

::= { dot11GasAdvertisementEntry 19 }

dot11GasQueryResponseLengthLimit OBJECT-TYPE

SYNTAX INTEGER {1..127}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object indicates the maximum number of octets an AP will transmit in one or more Query Response fields contained within GAS Comeback Response Action frame(s). A value of 127 means the maximum limit enforced is contained by the maximum allowable number of fragments in the GAS Query Fragment Response ID”

::= { dot11GasAdvertisementEntry 20 }

-- **********************************************************************

-- * End of dot11GasAdvertisement TABLE

-- **********************************************************************