IEEE P802.11
Wireless LANs

| Proposal for Addition of Personal Network to Network Types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |

Abstract

This proposal adds Personal Network to TGu defined network type list. It addresses LB 132 CID # 87 and 90.

This document is based on IEEE 802.11-2007 and 802.11u-d3.01.

- Introduction

TGu provides “Network Type” advertisement, which identifies the broad category of the WLAN network. It helps non-AP STAs with network discovery and selection. This proposal introduces “Personal Network” type into the 802.11 network type list.

7.3.2.59 Interworking information element

Change Table 7-43r as shown below:

| Network Type Codes | Meaning | Description |
| --- | --- | --- |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. These networks may or may not employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts area available. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http/https redirect or DNS redirection). |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. |
| 4 | Emergency network | The network supports Emergency Services. Further information is available from a native GAS query (see 7.3.3) |
| 5 | Peer to Peer Network | A small network of personal devices such as camera and computer, where one device assumes the role of AP. |
| 5 6 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |