IEEE P802.11
Wireless LANs

| Proposal for Advertisement Protocol ID to carry LoST protocol messages |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

P802.11u-d3.0 provides methods to facilitate the access of non-AP STAs to information stored in external databases, while in state-1, mainly for the purpose of network selection.

This document proposes to allow non-AP STAs in state-1 to query PSAP databases to download local emergency dialstrings and to translate location to PSAP URIs, necessary to place emergency calls.

This proposal is based on TGu draft D3.01.

2. Normative references

Insert the following new references into clause 2 maintaining the ordering:

IANA, EAP Method Type Numbers, http://www.iana.org/assignments/eap-numbers.

IANA, PPP Protocol Numbers, http://www.iana.org/assignments/ppp-numbers.

IEEE P802.21/D10.00, “Draft IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services”, April 2008.

IETF RFC 2903 “Generic AAA architecture”, C. de Laat, G. Gross, L. Gommans, J. Vollbrechtm and D. Spence, August 2000 (status informational).

IETF RFC 3986, Uniform Resource Identifier (URI): Generic Syntax, January 2005.

ISO 3166-1, http://www.iso.org/iso/iso_catalogue/catalogue_tc/catalogue_detail.htm?csnumber=39719.

OASIS Emergency Management Technical Committee, “Common Alerting Protocol Version 1.1” http://xml.coverpages.org/CAPv11-12649.pdf, April 2005

OASIS Emergency Management Technical Committee, “Emergency Data Exchange Language (EDXL) Distribution Element, v. 1.0”. OASIS Standard EDXL-DE v1.0, http://docs.oasis-open.org/emergency/edxl-de/v1.0, May-2006.

RFC5222 “LoST: A Location-to-Service Translation Protocol”, August 2008

- 7. Frame Formats

| Table 7-43u—Advertisement Protocol ID definitionsName | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Native Query Protocol | 0 |  |  |  |  |  |
| 0 MIH Information Service | 1 |  |  |  |  |  |
| 1 MIH Command and Event Services Capability Discovery | 2 |  |  |  |  |  |
| 2 Emergency Alert System (EAS) Service | 3 |  |  |  |  |  |
| 3 3GPP Network broadcast information | 4 |  |  |  |  |  |
| 4 Location-to-Service Translation Protocol | 5 |  |  |  |  |  |
| 4 Reserved | 6-220 |  |  |  |  |  |
| 5-220 Vendor Specific | 221 |  |  |  |  |  |
| 221 Reserved | 222-255 |  |  |  |  |  |