IEEE P802.11
Wireless LANs

| TGz LB 135 Peer U-APSD proposed text changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alexander Safonov | IITP RAS | Bolshoy Karetny per 19, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Dongan-gu, Anyang-shi, Kyungki-do, 431-749, Korea | +82-2-526-4225 | yhseok@lge.com |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document contains LB 135 comment resolutions related to section 11.2.1.12 Peer U-APSD.

3. Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form “3.z<x>” where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline document.

Insert the following new definitions into Clause 3:

3.z3 Peer U-APSD mode: A power save mode which may be used between two STAs which have setup a Tunneled Direct Link Setup (TDLS) link.

11. MLME

Replace 11.2.1.12 with the following text:

11.2.1.12 Peer U-APSD

A STA supports Peer U-APSD if dot11TDLSPeerUAPSDImplemented is true. Support of this capability shall be signaled through the Peer U-APSD subfield in the Extended Capabilities element included in the TDLS Setup Request or TDLS Setup Response frame. Support for Peer U-APSD means that the STA has capability to buffer frames for the peer STA which operates in Peer U-APSD and deliver them during unscheduled service periods.

To configure Peer U-APSD at a STA, all four U-APSD Flag bits inside the QoS Info subfield of the QoS Capability element carried in a TDLS Setup Response or the QoS Info subfield of the EDCA Parameter Set element carried in a TDLS Setup Confirm frame shall be set to 1.

A STA which configured Peer U-APSD at a TDLS peer STA may enter power save mode by setting the Power Management field in the Frame Control field of an acknowledged MPDU transmitted to the TDLS peer STA over the direct link. The STA which transmitted the frame with the Power Management bit set is then referred to as a PU sleep STA. The STA receiving the frame with the Power Management bit set is referred to as a PU buffer STA. A PU sleep STA may be a PU buffer STA at the same time.

11.2.1.12.1  Peer U-APSD Behavior at the PU buffer STA

MSDUs received at a PU buffer STA destined for a PU sleep STA shall be temporarily buffered at the PU buffer STA. The algorithm to manage this buffering is beyond the scope of this standard, with the condition that the PU buffer STA shall preserve the order of arrival of frames on a per-TID, per-STA basis.

A PU buffer STA shall transmit a unicast Peer Traffic Indication frame to a PU sleep STA if and only if the following conditions are met:

- a new frame arrives at the PU buffer STA, the frame being destined for a PU sleep STA,

- the frame buffer at the PU buffer STA contains no other frames with the same AC which are destined for the PU sleep STA,

- and no SP has occurred for that AC and for that PU sleep STA, for a period of dot11TDLSPeerUAPSDIndicationPeriod prior to the arrival of the new traffic

The transmitted Peer Traffic Indication frame shall indicate the non-empty AC(s), by setting the corresponding AC Traffic Available subfield to 1. The Peer Traffic Indication frame shall be transmitted through the AP path.

11.2.1.12.2  Peer U-APSD Behavior at the PU sleep STA

When a PU sleep STA receives a Peer Traffic Indication frame indicating that frames are buffered at the PU buffer STA for one or more ACs, the PU sleep STA shall initiate an unscheduled SP with the PU buffer STA to retrieve the buffered MSDU(s) for the AC(s) for which no unscheduled SP is currently active.

The procedures to trigger and terminate an unscheduled SP between a PU buffer STA and a PU sleep STA are described in 11.2.1.4 and 11.2.1.5, where the PU buffer STA shall take the role of the AP and the PU sleep STA shall take the role of the non-AP STA using U-APSD.

Annex D (normative) ASN.1 encoding of the MAC and PHY MIB

Replace dot11TDLSPeerUAPSDImplemented element with the following:

dot11TDLSPeerUAPSDImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Peer U-APSD.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry 95 }