IEEE P802.11
Wireless LANs

| LB132: Putting AP Civic Location MIB in CAType Order |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-9 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This document re-orders the objects in the dot11ApCivicLocation so they are in CAType order (cf. RFC-4776).  The changes in this submission are only to Annex D.  This proposal resolves the following LB132 CIDs: 844.

This document is based on IEEE 802.11u-d3.01.

Change the following text as shown below:

# Annex D

-- **********************************************************************

-- * dot11ApCivicLocation

-- **********************************************************************

dot11ApCivicLocation       OBJECT-TYPE

SYNTAX

MAX-ACCESS     not-accessible

STATUS         current

DESCRIPTION

"This table represents location of the AP in civic format

per RFC-5139 and RFC-4776"

::= { dot11imt 4 }

dot11ApCivicLocation OBJECT-TYPE

SYNTAX Dot11ApCivicLocation

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"AP location in Civic coordinates."

Dot11ApCivicLocation ::=

SEQUENCE {

dot11ApCivicLocationCountry	OCTET STRING,

dot11ApCivicLocationA1		OCTET STRING,

dot11ApCivicLocationA2		OCTET STRING,

dot11ApCivicLocationA3		OCTET STRING,

dot11ApCivicLocationA4		OCTET STRING,

dot11ApCivicLocationA5		OCTET STRING,

dot11ApCivicLocationA6		OCTET STRING,

dot11ApCivicLocationPrd		OCTET STRING,

dot11ApCivicLocationPod		OCTET STRING,

dot11ApCivicLocationSts		OCTET STRING,

dot11ApCivicLocationHno		OCTET STRING,

dot11ApCivicLocationHns		OCTET STRING,

dot11ApCivicLocationLmk		OCTET STRING,

dot11ApCivicLocationLoc		OCTET STRING,

dot11ApCivicLocationNam		OCTET STRING,

dot11ApCivicLocationPc		OCTET STRING,

dot11ApCivicLocationBld		OCTET STRING,

dot11ApCivicLocationUnit	OCTET STRING,

dot11ApCivicLocationFlr		OCTET STRING,

dot11ApCivicLocationRoom	OCTET STRING,

dot11ApCivicLocationPlc		OCTET STRING,

dot11ApCivicLocationPcn		OCTET STRING,

dot11ApCivicLocationPobox	OCTET STRING,

dot11ApCivicLocationAddcode	OCTET STRING,

dot11ApCivicLocationSeat	OCTET STRING,

dot11ApCivicLocationRd		OCTET STRING,

dot11ApCivicLocationRdsec	OCTET STRING,

dot11ApCivicLocationRdbr	OCTET STRING,

dot11ApCivicLocationRdsubbr	OCTET STRING,

dot11ApCivicLocationPrm		OCTET STRING,

dot11ApCivicLocationPom		OCTET STRING

}

dot11ApCivicLocationCountry OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..1))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the two uppercase characters which

Which correspond to the alpha-2 codes in ISO 3166-1

.  Example: US.”

::= { dot11ApCivicLocation 1 }

dot11ApCivicLocationA1 OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..2))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the national subdivisions (state,

Region, province, prefecture).  Example: New York.

The A1

element is used for the top level subdivision within a

country.  In the absence of a country-specific guide on how to

use the A-series of elements, the second part of the ISO 3166-

2 code [ISO.3166-2] for a country subdivision SHOULD be used.

The ISO 3166-2 code is a formed of a country code and hyphen

plus a code of one, two or three characters or numerals.  For

the A1 element, the leading country code and hyphen are

omitted and only the subdivision code is included.

For example, the codes for Canada include CA-BC, CA-ON, CA-QC;

Luxembourg has just three single character codes: LU-D, LU-G

And LU-L; Australia uses both two and three character codes:

AU-ACT, AU-NSW, AU-NT; France uses numerical codes for

mainland France and letters for territories: FR-75, FR-NC.”

::= { dot11ApCivicLocation 2 }

dot11ApCivicLocationA2 OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the county, parish, gun (JP),

District (IN).  Example: King’s County."

::= { dot11ApCivicLocation 3 }

dot11ApCivicLocationA3 OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the city, township, shi (JP).

Example: New York."

::= { dot11ApCivicLocation 4 }

dot11ApCivicLocationA4 OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the city division, borough, city

District, ward, chou (JP).  Example: Manhattan."

::= { dot11ApCivicLocation 5 }

dot11ApCivicLocationA5 OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the neighborhood, block.  Example:

Morningside Heights."

::= { dot11ApCivicLocation 6 }

dot11ApCivicLocationA6 OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the street.  Example: Broadway.

The A6

element is retained for use in those countries that require

this level of detail.  Where A6 was previously used for street

names in [RFC4119], it MUST NOT be used, the RD element MUST

be used for thoroughfare data.  However, without additional

information these fields MUST not be interchanged when

converting between different civic formats.  Where civic

address information is obtained from another format, such as

the DHCP form [RFC4776], the A6 element MUST be copied

directly from the source format."

::= { dot11ApCivicLocation 7 }

dot11ApCivicLocationPrd OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the leading street direction.

Example: N, W."

::= { dot11ApCivicLocation 8 }

dot11ApCivicLocationPod OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the trailing street suffix.

Example: SW."

::= { dot11ApCivicLocation 9 }

dot11ApCivicLocationSts OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the street suffix.  Example: Avenue,

"Platz, Street.”

::= { dot11ApCivicLocation 10 }

dot11ApCivicLocationHno OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..7))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the numeric part only of the

House number.  Example: 123."

::= { dot11ApCivicLocation 11 }

dot11ApCivicLocationHns OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..7))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the house number suffix.  Example:

A, ½."

::= { dot11ApCivicLocation 12 }

dot11ApCivicLocationLmk OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the landmark or vanity address.

Example: Low Library."

::= { dot11ApCivicLocation 13 }

dot11ApCivicLocationLoc OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains additional location information.

Example: Room 543."

::= { dot11ApCivicLocation 14 }

dot11ApCivicLocationNam OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Name (residence, business,

or office occupant. Example: Joe’s Barbershop."

::= { dot11ApCivicLocation 15 }

dot11ApCivicLocationPc OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the postal code.  Example:

10027-0401."

::= { dot11ApCivicLocation 16 }

dot11ApCivicLocationBld OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the building (structure).

Example: Hope Theater."

::= { dot11ApCivicLocation 17 }

dot11ApCivicLocationUnit OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..7))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the unit (apartment, suite).

Example: 12a."

::= { dot11ApCivicLocation 18 }

dot11ApCivicLocationFlr OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the floor number.  Example: 5."

::= { dot11ApCivicLocation 19 }

dot11ApCivicLocationRoom OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..7))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the room.  Example: 450F."

::= { dot11ApCivicLocation 20 }

dot11ApCivicLocationPlc OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the place type.  Example: office."

::= { dot11ApCivicLocation 21 }

dot11ApCivicLocationPcn OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the postal community name.

Example: Leonia."

::= { dot11ApCivicLocation 22 }

dot11ApCivicLocationPobox OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the post office box (P.O. Box).

Example: U40."

::= { dot11ApCivicLocation 23 }

dot11ApCivicLocationAddcode OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the additional code.

Example: 13203000003."

::= { dot11ApCivicLocation 24 }

dot11ApCivicLocationSeat OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the seat (desk, cubicle,

Workstation).  Example: WS 181”.

::= { dot11ApCivicLocation 25 }

dot11ApCivicLocationRd OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the primary road or street.

Example:Broadway."

::= { dot11ApCivicLocation 26 }

dot11ApCivicLocationRdsec OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the road section.  Example: 14.

In some

countries a thoroughfare can be broken up into

sections, and it is not uncommon for street numbers to be

repeated between sections.  A road section identifier is

required to ensure that an address is unique.  For example,

West Alice Parade has 5 sections, each numbered from 1; unless

the section is specified 7 West Alice Parade could exist in 5

different places.

::= { dot11ApCivicLocation 27 }

dot11ApCivicLocationRdbr OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the road branch.  Example: Lane 7."

Minor streets can share the same name, so that they can only

Be distinguished by the major thoroughfare with which they

intersect.  For example, both West Alice Parade, Section 3

and Bob Street could both be interested by a Carol Lane.  This

element is used to specify a road branch where the name of the

branch does not uniquely identify the road.  Road branches MAY

also be used where a major thoroughfare is split into

sections.”

::= { dot11ApCivicLocation 28 }

dot11ApCivicLocationRdsubbr OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the road sub-branch.  Example:

Alley 8."

::= { dot11ApCivicLocation 29 }

dot11ApCivicLocationPrm OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the road pre-modifier.  Example:

Old."

::= { dot11ApCivicLocation 30 }

dot11ApCivicLocationPom OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..15))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the road post-modifier.  Example:

Extended."

::= { dot11ApCivicLocation 31 }

-- **********************************************************************

-- * End of dot11ApCivicLocation

-- **********************************************************************

# Annex P

Insert the following references to the end of the bibliography list:

[+1] IETF RFC-4776, “Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information”, H. Schulzrinne, November 2006

[+1] IETF RFC-5319, “Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)”, M. Thomson, February 2008.