IEEE P802.11
Wireless LANs

| Proposal for Service Advertisement, Network Discovery & Selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

P802.11u-d3.01 provides non-native GAS mechanism for non-AP STAs to find out the network capabilities and services without connecting and reconnecting with the network. However, that method requires deployment of large information services which is not be feasible for light weight small networks and usage models. This proposal provides methods to enable the network services advertisement which will help with network discovery and selection.

This document is based on IEEE 802.11-2007 and 802.11u-d3.01.

- Introduction

IEEE 802.11u-d3.01 currently supports IEEE 802.21-IS which provides mechanisms for network services discovery. Using 802.11u Non-Native GAS mechanism, devices can query the 802.21 IS for the network services.

However, 802.21-IS is deployable for the large scale networks. It is not feasible for small networks, i.e., one AP with a few connected devices. What is lacking is the availability of similar behaviour in small networks where 802.-21-IS is not available.

This proposal introduces such methods that will help mobile devices to find out network services without going through the trial and error method of connecting and disconnecting with the network. It will greatly benefit the devices with the fast discovery of network services at smaller scale networks, resulting better network selection and improved battery life for mobile devices.

Note that this is not intended as a replacement to 802.21-IS.  Clearly this is not possible given the richness of the 802.21-IS query language and the small amount of information conveyed by the proposed solution. It will be practical in small networks.

    - Management frames

      - Association Request

Insert  the following to the contents of Table 7-10 as shown below:

Table 7-10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Service Description | The Service Description element is present if dot11InterworkingServiceEnabled is true. |

      - Reason Code field

Insert following item into Table 7-22 before “Reserved”, renumber when necessary:

Table 7-22—Reason codes

| Reason Code | Meaning |
| --- | --- |
| 52 | Connection is rejected because the device is disallowed to join the network. |
| 53 – 65535 | Reserved |

- 7.3.2	Information elements

Insert the following to the contents of Table 7-26 as shown below:

Table 7-26—Element IDs

| Information Element | Element ID | Length |
| --- | --- | --- |
| Device Service Description | <ANA> | variable |

Insert the following new subclause, renumbering clauses as necessary:

      - Device Service Description element

The Device Service Description information element contains information about the services provided by the device joining the network.  The Service Description element format is shown in Figure 7-95ay.

|  |  |  | B0 – B3 | B4 – B7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Number of Services Supported | Reserved | Device Service #1 | . . . | Device Service # n (optional) |
| Bits: |  |  | 4 | 5 |  |  |  |
| Octets: | 1 | 2 |  |  | variable |  | variable |

Figure 7-95ay—Device Service Description element format

The Length field is the length of the Device Service element. The value of the Length field is variable and depends on the number of Device Service fields and lengths present.

The Number of Services Supported field is a 4-bit field whose value is the number services provided by the device. A device may provide maximum of 8 de.

The Device Service field format is shown in Figure 7-95az.

|  | Length | Service Type Code | Supported Protocols |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure 7-95az— Device Service field format

The Length field is the length of Device Service field. Its value is variable and depends on the size of the other fields.

The Device Service field contains the Service Type code shown Table 7-43aa.

Table 7-43aa—Device Service Type codes

| Service Names | Service Type Codes |
| --- | --- |
| Unspecified | 0 |
| Generic Printing | 1 |
| Color Printing | 2 |
| Black & White Printing | 3 |
| Digital TV Display | 4 |
| Digital Picture Display | 5 |
| Projector | 6 |
| Digital Camera | 7 |
| Scanner | 8 |
| Copier | 9 |
| Fax | 10 |
| Phone | 8 |
| TV | 9 |
| Digital Player | 10 |
| CD Player | 11 |
| Video Player | 12 |
| Media Streaming | 13 |
| Game Console | 14 |
| Computer | 15 |
| Computer Monitor | 16 |
| Mouse | 17 |
| Keyboard | 18 |
| Data Storage | 19 |
| Security Camera | 20 |
| Temperature Controller | 21 |
| Reserved | 16 – 127 |
| Vendor Specific | 128 |
| Reserved | 129 – 255 |

The Device Service Name is the functionality supported by the device.

The Device Service Type code is the value which is assigned to corresponding device functionality.

The Supported Protocols field format is shown in Figure 7-95ba.

|  |  | B0 - B3 | B4 - B7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Length | Number of Protocols Supported | Reserved | Supported Protocol Type #1 | . . . | Supported Protocol Type #16 |
| Bits: |  | 3 | 4 | 4 |  | 4 |
| Octets: | 1 |  |  |  |  |  |

Figure 7-95bb — Supported Protocols field format

The Number of Supported Protocols field is 4-bit field. The device may support up to 16 protocols per service.

The Supported Protocol Type field contains the upper layer protocol type supported by the device. The Supported Protocol Type codes are shown in Table 7-43ab.

Table 7-43ab—Supported Protocol Type codes

| Supported Protocol Name | Supported Protocol Type codes |
| --- | --- |
| Plug and Play (PNP) | 0 |
| Bonjour | 1 |
| Reserved | 2-9 |
| Vendor Specific | 10 |
| Reserved | 11-15 |

- 7.3.3	Native Query Protocol information elements

Change the contents of Table 7-43x as shown below:

Table 7-43x—Native query protocol info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 1 | 7.3.3.1 |
| multiple SSID Set | 2 | 7.3.3.2 |
| Venue Name | 3 | 7.3.3.3 |
| Emergency Call Number Information | 4 | 7.3.3.4 |
| Emergency Public Network Access | 5 | 7.3.3.5 |
| Network Authentication Type | 6 | 7.3.3.6 |
| Roaming Consortium List | 7 | 7.3.3.7 |
| Device Services List | 8 | 7.3.3.8 |
| Reserved | 8 – 220 | n/a |
| Vendor Specific | 221 | n/a |
| Reserved | 222 – 255 | n/a |

Insert the following new subclause, renumbering clauses as necessary:

      - Device Services List

The Device Services List provides a list of services provided by the devices connected to the network. This list may be returned in response to a Native GAS Query Request.  The format of the Device Service List element is provided in Figure 7-95bb

|  | Info ID | Length | Status Code | Supported Service Count | Device Service #1 (optional) | Device Service #2 (optional) | . . . | Device Description #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 1 | variable | variable |  | variable |

Figure 7-95bb—Device Description list format

The Length field is 2-octet length of the Device Services List element and is equal to 2 plus the lengths of Device Services present in the element.

The Status Code is a 2-octet field whose value is drawn from Table 23.

The Device Service field format is shown in Figure 7-95az. It is variable length.