IEEE P802.11
Wireless LANs

| TGp LB 125 Comment Resolution for Clauses 1, 2, and 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Lee Armstrong | Armstrong Consulting, Inc. | 132 Fomer Road, Southampton, MA 01073 | 617-620-1701 | LRA@tiac.net |

Abstract

Proposed resolution to misc. CIDs  in Clauses 1, 2, and 3 from LB125.

|  | LB125 Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]

| 10 | Roy, Richard | 3.168a | 2 | 26 | TR | The concept of a WBSS is unnecessary. The additional functionality required to make STAs WAVE capable neither depends on nor does it require any concept of associating in any way with other STAs. As stated, this amendment specifies functionality that allows STAs to communicate outside the context of any BSS, and the introduction of the term/concept WBSS only confuses the matter, not to mention the implementer. | Remove the definition of WBSS. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | Roy, Richard | 3 | 2 | 19 | TR | The proposed modification to the definition of a BSS is technically problematic. To successfully join a BSS, a STA must receive more than one valid beacon frame, and the ONDEMANDBEACON frame is not transmitted periodically, rather only when commanded by the SME. Use of an ONDEMANDBEACON to create a BSS would require transmission of several such frames with the appropriate timing for other STAs to successfully join. | Remove the suggested changes to clause 3.16 leaving the definition of BSS unaltered. |
| 12 | Myles, Andrew | 3.168a | 2 | 27 | TR | The text states, "A WAVE BSS is initialized when a STA in WAVE mode sends a WAVE beacon." This seems to imply that a new BSS is initialized very time a WAVE Beacon is sent. I suspect this is not the intent | Clarify the intent and make appropriate changes |
| 13 | Engwer, Darwin | 3.168b | 2 | 30 | TR | "An information element that contains information provided by the MAC through the MLME_SAP." - The cited definition for WIE says nothing that distinguishes it from other IEs. Most IEs contain data obtained via the MLME_SAP. A better definition would state the purpose or use of the WIE. What is it used for? Why is it important (in the context of the 802.11p amendment and 802.11 in general)? | Change "An information element that contains information provided by the MAC through the MLME_SAP" to "An information element included in a WAVE beacon to ______________________". Or "A WAVE beacon information element used to ____________." Fill in the blank with the purpose of the WIE. Perhaps ____ = "advertise additional WAVE capabilities provided by the SME"? |
| 14 | Hart, Brian | 3.168b | 2 | 30 | T | Definition could apply to any element filled in my the MLME SAP | Insert WAVE or equivalent in the defn |
| 15 | Myles, Andrew | 3.168b | 2 | 30 | ER | A WAVE IE is defined. However, the definition of an IE belongs in clause 7 | Delete |
| 16 | Malinen, Jouni | 3.168b | 2 | 31 | TR | WIE is claimed to be provided by the MAC. However, MLME-ONDEMANDBEACON.request() is from SME to MLME (MAC), not from MAC.. Was this “by the MAC” supposed to be “to the MAC” or “by the SME”? | Replace “by the MAC” with “to the MAC”. |
| 18 | Bai, Fan | 3.168a | 2 | 33 | T | The case where only zero WIE is included in WAVE beacon should be explicitly explained and clarified. Otherwise, it could be confusing. |  |
| 21 | Thomson, Allan | 3 | 2 | 35 | TR | The description of WAVE mode does not describe what WAVE mode is | Change sentence to state what WAVE mode is |
| 22 | Emeott, Stephen | 3.168c | 2 | 36 | TR | The WAVE mode has enough similarities to the IBSS mode (e.g. stations communicate directly, do not associate with one another, sending beacons to advertise the presence of a BSS) that it should be called IBSS on-demand mode or IBSS WAVE mode. | Make the suggested change throughout the document |
| 23 | Roy, Richard | 3.168c | 2 | 36 | TR | WAVE is not a separate "mode" of operation of a STA. The WAVE amendment provides additional specifications that allow STAs to communicate (i.e., send data, management, and control frames) outside the context of any BSS. For example, in addition to all the normal 802.11 functionality, WAVE capable STAs can send data frames without first having to join a BSS. | Replace the definition of WAVE mode a definition of "WAVE capable STA (WC STA): a STA capable of transmitting and receiving data, control, and management frames outside the context of a BSS. WC STAs have dot11WAVECapable set to true." |
| 24 | Stanley, Dorothy | 3.168c | 2 | 36 | T | The definition does not describe the mode | Change to "a mode of operation enabling a rapidly moving station to communicate with a BSS or other stations" or similar |
| 26 | Hart, Brian | 3.168c | 2 | 39 | E | "Within the context of a … BSS" is unusual .11 language | Suggest "within … BSS", search and replace thru-out document |
| 28 | Roy, Richard | 3.168d | 2 | 42 | TR | Beacon management frames are being used for a purpose to which they are ill-suited. The beacon frame should not be overloaded with additional functionality that is orthogonal to its basic purpose, that of initiating and maintaining BSSes. The only feature of a beacon frame that is potentially useful for STAs communicating outside the context of a BSS is the accurate Timestamp in the beacon frame itself. As the Timestamp is used for optional synchronization, a Timing Synchronization management frame is the appropriate management frame to include in the amendment. Allowing this frame to optionally carry the WIE creates all the func`tionality necessary to allow WC STAs to operate successfully. | Remove the ONDEMANDBEACON frame and replace it with a Timing Synchronization management frame that has the accurate Timestamp required. |
| 29 | Stanley, Dorothy | 3.168d | 2 | 42 | T | The note should be in a different section, also Grammar - change from "a beacon frame for" to "a Bracon Frame in which" | as in comment |
| 33 | Roy, Richard | 3.168e | 2 | 49 | TR | Beacon management frames are being used for a purpose to which they are ill-suited. The beacon frame should not be overloaded with additional functionality that is orthogonal to its basic purpose, that of initiating and maintaining BSSes, and communication by WC STAs outside the context of a BSS is exactly that, communication without a BSS. A beacon frame is not required. | Remove the definition of WAVE beacon and all instances of the term from the document. This can be easily accomplished since the functionality (that of carrying an optional information element the contents of which are beyond the scope of 802.11) implemented by the use of this frame is beyond the scope of 802.11. This simple functionality (optionally carrying an IE) is better left to a specifically designed action frame and the proposed Timing Synchronization management frame. |
| 34 | Thomson, Allan | 3 | 2 | 50 | TR | "sent by a WAVE mode STA". What is a WAVE mode sta? | Define what a WAVE mode sta is |
| 35 | Myles, Andrew | 3.1683 | 2 | 50 | ER | The phrase, "sent by a WAVE mode STA" does not seem to add much to the definition because it is already implicit that only a WAVE mode STA would set the WAVE indication bit | Remove indicated phrase |
| 38 | Kenney, John | 3.168b | 2 | 30-31 | T | It is not necessary to have a separate definition for this IE. The meaning of WAVE IE is made clear when it is introduced in clause 7.2.3.1 and Table 7-8 (using these same words). Note also that the 3.168b definition has an error: "by the MAC" should be "to the MAC". Table 7-8 is correct. | Delete 3.168b. |
| 39 | Rai, Vinuth | 3.168b | 2 | 30-31 | T | WIE definition is uncessary, concept is well defined in clause 7 | Remove WIE definition |
| 40 | Stephenson, Dave | 3 | 2 | 30-33 | E | The WIE is defined. Why is it necessary to provide a definition for this IE. It is not standard practice to provide a definition for every IE as the definition is usually provided in clause 7.3.2. Adding unnecessary definitions just clutter this clause. | Delete the definition or move it to 7.3.2. |
| 41 | Stephenson, Dave | 3 | 2 | 42-51 | TR | The definitions for on-demand beacon and wave beacon are the same. This is confusing. | Delete on of the definitions and use the remaining definition consistently throughout the text of the amendment. |
| 42 | Kenney, John | 3.168e | 2 | 49-51 | E | Improve wording for clarity | Change the definition to: "An On-demand beacon frame sent by a STA in WAVE mode and encoded with the WAVE indication bit of the Extended Capabilities information element set to 1." |
| 46 | Roy, Richard | 4 | 3 | 6 | TR | The information element being described is not just restricted to use by WC STAs. Calling it a WAVE information element is misleading. | Rename the WIE to HLIE (higher layer information element). |

:

- Commenter’s Suggested Remedy (If appropriate):  [From Spreadsheet]

Various, see specific comments above.

- Background, Explanation, Discussion, etc.:

The motion in submission 11-08-1024-06-000p-no-wbss-no-beacon-comment-resolution.doc passed, resulting in the elimination of WAVE BSS and major modifications to the draft. All of these comments refer to parts of the draft that were modified by the 1024 motion. Such modifications either directly or indirectly resolve each of these comments. We term this to be “overtaken by events” (OBE).

- Recommended Resolution of the Comment:

None, these are all OBE.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to: Resolve each of the comments listed in this submittal as being countered by being OBE with the passing of the motion in 11-08-1024-07-000p-no-wbss-no-beacon-comment-resolution.doc.

Motion by: ____________________Date: _________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References: