IEEE P802.11
Wireless LANs

| Annex I and J Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| R. Roy | SRA (Affiliation: Connexis) |  | 650-861-3351 | dickroy@alum.mit.edu |

Abstract

This is a recommended change to the IEEE 802.11 (11p) document regarding the regulatory language in Annexes I and J.  Note this document is a revision of 11-07/2228r1 which was originally posted in March 2007.  The current document management system does not allow posting of revisions of this document.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Ed: Replace the last two rows of Table I.2 with the following two rows:

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| <ANA/ELS> ITS operations | FCC CFR 47 [B8], Sections 90.375, 90.377, 90.379, 90.383, 95.639, 95.1511 | Reserved | Reserved |
| <ANA/ELS+1>-255 | Reserved | Reserved | Reserved |

Ed: Replace the last three rows of Table I.3 with the following two rows:

- Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| <ANA/BLS> ITS operations | FCC CFR 47 [B8], Sections 90.375, 90.377, 90.379, 90.383, 95.639, 95.1511 | Reserved | Reserved |
| <ANA/BLS+1>-255 | Reserved | Reserved | Reserved |

Make the following changes to Annex J.

Annex J

(normative)

Country information element and regulatory classes

J.1 Country information and regulatory classes

WLANs implemented in accordance with this standard and the specifications and definitions referenced in it may be subject to equipment certification and operating requirements established by regional and national regulatory administrations. The specification establishes minimum technical requirements for interoperability taking into consideration established regulations at the time this standard was issued. These regional and national regulations are subject to revision and may be superseded. Regulatory requirements that do not affect interoperability are not addressed in this standard. Implementers are referred to the regulatory sources in Table I.1 and their successors for further information. Operation in countries within defined regulatory domains may be subject to additional or alternative national regulations.

The Country information element (see 7.3.2.9) allows a STA to configure its PHY and MAC for operation

when the regulatory triplet of Regulatory Extension Identifier, Regulatory Class, and Coverage Class fields

is present. The regulatory triplet indicates both PHY and MAC configuration characteristics and operational

characteristics. The First Channel Number field of subsequent subband triplet(s) is based on the

dot11ChannelStartingFactor that is indicated by the Regulatory Class field.

The regulatory class for the OFDM PHY points to a row in Tables J.1, J.2, or J.3, wherein elements (specifically the Channel starting frequency, Nominal channel bandwidth, and Channel set) that enable a STA to calculate the center frequencies and nominal channel bandwidth of a set of RF channels are found.  The tables also contain pointers to regulatory documents for each regulatory domain wherein further transmission requirements may be found.

The channel starting frequency variable is a frequency, used together with a channel number, to calculate a

channel center frequency.

Nominal channel bandwidth is the minimum frequency difference between nonoverlapping channel center frequencies.

The channel set is a list of integer channel numbers that are used to calculate a set of RF channels.  For a device to legally operate on any or all of these channels, they must be allowed under the relevant regulatory specifications.

The transmit power limits

given in Tables J.1 through J.3 are informative and in all cases regulatory constraints on those power limits apply.

An emissions limits set is an enumerated list each element of which points to a row in Table I.2 containing references to regulations where restrictions on emissions in various regulatory domains can be found.  These documents are subject to revision and change

Note - Specific transmit restrictions and limits are listed in I.2.

A behavior limits set is an enumerated list each element of which points to a row in Table I.3 containing behavior limits and references to regulations where behavior limits in various regulatory domains can be found.  These documents are subject to revision and change

Regulatory classes for operation in the United States are enumerated in Table J.1.

Table J.1—Regulatory classes in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Nominal Channel Bandwidth (MHZ) | Channel set | Transmit Power limit (mW) | Transmit Power limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 20 | 36, 40, 44, 48 | 40 | — | 1 | 1, 2 |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | — | 1 | 1 |
| 3 | 5 | 20 | 149, 153, 157, 161 | 800 | — | 1 | 1 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | — | 1 | 1 |
| 5 | 5 | 20 | 165 | 1000 | — | 4 | 10 |
| 6 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 25 | — | 5 | 9 |
| 7 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 500 | — | 5 | 9 |
| 8 | 4.89 | 10 | 11, 13, 15, 17, 19 | 50 | — | 5 | 9 |
| 9 | 4.89 | 10 | 11, 13, 15, 17, 19 | 1000 | — | 5 | 9 |
| 10 | 4.85 | 20 | 21, 25 | 100 | — | 5 | 9 |
| 11 | 4.85 | 20 | 21, 25 | 2000 | — | 5 | 9 |
| 12 | 2.407 | 25 | 1-11 | 1000 | — | 4 | 10 |
| 13 | 3.000 | 20 | 133, 137 | --- | 1W/MHz | 6 | 3, 4, 6, 11, 12 |
| 13 | 3.000 | 20 | 133, 137 | --- | 40mW/MHz | 6 | 3, 4, 5, 6, 11, 13 |
| 14 | 3.000 | 10 | 132, 134, 136, 138 | --- | 1W/MHz | 6 | 3, 4, 6, 11, 12 |
| 14 | 3.000 | 10 | 132, 134, 136, 138 | --- | 40mW/MHz | 6 | 3, 4, 5, 6, 11, 13 |
| 15 | 3.0025 | 5 | 131, 132, 133, 134, 135, 136, 137, 138 | --- | 1W/MHz | 6 | 3, 4, 6,11, 12 |
| 15 | 3.0025 | 5 | 131, 132, 133, 134, 135, 136, 137, 138 | --- | 40mW/MHz | 6 | 3, 4, 5, 6, 11, 13 |
| <ANA> | 5 | 5 | 171-185 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 10 | 171-185 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 20 | 172-183 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 30 | 173-182 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 40 | 174-181 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA>-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Regulatory classes for operation in Europe are enumerated in Table J.2.

Ed: Move the following sentence below Table J.2

Regulatory classes for operation in Japan are enumerated in Table J.3.

Table J.2—Regulatory classes in Europe

Ed: Add the indicated rows to Table J.2.

| Regulatory Class | Channel Starting Frequency (GHz) | Nominal Channel Bandwidth (MHZ) | Channel set | Transmit Power limit (mW) | Transmit Power limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <ANA> | 5 | 5 | 171-185 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 10 | 171-185 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 20 | 172-183 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 30 | 173-182 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA> | 5 | 40 | 174-181 | --- | --- | <ANA/ELS> | <ANA/BLS> |
| <ANA>-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Motion: Move to accept these changes to Annexes I and J and instruct the editor to update the latest 11p draft to include these changes.

Moved:

Seconded:

YES:

NO:

ABSTAIN: