IEEE P802.11
Wireless LANs

| Annex I and J Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| R. Roy | SRA (Affiliation: Connexis) |  | 650-861-3351 | dickroy@alum.mit.edu |

Abstract

This is a recommended change to the IEEE 802.11 (11p) document regarding the regulatory language in Annexes I and J.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This submission addresses changes to Annexes I and J.  The limits sets for the US were missing 90.379 which is the FCC reference to ASTM 2213, and the European regulatory document specifying the use of 5.9GHz bands in Europe.

Regulatory classes.

- External regulatory references

Change the emissions limits set in D4.02 Table I.2 as follows

| - Emissions limits sets |  |  |  |
| --- | --- | --- | --- |
| Emissions limits set | USA | Europe | Japan |
| <ANA> ITS radio service | FCC 47 CFR [B8], Sections 90.375, 90.377, 90.379, 95.639 and 95.1511 | ReservedETSI EN 302 571 | Reserved |
| 7<ANA+1>-255 | Reserved | Reserved | Reserved |

Change the behavior limits set in D4.02 Table I.3- Behavior limits sets, by combining the two rows into one and adding the missing reference to ASTM 2213 as well as the European reference so that it appears as follows:

| - Behavior limits sets |  |  |  |
| --- | --- | --- | --- |
| Behavior limits sets | USA | Europe | Japan |
| <ANA> ITS operations | FCC 47 CFR [B8], Sections 90.375, 90.377, 90.379, 95.639 and 95.1511 | ETSI EN 302 571 | Reserved |
| 16<ANA+1>-255 | Reserved | Reserved | Reserved |

Country information element and regulatory classes.

Change D4.02 Table J.1 and editor instructions to read as follows:

Insert 4 new entries to Table J.1, all with the same value of regulatory class, before the last row

and change the numbering of the last row accordingly as shown. Note that <ANA > means that

the number is to be assigned by ANA, <ANAE> refers to the ANA number assigned to the new

entry in Table I.2 through this amendment, while <ANAD> refers to the ANA numbers assigned to the new entries in Table I.3 through this amendment :

- Regulatory classes in the USA

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <ANA> | 5.0025 | 5 | 170-184 | 760 | 44.8 dBm | <ANAE> | <ANAD> |
| <ANA+1>1 | 5 | 10 | 171-184 | 760 | 44.8 dBm | <ANAE> | <ANAD>, |
| <ANA+2>1 | 5 | 20 | 172-183 | 10 | 23 dBm | <ANAE> | <ANAD> |
| <ANA+3>1 | 5 | 40 | 174-181 | 10 | 23 dBm | <ANAE> | <ANAD> |
| 16<ANA+4>-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

This regulatory class specifies a list of channels in the 5.9GHz band.  Current regulations only permit a subset of these channels.

Add the indicated rows and footnote to Table J.2

Table J.2—Regulatory classes for 5 GHz bands in Europe.

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |

| <ANA>1 | 5.0025 | 5 | 170-184 | 760 | 44.8 dBm | <ANAE> | <ANAD> |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <ANA+1>1 | 5 | 10 | 171-184 | 760 | 44.8 dBm | <ANAE> | <ANAD> |
| <ANA+2>1 | 5 | 20 | 172-183 | 10 | 23 dBm | <ANAE> | <ANAD> |
| <ANA+3>1 | 5 | 40 | 174-181 | 10 | 23 dBm | <ANAE> | <ANAD> |
| 16<ANA+4>-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

This regulatory class specifies a list of channels in the 5.9GHz band.  Current regulations only permit a subset  of these channels

1