IEEE P802.11
Wireless LANs

| Proposal to Update SSPN features |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Gabor Bajko | Nokia | 313 Fairchild dr Mountain View, CA 94043 | +1 650 625 2182 | gabor.bajko@nokia.com |

Abstract

This proposal is addresses LB132 SSPN comments.

This document is based on 802.11u-d3.01.

- MAC service definition

  - Detailed service specification

- 6.2.1 MAC Data services

The IEEE 802.11 MAC supports the following service primitives as defined in ISO/IEC 8802-2: 1998:

— MA-UNITDATA.request

— MA-UNITDATA.indication

— MA-UNITDATA.confirm

The LLC definitions of the primitives and specific parameter value restrictions imposed by IEEE Std 802.11

are given in 6.2.1.1 through 6.2.1.3.

      - MA-UNITDATA.request

6.2.1.1.4 Effect of receipt

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of an individually addressed frame of type data and subtypes 0 to 3 inclusive the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthBestEffortRate in the dot11InterworkingEntry identified by the source MAC address of the received frame.  The specific mechanism to perform rate limiting is outside the scope of this specification.: If the rate-limiting mechanism does not discard the frame, the following operations shall be performed by the AP’s MAC sublayer:

- dot11NonApStaBestEffortMSDUcount shall be incremented by 1

- dot11NonApStaBestEffortOctetCount shall be incremented by the number of octets in the MSDU

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of an individually addressed frame of type data and subtypes 8 to 11 inclusive (QoS data subtypes) and the priority is an integer in the range of 0 to 7 inclusive, then the , the AP’s MAC sublayer shall derive the access category from the priority using the mapping in Table 9-1.  The AP’s MAC sublayer shall retrieve the MIB variables listed below from the  dot11InterworkingEntry identified by the source MAC address of the received frame and perform the following operations:

- If the access category is AC_VO, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in  dot11NonApStaMaxAuthVoiceRate, the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate limiting mechanism does not discard the frame, then dot11NonApStaVoiceMSDUcount shall be incremented by 1 and dot11NonApStaVoiceOctetCount shall be incremented by the number of octets in the MSDU..

- If the access category is AC_VI, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthVideoRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaVideoMSDUcount shall be incremented by 1 and dot11NonApStaVideoOctetCount shall be incremented by the number of octets in the MSDU.

- If the access category is AC_BE, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthBestEffortRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaBestEffortMSDUcount shall be incremented by 1 and dot11NonApStaBestEffortOctetCount shall be incremented by the number of octets in the MSDU.

- If the access category is AC_BK, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthBackgroundRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaBackgroundMSDUcount shall be incremented by 1 and dot11NonApStaBackgroundOctetCount shall be incremented by the number of octets in the MSDU.

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of an individually addressed frame of type data whose priority is an integer in the range of 8 to 15 inclusive, then the , the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthHccaRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaHCCAMSDUcount shall be incremented by 1, and dot11NonApStaHCCAOctetCount shall be incremented by the number of octets in the MSDU.

      - MA-UNITDATA.indication

Change 6.2.1.2.4 as shown below:

        - Effect of receipt

The effect of receipt of this primitive by the LLC sublayer is dependent on theontent of the MSDU.

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of a group addressed frame of type data the AP’s MAC sublayer shall discard the frame if dot11NonApStaAuthSourceMulticast is FALSE in the dot11InterworkingEntry identified by the source MAC address of the received frame.  If dot11NonApStaAuthSourceMulticast is TRUE, the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthSourceMulticastRate in the dot11InterworkingEntry identified by the source MAC address of the received frame.  The specific mechanism to perform rate limiting is outside the scope of this specification.

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of an individually addressed frame of type data and subtypes 0 to 3 inclusive the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthBestEffortRate in the dot11InterworkingEntry identified by the source MAC address of the received frame.  The specific mechanism to perform rate limiting is outside the scope of this specification.: If the rate-limiting mechanism does not discard the frame, the following operations shall be performed by the AP’s MAC sublayer:

- dot11NonApStaBestEffortMSDUcount shall be incremented by 1

- dot11NonApStaBestEffortOctetCount shall be incremented by the number of octets in the MSDU

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of an individually addressed frame of type data and subtypes 8 to 11 inclusive (QoS data subtypes) and the priority is an integer in the range of 0 to 7 inclusive, then the , the AP’s MAC sublayer shall derive the access category from the priority using the mapping in Table 9-1.  The AP’s MAC sublayer shall retrieve the MIB variables listed below from the  dot11InterworkingEntry identified by the source MAC address of the received frame and perform the following operations:

- If the access category is AC_VO, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthVoiceRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaVoiceMSDUcount shall be incremented by 1 and dot11NonApStaVoiceOctetCount shall be incremented by the number of octets in the MSDU.

- If the access category is AC_VI, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthVideoRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaVideoMSDUcount shall be incremented by 1 and dot11NonApStaVideoOctetCount shall be incremented by the number of octets in the MSDU.

- If the access category is AC_BE, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthBestEffortRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaBestEffortMSDUcount shall be incremented by 1 and dot11NonApStaBestEffortOctetCount shall be incremented by the number of octets in the MSDU.

- If the access category is AC_BK, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthBackgroundRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonApStaBackgroundMSDUcount shall be incremented by 1 and dot11NonApStaBackgroundOctetCount shall be incremented by the number of octets in the MSDU.

At an AP for which dot11SspnInterfaceEnabled is TRUE, upon receipt of an individually addressed frame of type data whose priority is an integer in the range of 8 to 15 inclusive, then the , the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonApStaMaxAuthHccaRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then

dot11NonApStaHCCAMSDUcount shall be incremented by 1, and

dot11NonApStaHCCAOctetCount shall be incremented by the number of octets in the MSDU.

      - MA-UNITDATA.confirm

Change 6.2.1.3.4 as shown below:

        - Effect of receipt

The effect of receipt of this primitive by the LLC sublayer is dependent on the content of the MSDU.