IEEE P802.11
Wireless LANs

| Language and Venue Types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Roke Manor Research Ltd | Old Salisbury Lane, Romsey, Hampshire, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |

Abstract

This proposal addresses a number of comments related to the MIB, which resulted from LB132.  This proposal addresses the following CIDs: 17,18.

This document is based on IEEE 802.11u-D3.0

Modify the following section of clause 7.3.3.2. on page 29, starting at line 31.

The Venue Name field is shown in Figure 7-95bb. Each venue name field that is defined for an SSID is included in the native query response.

| Length | Language Code | Venue Name |
| --- | --- | --- |

Octets:	  1	           3		        variable

Table 7-95bb—Venue Name field

—The length sub-field is the length in octets of the venue name subfield. It is set equal to 1 plus the number of octets in the Venue name subfield

— "The language code field is an ISO-14962-1997 [B39] encoded string that defines the language used in the  venue name field and is either a two or three character language code selected from ISO-639 [B40]. A two character language code has a zero ("NULL" in ISO-14962-1997) appended to make it three octets in length. A language field consisting of three zeros is used to signal that no language is specified for this venue name".

—The Venue name sub-field is a UTF-8 formatted field containing the venue’s name. The maximum length of this field is 256 octets. UTF-8 format is defined RFC-3629 [B37].

Add the following biographical references to Annex P, renumbering the order sequence

[B39]	ISO 14962:1997, Space data and information transfer systems - ASCII encoded English

[B40]    ISO 639, Codes for the Representation of Names of Languages