IEEE P802.11
Wireless LANs

| TGz Draft 2.0 LB127 Peer PSM related CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: September 09, 2008 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michelle Gong | Intel Corporation | 2200 Mission College Blvd, Santa Clara, CA 95052 USA | +1-408-765-7994 | michelle.x.gong@intel.com |

Abstract

This document addresses Peer PSM related CIDs.

Instruction to the editor: Make changes as shown below.

7.3.2.67 Wakeup Schedule information element

The Wakeup Schedule information element contains information regarding the wakeup schedule for Peer Power Save Mode. The element information format is defined in Figure 7-z5.

|  | Element ID | Length | Start Time | Interval | Duration of Awake Window | Idle Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 4 | 4 | 2 |

Figure 7-z5—Wakeup Schedule element format

The Length field is set to 16.

The Start Time field is set to the lower order 4 octets of the TSF timer value at the start of the first Awake Window.

The Interval field is set to the time in microseconds between two successive Awake Windows.

The Duration of Awake Window field is set to the duration of the Awake Window, in microseconds.

The Idle Count field is set to the number of consecutively empty Awake Window before a peer STA sends a TDLS AP Path Switch Request frame.

11.2.1.13 Peer Power Save Mode

Peer Power Save Mode (Peer PSM) is a power save mechanism that can be used between peer STAs operating over a TDLS direct link. A STA supports Peer Power Save Mode if dot11TDLSPeerPSMImplemented is true. Support of this capability shall be signaled through the Peer PSM subfield in the Extended Capabilities element included in the Association Request frame body inside the TDLS Setup Request or TDLS Setup Response frame.

The following terminology is used throughout this section:

TDLS Peer PSM initiator: A STA which transmits a TDLS Peer PSM request frame

TDLS Peer PSM responder: A STA which transmits a TDLS Peer PSM response frame

TDLS Peer STA: A STA which is involved in a direct link or a direct link establishment

A STA which intends to enter peer power save mode shall send a Peer PSM Request frame to the peer STA, including a proposed wakeup schedule. When the TDLS peer STA accepts the proposed wakeup schedule, it shall respond with a Peer PSM Response frame indicating status code 0. Otherwise it shall respond with a Peer PSM Response frame indicating the appropriate status code for rejecting the schedule. An alternative schedule shall be included in the Peer PSM Response frame when the reason code is set to “Wakeup schedule rejected but alternative schedule provided”. The alternative schedule may be used by the TDLS Peer PSM initiator to generate a new Peer PSM Request frame.

After transmitting or receiving a Peer PSM Response frame indicating status code 0, both the TDLS Peer initiator and responder have established a wakeup schedule between them.  The wakeup schedule remains valid until the direct link is torn down or STAs explicitly update the existing schedule.

Either station may update an existing schedule by initiating a Peer PSM Request/Response exchange.  If the Peer PSM Response frame indicating status code 0, a new wakeup schedule is established for the direct link.  Otherwise, the old schedule still applies.  The new schedule takes effect after the termination of the on-going service period.

A TDLS peer STA enters TDLS Peer Power Save mode per direct link by setting the Power Management bit to one in an acknowledged MPDU transmitted to the peer STA. The power save status on one direct link is independent of the power save status on another direct link.  If at least one TDLS peer STA enters Power Save mode, both TDLS peer STAs on the direct link shall be awake at the beginning of the scheduled Awake Windows, and stay awake for the duration of the Awake Window or until the end of a TDLS Peer PSM service period.

A TDLS Peer PSM service period is a contiguous time during which one or more unicast frames are transmitted between two TDLS peer STAs when at least one STA is in Peer Power Save Mode.  A TDLS Peer PSM service period is initiated at the beginning of the Awake Window.  The TDLS Peer PSM service period is terminated at the end of the Awake Window when no frame exchange occurs within the Awake Window or when each STA has completed an acknowledged frame exchange with the EOSP bit set to one with the TDLS peer STA, subject to the retransmission rules defined in this section.

A STA shall remain awake until it has transmitted to and received from the peer STA an acknowledged MPDU with the EOSP bit set to one, subject to the retransmission rules defined in this section.

When both TDLS peer STA set the More Data ACK subfield in their QoS Capability information element, then the More Data bit inside an ACK frame set to zero shall have the same function as the EOSP bit inside a QoS frame set to one (i.e. to terminate a TDLS Peer PSM service period). In this case, a TDLS Peer PSM service period is terminated after an acknowledged frame exchange with the EOSP bit in the MPDU set to one and the More Data bit in the ACK set to 0.

To keep track of the connectivity over the direct link and to maintain the wakeup schedule, peer STAs shall start an acknowledged frame exchange at least once per Idle Count consecutive Awake windows, as a keepalive. For instance a QoS-Null frame may be used as a keepalive frame. When no successful frame exchange has occurred for Idle Count consecutive Awake Windows, a STA may send a TDLS AP Path Switch Request to the peer STA via the AP path.

If a STA does not receive an acknowledgment to a directed MPDU sent with the EOSP subfield set to 1 which terminates a TDLS Peer PSM service period, it shall retransmit that frame at least once within the same service period, subject to applicable retry or lifetime limit. The maximum number of retransmissions within the same service period is the lesser of the maximum retry limit and the MIB attribute dot11PeerSTAMissingAckRetryLimit. If an acknowledgment to the retransmission of this last frame in the same service period is not received, it may wait until the next PSP to transmit that frame, subject to its applicable retry or lifetime limit.