IEEE P802.11
Wireless LANs

| Proposal for Link Layer Service Advertisement, Network Discovery |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

P802.11u-d3.01 provides non-native GAS mechanism for non-AP STAs to find out the network capabilities and services without connecting and reconnecting with the network. However, that method requires deployment of large information services which is not be feasible for light weight small networks and usage models. This proposal provides methods to enable the network services advertisement which will help with network discovery and selection in Peer-to-Peer networks.

This document is based on IEEE 802.11-2007,  802.11u-d3.01 and 802.11-08/1156r1

- Introduction

IEEE 802.11u-d3.01 currently supports IEEE 802.21-IS which provides mechanisms for network services discovery. Using 802.11u Non-Native GAS mechanism, devices can query the 802.21 IS for the network services.

However, 802.21-IS is deployable for the large scale networks. It is not feasible for small networks, i.e., one AP with a few connected devices. What is lacking is the availability of similar behaviour in small networks where 802.-21-IS is not available.

Today, we see more and more consumer electronic devices, such as phones, cameras, media players, projectors, and computers enabled with Wi-Fi. However, as the mobile users move around, the devices may need to communicate among themselves. For instance, users can take picture with their camera and send it to their notebook for editing and displaying. This is where Peer-to-Peer (P2P) personal networking comes into picture. The idea is simple, you put Wi-Fi enabled devices beside each other, and they communicate with each other without requiring full fledge 802.11 infrastructure. As the name suggest, the P2P is used for communication among personal devices or connecting to higher layer services, using the 802.11 radios in the devices.

This proposal introduces methods that will help personal devices to discover the P2P networks around them and query services available in the network without going through the trial and error method of connecting and disconnecting with the network.

IEEE 802.11u-d3.01 currently provides a Network Type field in included in the Interworking element.  Using the Network Type field, a non-AP STA can quickly scan and discover the P2P networks by sending a Probe Request in which the Network Type field is set to “Peer-to-Peer”.  Then, only the APs configured with Network Type = “Peer-to-Peer” will reply with a Probe Response.

As P2P enabled devices forms the P2P network, a device assumes the role of control point or virtual AP in the network, mediates the communication within the P2P, and builds the list of device services available in the network and advertises them when queried.

Once, the P2P network is identified, using the Peer-to-Peer network type, as described above, the device can initiate the service discovery processes. In pre-associated state, non-AP STAs can query the P2P network for the available services it is interested in. If the service of interest is available in a particular P2P network, the client can then connect to the network. It can then make use of the services offered. In the absence of such functionality, the client would have to try to connect to the network, after finding out the service it needs is not available in the network, it would have to disconnect from the network and try to connect another network until it finds the service it needs.  The proposed link layer service advertisement and discovery solution removes the trial-error way of connecting and disconnecting from the networks until the client finds what it needs.

The proposed link layer service advertisement and discovery mechanisms will greatly benefit the personal devices with fast discovery of network services in P2P personal networks, resulting better network selection, quicker response and improved battery life for mobile devices.

Note that this is not intended as a replacement to 802.21-IS.  Clearly this is not possible given the richness of the 802.21-IS query language and the small amount of information conveyed by the proposed solution. It will be practical in small P2P networks.

    - Management frames

      - Association Request

Insert  the following to the contents of Table 7-10 as shown below:

Table 7-10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Service Description | The Service Description element is present if dot11PeerToPeerNetworkingEnabled is true. |

      - Reason Code field

Insert following item into Table 7-22 before “Reserved”, renumber when necessary:

Table 7-22—Reason codes

| Reason Code | Meaning |
| --- | --- |
| 52 | Connection is rejected because the service is already provided by the network |
| 53 – 65535 | Reserved |

- 7.3.2	Information elements

Insert the following to the contents of Table 7-26 as shown below:

Table 7-26—Element IDs

| Information Element | Element ID | Length |
| --- | --- | --- |
| Device Service Description | <ANA> | variable |

7.3.2.59 Interworking information element

Change Table 7-43r as shown below:

| Network Type Codes | Meaning | Description |
| --- | --- | --- |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. These networks may or may not employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts area available. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http/https redirect or DNS redirection). |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. |
| 4 | Emergency network | The network supports Emergency Services. Further information is available from a native GAS query (see 7.3.3) |
| 5 | Peer-to-Peer network | A small network of personal devices such as printer, camera and computers, where one device may assume the role of AP. |
| 5 6 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |

Insert the following new subclause, renumbering clauses as necessary:

      - Device Service Description element

The Device Service Description information element contains information about the services provided by the device joining the network.  The Service Description element format is shown in Figure 7-95ay.

|  |  |  | B0 – B3 | B4 – B7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Number of Services Supported | Reserved | Device Service #1 | . . . | Device Service # n (optional) |
| Bits: |  |  | 4 | 4 |  |  |  |
| Octets: | 1 | 2 |  |  | variable |  | variable |

Figure 7-95ay—Device Service Description element format

The Length field is the length of the Device Service element. The value of the Length field is variable and depends on the number of Device Service fields and lengths present.

The Number of Services Supported field is a 4-bit field whose value is the number services provided by the device.

The Device Service field format is shown in Figure 7-95az.

|  | Length | Service Type Code | Supported Protocols |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure 7-95az— Device Service field format

The Length field is the length of Device Service field. Its value is variable and depends on the size of the other fields.

The Device Service field contains the Service Type code shown Table 7-43aa.

Table 7-43aa—Device Service Type codes

| Service Names | Service Type Codes |
| --- | --- |
| Unspecified | 0 |
| Printing | 1 |
| Digital Displaying | 2 |
| Projecting | 3 |
| Digital Camera | 4 |
| Scanning | 5 |
| Copying | 6 |
| Fax | 7 |
| Phone | 8 |
| Media Playing | 9 |
| Computer | 10 |
| Computer Pointing | 8 |
| Keyboard Input | 15 |
| Data Storing | 16 |
| Reserved | 17 – 127 |
| Vendor Specific | 128 |
| Reserved | 129 – 255 |

The Device Service Name is the functionality supported by the device.

The Device Service Type code is the value which is assigned to corresponding device functionality.

The Supported Protocols field format is shown in Figure 7-95ba.

|  |  | B0 - B3 | B4 - B7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Length | Number of Protocols Supported | Reserved | Supported Protocol Type #1 | . . . | Supported Protocol Type #16 |
| Bits: |  | 4 | 4 |  |  |  |
| Octets: | 1 |  |  | 1 |  | 1 |

Figure 7-95bb — Supported Protocols field format

The Number of Supported Protocols field is 4-bit field. The device may support up to 16 protocols per service.

The Supported Protocol Type field contains the upper layer protocol type supported by the device. The Supported Protocol Type codes are shown in Table 7-43ab.

Table 7-43ab—Supported Protocol Type codes

| Supported Protocol Name | Supported Protocol Type codes |
| --- | --- |
| Plug and Play (UPnP), see [B39] | 0 |
| Bonjour , see [B40] | 1 |
| Reserved | 2-9 |
| Vendor Specific | 10 |
| Reserved | 11-15 |

- 7.3.3	Native Query Protocol information elements

Change the contents of Table 7-43x as shown below:

Table 7-43x—Native query protocol info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 0 | 7.3.3.1 |
| Venue Name | 1 | 7.3.3.3 |
| Emergency Call Number Information | 2 | 7.3.3.4 |
| Emergency Public Network Access Information | 3 | 7.3.3.5 |
| Network Authentication Type Information | 4 | 7.3.3.6 |
| Roaming Consortium List | 5 | 7.3.3.7 |
| Device Services List | 6 | 7.3.3.8 |
| Reserved | 7 – 220 | n/a |
| Vendor Specific Information | 221 | n/a |
| Reserved | 222 – 255 | n/a |

Insert the following new subclause, renumbering clauses as necessary:

      - Device Services List

The Device Services List provides a list of services provided by the devices connected to the network. This list may be returned in response to a Native GAS Query Request.  The format of the Device Service List element is provided in Figure 7-95bb

|  | Info ID | Length | Status Code | Supported Service Count | Device Service #1 (optional) | Device Service #2 (optional) | . . . | Device Service #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | 1 | Variable | variable |  | variable |

Figure 7-95bb—Device Description list format

The Length field is 2-octet length of the Device Services List element and is equal to 2 plus the lengths of Device Services present in the element.

The Status Code is a 2-octet field whose value is drawn from Table 23.

The Device Service field format is shown in Figure 7-95az. It is variable length.

Annex D

Change the dot11StationConfigEntry sequence list in the dot11StationConfig table by inserting the shown entries:

dot11StationConfigEntry::=
	SEQUENCE {

dot11StationID 				MacAddress,
		dot11MediumOccupancyLimit 			INTEGER,
		dot11CFPollable 			TruthValue,
		dot11CFPPeriod 			INTEGER,
		dot11CFPMaxDuration 			INTEGER,
		dot11AuthenticationResponseTimeOut 			Unsigned32,
		dot11PrivacyOptionImplemented 			TruthValue,
		dot11PowerManagementMode 			INTEGER,
		dot11DesiredSSID 			OCTET STRING,
		dot11DesiredBSSType 			INTEGER,
		dot11OperationalRateSet 			OCTET STRING,
		dot11BeaconPeriod 			INTEGER,
		dot11DTIMPeriod 			INTEGER,
		dot11AssociationResponseTimeOut 			Unsigned32,
		dot11DisassociateReason 			INTEGER,
		dot11DisassociateStation 			MacAddress,
		dot11DeauthenticateReason 			INTEGER,
		dot11DeauthenticateStation 			MacAddress,
		dot11AuthenticateFailStatus 			INTEGER,
		dot11AuthenticateFailStation 			MacAddress,
		dot11MultiDomainCapabilityImplemented 			TruthValue,
		dot11MultiDomainCapabilityEnabled 			TruthValue,
		dot11CountryString 			OCTET STRING,
		dot11SpectrumManagementImplemented 			TruthValue,
		dot11SpectrumManagementRequired 			TruthValue,
		dot11RSNAOptionImplemented 			TruthValue,
		dot11RSNAPreauthenticationImplemented 			TruthValue,
		dot11RegulatoryClassesImplemented 			TruthValue,
		dot11RegulatoryClassesRequired 			TruthValue,
		dot11QosOptionImplemented 			TruthValue,
		dot11ImmediateBlockAckOptionImplemented 		TruthValue,
		dot11DelayedBlockAckOptionImplemented 			TruthValue,
		dot11DirectOptionImplemented 			TruthValue,
		dot11APSDOptionImplemented 			TruthValue,
		dot11QAckOptionImplemented 			TruthValue,
		dot11QBSSLoadOptionImplemented 			TruthValue,
		dot11QueueRequestOptionImplemented 			TruthValue,
		dot11TXOPRequestOptionImplemented 			TruthValue,
		dot11MoreDataAckOptionImplemented 			TruthValue,
		dot11AssociateinNQBSS 			TruthValue,
		dot11DLSAllowedInQBSS 			TruthValue,
		dot11DLSAllowed 			TruthValue,

dot11InterworkingServiceImplemented 			TruthValue,
		dot11InterworkingServiceEnabled			TruthValue,					dot11QosmapImplemented 			TruthValue,
		dot11QosMapEnabled 			TruthValue,
		dot11EbrImplemented 			TruthValue,
		dot11EbrEnabled 			TruthValue,
		dot11GasTimPeriod 			INTEGER,
		dot11GasTimTimeToSuspend 			INTEGER,
		dot11ESNetwork 			TruthValue,

dot11GasMulticastRepititions			INTEGER,

dot11GasResponseBufferingTime			INTEGER,

dot11PeerToPeerNetworkingImplemented			TruthValue,
		dot11PeerToPeerNetworkingEnabled			TruthValue
	}

Insert the following elements at the end of the dot11StationConfigTable element definitions

dot11PeerToPeerNetworkingImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the STA is capable of peer-to-peer networking. A STA setting this to TRUE implements peer-to-peer networking service. When this is set to FALSE, the STA does not implement peer-to-peer networking Service.”

DEFVAL (false)

::= {dot11StationConfigEntry 1}

dot11PeerToPeerNetworkingEnabled OBJECT-TYPE

SYNTAX TruthValueMAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the capability of the STA to network with “Peer-to-Peer networks is enabled. The capability is disabled otherwise.”

DEFVAL (false)

::= {dot11StationConfigEntry 2}

(Informative)

Annex T

(Informative)

Interworking with External Networks

Change the sub-clause text introduced in 802.11-08/1156r1 by inserting the text as shown below:

Annex T.1	Network Selection

Interworking Service provides features to support the network selection process a non-AP STA uses to choose the network with which to associate.  Generic Advertisement Service provides a non-AP STA access to an 802.21-IS which can provide a rich set of information to aid the network selection process.  In addition, Interworking Service provides lightweight features which also facilitate this process.  The following paragraphs describe several use cases illustrating how these features may be used to aid in network selection.  The uses cases are:

- Airport: a business traveler needs to connect via an airport hotspot to her enterprise network to download email and information from the customer database.

- Tourist: a tourist in downtown London wants to use the internet to make dinner reservations and later becomes ill and needs to contact emergency services.

- Shopping: a woman living in Silicon Valley visits the local shopping mall and uses her smartphone to discover items on sale.

- Sales meeting: a business woman visiting a customer accesses their guest network.

- Museum: a visitor to a museum uses his smartphone to obtain virtual docent service.

- Peer-to-Peer: a camera user wants to show his pictures to his friend in a conference room and upload some of them to his friend’s notebook computer.

Peer-to-Peer

John has taken some pictures using his P2P enabled camera. During a meeting in a conference room, John wants to show his picture to Jack using the projector in the room and upload some pictures to Jack’s P2P enabled notebook.

- The P2P enabled camera’s non-AP STA performs an active scan by transmitting a Probe Request frame containing the wildcard SSID and an Interworking element with Network Type field set to “Peer-to-Peer Network”.  In response, it receives Probe Response frames from several of the APs for the SSID with one of them “Jack’s P2P Network”.

- The Probe Response received by the camera indicated the following capabilities:

  - Extended capabilities element indicates: AP provides Interworking Service

  - Interworking element indicates: Network Type = 6 (Peer-to-Peer Network) Internet Access = 0 (Internet access not available), ASRA = 0 (there is no additional step required for network access)

  - Advertisement Protocol element indicates Jack’s notebook AP supports Native GAS.

  - There is no RSN element present in the received beacon frame.

- Since the AP indicated Interworking service is available, the camera’s non-AP STA uses the MLME-GAS.request primitive to invoke Native GAS

- The Native-GAS response indicates the Jack’s P2P network has the Projecting Service.

- Since the camera’s SME now knows it should be able to successfully associate to the network, it associates to Jack’s P2P enabled AP in the notebook.

- John’s camera passes the Device Services element, including phone and camera services in the Association request.

- After successful association, the Jack’s P2P AP in the notebook adds the camera and phone services provided by the camera to its Device Service list.

- The camera connects to the projector using upper layer protocols and the user starts showing the pictures in the projector.

- Jack likes some of the pictures and asks John to upload them to his notebook.

- John uploads the pictures to Jack’s computer utilizing P2P.

- When camera disassociates from the Jack’s P2P network, the P2P control point updates its Service list, removing the services provided by the camera.

Annex P: Bibliography

General

Insert the following references to the end of the bibliography list.

[B39] UPnP Standard, UPnP™ Forum Universal Plug and Play Device Standards, www.upnp.org/resources/standards.asp

[B40] Bonjour Protocol Specifications, http://developer.apple.com/networking/bonjour/specs.html