IEEE P802.11
Wireless LANs

| TGp LB 125 Resolution of Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x 175 | justin.mcnew@kapsch.net |

Abstract

This submission proposes the removal of Timing Information Element (TIE).

|  | LB125 Comment Resolution |
| --- | --- |

Introduction: The TIE is undefined.  Removing the TIE would result in the following changes to document 1024r7:

Table 7-18b— Timing and Higher Layer Information frame body

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Timestamp |  |
| 2 | Capability |  |
| 3 | Supported Rates IE | ID=1 Optional |
| 4 | Country IE | ID=7 Included if dot11MultiDomainCapabilityEnabled or dot11SpectrumManagementRequired is true. Optional |
| 5 | EDCA IE | ID=22 Optional |
| 6 | Higher layer information element (HLIE) (s) | ID=69 Optional (Zero or more HLIEs may be included in one frame. No HLIEs may be included if the frame is intended to transmit timing information only.) |
| 7 | Extended Capabilities IE (ID = 127 per 11y and 802.11-2007) | ID=127 Optional |
| 8 | Timing Information Element | ID=<ANA> Optional |

Insert the TIE and HLIE rows below into Table 7-26 - Element IDs.  Note that <ANA> means that the number is to be assigned by ANA.

Table 7-26—Element Ids

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| TIE (see 7.3.2.81) | <ANA> | 3 to 257 |  |
| HLIE (see 7.3.2.80) | 69 | 3 to 257 |  |

7.3.2.81 Timing information element (TIE)

Editor: Requires further discussion and acceptance of 11-07/2229r1

10.3.42.3.2 Semantics of the service primitive

This primitive provides the following parameters:

MLME-TIMING_HL_INFO.indication(

Timestamp,

Local Time,

Capability Information,

Extended Capabilities Information,

BasicRateSet,

OperationalRateSet,

Country,

TIE contents,

HLIE contents,

EDCA parameter set,

RCPI,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Timestamp | Integer | N/A | The timestamp of the received frame. |
| Local Time | Integer | N/A | Local Time is the value of a station's TSF timer at the start of reception of the first octet of the timestamp field of the received higher layer and timing information frame. |
| Capability Information | As defined in frame format | As defined in frame format | The announced capabilities of the STA. |
| Extended Capabilities Information | As defined in 7.3.2.27 | 1 - 255 octets | The announced extended capabilities of the STA. |
| BasicRateSet | Set of Integers | 1 - 127 inclusive (for each integer in the set) | The set of data rates that must be supported by all STAs that desire to communicate with this STA outrside the context of a BSS. A receiving STA must be able to receive and transmit at each of the data rates listed in the set. |
| Operational RateSet | Set of Integers | 1 - 127 inclusive (for each integer in the set) | The set of data rates that the transmitting STA desires to use for communication outside the context of a BSS. A receiving STA must be able to receive at each of the data rates listed in the set. This set is a superset of the basic rate set announced by the STA. |
| TIE | As defined in TBD | As defined in TBD | Timing information element |
| HLIE | As defined in 7.3.2.80 | As defined in 7.3.2.80 | Higher layer information element. |
| EDCA parameter set | As defined in frame format | As defined in frame format | The values from the EDCA Parameter Set information element if such an element was present, else null. |
| RCPI | Integer | 0-255 | RCPI is the measured value of received channel power on the received Timing-and-higher-layer-information frame. |

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to remove the Timing Information Element from 802.11p, instruct the editor to remove the changes recommended in document 1024r7 from D4.02.

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: