IEEE P802.11
Wireless LANs

| Proposed Resolutions for Comment Group #3 and Various Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-10-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 264 3759 | Kapil.sood@intel.com |

Abstract

This document contains proposed resolutions to the following comments from SB on D6.0

- Comment Group #3 (CID 260, 264, 261, 240, 133)

- CID # 241

- CID # 251

- CID # 259

All changes apply to TGw draft D6.0, unless specified otherwise.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# A. Comment Group #3

Proposed Resolution to CID 260 as “Counter”, “See CID 264”

Proposed Resolution to CID 261 as “Counter”, “See CID 264”

Proposed Resolution to CID 240 as “Counter”, “The entire section has been replaced by TIE defined by 802.11r.  In addition, see CID 264.”

Proposed Resolution to CID 133 as “Accept”

Proposed Resolution to CID 264 as “Counter”, “Changes in doc# 11-08-1231-r0”

Resolution for CID 264 is given below:

Change 8.4.3., page 27, line 40

An RSNA-enabled AP shall use Table 8-1a and the Management Frame Protection Capable and Management Frame Protection Required advertised in the RSN IEs to determine if it may associate with a non-AP STA. An RSNA enabled non-AP STA shall use Table 8-1a and the values of the Management Frame Protection Capable and Management Frame Protection Required bits advertised in the RSN IEs to determine if it may associate with an AP.  Management Frame Protection is enabled when dot11RSNAProtectedManagementFramesEnabled  is set to 1.  Management Frame Protection is negotiated when an AP and non-AP STA set the Management Frame Protection Capable field to 1 in their respective RSN IEs in the (re) association procedure, and both parties confirm the Management Frame Protection Capable bit set to 1 in the 4-way Handshake, FT 4-way Handshake, or the FT Fast BSS Transition protocol.

Change 8.4.4., page 28, line 42

In order to establish a connection with a peer STA, an RSNA enabled STA that implements Management Frame Protection shall use Table 8-1b and the MFPC and MFPR values advertised in the RSN IEs exchanged in the 4-Way Handshake initiated by the Authenticator of the STA with the larger MAC address to determine if the communication is allowed.  Management Frame Protection is enabled when dot11RSNAProtectedManagementFramesEnabled  is set to 1.  The STAs negotiate use of protection of management frames when the both STAs set the Management Frame Protection Capable field to 1 in their respective RSN IEs in the (re) association procedure, and both parties confirm the Management Frame Protection Capable bit set to 1 in the 4-way Handshake.

Change 5.4.2.4., page 2, line 54

Change 5.4.3.2., page 3, line 6

Change 5.8.2.1., page 5, line 6

Change 5.8.2.1., page 5, line 17

Change 5.8.2.2., page 6, line 40

Change 5.8.6., page 6, line 48

Change 7.3.2.25.1., page 15, line 3

Change 8.3.1., page 21, line 21

Change 8.3.1., page 21, line 36

Change 8.3.3.3.3., page 23, line 13

Change 8.3.4.6., page 26, line 10

Change 8.7.2.1a., page 43, line 40

Change 11.19., page 53, line 48

Change 7.3.2.25.1., page 15, line 3

Management Frame Protection is negotiated

# B. CID# 241

Proposed Resolution to CID 241 as “Counter”, “See Text in 11-08-1231-r0”

Replace 8.3.4.4, 8.3.4.5, and 8.3.4.6, page 25 line 16 to page 26 line 12, as follows:

8.3.4.4 BIP replay protection

The MMIE Sequence Number field represents a sequence number whose length is 6 octets.

When Management Frame Protection is negotiated, for initialization for replay counters, the receiver shall maintain a 48-bit replay counter for each IGTK. The receiver shall set the receive replay counter to the value of the IPN in the IGTK KDE provided by the Authenticator in either the 4-way Handshake, FT 4-way Handshake, FT Handshake, or Group Key Handshake.  The transmitter may reinitialize the sequence counter when the IGTK is refreshed.  See 8.3.4.5 and 8.3.4.6.for per packet BIP processing.

Note - when the IGTK PN space is exhausted, the choices available to an implementation are to replace the IGTK or to end communications.

8.3.4.5 BIP transmission

When a STA transmits a protected broadcast/multicast Robust Management frame it shall:

a) Select the IGTK currently active for transmission of frames to the intended group of recipients and construct the MMIE (see 7.3.2.55) with the MIC field masked to zero and the KeyID field set to the corresponding IGTK KeyID value. The transmitter shall select a valid replay counter for this IGTK and insert this value into the MMIE Sequence Counter field.  The transmitter shall insert a monotonically increasing non-negative integer into the MMIE IPN field.

b) Compute AAD as specified in 8.3.4.3.

c) Compute AES-128-CMAC over the concatenation of (AAD || Management Frame Body || MMIE), and insert the 64-bit output into the MMIE MIC field.

d) Compose the broadcast/multicast Robust Management frame as the IEEE 802.11 header, management frame body, MMIE, and FCS. The MMIE shall appear last in the frame.

e) Transmit the broadcast/multicast Robust Management frame.

8.3.4.6 BIP reception

When a STA receives a broadcast/multicast Robust Management frame protected by BIP, it shall: 
a) Identify the appropriate IGTK key and associated state based on the MMIE KeyID field. If no such IGTK exists, silently drop the frame.

b) The STA shall perform replay protection on the received frame.  The receiver shall interpret the MMIE IPN field as a 48-bit unsigned integer. It shall compare this MMIE IPN integer value against the receive replay counter for the IGTK identified by the MMIE Key ID field. If the integer value from the received MMIE IPN field is less than or equal to the replay counter value for this IGTK, the receiver shall discard the frame and increment the dot11RSNAStatsCMACReplays counter by 1.

c) If the replay protection succeeds, the receiver shall compute AAD for this management frame, as specified in 8.3.4.3.

d) The receiver shall extract and save the received MIC value, and compute the AES-128-CMAC over the concatenation of (AAD || Management Frame Body || MMIE) with the MIC field masked to zero in the MMIE. If the result does not match the received MIC value, then the receiver shall discard the frame and increment the dot11RSNAStatsCMACICVErrors counter by 1.

e) The receiver shall update the replay counter for the IGTK identified by the MMIE Key ID field with the integer value of the MMIE IPN field.

If Management Frame Protection is enabled, broadcast/multicast Robust Management frames that are received without BIP protection shall be discarded.

# C. CID# 251

Proposed Resolution to CID 251 as “Reject”, “The text in 11.20 states all conditions in which a non-AP STA shall send a ping response.  If a non-AP STA would like to delete its PTK (say, if compromised), then it shall issue a new plaintext Associate to the AP, and per procedure specified in 11.20, does not have to respond to the ping request.”

# D. CID# 259

Proposed Resolution to CID 259 as “Reject”, “The text proposed herein is consistent with the format and layout of the Association (11.3.2.2) and Reassociation (11.3.2.4) procedures in the base draft 802.11-2007.  Also, procedures for Association and Reassociation are different, as in reassociation procedure, there is consideration for FT.”