IEEE P802.11
Wireless LANs

| HT Features in 11s Mesh Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408-467-8436 | Liwen.chu@st.com |
| George Vlantis | STMicroelectronics | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408-451-8109 | George.vlantis@st.com |
| Yuichi Morioka | Sony Corporation |  | +81 3-5448-4017 | Yuichi.Morioka@jp.sony.com |
| Kazuyuki Sakoda | Sony Corporation |  | +81 3-5448-4017 | KazuyukiA.Sakoda@jp.sony.com |
| Vincenzo Scarpa | STMicroelectronics | via per Arnesano 16, c/o Distretto Tecnologico, 73100 Lecce - Italy | +39 083-235-3124 | Vincenzo.Scarpa@st.com |

Abstract

The document contains normative text to use HT features in 11s mesh network. 11-08-0798-05-000s-HT-Features-in-mesh-network.ppt is the related presentation.

7.3.2.57 HT Cappabilities element

7.3.2.57.2 HT Cappabilities info field

Add the following text to the end of section7.3.2.57.2:

The following subfields should be reserved for an MP: Tx STBC, Rx STBC, PSMP Support.

7.3.2.57.5 HT Extended Cappabilities field

Add the following text to the end of section7.3.2.57.5:

The following subfields should be reserved for an MP: PCO.

Add the following new section to the end of section 9.13.3.4:

9.13.3.4a Protection rules for HT MP in a mesh network

The HT protection field in an MP may be set to no protection mode only if:

- All STAs detected in the primary or the secondary channel are HT STAs, and

- All MPs that are one-hop neighbors of the transmitting MP to be a member of this mesh network are either:

  - 20/40 MHz HT MPs in a 20/40 MHz mesh network, or

  - 20 MHz HT STAs in a 20 MHz mesh network.

The HT protection field in an MP may be set to non-member protection mode only if:

- A non-HT STA is detected in either the primary or the secondary channel or in both the primary and secondary channels, that is not known by the transmitting MP to be a member of this mesh network, and

- All MPs that are one-hop neighbors of the transmitting MP to be a member of this mesh network are HT MPs.

The HT protection field in an MP may be set to 20 MHz protection mode only if:

- All STAs detected in the primary and all STAs detected in the secondary channel are HT STAs and all MPs that are one-hop neighbors of the transmitting MP are HT MPs, and

- The mesh network is a 20/40 MHz mesh network, and

- There is at least one 20 MHz HT MP that is one-hop neighbor of the transmitting MP.

The HT protection field in an MP is set to non-HT mixed mode otherwise.

If two peer HT MPs report the same protection mode in HT protection field, the protection mechanisms of the related mode will be used to protect the transmission between the peer HT MPs.

If an HT MP and its peer HT MP report different protection modes in HT protection field, the following rules shall be used:

- If an HT MP or its peer HT MP reports non-HT mixed mode, the protection mechanisms of non-HT mixed mode shall be used to protect the transmission between the peer HT MPs.

- If an HT MP or its peer HT MP reports non-member protection mode and non-HT mixed mode is not reported by any of these HT MPs, the protection mechanisms of non-member protection mode shall be used to protect the transmission between the peer HT MPs

- If an HT MP or its peer HT MP reports 20 MHz protection mode and neither non-HT mixed mode nor non-member protection mode is reported by any of these HT MPs, the protection mechanisms of 20 MHz protection mode shall be used to protect the transmission between the peer HT MP.

Update section 11.9.7.2a as following:

11.9.7.2a Selecting and advertising a new channel in a mesh

If an MP detects the need to switch the channel of a PHY (e.g., due to regulatory requirement for radar avoidance), the MP should inform peer MPs to which a mesh link has been established.

Once the MP identifies the candidate channel to switch its PHY to, it creates a new candidate channel precedence indicator value by adding a pseudo-random number to the current channel precedence value. The random value shall be in the range 0 to 8191 inclusive. The random value shall be selected in a manner that minimizes the probability of MPs generating the same number, even when those MPs are subjected to the same initial conditions. It is important that designers recognize the need for statistical independence among the random number streams among MPs.

The MP then executes the UCG switch procedure described in 11B.4.3.

Before an MP starts a 20/40 MHz mesh network it shall perform a minimum of dot11BSSWidthChannelTransitionDelayFactor overlapping BSS/mesh network scans to search existing BSS/mesh network.

If necessary, a 20/40 MHz mesh network may be changed to a 20 MHz mesh network and a 20 MHz mesh network may be changed to a 20/40 MHz mesh network.

When a mesh network switches from a 20 MHz mesh network to a 20/40 MHz mesh network or switches from a 20/40 MHz mesh network to a 20 MHz mesh network, two peer MPs may need to renegotiate MDAOP. When a mesh network switches from a 20/40 MHz mesh network to a 20 MHz mesh network, some MDAOP may need to be deleted to make sure that the MAF of each MP to not exceed its MAF Limit.

When a mesh network switches from a 20 MHz mesh network to a 20/40 MHz mesh network or switches from a 20/40 MHz mesh network to a 20 MHz mesh network, an MP may need to do path maintenance to find optimized path and to avoid congestion.

An MP starts a 20/40 MHz mesh network has the same requirement as an IDO STA starts a 20/40 MHz BSS.

When transmitting 40MHz frames, an MP shall follow the CCA rules as defined in section 11.14.9 and the NAV rules as defined in 11.14.10.

Note: 11.14.4.4 can be used for 40MHz transmission restriction of an MP since it defines restriction on non-AP STAs that are not a member of an infrastructure BSS.