IEEE P802.11
Wireless LANs

| Normative Text for Amendment for EASN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-5 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ding Zhiming | Huawei | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835663 | dingzhiming@huawei.com |
| Stephen McCann | Roke Manor Research Ltd | Roke Manor, Old Salisbury Lane, Romsey, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |
| Shu Guiming | Huawei | Bldg 7,Huawei Technologies Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835543 | shugm@huawei.com |
| Celine Liu | Huawei | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835837 | celineliu@huawei.com |

Abstract

This document is a normative text proposal for EASN. The text is aligned with IEEE P802.11u/D4.01, October, 2008

Insert the following definitions in “4. Abbreviations and acronyms” (Page 4)

4. Abbreviations and acronyms

Insert the following new abbreviations and acronyms into clause 4 in alphabetical order:

802.x LAN	IEEE 802 based local area networks such as 802.3 and 802.11

AAA		authentication, authorization, and accounting

ASRA		additional step required for access

DN		destination network

EAS		emergency alert system

EASN		emergency alert system notification

EBR		expedited bandwidth request

EDT		eastern daylight time

EICI		Emergency Information Change Indication

ES		emergency services

ESO		emergency services only

EST		eastern standard time

GAS		 generic advertisement Service

HESSID 	homogenous ESS identifier

MICS		media independent command service

MIES 		media independent event service

MIH 		media independent handover

MIIS		media independent information service

MLPP		multi-level precedence and preemption

NAI 		network access identifier

PHB		per-hop behavior

PoS		point of service

Modify  Figure 7-95ao in Clause 7.3.2.59  (Page 19)as follows:

7.3.2.59 Interworking information element

The Interworking information element contains information about the interworking service capabilities of a STA as shown in Figure 7-95ao.

| - Interworking element format |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |  |  |  |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN-Supporting | Emergency Network | Venue Group | 802.11 Venue Type | HESSID (optional) | EASN (optional) |  |
|  |  |  |  |  |  |  |  |  |  |  | EASN-Available | EICI |
| Bits |  |  | 4 | 1 | 1 | 1 | 1 |  |  |  | 1 | 7 |
| Octets: | 1 | 3 or 9 |  |  |  |  |  | 1 | 1 | 0 or 6 | 1 |  |

Figure 7-95ao—Interworking element format

The value of the Length field is the length of the Interworking capabilities field. The value of the length field is 3 if no HESSID is present or 9 if a HESSID is present.

The Network Types are shown in Table 7-43r. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See U.4 for informative text on usage of fields contained within the Interworking element.

Modify the text in Clause 7.3.2.59 (Page 20-23) as follows:

Table 7-43r—Network Type codes

Bit 4 is the Internet field. If this field is set to 1, the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet. If the 802.11 Network Type is set to 2, 3, or 14, then this field is set to 1 when the network provides connectivity to the Internet, and it is set to 0 when the network provides unspecified connections to external networks. When the 802.11 Network Type is set to a value other than 2, 3, or 14, this field is always set to 0.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 to indicate that the network requires a further step for access. For more information, refer to Native Query Protocol Native Info Network Authentication Type Information in 7.3.3.5.

Bit 6 is the Emergency Alert System Notification (EASN) capability field.  It is set to 1 to indicate that the IEEE 802.11 infrastructure supports EASN mechanism, otherwise it is set to 0. It is named as EASN-Supporting.

Bit 7 is the Emergency Network field. When set to 0, this field indicates that no emergency services are directly configured to be reachable through te SSID. When set to 1, this field indicates that higher layer emergency services are reachable through the SSID, in addition to any other services offered. Emergency services may be supported either by an ESO (emergency services only) network or a network that supports emergency services accessible with public credentials. An AP advertises this capability if at least one of the two methods is supported.

For a network to be identified as an Emergency Services (ES) network, location capability is enabled on an AP if the AP is located in a regulatory domain that requires location capabilities. In addition, this field is only advertised if all of the following are true (see U.4.2 for further information):

—If an ESO network, the SSID is configured for open authentication and used exclusively for providing emergency services

—If an Emergency Services network with public credentials, the SSID is configured for RSN authentication

—dot11QosOptionImplemented is true

—Expedited Bandwidth Request Service capability field is set to 1

—Network supports end-to-end QoS from non-AP STA to all APs in ESS to PSAP. It is outside the scope of the Interworking Service how this information is known to the AP

The Venue Group and 802.11 Venue Type fields are both one octet values selected from Table 7-43s and Table 7-43t respectively. The entries in Table 7-43s and Table 7-43t are drawn from the International Building Code’s Use and Occupancy Classifications [B33]. An AP sets the default value of Venue Group to Residential and the default value of 802.11 Venue Type to unspecified. A non-AP STA sets the default value of Venue Group to unspecified and the default value of Venue Type to unspecified.

Table 7-43s—Venue Group codes and descriptions

Table 7-43t—802.11 Venue Type assignments

The HESSID field specifies the value of HESSID, see 11.18.1. A non-AP STA uses this field to indicate the desired HESSID in an active scan. The HESSID field is optionally present depending on the setting of the HESSID Present field in the Interworking capabilities field.

The EASN octet is an optional field, only exists while the single bit B6 EASN-Supporting is set to 1. It is formed by two subfields, one is a single bit named EASN-Available that indicates the EAS availability, and the other is the remaining bits named EICI (Emergency Information Change Indication). The single bit EASN-Available is set to 1 to indicate that a higher layer EAS is available at the AP for download, and 0 to indicate that no higher layer EAS is available. The EICI is used to indicate the status and the change of the emergencies. When a first EAS message is active, the EASN-Available bit is set to 1, the EICI is set to non-zero; then the value of EICI is increment by 1 upon a new emergency is alerted or an existing EAS message becomes inactive; the EASN-Available bit is set to 0 if all the existing EAS messages become inactive; if a new emergency is alerted thereafter, the EICI value is incremented upon its previous value. The EICI value restores to 1 after it reaches 127 upon one more change. When EASN-Available bit is set and the value of EICI changes, a non-AP STA requests further information from the higher layers to receive the full EAS information. This information is transported over GAS using a suitable advertisement protocol as described in 11.18.5.

Modify the text in 11.18.5 ( Page 73) as follows:

on this access category and deny all subsequent ADDTS request frames with TID=1 or 2, or if the ACM field for AC_BK is set to 0 then the non-AP STA shall be disassociated.

—dot11NonAPStationHCCAOctetCount is compared to dot11NonAPStationAuthMaxHCCAOctets.When the value of the authorized maximum octet count is exceeded, then the HC shall delete all admitted TSs with access policy = HCCA or HEMM and deny all subsequent ADDTS request frames with access policy = HCCA or HEMM.

—The sum of dot11NonAPStationVoiceOctetCount, dot11NonAPStationVideoOctetCount, dot11NonAPStationBestEffortOctetCount, dot11NonAPStationAuthMaxBackgroundOctets, and dot11NonAPStationHCCAOctetCount is compared to dot11NonAPStationAuthMaxTotalOctets. When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

When a non-AP STA has exceeded the resource limits listed above, the AP shall terminate the connection. The AP SME invokes the disassociation procedure defined in 11.3.2.7 by issuing an MLME-DISASSOCIATE.request with the reason code “Authorized Access Limit Reached”.

11.18.5 Interworking Procedures: Emergency Alert System (EAS) Support

The Emergency Alert System (EAS) provides alerts, typically issued by authorities, to be transmitted, upon request, to non-AP STAs using the GAS protocol, from an AP within the IEEE 802.11 infrastructure. The mechanism provided by the interworking service uses a single bit EASN-supporting field and an optional octet EASN field in the Interworking element Figure 7-95ao, transmitted from an AP, to indicate whether or not any alert is available within the IEEE 802.11 infrastructure, and the change of the EAS messages info. The EASN-supporting field is set to 0 by a non-AP STA.

If an external network emergency alert is received by the IEEE 802.11 infrastructure, or existing emergency alerts are updated, each AP then advertises the option for the EAS mechanism, setting the Interworking element EASN field’s subfield EASN-Available to 1, and modifies the EICI subfield. Once the non-AP STA receives this advertisement, it utilizes GAS to retrieve the alert message from the AP, typically formatted using the Common Alert Protocol (CAP).

By utilizing GAS, the emergency functionality of the interworking service is extended to provide EAS support for client devices.

The CAP allows the collection and distribution of “all-hazard” safety notifications and emergency warnings across information networks and public alert systems used by first responders.

Authorization and integrity protection of the message is the responsibility of the authority and is out of scope of this standard.

11.18.6 Support for QoS Mapping from External Networks

Maintaining proper end-to-end QoS is an important factor when providing Interworking Service. This is because the external networks may employ different network-layer (Layer 3) QoS practices. For example, the use of a particular differentiated services code point (DSCP) for a given service may be different between different networks. To ensure the proper QoS over-the-air in the IEEE 802.11 infrastructure, the mapping from DSCP to UP for the corresponding network needs to be identified and made known to the STAs. If an inconsistent mapping is used then:

—Admission control at the AP may incorrectly reject a service request, because the non-AP STA used the incorrect UP.

—Non-AP STAs may use the incorrect value for User Priority in TSPEC and TCLAS elements.

—The user may be given a different QoS over the IEEE 802.11 network.