IEEE P802.11
Wireless LANs

| TGp LB 125 Various Comments Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x 175 | justin.mcnew@kapsch.net |

Abstract

|  | LB125 Comment Resolution |
| --- | --- |

- COMMENT:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 86 | Dickey, Susan | 6.2.1.1.2 | 3 | 48 | T | It is a serious error to add a lower-level MAC state parameter like BSSID to the data plane service primitive. Consider that by the time a higher-level data plane entity makes a request based on a BSSID received in an MA-UNITDATA.indication, that BSSID may already be obsolete. What those requesting this functionality really want is the ability to send to any and all WAVE mode STAs from whom a beacon has been received with a WIE indicating that the STAs are interested in the communication. This is the responsibility of the SME above the MAC, to look at higher-level protocol headers and tell the MLME to join a particular WAVE BSS (i.e, transmit on a particular channel) as required. It is a non-standard form of routing, and out of scope for 802.11. | This is a bad idea, and the desired functionality is out of scope for 802.11. Remove all the proposed changes to 6.2.1.1.2, 6.2.1.2.2 and 6.2.1.3.2 from the 802.11p draft. You may want to check the MLME SAP to make sure that sufficient information to allow higher layer protocols to direct communication as desired is available; passing the WIE in the MLME to SME SAP as is already done ought to be sufficient. | Decline |
| 411 | Marshall, Bill | A | 24 | 21 | TR | nothing in the normative text attaches the 5.9GHz PHY to the MAC extensions for WAVE BSS Support. While they will likely be implemented together, the PICS should reflect the normative requirements of the standard. As such, a separate row in the IUT configuration should be added for WAVE support | Add another row at line 21, with next available CF<n>, WAVE support, 11.18, O.2, Yes/No. In the following PICS entries, change the ones that are concerned with MAC functionality (A.4.4 and A.4.15) to reference this new CF<n>, and leave the ones that are purely PHY-related (A.4.8) to CF6A. | Accept – add support for outside the context of a BSS |
| 414 | Adrian, Stephens | A.4.4.4 | 25 | 50 | TR | "WAVE MAC resumes operation within 2 TUs" Where does this requirement come from? It cannot be clause 7 or clause 10? | Add a reference to defining normative text in clause 11 or remove this requirement. | Accept – add text to clause 1 (simple paragraph that states when dot11currentchannelnumber is changed MAC operation shall resume in less than 2 TU). Add some explanation in clause 5 also about how the SME may change the PHY to a different channel based on application requests. Remove requirement from Clause 10. |
| 472 | Myles, Andrew | General | 100 | 18 | TR | In the last LB I commented: During the San Francisco meeting, a presentation was given that claims experiments show adjacent channel interference is a significant problem when 11p is used in a way similar to the way it is used by IEEE 1609 I requested: Please either explain how 11p can be used in its current form or make appropriate modification to either 11p and/or IEEE 1609 so that 11p can be used The request was declined with The judgment of this task group is that the most effective solutions (e.g: channel management) to this potential problem are out of the scope of this 11p amendment. My response in this LB is: It is not out of scope of this WG to understand whether or not a proposed amendment will be effective in its intended context. The only conclusion I can draw from the somewhat evasive answer is that the questions and issues raised in San Franciso are still open. | Please either explain how 11p can be used in its current form or make appropriate modification to either 11p and/or IEEE 1609 so that 11p can be used. | Decline – multiple simultaneous channel usage is no longer a requirement. |
| 473 | Myles, Andrew | General | 100 | 19 | TR | In the last LB I commented: …it is now a set of mechanisms without any obvious context. I suggested: Rewrite the document as a standalone standard that references 802.11 but does not amend it. This should be a relatively simple process given the way the document is now written The TG responded: The new draft addresses most of the comment concerns, but the PAR specifically identifies this as an amendment rather than a stand-alone document. See clause 2 of document 2995r0 for more details. I now respond: The lack of text in the amendment explaining the context of these seemingly random features that have no relevance to the majority of 802.11 users is of great concern. In particular, it detracts further from the base standard and has the potential to confuse. | At this point there are a few choices to remedy the situation: * Withdraw the PAR, which will make the problem go away * Change the PAR so that 11p is a standalone standard, with context added * Add context to the current draft, probably in clause 5 or cluse 11 I would be happy with any of these choices but would prefer the second option | Decline. 802.11p specifically defines operation for exchanging data frames without the overhead of BSS-related features. This is critical to the environment in which the standard will be used (low latency requirements). |

- Background

This submission proposes the resolutions to comments 86, 411, 414, 472 and 473.

- Recommended Resolution of the Comments:

Resolve the comments as noted in the above table and change the text in D4.0/D4.02 as described below.

A.4 PICS proforma--IEEE Std 802.11™—2007 Edition

A.4.3 IUT Configuration

Change the insertion at page 24, lines 18-21 (addressing comment 411) of P801.11p D4.0 as follows (i.e. add the row beginning with “*CF<n>” to the table):

NOTE TO EDITOR: In *CF<n>, use the next available number for <n>.

Insert the following into A.4.3:

| Item | Features | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF6A | OFDM PHY for the 5.9 GHz band | -- | O.2 | Yes [] No[] |
| *CF<n> | WAVE support | 11.a | O.2 | Yes [] No[] |

A.4.4 MAC protocol

For all tables for the insertions in subclause A.4.4 (including those for subclauses A.4.4.1, A.4.4.2 and A.4.4.4), from page 24 line 35 to page 25, line 52 (addressing comment 411) of P801.11p D4.0, in the “Status” column of each table, change each instance of:

CF6A:M

to:

CF<n>:M

Note that CF<n> is defined in the instructions for A.4.3 above.

A.4.4.4 MAC addressing function

For the table for the insertion in at page 25, lines 46-52 (addressing comment 414) of P801.11p D4.02, change the entry at row 3, column 3 as follows (NOTE TO EDITOR: “11.a” is the appropriate subclause number):

Clause 7

10.3.9.1

Clause 11.a

11.a STAs communicating outside the context of a BSS

Insert the following as the second paragraph of subclause 11.a (addressing comment 414):

The SME may change the current channel of the PHY based on requests from applications using the medium.  When the dot11CurrentChannelNumber parameter is changed by the SME, MAC operation shall resume in less than 2 TU.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the recommended changes to P802.11p D4.0 noted above and instruct the editor to make these changes to P802.11p D4.0.

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: