IEEE P802.11
Wireless LANs

| TGp - Changes to P802.11D4.02 Nov 5 Draft |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Alastair Malarky | Mark IV Industries | 6030 Ambler Drive, Mississaugua, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |

Abstract

This submission provides suggested corrections to the November 5 2008 draft of P802.11p.

For the most part the changes are clarification.  One technical change proposed is the addition of a Vendor Specific IE into the new Timing and higher layer management frame.

|  | LB125 Comment Resolution |
| --- | --- |

- Background and Rationale:

A review of the November 5th draft of P802.11p-D4.02 highlighted that the result of other comment resolutions has introduced some minor inconsistencies and omitted one information element.  This submission poposes changes to rectify these.

The rationales for the minor changes is shown in the table below.

| Para | Location | Action | Rationale |
| --- | --- | --- | --- |
| 1.2 | Page 2 line 9 | Delete the insertion of “and outside the context of”. | This is duplication. This capability is addressed by the new indented sentence added immediately following. |
| 7.1.3.3.3 | Page 5 line 30 | Change “basic service set identifier” to “basic service set identification” or “BSSID” | Incorrect parameter name |
| 7.1.3.3.3 | Page 5 line 31 | Change “the address fields as shown in Table 7-7 (instead of any BSSID value in the MIB).” To “the address fields containing BSSID values as shown in Table 7-7, instead of using any BSSID value in the MIB.” | Clarifies only BSSID address field changed. i.e. not RA and TA. |
| 7.2.3.a | Page 6 line 28 | Change “If the dot11MultiDomainCapabilityEnabled attribute is true, a STA shall include a Country information element in the transmission of Timing and higher layer information frames.” to “Under the conditions identified in Table 7-18b, a STA shall include a Country information element in the transmission of Timing and higher layer information frames.” | Only identifies one attribute to control whether Country IE included, but table indicates two possibilities. Also avoids duplication. |
| 7.3.2.2 | Page 7 line 33 | Change “The Supported Rate information in Beacon and Probe Response management frames is delivered to the management entity in a STA via the BSSBasicRateSet parameter in the MLME-SCAN.confirm and MLME-TIMING_HL_INFO.indication primitives.” to “The Supported Rate information in Beacon and Probe Response management frames is delivered to the management entity in a STA via the BSSBasicRateSet parameter in the MLME-SCAN.confirm primitive. The Supported Rate information in the Timing and Higher layer management frame is delivered to the management entity in a STA via the BasicRateSet parameter in the MLME-TIMING_HL_INFO.indication primitive.” | This statement is incorrect. The added primitive doesn’t apply to beacon or probe response frames. Also should use BasicRateSet in outside-BSS mode. |
| 10.3.d.1.2 | Page 14 line 38 | Should be BasicRateSet | Should use BasicRateSet in outside-BSS mode. |
| 17.3.10.2 | Page 19, line 26 | Change “requiring low bit error rate (BER)” to “requiring improved immunity to nearby interfering STAs” | The filtering does not improve the BER performance, it only allows you to maintain the BER performance in the event of interference. |
| 17.3.10.3 | Page 20, line 5 | Change “requiring low bit error rate (BER)” to “requiring improved immunity to nearby interfering STAs” | The filtering does not improve the BER performance, it only allows you to maintain the BER performance in the event of interference. |

The one main technical proposal herein is the addition of the Vendor Specific information element into the new Timing and higher layer management frame.  The rationale for this as follows.

The Vendor Specific information element is included in IEEE 802.11 for:

The Vendor Specific information element is used to carry information not defined in this standard within a single defined format, so that reserved information element IDs are not usurped for nonstandard purposes and so that interoperability is more easily achieved in the presence of nonstandard information.

Prior to LB125 resolution, operation for the WAVE mode supported this element in the Extended Beacon management frame.  However this frame is no longer used for “operation outside a BSS”.  The only management frame that is supported for this operation is the new Timing and higher layer management frame.  However the current draft does not include such an element.  It is poposed this is added to Table 7-18b, and also to 10.3.d.1.2 and 10.3.d.3.2.

- Proposed Modifications to P802.11p

The revised text for the P802.11p amendment is shown below.  For the convenience of reviewers, changes to the draft are shown in blue.  Instructions in green are identification to the P802.11 editor of what changes are required from the baseline of P802.11pD4-02 + Nov 05.

Editor to modify change to 1.2first indented statement as shown below

1.2 Purpose

Change the first indented statement as follows:

— Describes the functions and services required by an IEEE 802.11™-compliant device to operate within and outside the context of ad hoc and infrastructure networks as well as including the aspects of STA mobility (transition) within those networks.

Editor to change text for modification of second paragraph of 7.1.3.3 as shown below

Insert after the second paragraph of 7.1.3.3.3:

When STAs are communicating outside the context of a BSS, the value of the BSSID field may take on any 48-bit value, and, while not identifying a BSS, may be set by a higher layer or the SME. When the optional basic service set identifier BSSID parameter in the MA-UNITDATA.request is present, the value of that parameter shall be used to set the values of the address fields containing BSSID values as shown in Table 7-7, (instead of using any BSSID value in the MIB).

Editor to change text for inclusion of first paragraph of 7.2.3.a as shown below

7.2.3.a Timing and higher layer information frame format

The frame body of a management frame of subtype Timing and higher layer information contains the information shown in Table 7-18b. If the dot11MultiDomainCapabilityEnabled attribute is true Under the conditions identified in Table 7-18b, a STA shall include a Country information element in the transmission of Timing and higher layer information frames.

Editor to add the following row to Table 7-18b submission

| Last | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |
| --- | --- | --- |

Editor to replace text for change first sentence in third paragraph of 7.3.2.2 as shown below

Change the first sentence in the first paragraph as follows:Add a new sentence after the first sentence in the third paragraph

The Supported Rate information in Beacon and Probe Response management frames is delivered to the management entity in a STA via the BSSBasicRateSet parameter in the MLME-SCAN.confirm and the MLME-TIMING_HL_INFO.indication primitives.  The Supported Rate information in the Timing and higher layer management frame is delivered to the management entity in a STA via the BasicRateSet parameter in the MLME-TIMING_HL_INFO.indication primitive.

Editor to add additional parameter to the primitive list in 10.3.d.1.2 as shown

10.3.d.1.2 Semantics of the service primitive

This primitive provides the following parameters:

MLME-TIMING_HL_INFO.request(

Capability Information,

Extended Capabilities Information,

BasicRateSet,

OperationalRateSet,

Country,

EDCA parameter set,

HLIE contents,

VendorSpecificInfo

)

Editor to change entry for BSSBasic RateSet in table in 10.3.d.1.2 as shown

| BSSBasicRateSet | Set of Integers | 1 - 127 inclusive (for each integer in the set) | The set of data rates that must be supported by all STAs that desire to communicate with the transmitting STA (i.e., the STA transmitting the Timing and higher layer information frame). A receiving STA must be able to receive and transmit at each of the data rates listed in the set. |
| --- | --- | --- | --- |

Editor to add following row to end of table in 10.3.d.1.2

| Vendor-Specific Info | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |
| --- | --- | --- | --- |

Editor to add additional parameter to the primitive list in 10.3.d.3.2 as shown

10.3.d.3.2 Semantics of the service primitive

This primitive provides the following parameters:

MLME-TIMING_HL_INFO.indication(

Timestamp,

Local Time,

Capability Information,

Extended Capabilities Information,

BasicRateSet,

OperationalRateSet,

Country,

EDCA parameter set,

HLIE,

RCPI,

VendorSpecificInfo

)

Editor to add following row to end of table in 10.3.d.3.2

| Vendor-Specific Info | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |
| --- | --- | --- | --- |

Editor to add change 1st sentence in 17.3.10.2 as shown

An optional enhanced performance specification is provided for systems requiring low bit error rate (BER) improved immunity to nearby interfering STAs.

Editor to add change 1st sentence in 17.3.10.3 as shown

An optional enhanced performance specification is provided for systems requiring low bit error rate (BER) improved immunity to nearby interfering STAs.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the recommendations of this submission, and instruct the editor to incorporate the proposed revisions provided into the amendment.

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References:

The following documents have been considered in the assessment herein:

- IEEE Std 802.11-2007

IEEE P802.11p-D4.02+Nov 05