IEEE P802.11
Wireless LANs

| TGp LB 125 Changes to Subclause 11.a Clarifying “Outside the Context of a BSS” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x 175 | justin.mcnew@kapsch.net |

Abstract

This submission proposes to change subclause 11.a of 802.11p Draft 4.02-NOV05 to clarify “outside the context of BSS”.

- Background

Subclause 11.a of Draft 4.02-NOV05 requires clarification of the concept of “outside the context of a BSS”, including simultaneity of membership in a BSS and usage of EDCA parameters.

- Recommendation:

Subclause 11.a of Draft 4.02-NOV05 should be changed as follows:

NOTE TO EDITOR: “dot11[appropriate MIB attribute]” should be replaced by the name of the appropriate MIB attribute.

Replace the insertion on page 18, lines 26-34, which currently reads:

Insert the following new subclause, 11.a, after the last subclause in 11 inserting the appropriate subclause number:

11.a STAs communicating outside the context of a BSS

A STA may communicate outside of the context of a BSS. A STA communicating outside the context of a BSS may transmit and receive frames without scanning or MAC sublayer authentication or association procedures.  A STA may be a member of a BSS and also communicate outside the context of a BSS.

with:

Insert the following new subclause, 11.a, after the last subclause in 11 inserting the appropriate subclause number:

11.a STAs communicating outside the context of a BSS

A STA may communicate outside of the context of a BSS. A STA communicating outside the context of a BSS may transmit and receive frames without scanning or MAC sublayer authentication or association procedures. When dot11[TBD] is FALSE a STA shall not communicate outside the context of a BSS.  When a STA is a member of a BSS or is the point coordinator within a BSS, dot11[TBD] shall be set to FALSE. When dot11[TBD] is TRUE a STA shall not communicate within a BSS.  A STA that communicates outside the context of a BSS shall use only one EDCA parameter set for accessing the medium on the current channel. The EDCA parameter set used outside the context of a BSS is either the default EDCA parameter set specified in table 7-37a or it is set by the SME in dot11EDCATable.  If a STA does not have dot11[TBD] defined, the attribute should be regarded as FALSE.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the recommended changes to P802.11p D4.02-NOV05 noted above and instruct the editor to make these changes to P802.11p D4.0.

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: