IEEE P802.11
Wireless LANs

| Addition of 3GPP cellular network information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| George Bumiller | Research In Motion | 39 Lakeview Terrace, Ramsey, NJ 07446 USA | +1 201 327 1136 | g.bumiller@ieee.org |
| Stephen McCann | Roke Manor Research Ltd (Roke) | Roke Manor Research Ltd Old Salisbury Lane Romsey, Hampshire SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| George Babut | Rogers Communications Inc. | Rogers Communications Inc. Rogers Brampton Campus 8200 Dixie Road Brampton, ON L6T 0C1 CANADA | +1 647 747 4225 | George.Babut@rci.rogers.com |
| Colin Blanchard | BT Group, plc | BT Adastral Park MLB1 PP8 Martlesham Heath, UK | +44 1473 605 353 | colin.blanchard@bt.com |
| Brian Daly | AT&T | PO Box 97061 7277 164th Avenue Redmond, WA 98073-9761 | +1 425 580 6873 | brian.k.daly@att.com |
| Necati Canpolat | Intel Corp. | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |

Abstract

The inclusion of the IEEE 802.11u D_3.0 in 3GPP TS 24.234, and the interest of other 3GPP groups in making use of the Interworking with External Networks capability, is described in another contribution.

This proposal addresses the following comments from LB137: CID 4036, 4037, 4038, 4307, 4308, 4309, 4584, 4587, 4588, 4589,

This document is based on IEEE 802.11u-D4.01

- 2. Normative references

- Insert the following new reference into 2 maintaining the ordering:

3GPP TS 24.234 v8.1.0, “3GPP System to Wireless Local Area Network (WLAN) interworking; WLAN User Equipment (WLAN UE) to network protocols; Stage 3 (Release 8)”

- 7. Frame formats

- 7.3 Management frame body components

- 7.3.3 Native Query Protocol query elements

Modify Table 7-43x as shown below:

| - Table 7-43x - Native Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | Native Info Element (clause) |
| Reserved | 0-255 | n/a |
| Capability List Information | 256 | 7.3.3.1 |
| Venue Name Information | 257 | 7.3.3.2 |
| Emergency Call Number Information | 258 | 7.3.3.3 |
| Emergency Public Network Access Information | 259 | 7.3.3.4 |
| Network Authentication Type Information | 260 | 7.3.3.5 |
| Roaming Consortium List | 261 | 7.3.3.6 |
| AP Date, Time and Time Zone information | 262 | 7.3.3.8 |
| IP Address Type Availability | 263 | 7.3.3.9 |
| NAI Realm | 264 | 7.3.3.10 |
| 3GPP Cellular Network Broadcast information | 265 | 7.3.3.11 |
| Reserved | 266- 56796 | n/a |
| Vendor Specific Information | 56797 | 7.3.3.7 |
| Reserved | 56798 - 65535 | n/a |

Insert the following new subclauses, renumbering clauses as necessary:

- 7.3.3.11 3GPP Cellular Network Broadcast information

The 3GPP Cellular Network Broadcast information contains Cellular information such as Network advertisement information e.g. network codes and country codes to assist a 3GPP non-AP STA in selecting an AP to access 3GPP networks.  The format of this element is shown in Figure 7-95bo.

|  | Info ID | Length | Status Code | Payload |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable |

Figure 7-95bo—3GPP Cellular Network Broadcast list format

The Length field is 2-octet length of the Capabilities List element and is equal to 2 plus the size of the Payload field.

The Status Code is a 2-octet field whose value is drawn from Table 23.

The Payload field is a generic container whose contents are defined in Annex A of 3GPP TS 24.234 v8.1.0.